Installation 16-bit Ethernet Adapter in Microsoft Window WorkGroup v3.1

---------------------------------------------------------------------

I. Follow Window Workgroup installation instructions (Setup):

II. Select 'NE2000 Compatible' in network adapter configuration and

input IRQ, I/O address

III. Complete Installation

IV. When the Window WorkGroup installation is complete, you may use any

text editor to edit PROTOCOL.INI file (under C:/Windows).

Following is the PROTOCOL.INI file by selecting "NE2000 compatible"

and IRQ=3, I/O=300:

[network.setup]

version=0x3100

netcard=ms$ne2clone,1,MS$NE2CLONE

transport=ms$netbeui,MS$NETBEUI

lana0=ms$ne2clone,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NE2CLONE]

DriverName=MS2000$

IOBASE=0x300

INTERRUPT=3

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=MS$NE2CLONE

LANABASE=0

Following is sample file of Config.sys:

FILES=70

BUFFERS=40

DEVICE=HIMEM.SYS

device=emm386.exe 1024 frame=e000 ram

dos=high,umb

device=C:\WINDOWS\protman.dos /i:C:\WINDOWS

device=C:\WINDOWS\workgrp.sys

device=C:\WINDOWS\NE2000.DOS

stacks=9,256

V. Edit the PROTOCOL.INI file and modified as follows.  The only line

need to be modified is DriverName.  You may also change the IRQ and

I/O address according to the hardware configuration.

[network.setup]

version=0x3100

netcard=ms$ne2clone,1,MS$NE2CLONE

transport=ms$netbeui,MS$NETBEUI

lana0=ms$ne2clone,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NE2CLONE]

DriverName=NE2000$

IOBASE=0x300

INTERRUPT=3

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=MS$NE2CLONE

LANABASE=0

VI.  You need not change Config.sys file if you select 'NE2000

Compatible' in installation.  If you do not select 'NE2000

Compatible', make sure follow device driver is inserted into

Config.sys:

device=C:\WINDOWS\NE2000.DOS

VII. Copy NE2000 NDIS driver:

Locate the NE2000 NDIS driver in the driver diskette comes with

the adapter: A:\MSLANMAN.DOS\DRIVERS\ETHERNET\NE2000\NE2000.DOS,

and copy the files to C:\Windows subdirectory

Installation 8-bit Ethernet Adapter in Microsoft Window

WorkGroup v3.1 in a DOS PC

-------------------------------------------------------------------

I. Follow Window Workgroup installation instructions (Setup):

II. Select 'NE1000 Compatible' in network adapter configuration and

input IRQ, I/O address

III. Complete Installation

IV. When the Window WorkGroup installation is complete, you may use any

text editor to edit PROTOCOL.INI file (under C:/Windows).

Following is the PORTOCOL.INI file by selecting "NE1000 compatible"

and IRQ=3, I/O=300:

[network.setup]

version=0x3100

netcard=ms$ne1clone,1,MS$NE1CLONE

transport=ms$netbeui,MS$NETBEUI

lana0=ms$ne1clone,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=MS$NE1CLONE

LANABASE=0

[MS$NE1CLONE]

DirverName=MS1000$

IOBASE=0x300

INTERRUPT=3

Following is sample file of Config.sys:

FILES=70

BUFFERS=40

DEVICE=HIMEM.SYS

device=emm386.exe 1024 frame=e000 ram

dos=high,umb

device=C:\WINDOWS\protman.dos /i:C:\WINDOWS

device=C:\WINDOWS\workgrp.sys

device=C:\WINDOWS\NE1000.DOS

stacks=9,256

V. Edit the PROTOCOL.INI file and modified as follows.  The only line

need to be modified is DriverName.  You may also change the IRQ and

I/O address according to the hardware configuration.

[network.setup]

version=0x3100

netcard=ms$ne1clone,1,MS$NE1CLONE

transport=ms$netbeui,MS$NETBEUI

lana0=ms$ne1clone,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=MS$NE1CLONE

LANABASE=0

[MS$NE1CLONE]

DriverName=NE1000$

IOBASE=0x300

INTERRUPT=3

VI.  You need not change Config.sys file if you select 'NE1000

Compatible' in installation.  If you do not select 'NE1000

Compatible', make sure follow device driver is inserted into

Config.sys:

device=C:\WINDOWS\NE1000.DOS

VII. Copy NE1000 NDIS driver:

Locate the NE1000 NDIS driver in the driver diskette comes with the

adapter:  A:\MSLANMAN.DOS\DRIVERS\ETHERNET\NE2000\NE1000.DOS, and copy

the files to C:\Windows subdirectory

Installation 8-bit ARCnet Adapter in Microsoft Window WorkGroup v3.1

--------------------------------------------------------------------

I. Follow Window Workgroup installation instructions (Setup):

II. Select 'ARCNET Compatible' in network adapter configuration and

input IRQ, I/O address and Memory address

III. Complete Installation

IV. When the Window WorkGroup installation is complete you may use any

text editor to edit PROTOCOL.INI file (under C:/Windows).

Following is the sample PROTOCOL.INI file by selecting ARCNET

compatible and IRQ=2, I/O=2E0 and Memory=D000

[network.setup]

version=0x3100

netcard=ms$arcclone,1,MS$ARCCLONE

transport=ms$netbeui,MS$NETBEUI

lana0=ms$arcclone,1,ms$netbeui

[protman]

DriverName=PROTMAN$

PRIORITY=MS$NETBEUI

[MS$NETBEUI]

DriverName=netbeui$

SESSIONS=10

NCBS=32

BINDINGS=MS$ARCCLONE

LANABASE=0

[MS$ARCCLONE]

DriverName=SMC_ARC$

interrupt=2

iobase=0x2E0

memorybase=0xD000

Following is sample file of Config.sys:

FILES=70

BUFFERS=40

DEVICE=HIMEM.SYS

deivce=emm386.exe 1024 frame=e000 ram

dos=high,umb

device=C:\WINDOWS\protman.dos /i:C:\WINDOWS

device=C:\WINDOWS\workgrp.sys

device=C:\WINDOWS\SMC_ARC.DOS

stacks=9,256

V. The 8-bit ARCnet Adapters are fully compatible with Microsoft

Window Workgroup ARCnet driver; therefore, you need not do any

modification.  You may start the WorkGroup immediately.