Installation Guide for Windows For Workgroups 3.11

========================================================

This file contains the information about how to install LE8029R.386 Driver

(NDIS 3.0) into Windows For Workgroup 3.11. Install NDIS 3.0 Driver into

Windows For Workgroups as following procedures :

The Windows For Workgroups 3.11 is assumed that has been installed

in your computer without any network driver installed.

a. Turn your computer off.

b. Install the LE-8029R Ethernet Adapter to your computer.

c. After the adapter installation, turn your computer on.

d. Make sure your computer is under DOS.

e. Start your Windows for Workgroups 3.11.

f. Select "Network" icon.

g. Select "Network Setup" icon.

h. Select "Drivers" bar.

i. Select "Add Adapter" bar.

j. Select "Unlisted or Updated Network Adapter" item bar on the Adapter

list.

k. System prompts you to insert driver diskette and enter the path of

the driver directory.

l. Insert the driver diskette into drive A: or B: and enter A:\WFW\3.11

or B:\WFW\3.11

m.          "LE8029R Card (REALPCI)" appears in the list and select it.

n. Select "OK" bar.

o.          "LE8029R Card (REALPCI) " bar appears. It

indicates the NDIS driver has been copied into your Windows system.

p. Select "Close" bar.

q. Select "OK" bar.

r. Windows prompts you to insert several WFW system diskettes.

s. When message "Windows Setup has modified your SYSTEM.INI and

PROTOCOL.INI...    ....." appears, select "OK" bar.

t. Select "Restart Computer" bar.

u. Now, the Windows for Workgroup 3.11 is ready for your Ethernet

Adapter.