WESTERN DIGITAL

UNIX STREAMS DRIVER FOR USE WITH

"PLUS"-FAMILY LAN ADAPTER BOARDS

RELEASE INFORMATION

Version 1.1          26 August 1988

This driver was specifically designed for use with the

INTERACTIVE Systems' 386/ix (*) version of UNIX (*) System V.3

for 386 AT (*) class machines.  This is the only system that it

has been tested on to date.

It should operate on the AT&T 386 release, the Microport (*)

release, and others, using different installation procedures.

Versions of this driver have already been incorporated in some

UNIX systems; consult your UNIX vendor.

This driver supports Western Digital "PLUS" (WD8003) family

Ethernet and StarLAN LAN adapter boards:

Ethernet:     EtherCard PLUS (WD8003E, WD8003EBT, WD8003EB)

EtherCard PLUS/A (WD8003ET/A)

Ethernet Twisted Pair:

EtherCard PLUS/TP (WD8003WT)

StarLAN:      StarCard PLUS (WD8003S)

StarLink PLUS (WD8003SH)

StarCard PLUS/A (WD8003ST/A)

OPERATIONAL CONSIDERATIONS

This version of the driver is designed to work with INTERACTIVE

Systems' 386/ix Version 1.0.6 or later.  If attempting to use

this driver with Version 1.0.5, an error in the kernel link may

occur with the symbol "machenv" undefined.  To use this driver

with Version 1.0.5 (AT bus machines only) add the lines:

#ifdef AT386

struct machenv machenv = {0};

#endif

to the file /etc/atconf/modules/kernel/space.c.  The wd driver

uses the machenv structure to determine if it is running on an

AT or Micro Channel (*) bus machine.  On other versions of UNIX,

a structure

struct machenv {

unsigned char machine;

unsigned char adapter;

unsigned char drive;

unsigned char special;

} machenv;

may be defined, with machenv.machine being set to 2 for Micro

Channel bus machines, and some other value (0 or 1 suggested)

for AT bus machines.

MODIFICATION HISTORY

<< Version 1.1 -- Current version:  August 1988 >>

o Micro Channel bus adapters are now supported.

o This version now supports use of more than the first 8K of RAM

on the WD8003EBT board by changing the WDBOARDSIZE definition

in /etc/atconf/modules/wd/space.c.

o The driver will no longer allow opens if it does not detect an

adapter present.  This previously could have caused a system

crash if an attempt to communicate with the driver occurred

when no adapter was installed in the system.

o Several external symbols which did not start with the "wd"

prefix have been made static to reduce the chance for conflict

with other drivers when linking the kernel.

o A problem with sending some Ethernet-style packets to a process

expecting only 802.2 LLC packets has been fixed.

o A problem was fixed which could have caused a system crash if a

corrupted packet was read (usually due to having another board

in the system which is responding to the same Shared Memory

Address range).

o The default major device number was changed from 35 to 30 to be

compatible with INTERACTIVE Systems' 386/ix Version 1.0.6

device numbers.

<< Version 1.0 -- February 1988 >>

Original version.

(*) UNIX is a registered trademark of AT&T.  INTERACTIVE and

386/ix are trademarks of INTERACTIVE Systems Corporation.

Microport is a trademark of Microport Corporation.  AT is a

registered trademark and Micro Channel is a trademark of

International Business Machines Corporation.

#######################################################