NWPATCH

NetWare Driver Patch Utility

User Document

NWPATCH.EXE is a patch utility for Novell NetWare (*) drivers

that operate with Western Digital (WD) "PLUS" (WD8003) family

LAN adapter boards.  This utility will patch the following files:

NetWare v2.0a:	(WD driver versions 2.05 or later)

SHELLDRV.OBJ - WORKSTATION

ACOMDRV.OBJ - SERVER    BCOMDRV.OBJ - SERVER

CCOMDRV.OBJ - SERVER    DCOMDRV.OBJ - SERVER

NetWare v2.1:	(WD driver versions 2.10.0 or later)

WD8003SH.OBJ - WORKSTATION

WD8003AD.OBJ - SERVER   WD8003BD.OBJ - SERVER

WD8003CD.OBJ - SERVER   WD8003DD.OBJ - SERVER

NetWare ELS:	ANET3.COM - WORKSTATION

NET$OS.EXE - SERVER

NOTE

NetWare drivers for Micro Channel adapters configure

themselves automatically and never need to be patched.

A COPY of the file to be patched should be put in your default

drive/directory before the patch program is executed.  It is not

safe to patch a driver unless there is a backup copy of the

original driver saved on a different disk or directory.

The patch program can then be started by typing NWPATCH.  The

user should know:

o which Novell version of NetWare the driver is from;

o whether the driver is for a workstation or server;

o for a server driver, whether it is to be the primary,

secondary, etc.;

o for NetWare v2.0a drivers, what the WD driver version

number or the creation date of the original driver is;

o for NetWare ELS, what the WD driver version number

is (using the ANET3.COM program, typing "ANET3 i" will

cause the version number to be displayed).

Follow the instructions given by the patch utility to change

the:

PC INTERRUPT (IRQ)

BASE I/O ADDRESS

SHARED RAM MEMORY BASE ADDRESS

The patch utility DOES NOT do error checking, so please ensure

that the new values entered are VALID!  When the parameters

displayed match the values desired, enter 'E' or 'e' to save

them in the appropriate driver file.

To quit the program without making any changes in the driver

file, enter 'Q' or 'q'.

(*) NetWare is a registered trademark of Novell, Inc.

####################################################################