WESTERN DIGITAL

DIAGNOSTIC PROGRAM FOR

"PLUS"-FAMILY LAN ADAPTER BOARDS

OPERATING INSTRUCTIONS

This document contains the procedures for testing the following

"PLUS" - Family LAN adapter boards:

XT/AT Bus Adapters:

StarLAN     StarCard PLUS                (WD8003S)

StarLink PLUS                (WD8003SH)

Ethernet    EtherCard PLUS               (WD8003E)

EtherCard PLUS With 32K RAM  (WD8003EBT)

And Boot ROM Socket

EtherCard PLUS - software    (WD8003EB)

configured

EtherCard PLUS TP            (WD8003WT)

Twisted Pair

Micro Channel Bus Adapters:

StarLAN     StarCard PLUS/A              (WD8003ST/A)

Ethernet    EtherCard PLUS/A             (WD8003E/A)

<< DISK FILES >>

DIAGNOSE.EXE        Diagnostic Program

$RUN.OVL      --

SYS$MSG.DAT     |

SYS$HELP.DAT    |-- Support FIles

SYS$ERR.DAT     |

DIAGHLP.HLP   --

USER.DOC            User Documentation (this file)

All Diagnostic Program files must be in the same directory for

proper operation.

<< OVERVIEW >>

The Diagnostic Program is used to verify the correct

configuration and operation of a Western Digital LAN Adapter

board before installing the network software.

<< BEFORE STARTING THE PROGRAM >>

1. Be sure you know the configuration of any adapters that use

jumpers to set the configuration.

2. After turning off the PC's power, insert the adapter in the PC.

** Ethernet boards must be on a properly terminated cable

(use a BNC "T" connector with two 50 Ohm terminators or

an AUI loopback connector).

3. Boot the PC to a DOS prompt.

** Memory resident (TSR) programs and device drivers should

not be loaded while the Diagnostic Program is active, so

you may need to temporarily rename your CONFIG.SYS and

AUTOEXEC.BAT files.  The Diagnostic Program is always run

without loading any network software.

<< STARTING THE PROGRAM >>

1. To run the Diagnostic Program change to directory with the

Diagnostic Program (\DIAGNOSE on the floppy disk,

\WDPLUS\DIAGNOSE on a hard disk) type DIAGNOSE and press

Enter.

2. Read the opening screen and press Enter.

** Press F1 any time in this program to get help

3. Press Enter to select a board to test.  More than one board

may be displayed depending on the number installed in the

system.

4. If an error message is displayed then a board is not present.

** The board type and the Base I/O should appear. If two

boards are in the machine both boards should be listed.

Every network board in a PC must have a unique IRQ and

Base I/O.

** If only one board is displayed, and more than one are in

the PC, then the Base I/O on the boards is the same.  Turn

off the PC and check the Base I/O on all boards.  Change

the Base I/O's so that each board has a unique value.

Restart the PC and rerun the Diagnostic Program.

5. Select the adapter to test and press Enter.  The board

configuration is displayed.

Board Type - Displays the board type

Base I/O - found by searching all base I/O's

IRQ - defaults to three (3) regardless of what IRQ the

board is actually set to (see note 1)

RAM size - defaults to most appropriate setting (see note 1)

RAM address - always defaults to D0000

ROM size - defaults to most appropriate setting (see note 1)

ROM status - defaults to most appropriate setting (see

note 1)

NOTE 1:  The EtherCard PLUS EB and Micro Channel boards can

correctly read the configuration information off the card.

** Read the file SYSMAP.DOC on SuperDisk Drivers disk

for a thorough understanding of the various settings

available on Western Digital network adapters.

6. Press any key to continue.

<< SET CONFIGURATION >>

1. Go to SET CONFIGURATION by pressing S or using the Up/Down

arrows if the IRQ, RAM address, RAM Size, or ROM Size and Address

need to be changed from the displayed value.

2. The board's network address is displayed.

3. Use the Up/Down arrows to move to the setting that needs to be

changed.

4. Press Enter and a menu of options is displayed.  Again use

the Up/Down arrows to position the highlight bar on the proper

option and press Enter to select a new choice.

** The Base I/O and IRQ must match the board settings

** The RAM address selected here is only for the purpose of

this program and has no effect on any other network

software used afterward.

5. If an invalid selection is made the program will not stop the

user from making that selection.

6. Press F10 to save the changes or ESC to exit without change.

<< BASIC ADAPTER TEST >>

1. The BASIC ADAPTER TEST must be run to verify correct

operation.

2. Press B or move the highlight bar using the Up/Down arrows

over the BASIC ADAPTER TEST option in the menu and press Enter.

3. This selection tests the basic board functions.  The test

takes less than a minute and displays "ALL BASIC ADAPTER TESTS

COMPLETED SUCCESSFULLY" and "PRESS ANY KEY TO CONTINUE" upon

successful completion.

4. The test has failed if any of these conditions occur:

A) An error message displays at the bottom of the screen in a

RED box; or

B) The test hangs up at a certain blinking line and never

completes.  The blinking line is the test that failed.

** Read the error message or blinking line and write it

down!!  Then reboot the PC.

** Use the error message or blinking line to find the

problem.

Common problems include not terminating an Ethernet board and

hardware or software conflicts with IRQ, Base I/O, or RAM

address.

** A board may be considered defective if it does not

pass this test and an identical board with the same

jumper settings does pass.

5. If the board fails recheck the board settings, try a

different BNC "T" connector and terminator, or remove other

boards from the PC.

<< INITIATE TEST MESSAGE and RESPOND TO TEST MESSAGES >>

1. The best test environment is two PCs directly cabled,

but this test can be run with any PCs on the network.

2. These two tests must be run together on two different

machines, both of which must have passed the basic adapter test.

3. In this example we will refer to COMPUTER A and COMPUTER B.

Start with COMPUTER A

Press R or move the highlight bar using the Up/Down arrows

to Respond To Test Messages, and press Enter.

Press Enter again and a box opens in the middle of the

screen with the message:

Responding to test messages

Press any key to stop

COMPUTER A is now ready to respond to test messages and is

called the Responder.

Go to COMPUTER B

Press I or move the highlight bar using the Up/Down arrows to

Initiate Test Messages, and press Enter.  There are two

methods to perform the test.

1. Press Enter to begin the test.  The test will perform

1000 iterations after finding the first available

responder.

2. If you have more than one responder on the network

press F2 and all active responders are displayed with

their network addresses.  Select the desired responder

by using the Up/Down arrows and pressing Enter.

Press Enter and the test will perform 1000 iterations.

** If an error message, No Responder Found, is displayed

then the two computers cannot communicate across the

network.  The normal reason for this test to fail is

cabling, "T" connectors, or terminators that are bad.

When the test is complete press F4 on both computers to find the

test results.

** A small number of collisions is normal on a live

network or one with multiple test initiators, however

Alignment or CRC errors might mean that the board that

initiated the messages (COMPUTER B) is bad.

Press F6 to clear the results for the next test, then

press ESC.

At this point the test of the adapters is complete.

There are further options available on the initiating computer

(COMPUTER B).  The option menu is selected by pressing F6.

F6 will allow you to set parameters such as the frame

length.  The Up/Down arrows are used to select the

parameters and the spacebar to change parameter values.

Set the Iterations to non-stop to run a continuous test.

To stop the continuous test press any key.

This concludes this additional options test.  There are

other choices available under the additional options test.

<< ADVANCED USE >>

The Diagnostic Program is also designed to operate from the

command line.  This method of operation is especially

appropriate if your adapter uses a non-factory configuration.

Using the command line setup, you bypass having to constantly use

the SET CONFIGURATION portion of the program.

The following lists the command line parameters available:

/base:aaa       Specifies base I/O address.

/irq:n          Specifies interrupt level (IRQ).

/ram:aaaaa      Specifies base address of the on-board RAM.

/rom:aaaaa      Specifies base address of the boot ROM.

/adapter:n      Specifies adapter to test, see note 1.

/slot:n         Specifies Micro Channel slot adapter to test.

/send:xxxxxx    Initiates test messages to network address

'xxxxxx'.  Each 'x' is a two digit hex value

(e.g. C0).

/send           Initiates test messages to all responders.

/respond        Echoes received test messages.

/test           Runs Basic Adapter Test.

/v              Displays program version and copyright info.

/?              Displays this help screen.

Note 1: Adapter is selected using Base I/O for systems with more

than one network adapter.  If /ADAPTER:2 was used then the board

with the second Base I/O (in ascending order) is tested.

These command line parameters can be used in combination in

batch files.  For example:

diagnose /irq:7 /respond  Initiates program using IRQ 7 and starts

the computer in a responder mode.

diagnose /base:320 /test  Initiates program using a Base I/O of

320h and runs the Basic Test Program.

<< SUPPORT >>

If you have any additional questions regarding your Diagnostic

Program contact your dealer or distributor technical support

department.

<< TRADEMARKS >>

Micro Channel is a registered trademark of International

Business Machines Corporation.