This is a reference for the ENET diagnostic program Version

2.10 (ENETDiag Ver. 2.10). With the proper setup, this program can

test the following network peripherals:

a) ReadyLINK Multi-media Concentrator

b) ReadyLINK 10 BASE-T Concentrator TP2000

c) COMPEX ENET Repeater

d) ReadyLINK ENET16/U

e) ReadyLINK ENET16-COMBO

f) ReadyLINK ENET16-TP

g) ReadyLINK ENET/U

This appendix will be divided into three sections:

a) "Starting up":

It tells you how to start the program.

b) "Menus and Options":

It provides guild-lines to each menu item and dialogue

box. It is arranged in the order that the menus appear on

the screen.

c) "Setup for testing":

It tells you how to setup the system to test

different network peripherals.

A) Starting up

You can run the ENETDiag Ver. 2.10 in the DOS environment

only.  By typing

ENETDiag [option]

at the DOS prompt, the diagnostic program will be invoked.

The command-line options are case-insensitive and are -t ,

-f(filename) or /t, /f.

By default, the AUI (attachment unit interface) cable and

coaxial cable modes are selected. The option, -t, sets the

program for using twisted-pair cable in the network.

NOTE: The type of medium you have selected is displayed on

the top right-hand corner of the main menu.

The option, -f(filename), causes the diagnostic program to

load the configuration file as the initial settings for the

cards.

NOTE: The argument, (filename), should include the full

pathname, where applicable.

B) Menus and Options

The arrangement of the menus and options are shown below.

Main

�

�������������������������������������������Ŀ

�        �                   �              �

Setup     Test                View           Exit

�        �                   �              �

�Edit    �Initialization     �Test Log      �Resume

�        �                   �              �

�Load    �RAM                �Jumper        �Exit to DOS

�        �                   �Setting

�Save    �IRQ                �

�                   �Card ID

�Loopback

�

�Tx and Rx

Setup Menu

*****************************************************************

This menu appears on the top left of the menu bar. It lets

you set up the test environment. You can load a file storing the

settings used before and can save the current setup in the file

for further use.

The menu provides you with three commands: Edit, Load and

Save.

������Ŀ

� Edit �

��������

This command allows you to configure each card to be tested.

The maximum number of cards supported by the program is four (4).

In this menu, you can use Left and Right arrow keys ( <- and

-> ) to choose different card and use Up and Down arrow keys ( #

and # ) to choose the option of a card. If you want to change the

setting of an option, you can press "ENTER" key when the

selection bar is over the option. Then a small selection menu

will appear on the top left hand corner. Use the arrow keys to

choose the setting you like and press "ENTER" to confirm the

change. The setting is updated on the "Edit" screen immediately

after leaving the selection menu. Sometimes, the settings of

other options are changed if the setting of a related option is

altered. For example, the option "RAM Address" is changed to "NA"

(not applicable") if you change the mode to "NE2000XT".

Press "ESC" key to quit from the selection menu. Pressing it

again in the "Edit" screen, lets you quit from the "Edit" menu.

This menu provides the following options:

a) Status:

The status of any card you want to test has to be

set to "SELECT" before the program will recognize the

card and test it. Any card with status, "UNSELECT",

will be regarded as un-installed even it is installed

in your system.

b) Type:

This program supports two types of ethernet

network adapters: 16-bit adapters including:

i)   ReadyLINK ENET16/U;

ii)  ReadyLINK ENET16-COMBO &

iii)  ReadyLINK ENET16-TP)

and one 8-bit adapter including:

i) ReadyLINK ENET/U.

c) Mode:

Our network adapters can emulate different working

modes in order to support the device drivers provided

by other companies such as Western Digital.

For an 8-bit network adapter, two modes are

supported:

i) WD8 (8-bit) mode:

This is the Western Digital mode.

ii) NE1000 mode:

This mode is software-compatible with the

Novell NE1000 network adapter.

For a 16-bit adapter, it emulates up to five

modes:

i) CPX16 (16-bit) mode:

This is the Compex mode.

ii) WD-Plus:

An adapter configured in this mode is

software-compatible with the Western Digital

Ethercard Plus16 adapter.

iii) NE1000:

If the card is plugged in a 8-bit or 16-bit

slot and is programmed to NE1000 mode, it

works exactly as ENET/U in NE1000 mode.

NOTE: This mode is supported only if the network adapter

uses the COMPEX ASIC chip EN90293-118.

iv) NE2000XT:

An adapter configured in this mode is

software-compatible with the Novell NE2000

card. This mode is used if you want to use

the NE2000 mode and the card is plugged in a

8-bit slot.

v) NE2000AT:

This mode is the same as NE2000XT except that

the card is installed in a 16-bit slot.

d) RAM size:

This option allows you to specify the size of RAM

on the adapter. On an 8-bit adapter, 8K or 32K RAM can

be chosen. Size of RAM on the 16-bit adapter is either

16K or 64K.

e) RAM address:

With the exception of the Novell mode, the memory

on the adapter is mapped to the system memory map by

the diagnostic program. This option specifies the base

address on the system memory map.  Except WD-Plus mode,

which occupies 16K system memory spaces, all modes only

use 8K memory spaces.

NOTE: Please remove the BOOT ROM from the network

adapter when testing your card(s) because it will

occupy the system memory spaces.

NOTE: Before setting this option, MAKE SURE that the

base address does not conflict with addresses occupied

by other adapters. If two adapters use same memory

spaces, the program MAY HANG.

f) I/O address:

A set of registers on the adapter are configured

by the diagnostic program. You use this option to

specify the base I/O address of each card.

NOTE: Before setting this option, CHECK the base I/O

addresses of other adapters carefully.

g) IRQ number:

Each adapter occupies one system interrupt line.

NOTE: CHECK this option carefully before starting the

test, otherwise the system MAY HANG.

When the program is invoked, all cards have their default

options but their status are set to "UNSELECT".  All settings of

options you changed in the program will not be saved

automatically when you exit. If you want to use the same set of

settings again, you can use option, "Save", to store them in a

file.

������Ŀ

� Load �

��������

The configuration file contains the information about the

settings of each card. This file is not a text file and can not

be edited by the user. It is protected by the stored checksum.

The file is corrupted if the calculated checksum is different

from the stored one.

The command lets you load a configuration file from the

diskette. After loading it, the settings on the "Edit" menu will

be overwritten.  The length of filename (including the pathname)

is restricted to 30 letters only.

������Ŀ

� Save �

��������

This menu lets you save the current settings on the "Edit"

menu. The diagnostic program calculates a checksum to ensure

integrity of the configuration file. Similarly, the length of the

filename (including the pathname) is also limited to 30 letters.

Test Menu

*****************************************************************

After preparing all settings of the cards, you can start the

tests in this menu. Five tests are available for you to test the

functionality of different parts of the network adapter. However,

you should NOTE that sometimes the test results of a test may

depend on results of other test. For this reason, you SHOULD

follow the test sequence on the menu, otherwise, the test results

may be INVALID.

The layout of the test menus are similar and are designed

for four cards. Only the test results of the cards under the test

will be shown on the screen. An message, "UNSELECT" is displayed

if the card is not under the test.

In each menu, there are several options for you to choose.

They are listed as follows:

a) Iteration:

You can specify the number of iterations the test

performs. If the iteration is greater than 30000, then

the program will run infinite times. The duration of

the test depends the numbers of the cards under the

test and the iterations. By default, the iteration

counter is set to 100.

b) Test Mode

The test mode specify how to stop the test when

the test finds any error. In this option, "Test Mode",

you can choose one of the following:

i) Stop if errors

In this mode, the test will be stopped when a

error is found, even if the iteration count

is not completed. The program shows the count

of successful iterations during the test and

shows the test result, either "Passed" or

"Failed" when the test is stopped. If an

error is found, the cause of the error will

be shown on the screen for debugging purpose.

ii) Continue:

If you want to obtain the failure rate

(failed iterations / total iterations) of a

test, you can choose this mode. Under this

mode, the test will be stopped only at the

end of the test according to the iteration

count specified. During the test, the program

updates the counts of the successive and

failed iterations according to the test

results. You should note that the program

does not provide the causes of the errors in

this mode.

You can stop the test at any time manually by

pressing the any key.

NOTE: DON'T press the keys too frequently

after starting the test.

c) Start

This command starts the test at any time. Once you

start the test, the test results will be shown on the

screen. If you want to re-start a test, this command

can be used. But the program will request you to

confirm that you wish to clear the previous test

results before starting a new test on all installed

cards. Moreover, this command will start initialization

test on the cards automatically at the first time that

you test the card(s).

d) View

This command lets you view the causes of error(s)

of the card(s) in the latest test. During the test, the

error messages for each card will be shown in a message

box and will be erased from the screen if you continue

the test on the other cards. You can use this command

to review these error messages on all cards after the

test. This command is supported only in "Stop if

errors" mode.

e) Exit

In the test menu, you can quit only when you

choose this option.

In order to let you start the test properly, the following

paragraphs will describe the setup and the function of each test.

����������������Ŀ

� Initialization �

������������������

a) Initialization Test

This test should be done before starting any test.

You can start the test in this menu manually or the

program will start it automatically before other tests

start in other test menus. In this test, all registers

will be tested and verified. If a card passes this

test, the card ID will be shown on the screen. Other

tests will not check a card that has FAILED in this

test.

You should plug OUR network adapter in the proper

slot and set the base address of I/O map to the correct

location.

�����Ŀ

� RAM �

�������

b) RAM Test

The program checks every memory module on the

network adapter by the read-after-write tests. These

tests can verify the address and data paths and the

memory modules separately.

You should choose a proper base address on the

system memory map for the memory on the network

adapter. A wrong base address will cause the data

crashes and the system to halt. Moreover, the size of

RAM should be correctly stated.

�����Ŀ

� IRQ �

�������

c) IRQ Test

This test checks the interrupt line between the

network adapter and the system board. The test

initializes the transmission of frames through the

adapter and then checks the interrupt line on the

system. You can proceed with this test even if the

adapter fails in the RAM Test.

You should choose a interrupt line that DOES NOT

conflict with other adapters' settings, otherwise the

system MAY HANG. Moreover, the adapter must be properly

terminated.

����������Ŀ

� Loopback �

������������

d) Loopback Test

This test checks the loopback circuits on the

adapter. It can't be proceeded if either RAM and IRQ

Tests failed.

You should terminate the adapter properly.

�����������Ŀ

� Tx and Rx �

�������������

e) Transmit (Tx) and Receive (Rx) Test

Unlike the other tests, this test requires at

least two inter-connected network adapters. The test

will initialize the transmission of frames from every

card to the others at any time. Its objective is to

test the functionality of the network adapter under the

network traffic conditions. In this test, the card is

fully tested. A card that has passed all above tests,

can then proceed with this test.

View Menu

*****************************************************************

This menu enables you to check the latest test results, the

card's configuration and the card identification code.

����������Ŀ

� Test Log �

������������

In each test menu, you can only view the test results for a

card under that test. If you want all test results for a card,

you can use this menu.

In this menu, all test results of the five tests will be

shown. For different test mode, the display format is different.

If a test on the card uses "Continue" mode, the "Passed" or

"Failed" message will be shown only. If "Stop at any error" mode

is used, both counts of successive and failed iterations are

displayed together. The count of successive iterations is shown

on the left hand side.

The program will search for a card tested before and shows

its test results. If you have not started any tests before, the

menu shows nothing. You should NOTE that all test results will be

cleared if you change the setting in the "Setup-Edit" menu.

You can use the "Choose" command to view the test results of

another card provided that the card has been tested.

����������������Ŀ

� Jumper Setting �

������������������

This command lets you to check that the jumper settings on

the network adapter board are correctly matched to the settings

you have chosen in the "Setup-Edit" menu. Three sets of jumpers:

a) mode and memory size

b) I/O base address

c) IRQ

can be checked here.

This command is used before installing the network adapters

on the system. So you should use this command to verify the

jumper settings on the adapter after making any changes. The

settings of any card with status "SELECT" will be shown in the

menu. You can view another by using "Choose" command. However,

you can not change the jumper settings in this menu.

���������Ŀ

� Card ID �

�����������

Each card has a unique identification (ID) code, called card

ID. During the Initialization Test, this ID is read from the card

and is shown on the screen if it can pass the test. This command

lets you to view cards's ID that have passed the Initialization

Test.

C) Setup for testing

This diagnostic program can test two types of network

peripherals: concentrator (or repeater) and network adapter.

Before testing the network adapters, you should carefully

set all jumper settings and use "View-Jumper Setting" in ENETDiag

to check the settings. Then install them on your system and

connect them through the appropriate cables. If coaxial cable is

used, you should check all terminations to ensure that all of

them are connected TIGHTLY. Then you can start the diagnostic

program.

For testing of the concentrators or repeaters, the setup

procedures are similar. But the way to interpret the results are

different. If you have confirmed that the network adapters that

are used to test the repeaters/concentrators are OK, then the

test results are referring to the ports on the

repeater/concentrator instead.

NOTE: If you want to test a concentrator or repeater using

more than one station, you should not start the Interrupt

Test or the Tx-and-Rx Test while any one of the stations

is running the Loopback Test.