====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER     =

=     Driver Installation Procedure                =

= >>> Windows NT 3.5 & 3.51                        =

====================================================

The WINNT 3.5x driver files are in the \WINNT subdirectory of this diskette.

1. Start Windows NT. From the Main Window, double click on the Control Panel.

In the Control Panel window, double-click on the Network icon. The "Network

Settings" window will appear.

2. In Network Settings, double-click on "Add Adapter".

3. To add the CN100TX(B), choose the item "<Other> Requires Disk from

Manufacturer".

4. Input the drive and path for this diskette's Win NT driver files, e.g.

A:\WINNT\35

Click on "OK". The files will automatically be copied to the hard disk.

5. In the "Select OEM Option" window, click on OK.

6. A dialog box for cable selection will appear. The default value is

"AutoSense". Click on "Configure..." to change the setting.

a) If the CN100TX(B) will be attached to an Auto-Negotiation or

Half-Duplex network device, "AutoSense" is the correct setting and

the default does not need changing. The driver will then

automatically set a correct line speed and opearting mode to

communicate with the attached device.

b) If users want to connect the CN100TX(B) to a Full-Duplex device, then

they must choose "100BaseTx Full_Duplex" or "10BaseT Full_Duplex"

depending on the speed of the attached device.

Refer to the User's Manual or README.1ST file, section "Auto-Negotiation

with the Attached Device" for more details on Full-Duplex connections.

7. Return to the Network Settings window and click on the Binding button.

Then click OK.

8. Reboot.

**NOTE: Please unload the SCSI driver from the device icon of the control

panel before rebooting. Otherwise, it conflicts with the network

driver.

###