Jun. 1995

DE-220 SERIES ETHERNET CARD

NextStep DRIVER PROGRAM

===============================================

Do as the following procedure to install D-Link DE-220 driver in NextStep.

1. Run setup utility "SETUP220.EXE" in DOS to select IO port, IRQ and media

type. Note: the default IRQ in NextStep DE-220 driver is 10.

2. Login "root" to NextStep.

3. Insert the DE-220 driver diskette into the floppy diskette.

4. Make a temporary directory and copy all of the files of the DE-220 driver

into this temporary directory.

5. Change to this temporary directorary and type "chmod +x setup".

6. Type "./setup" to setup the files of driver.

7. Use File Filewer to click the "DLDE220.config" to install DE-220 dirver.

8. Select Network icon and then select item "D-Link Ethernet Adapter DE-220"

to add this driver to the system.

9. Select the used IO port address and IRQ level.

10.Save the configuration.

11.Reboot the system.