=====================================================

How to install an PC-NFS workstation with AE-200

Ethernet Adapter and driver on an MS-DOS computer

=====================================================

1. First-time installation on a dual-floppy PC without a hard disk

---------------------------------------------------------------

a. Create a bootable DOS diskette (containing the necessary

DOS programs).

- Format a blank diskette using DOS FORMAT command with

/s option.

- Use DOS COPY command to copy ANSI.SYS file to the

formatted diskette.

- Make a directory "DRIVERS" under the root directory.

- Copy the AE-200.SYS driver file from \PCNFS directory on

the AE-200 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on the formatted diskette.

b. Install PC-NFS on a Bootable DOS diskette as a PC-NFS

Bootable diskette for your PC-NFS workstation.

- Insert the PC-NFS Installation Diskette (#1) into

drive A: and type A: and press <Enter>.

- Insert the formatted bootable DOS diskette into drive B:.

- Type install and press <Enter>.

- Select a proper display mode for your PC.

- Enter the letter of the drive where you want to install

PC-NFS. Since the formatted diskette is inserted in drive

B:, you should type B: and press <Enter>.

- Select the type of network connection.

"Ethernet" is highlighted. Press <Enter> to select "Ethernet".

- Select the network adapter you will use for PC-NFS.

Since AE-200 Ethernet Adapter does not appear in the

list of the supported Ethernet adapters, highlight "Other"

and press <Enter>.

- When you select "Other", the next screen asks you to enter

a line specifying the device driver you want to use as shown

below. The installation program adds the line you type to

the CONFIG.SYS file in the formatted DOS diskette in drive B:.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=A:\DRIVERS\AE-200.SYS /pxx /ixx

p for io_addr : The i/o balse address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the adapters

AE-200. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the adpater AE-200.

The default value is 3.

- Follow the on-screen instructions to copy the files from

the PC-NFS distribution diskettes to the bootable DOS

diskette, until the installation program displays the final

screen, instructing you to remove the distribution diskette

and give the Server Diskette to your system administrator.

- The bootable DOS diskette now becomes an PC-NFS bootable

diskette for you computer. PC-NFS installation is complete.

- Press <Enter> to start the NFSCONF configuration program.

- Please follow the instructions described in Chapter 3 of

the "PC-NFS Installation Guide" for configuring PC-NFS for

your PC and network.

c. Power off your computer and install the AE-200 Ethernet

Adapter onto your computer as the instructions described

in the "AE-200 Ethernet Adapter Installation Guide".

d. Reboot your computer with the bootable PC-NFS diskette.

e. Now, your computer is ready for your PC-NFS connection.

2. If you have a single-floppy PC without a hard disk

--------------------------------------------------

You cannot install PC-NFS on a single-floppy PC without a hard

disk; however, you can create a PC-NFS boot diskette that

contains the necessary files to run PC-NFS on your single-floppy

PC. Follow the following procedure.

a. Create a bootable DOS diskette (containing the necessary

DOS programs).

- Format a blank diskette using DOS FORMAT command with

/s option.

- Use DOS COPY command to copy ANSI.SYS file to the

formatted diskette.

- Create an \DRIVERS directory on the boot diskette using

the DOS MKDIR command.

- Create an \NFS directory on the boot diskette using the

DOS MKDIR command.

b. Copy the necessary PC-NFS files from the PC-NFS distribution

diskettes to the \NFS directory on the DOS boot diskette.

Please refer to the page 31 and page 32 for the file names

required to be copied from the distribution diskettes to

the DOS boot diskette.

c. Copy the AE-200.SYS driver file from \PCNFS directory on

the AE-200 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on the DOS boot diskette.

d. Create AUTOEXEC.BAT and CONFIG.SYS files, using any PC text

editor, such as Edlin. The sample AUTOEXEC.BAT and CONFIG.SYS

are shown below:

AUTOEXEC.BAT

--------------------------------------------

PATH = A:\NFS

SET NFSDRIVE=A

PRT *

NFSRUN

--------------------------------------------

CONFIG.SYS

--------------------------------------------

DEVICE = ANSI.SYS

BUFFERS = 20

FILES = 20

DEVICE = \NFS\PCNFS.SYS

DEVICE = \NFS\SOCKDRV.SYS

DEVICE = \DRIVERS\AE-200.SYS /pxx /ixx

LASDRIVE = V

--------------------------------------------

Note : the options for DEVICE = \DRIVERS\AE-200.SYS /pxx /ixx

are described as below:

p for io_addr : The i/o balse address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the adapter

AE-200. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the adpater AE-200.

The default value is 3.

e. Remove your boot diskette and insert the PC-NFS Configuration

Diskette (#3) in drive A.

- Type SET NFSDRIVE=A and press <Enter>.

- Type b:nfsconf and press <Enter>

- When prompted, insert your boot diskette into drive A.

- Follow instructions in Chapter 3 of "PC-NFS Installation

Guide" to configure PC-NFS.

f. Power off your computer and install the AE-200 Ethernet

Adapter onto your computer as the instructions described

in "AE-200 Ethernet Adapter Installation Guide".

g. Reboot your computer with the bootable PC-NFS diskette.

h. Now, your computer is ready for your PC-NFS connection.

3. First-time installation on a bootable Hard Disk or Partition

------------------------------------------------------------

a. Install PC-NFS on a bootable hard disk

- Create a \DRIVERS directory in your hard disk drive C, for

example.

- Copy the AE-200.SYS driver file from \PCNFS directory on

the AE-200 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on your hard disk drive C.

- Insert the PC-NFS Installation Diskette (#1) into

drive A: and type A: and press <Enter>.

- Type install and press <Enter>.

- Select a proper display mode for your PC.

- Enter the letter of the drive where you want to install

PC-NFS. Press <Enter> to accept the default drive C, or

type another drive letter and press <Enetr>. In the

following instructions, we use drive C as an example.

- Select the type of network connection.

"Ethernet" is highlighted. Press <Enter> to select "Ethernet".

- Select the network adapter you will use for PC-NFS.

Since AE-200 Ethernet Adapter does not appear in the

list of the supported Ethernet adapters, highlight "Other"

and press <Enter>.

- When you select "Other", the next screen asks you to enter

a line specifying the device driver you want to use as shown

below. The installation program adds the line you type to

the CONFIG.SYS file in the formatted DOS diskette in drive B:.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=C:\DRIVERS\AE-200.SYS /pxx /ixx

p for io_addr : The i/o base address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the adapter

AE-200. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the adpater AE-200.

The default value is 3.

- Follow the on-screen instructions to copy the files from

the PC-NFS distribution diskettes to the bootable DOS

diskette, until the installation program displays the final

screen, instructing you to remove the distribution diskette

and give the Server Diskette to your system administrator.

- Press <Enter> to start the NFSCONF configuration program.

- Please follow the instructions described in Chapter 3 of

the "PC-NFS Installation Guide" for configuring PC-NFS for

your PC and network.

b. Power off your computer and install the AE-200 Ethernet

Adapter onto your computer as the instructions described

in the "AE-200 Ethernet Adapter Installation Guide".

c. Now, your computer is ready for your PC-NFS connection.

4. If your computer already has PC-NFS software installed

------------------------------------------------------

a. Locate the directory path where you place your previous

network driver file.

We choose C:\DRIVERS as an example.

- Copy the AE-200.SYS driver file from \PCNFS directory on

the AE-200 Ethernet Adapter Driver Diskette to the \DRIVERS

directory on your hard disk drive C.

b. Change the current directory to the root directory of your

installed PC-NFS files by using DOS CD command.

c. Start nfsconf program.

- Type nfsconf and press <Enter>.

- Select "Network" from the Main Configuration Screen.

- Select "Direct connect"

- Back to main screen.

- Select "Adapter" configuration.

- Select <Ethernet E>.

- Select " <Other> Use a manufacturer's driver: "

- When you select "Other", the next screen asks you to enter

a line specifying the device driver you want to use as shown

below. The installation program adds the line you type to

the CONFIG.SYS file in you hard disk drive C.

Config.sys line for your driver

:_

At the prompt, enter the following line and press <Enter>.

DEVICE=C:\DRIVERS\AE-200.SYS /pxx /ixx

p for io_addr : The i/o base address setting on the Adapter

The value can be one of 200, 220, 240, 260

300, 320, 340, and 360 for the adapter

AE-200. The default value is 300.

i for int_level : Hardware interrupt setting on the Adapter

The value can be one of 2,3,4,5,10,11 or

12 for the adpater AE-200.

The default value is 3.

- Review or change other Configurations as instructions

described in your "PC-NFS Installation Guide".

d. Power off your computer and install the AE-200. Ethernet

Adapter onto your computer as the instructions described

in the "AE-200 Ethernet Adapter Installation Guide".

e. Now, your computer is ready for your PC-NFS connection

with AE-200 Ethernet Adapter.