***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > Wollongong Pathway                       *

***********************************************

The driver for Wollongong Pathway is located in the \NDIS_20 subdirectory

of the PowerNIC diskette.

1. Make a directory called NDIS on your hard disk.

2. Copy E200P.DOS and PROTOCOL.INI from the \NDIS_20 subdirectory of this

PowerNIC diskette to the C:\NDIS directory.

3. Change diskettes and run the Pathway Setup Program, PWSETUP

4. After the "Pathway Setup Program for DOS" window appears, choose

"Installation".

5. In "Installation", select "Pathway Run Time". From "Pathway Run Time",

choose "Quick Setup for NDIS".

6. In the "Pathway Runtime Quick Installation for NDIS" window

input the appropriate floppy disk drive letter as the source drive.

The destination should be "C:\PATHWAY".

Also, set the IP address and the number of subnet mask bits.

7. In the "Driver NDIS and Kernel Configuation" window, enter the path

"C:\NDIS" to the device driver and the PROTOCOL.INI file

8. There is a screen editor for modifying the configuations with

respect to I0BASE, Hardware Interrupt, and Driver Name. To exit

the editor press the F4 key. Zero values of IRQ and IO address

are used to autodetect the CN200/600/650Eplus adapter.

9. Next, use the select bar to change the software interrupt and

network frame type.

10. The Pathway Setup program will copy all relevant files and modify

the CONFIG.SYS file as well as the AUTOEXEC.BAT file automatically.

11. Reboot.