====================================================

=     CN100TX(B) PCI-BUS FAST ETHERNET ADAPTER     =

=     Driver Installation Procedure                =

= >>> Windows 95                                   =

====================================================

The Windows 95 driver files are in the \WIN95 subdirectory of this diskette.

Windows 95 has three versions so far. Version 0, A and B. You can select

the "System" icon in "Control Panel" box. On the System dialogue box,

the "40.00.950" message is version 0, "40.00.950 A" is version A and

"40.00.950 B" is version B. For version 0 and A, please follow the

"New Hardware Found" installation procedures. For version B (OSR2), go

to "Update Device Driver Wizard" installation procedures.

1. Insert CN100TX(B) Fast Ethernet Adapter into your computer's PCI-bus slot.

2. Connect the network cable to the RJ-45 connector.

3. Turn on the computer to run Windows 95.

"New Hardware Found" installation procedures:

4. When "New Hardware Found" window shows up, move mouse pointer to "Driver

from disk provided by hardware manufacturer", and then push "OK".

5. When "Install From Disk" window shows up, type "A:\WIN95" or "B:\WIN95"

in the input drive and path box where the driver is stored.

6. After the driver information shows up, select (highlight)

the "CN100TX(B) Fast Ethernet Adapter" and click "OK".

7. When "Insert disk" window shows up, push "OK" button and then choose

a correct path to load *.dll files, i.e. D:\WIN95.

8. After "System Setting Change" window shows up, the installation is done.

Remove your installation disk, push "YES" to restart Windows 95,

and the driver will function.

9. Installation is completed for Windows 95 version 0 and A.

"Update Device Driver Wizard" installation procedures:

If your Windows 95 is version B (OSR2), you must CAREFULLY install

CN100TX(B) LAN card drivers from the release diskette, due to that

this version's "Update Device Driver Wizard" utility has poor error

recovery ability. (If you make mistakes during the installation,

refer to Appendix A for the error recovery). Please follow the

installation steps in details below.

4. When the "Update Device Driver Wizard" dialogue box shows up, and

in it, it says "This wizard will complete the installation of

"PCI Ethernet Controller", put this CN100TX(B) LAN card's release

diskette to floppy drives and make sure the the drive's door is

CLOSED. Then click "Next" button ONE TIME only. (Don't rush to push the

"Next" button again. The rush "Next" button clicked would be assumed as

"Finished" and may cause error installations (Appendix A for error

recovery). It is going to take 30 seconds for Windows 95 to search the

card's driver. Then, another message "Windows found the following updated

driver for this device: CN100TX(B) Fast Ethernet Adapter" shows up,

then click "Finish" button to complete the driver installation.

(If it shows "Windows was unable to locate a driver for this device.",

You may also click "Other Locations" and type A:, A:\WIN95, B:, or

B:\WIN95, then click "OK" button). After the "Finish" button id clicked,

Windows starts to copy files from Windows 95 CD-ROM to your hard

disk. This is going to take around two minutes, please wait.

5. When the "Copying Files" dialogue shows up to request the directory

for the "DC21X4.SYS" file. Type "A:\WIN95" or "B:\WIN95" where A:

or B: drive is the place you put the release dskette in, then click "OK".

Windows 95 also request Windows95 CD directory to copy network-

related files. Please type the your Windows 95 CD directory name.

(Traditionally, it is D:\WIN95 or D:\WINDOWS).

6. Restart your PC to complete the installation.

Appendix A: (Valid only for WIndows 95 version B installation)

No matter what mistakes you made in the version B's installtion procedures

above and can not install the card to Windows 95. The "Update Device Driver

Wizard" utility still REGISTERS to Windows 95 that the card is installed

(with incoomplete error info., of sourse). Even though you carefully follow

the installation procudures above for the second time to successfully install

the driver, Windows 95 still treats the card as bad because the error data

was already REGISTERED. Therefore, you have to remove the REGISTERED errors,

before you try to install the driver again. The following describes the

details to remove the registered errors.

1. Click "Network" icon in the "Control Panel" box and remove the

CN100TX(B) Fast Ethernet Adapter you just successfully installed if any.

2. Select "Start" button (left-down corner) and select "Run" item.

Type REGEDIT, the registry edidor utility , then click "OK" button to

execute it.

3. In the Registry Editor dialogue, double-click "HKEY_LOCAL_MACHINE" item,

double-click "Enum", then double-click PCI item. Select the

"VEN_1011&DEV_0019", then delete it, which is the error registered.

Close Register Editor dialogue.

4. Reboot Windows 95 again to carefully follow the installtion procedures

above again.

5. (OPTIONAL) SETTING CONNECTION TYPE

"AutoSense" is the default setting for "Connection Type" in Windows 95.

If the CN100TX(B) adapter will be connected to an Auto-Negotiation or a

standard ("Half-Duplex") 100BaseTX or 10BaseT network device, "AutoSense"

is correct and users do not need to change the default setting. The driver

will automatically set the correct line speed and operating mode to

communicate with the attached device.

If the CN100TX(B) will attach to a dedicated Full-Duplex device, then

the "Connection Type" setting must be changed. Follow these steps:

a) Start Windows 95. Select "My Computer" then "Control Panel".

Double-click on the "Network" icon in the Control Panel.

b) From the "Network" window, select the "Configuration" tab.

c) In the Configuration window, under "Components" highlight the

"CN100TX(B) Fast Ethernet Adapter" and click on "Properties".

d) In the window titled "CN100TX(B) Fast Ethernet Adapter Properties"

select the "Advanced" tab. There is a window labelled "Property".

Click on "Connection Type", and then choose "100BaseTX Full_Duplex" or

"10BaseT Full_Duplex" (whichever is appropriate) from the list to the

right. After choosing the Connection Type, click on "OK".

e) Click again on the next "OK" and then on "Yes" to save the change and

restart the system.

(Refer to the User's Manual or the README.1ST file's "Auto-Negotiation

with the Attached Device" section for more details on line speed and

Full-Duplex connections.)

###