STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 3.1 Server driver

For EtherCard PLUS Family LAN Adapters

RELEASE INFORMATION

Version 3.01             19 March 1992

VERSION HISTORY

-- Version 3.01 (Current Release) --

1. Fixed the "Interrupt Lost" problem on fast 386/486 servers.

-- Version 3.00 --

1. Increased the number of transmission buffers from one to three, resulting

in increased performance on packet transmissions.

2. Changed driver short name to 'SMCPLUSS'.

3. Changed driver long name to 'Standard Microsystems Star/EtherCard

PLUS Server Driver'

4. Changed copyright to Standard Microsystems Corp.

5. Fixed the bug in the LLC control field when transmitting

Ethernet 802.2 packet Type 2 packets.

-- Version 2.07 --

1. Fixed problem when loading multiple protocols on 8003E & 8013EBT boards

would cause the system to hang.

2. Fixed problem where ParseDriverParameters would not automatically match

drivers when loading a second protocol on 8003E & 8013EBT boards.

3. Increased maximum packet size to 1514 from 1510.

-- Version 2.06 --

1. Matched driver name and version number to pass NOVELL

integrity test.

2. Fixed board reset which would toggle driver shutdown status. Board reset

now clears driver shutdown status.

-- Version 2.05 --

1. Fixed the bug in the LLC type field when transmitting

Ethernet 802.2 packet Type 1 packets.

2. Supported 8013EP/A and 8013WP/A adapters.

-- Version 2.04 --

Added version number and date to CONFIG command.

No code change was made from the previous version.

-- Version 2.03 --

This version contains modifications for the WD83C690 Network

Interface Controller. This version or later versions must be

used with adapters which contain the WD83C690.

-- NOTE --

Use of adapters which contain the WD83C690 with previous

versions of this driver may lead to data integrity problems.

This version has been tested by Western Digital.

-- Version 2.02 --

This version corrects a problem when using 8013 adapters in 16

bit mode with 16 bit VGA adapters in 16 bit mode. This version

also corrects a problem which caused the Operating System to

Abend (abnormal end) when IRQ, RAM Base, or RAM Size parameters

where entered  on the command line when using any of the following

software configurable adapters: 8003EB, 8003W, 8003EP, 8013EP, 8013W,

or 8013EW.

-- Version 2.00 --

This is the first offical release of the NetWare 386 v3.1

driver. It conforms to the the ODI driver specification

for NetWare 386 version 3.1. This driver supports the

multiple MAC protocol and network management functions of

NetWare 386 v3.1. This driver does NOT function with NetWare

386 version 3.00.  If a driver for Netware 386 version 3.00

is required, refer to version 1.00 of this driver.

This driver has one known problem, it will not function

with the 8013EBT adapter when used with other 16 bit adapters

(ie. 16-bit VGA) in 16 bit mode.  This problem will be fixed

in the next release.

EtherCard PLUS, TokenCard, and SuperDisk are trademarks, and SMC and

Standard Microsystems are registered trademarks of Standard Microsystems

Corporation. Other trademarks mentioned herein belong to their respective

companies.