LE8029R driver for NetWare 2.x and 3.11

Client Installation

========================================

This section tells you how to install the adapter into a workstation

that uses DOS ODI. The ODI (Open Data-Link Interface) allows you to

* Expand your network by using multiple protocols without adding

network boards to the workstation.

* Communicate with a varity of workstations and file servers via

different protocols without rebooting the workstation.

* Protect your investment because protocols send packets through

the LAN driver written to ODI specifications.

* Because one LAN driver supports several protocols, you have fewer

hardware components to support.

When using ODI shells, no files need to be generated. To install

DOS ODI workstations, you need the following:

* A working copy of the NetWare WSGEN diskette

* A DOS system diskette to make master workstation diskette

The procedure to create the master ODI workstation diskette is as follows:

1. Create system diskette by using DOS FORMAT command with /S parameter.

2. Copy the shell file you want from your NetWare WSGEN diskette to

the master workstation diskette.

* If you will use conventional memory in your workstation, use

NETx.COM.

* If you will use expanded memory (LIM 4.0 EMS compatible) memory,

use EMSNETx.EXE.

* If you will use extended memory (XMS 2.0 compatible) memory,

use XMSNETx.EXE.

3. Copy the required workstation files from the DOSODI directory of the

WSGEN diskette to the master workstation diskette. The following files

are required to communicate with a NetWare server:

* LSL.COM (V2.11 or above)

* The LAN driver for the Ethernet adapter.

-- The file LE8029.COM, which is located in the directory

\DOSODI of the this driver diskette.

* The protocol stack file (IPXODI.COM, for example)

Optional

* NETBIOS.EXE, INT2F.COM

4. Create the CONFIG.SYS file. This file must have, at minimum, the

following:

* FILES=XX (XX at least a value of 20)

* BUFFERS=XX (XX at least a value of 30)

* A line to load a memory manager if you are using such a program.

5. Put the executable files you selected into an AUTOEXEC.BAT file

on the master workstation diskette. Load the files in the following

order:

* Link support layer

* LAN drivers

* Protocol stacks

* The shell

The following is a sample AUTOEXEC.BAT file.

LSL

LE8029    <----- for the adapter series

IPXODI

NETx

6. Create a NET.CFG file on the master workstation diskette (optional).

Usually, you do not need a NET.CFG file because the established

defaults are adequate for your network. However, you might need to

create a NET.CFG file if, for example, you are using different

configurations other than the defaults or you are using a protocol

other than IPX ODI.

There is one sample NET.CFG file in the directory \DOSODI of this

driver diskette.

Please refer to NetWare manuals for the further information about

NET.CFG file.

7. Label the master workstation diskette with the name of the LAN driver

(such as "Master Workstation LE8029R") and list the configuration

settings on the diskette.

8. Login to the network with the master diskette to ensure that the

files you created, such as AUTOEXEC.BAT and CONFIG.SYS. can connect

to the network.

============================================

IMPORTANT NOTE TO NetWare 3.11 or 2.2 CLIENT

============================================

Both Netware 3.11 and 2.2 servers normally use ETHERNET_802.3 as the

default Ethernet frame type instead of using ETHERNET_802.2, which

was recommended default frame type for Netware 3.12 and 4.01.

The ODI driver \DOSODI\LE8029.COM in this driver diskette was designed

to support multiple frame types and use ETHERNET_802.2 as default.

If your station is the client of Netware 2.2 or 3.11 and the server

was configured to support frame type of ETHERNET_802.3 only, you

have to edit the NET.CFG file of your station to support ETHERNET_802.3

by adding the FRAME statement under the LINK DRIVER heading as below:

LINK DRIVER LE8029

FRAME Ethernet_802.3

After NET.CFG file is changed, re-load the client software again.

LE8029R driver for NetWare 3.12 and 4.01

Client Installation

========================================

This information covers the following three sections:

1. LAN Driver installation using LE8029R driver

2. About Netware DOS Requester

----------------------------------------------------------------------------

1. LAN Driver Installation

-----------------------

During Netware DOS Client software installation

The LE8029R ODI driver files have been properly structured and prepared

in the driver diskette under \DOSODI sub-directory for Netware 3.12/4.01

DOS Client installation. Select one adapter model for your installation.

2. Netware DOS Requester

---------------------

This sub-directory \DOSODI contains all the necessary files required for

Netware 3.12 and 4.01 Client installation. During the client installation

process, the install utility copies LE8029.COM, VLM manager and modules

to a directory named NWCLIENT (unless you specify otherwise). The install

utility will also modify the CONFIG.SYS and AUTOEXEC.BAT files to incorporate

the required statements if you allow the utility to perform this step.

Netware 3.12 and 4.01 use the DOS Requester as a replacement for the shell,

NETX.EXE. The DOS Requester is a set of files loaded into the client

station's memory. The Requester has two components:

. Manager (VLM.EXE)

. Virtual Loadable Modules (VLMs)

After client installation, DOS client configuration files are modified as

follows:

* CONFIG.SYS ---- The follwoing command line was included in CONFIG.SYS.

LASTDRIVE=Z

* AUTOEXEC.BAT -- The following batch command line was included in

AUTOEXEC.BAT to load the networking files from \NWCLIENT.

@CALL C:\NWCLIENT\STARTNET

* STARTNET.BAT -- It will load ODI environment and load VLM.EXE. The command

include the following:

@ECHO OFF

C:

CD \NWCLIENT

SET NWLANGUAGE=ENGLISH

LSL

LE8029

IPXODI

VLM

* NET.CFG ------- The following example shows the NET.CFG file after the

client installation utility has been executed.

Link Driver LE8029

FRAME Ethernet_802.2

Netware DOS Requester

FIRST NETWORK DRIVE = F

USE DEFAULT = OFF

VLM = CONN.VLM

VLM = IPXNCP.VLM

VLM = TRAN.VLM

VLM = SECURITY.VLM

VLM = NDS.VLM

VLM = BIND.VLM

VLM = NWP.VLM

VLM = FIO.VLM

VLM = GENERAL.VLM

VLM = REDIR.VLM

VLM = PRINT.VLM

VLM = NETX.VLM

The Netware DOS Requester is the prefered DOS Client connection softwares.

Please refer to Netware 3.12/4.01 manuals for more information.