***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > Personal NetWare                         *

***********************************************

The driver for Personal NetWare 1.x is stored in the \DOS_ODI subdirectory

of the PowerNIC diskette.

1. First install Personal NetWare in the PC.

2. Copy the driver file to the Personal NetWare subdirectory.

At the DOS prompt, type the following commands:

COPY A:\DOS_ODI\E200P.INS C:\NWCLIENT

COPY A:\DOS_ODI\E200P.COM C:\NWCLIENT

NOTE: This assumes the diskette is in drive A.

3.  a) Run Personal NetWare's SETUP.EXE to enter the main menu.

b) Choose the "Primary Network Interface Card" item, and press ENTER to

select "CN200/600/650Eplus Ethernet Adapter".

c) Return to the main menu and choose "Configure Primary Interface Card"

to configure the "INT", "PORT" and "Media Frame Type(s)".

d) At the main menu, press "Save Changes and Exit".

4. Execute STARTNET.BAT.