Installation Guide for Lantastic/AI

==========================================

To install Lantastic/AI v4.x or Lantastic/AI v5.x software on a workstation,

follow the instructions.

Installation Porcedures :

a. Run your existing DOS system as you usually do.

b. Place the Lantastic/AI Network Operating System Disk #1 in

floppy drive A: or B:

c. Change to the drive containing the diskette and type

"INSATLL" at the system prompt, then press <Enter>.

d. Choose NE2000 as your Ethernet Hardware Adapter.

e. Follow all the operations as you are prompted on the screen.

f. After installing the Netware Operating System, reboot your system.

--- Sample AUTOEXEC.BAT file ---

:

call C:\LANTASTI\STARTNET.BAT

:

:

--- Sample STARTNET.BAT file for Lantastic/AI v4.x ---

:

@ECHO OFF

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

:

:

--- Sample STARTNET.BAT file for Lantastic/AI v5.x ---

:

@ECHO OFF

SET LAN_DIR=C:\LANTASTI.NET

SET LAN_CFG=C:\LANTASTI

PATH C:\LANTASTI;%PATH%

SHARE /L:200

NEX000 IRQ=3 IOBASE=300 16-BIT VERBOSE

AILANBIO

REDIR ADMIN LOGINS=3

SERVER C:\LANTASTI.NET

NET LOGIN/WAIT \\ADMIN ADMIN

NET LPT TIMEOUT 10

NET POSTBOX

:

: