***********************************************

*  CNet ISA-Bus PowerNIC Driver Installation  *

*  > Windows for Workgroups 3.1               *

***********************************************

The DOS NDIS driver for Windows for Workgroups is located in the

\NDIS_20 subdirectory of the PowerNIC diskette.

1. From the Control Panel, click on the "Network" icon.

2. If there is no 200E NDIS driver installed on the hard disk, select

"Unlisted or Updated Network Adapter". Specify "A:\NDIS_20" as the path

(assuming this diskette is in drive A). Skip to Step 4.

3. Otherwise, select "CNet CN200E Series Ethernet Adapters" in

the network adapter list.

4. Specify the correct configurations of NIC, including I/O Base

and INTERRUPT (IRQ). I/O Base and IRQ values should be set to

zero's for autosense or to the same values specified in the

PowerNIC SETUP program.

5. Save the configurations.

6. Exit and reboot PC to put the change into effect.

NOTE: If you had previously installed the driver in WFW and want to install

it from the diskette again, you must first delete the related OEM?.INF

file from the \WINDOWS\SYSTEM subdirectory.