* * * * * * * * * * * * * * * * * * * * * * *

BitCom Version 5.3 Release Note

October, 1992

(C) Copyright 1988-92 BIT Software Inc.

All Rights Reserved

* * * * * * * * * * * * * * * * * * * * * * *

________________________________________________________________________

INSTALLING BITCOM

BitCom 5.3 includes an installation program that will automatically

install BitCom.

To install BitCom:

1.  Insert the BitCom Program disk in Drive A (or Drive B).

2.  Type

A:\INSTALL (or B:\INSTALL if you are using your B Drive)

and press [Enter].

3.  Follow the instructions as they appear on screen.

*Note* If you are using a color monitor, delete the file $NOCOLOR

from the BitCom directory after you finish installation.

________________________________________________________________________

NEW FEATURES

*  Three New File-Transfer Protocols

- Xmodem 1K - This protocol is similar to Xmodem CRC, except it sends

1K-byte blocks of data, instead of 128-byte blocks. This protocol

increase the transfer rate, especially when using an error-correcting

modem.

- Xmodem 1K-G - This protocol is similar to Xmodem 1K, but it does not

use handshaking. This reduces overhead but also increases the chance

of errors cropping up into your data. Generally, you should only use

this protocol with error-correcting modems.

- Zmodem - Zmodem is an error-correcting, "streaming" protocol that

sends data in a steady stream, inserting error-checking codes at

specific intervals. As the other end gets the data, it check the

error-checking code. This protocol also features automatic

downloading and file recovery.

When possible, use the Zmodem protocol.

To see a description of the Zmodem Parameters Menu, refer to the

file PARAM.TXT, which is located in the BitCom directory.

*  Supports High-Speed Modems.

BitCom now supports baud rates of up to 57,600 bps.

*Note* When using an high-speed modem with error-correcting, set Flow

Control to RTS/CTS.

*  Selectable Connection Type.

BitCom now lets you easily choose the modulation technique that your

modem uses. Normally, BitCom will use your modem's default standard.

However, in the Type field of the Communication Parameters Menu, you

can now specify a modulation technique.

For example, if your modem by default uses V.42bis autoreliable mode,

but you want to use V.42bis reliable mode, you would choose V.42bis

Reliable in the Type field of the Communication Parameters Menu.

BitCom would then override your modem's default mode by sending a

special command to your modem, placing it in Reliable mode.

Also, if you are using a standard 2400-baud modem, BitCom can emulate

MNP 5 or MNP 4 -- in the software -- giving you the benefits of an

error-free connection and automatic data compression without an

MNP-equipped modem. You can tell BitCom to emulation MNP 5 or MNP 4 by

choosing either the MNP 5 (EMUL) or MNP 4 (EMUL) option.

In the Type field of the Communication Parameters Menu, you can choose

one of the following options.

*Note* You can only choose modulation techniques that your modem

supports. For example, if you are using a standard 2400-baud

modem, you cannot choose V.42bis Autoreliable. You can,

however, tell BitCom to emulate MNP 5 or MNP 4 using a standard

asynchronous modem.

- Default

When selected, BitCom will not send an initialization string,

leaving your modem in its default mode.

- Standard Asynchronous

When selected, BitCom will tell your modem to make a standard,

asynchronous connection, even if your modem supports V.42bis or

MNP 5.

- V42bis Autoreliable

This option places your modem in V.42bis auto-reliable modem. In

this mode, your modem will automatically detect whether it can make

a connection using the V.42bis, V.42 or MNP protocol. If it cannot,

it will automatically "fall back" to a standard asynchronous mode.

- V42bis Reliable

This option places your modem in V.42bis reliable mode. In this

mode, your modem will only make a connection using the MNP, V.42 or

V.42bis protocol. If it cannot, it will disconnect. Only use this

mode when you know the remote system supports MNP, V.42 or V.42bis.

- V42 Autoreliable

This option places your modem in V.42 auto-reliable modem. In this

mode, your modem will automatically detect whether it can make a

connection using either the V.42 or MNP protocol. If it cannot, it

will automatically "fall back" to a standard asynchronous mode.

- V42 Reliable

This option places your modem in V.42 reliable mode. In this mode,

your modem will only make a connection using either the MNP or V.42

protocol. If it cannot, it will disconnect. Only use this mode when

you know the remote system supports MNP or V.42.

- MNP 5 (EMUL)

This option tells your modem to emulate MNP 5 error-correction and

data compression. This gives the benefits of MNP 5 with a

standard, 2400-baud modem.

*Note* Do NOT use this option if your modem supports V.42bis, V.42

or MNP 5.

- MNP 4 (EMUL)

This option tells your modem to emulate MNP 4 error-correction. This

gives the benefits of MNP 4 with a standard, 2400-baud modem.

*Note* Do NOT use this option if your modem supports V.42bis, V.42

or MNP 5.

- MNP 5

This option tells your modem to use MNP 5 error-correction and data

compression, even if it capable of a V.42bis connection.

- MNP 4

This option tells your modem to use MNP 4 error-correction, even if

it capable of a V.42bis connection.

In the Communication Selection Menu, BitCom will indicate the

modulation technique you choose with a letter in the TYP field. Each

technique has been assigned a letter. For example, if you set the Type

field to V.42bis Auto-reliable, the TYP field in the Communication

Selection Menu will show a "B."

*Note* Different modems might use different initialization strings. If

you get an error after choosing one of the above options, open

the Initialization String Menu and check the initialization

strings. Then check your modem's manual for the correct AT

command string.

*  Initialization String Menu.

BitCom now lets you edit or add initialization strings it will send to

your modem.

To specify an initialization string, choose the Connection Type

command from the Communication Configuration Menu. This will open the

connection Type Initial Prefixes Settings Menu. Each rows will

show the current initialization strings for each modulation technique.

In the blank rows, you can specify additional initialization strings.

The name you entered in the Connection Type field will appear as an

option in the Type field of the Communication Parameters menu.

*Note* The entries in this menu must be sequential; there can be no

blank row between entries.

*Note* Before entering a new initialization string (or modifying an

existing one), check your modem's manual for the proper AT

commands.

After entering a new initialization string, you can then choose it

from the Type field of the Communications Parameters.

*  Selectable IRQ Settings.

BitCom lets you assign the interrupt request line (IRQ) that it will

use for each COM port. BitCom support IRQ 0 to IRQ 15. You can change

the IRQ line that BitCom will use for each COM port from the IRQ

Settings Menu.

To open the IRQ Settings Menu, choose the SetIRQ command from the

Communication Configuration Menu.

Assigning a unique interrupt request line (IRQ) to each serial port

enables you to use COM 1 and COM 3, or COM 2 and COM 4,

simultaneously.

Most modems and other serial devices are preset to use just two IRQ

lines for its four COM port settings; COM 1 and COM 3 usually use

IRQ 4, while COM 2 and COM 4 use IRQ 3. Thus, if you are using COM 1,

you cannot use COM 3 at the same time  unless COM 1 and COM 3 are

configured to different IRQ lines.

More and more modems and I/O serial cards, however, let you can change

their IRQ line setting. For example, by configuring your internal

modem to IRQ 2, you could set your mouse to COM 1 and your modem to

COM 3. Consult the documentation that came with your internal modem or

I/O serial card for instruction on how to change the IRQ line setting.

Changing IRQ settings usually requires you to change jumper switches.

Before changing the IRQ setting for a COM port, determine which

IRQ settings your other serial ports are using. Also be sure to check

that your modem is set to the new IRQ. For example, if you want to set

COM 3 to IRQ 5, you must first make sure that your modem is set to use

IRQ 5. If you are using an internal modem, you must change the IRQ

setting of the modem itself. If you are using an external modem, you

must change the IRQ setting of the serial card that the modem is

connected to.

*Note* Not all modems and serial cards allow you to change their IRQ

setting. If you are using an internal modem consult your

modem's manual on how to change its IRQ number. If you are

using an external modem that is connected to a serial card,

consult the serial card's manual.

*  New Built-in Script Variable.

Added a new built-in variable, "@AutoSpd," to BitCom's script

language. When set to yes, "@AutoSpd=1," BitCom will automatically

match the baud rate to the carrier speed. For example, if you set

BitCom's baud rate to 9600, but you make a connection at 2400, BitCom

will automatically lower your baud rate to 2400. When set to no,

"@AutoSpd=0," BitCom will not automatically match baud rates.

Do not set this variable to yes when using modems with data

compression; otherwise your modem might not be able to take advantage

of the increased throughput.

________________________________________________________________________

Changes to the User Interface

*  The Protocol Menu has three new protocols -- Xmodem 1K, Xmodem-1K-G

and Zmodem. In addition, the protocol Ymodem (Batch) is on a

separate line for easier selection.

*  A new menu, called Connection Type Initial Prefixes Settings, lets

your add or edit initialization strings that BitCom will send to your

modem. You can open this menu by choosing the Connect Type command

from the Communication Configuration Menu.

*  The MNP field in the Communication Selection Menu has been changed to

Type. This field lets you select the modulation technique (MNP 5,

V.42bis Autoreliable, etc.) that BitCom will tell your modem to use.

Also, the MNP field in the Communications Parameter Menu has

been changed from MNP to Type. This field will indication with a

letter the kind of modulation technique BitCom is using. If BitCom is

using your modem's default modulation technique, this field will be

left blank.

________________________________________________________________________

Corrections to the Manual

*  Running Host Mode

To run BitCom's Host Mode, go to the DOS prompt and type:

BITCOM @REMOTE.SCP

and press [Enter]. You will see the message "Running Remote Access -

Auto Answer."

*  Technical Support

Bit Software's technical support numbers are now

Voice: (510) 490-9470   Fax: (510) 490-2939

________________________________________________________________________

Additional Documentation

Include with BitCom are the following three text file:

EMULATE.TXT

PARAM.TXT

REFER.TXT

SCRIPT.TXT

These files give further details about BitCom's parameters and

capabilities. The file EMULATE.TXT describes BitCom's emulation files.

The file PARAM.TXT describes the option in BitCom parameter's menus.

The file REFER.TXT explains BitCom's error messages, includes an ASCII

code table, and a table of display attributes and color codes.

The file SCRIPT.TXT explains BitCom powerful Script language.

Thank you for your continued support.

Bit Software Inc.