FUTURE DOMAIN CORPORATION

FDU-CD

CD-ROM Device Driver Version 2.20

I. GENERAL PRODUCT OVERVIEW:

1) Product description:

The FDU-CD CD-ROM device driver program is to be used with

Future Domain SCSI Host adapters to support one or more CD-ROM

drives on a DOS-based host system.  The device driver can support

from 1 to 24 CD-ROM drives (via up to 4 SCSI adapters) on the

PC, PC/XT, PC/AT and Micro Channel Architecture (PS/2) environments.

2) Table of supported CD-ROM drives:

MANUFACTURE     MODEL NUMBERS

-----------     ----------------------------------

Chinon          CDX-431

Denon           DRD-253

Hitachi         CDR-3650U, CDR-1750S

LMSI            CM-131, CM-214

NEC             CDR-35, CDR-36, CDR-72, CDR-73, CDR-80, CDR-83

Panasonic       CR-501, LK-MC501

Pioneer         DRM-600/610

Sony            CDU-541, CDU-6110, CDU-6111, CDU-6211,

CDU-7211

Texel           DM-3020, DM-3021 DM-3120, DM-5020, DM-5021, DM-5120

Toshiba         XME-2100A, XME-3100A, XME-3201B, XME-3301TA,

XME-5100A

NOTE: The Denon CD-ROM drive is supported only on our TMC-8xx (8-bit)

adapters.  When used with our TMC-16xx (16-bit) adapters, DOS

will not be able to recognize media changes.

II. SYSTEM REQUIREMENTS:

- Minimum 512KB of System Memory needed by INSTALL program.

(This amount of memory is required by the automatic installation

program - INSTALL.EXE.  Refer to "Manual Installation" in the

CD-ROM device driver Installation Manual Guide, if you cannot

run the automatic installation program.)

- MS or PC DOS Version 3.x - 5.0x.

- MS DOS Extensions Version 2.20 (may be included)

- A Hard Disk Drive. (Optional)

(Not necessary for device driver or host adapter but

may be necessary for some CD-ROM application programs.)

III. CHANGES FROM PREVIOUS RELEASE:

1) Additional drives added:

MANUFACTURE     MODEL NUMBERS

-----------     ----------------------------------

LMSI            CM-214

NEC             CDR-36, CDR-73, CDR-83

Poineer         DRM-600/610

2) Changes were made to the driver to provide support for the Microsoft

Multimedia Extensions V1.0.

IV. INSTALL/TECH TIPS:

1) Quick tips:

a) New command line parameter - /I

If this switch is not specified, the driver will not return an

"Invalid Disc Change" error if the SAME disc is reinserted into

the drive.  This may cause a problem if two different disc's

having the same volume size are being used.

If you are experiencing this problem, specifying the /I switch

will force the driver to always return an "Invalid Disc Change"

error anytime a disc is changed. (SAME or DIFFERENT disc)

usage: DEVICE=C:\DEV\FDCD.SYS /D:MSCD000 /I

b) Denon DRD-253 DIP switch settings.

You must set the DIP switches appropriately for the drive to use

2048 bytes per sector.  This is done by setting switches SW4 and

SW5 to the ON (down) position.

c) LASTDRIVE = Z in the CONFIG.SYS file.

The install program will default to using drive letter 'Z'.

This drive letter should be changed so that DOS does not reserve

more drive letters than are necessary.

example: if you have one fixed drive with two partitions using

drive letter C & D and one CD-ROM drive using drive

letter E, set LASTDRIVE to E (LASTDRIVE = E). This is

very important for Network users so that drive letters

will be available to access Network drives as in a

Novell environment.

d) You can remove the BIOS chip on our SCSI controller if there

are no fixed drives attached.  This will allow the system to

boot-up much more quickly since the SCSI bus is not scanned

by the BIOS upon boot-up.

2) The following Software has been thoroughly tested with the

supported drives:

DISCOVERY SYSTEMS:

- Sampler

MICROSOFT:

- TESTDRV.EXE (CD-ROM device driver test program)

- CDSPEED.EXE (CD-ROM Data Streaming Transfer Rate Test V1.00a)

- Windows 3.0 with Multimedia extensions V1.0

- BookShelf

- Programmer's Library

- Musicbox with a variety of audio discs

NATIONAL GEOGRAPHIC SOCIETY:

- Mammals: A Multimedia Encyclopedia

SIERRA ONLINE:

- Mother Goose

SOFTWARE TOOLWORKS

- CD GAME PACK

- ILLUSTRATED ENCYCLOPEDIA

- US ATLAS

- WORLD ATLAS

NOTE: The driver will work under Windows 3.0 in REAL, STANDARD and

ENHANCED mode.  Depending on the application being used, you

may be limited to using REAL mode.  Contact the manufacture of

the application for compatibility under Windows.

3) Limitations:

a) The following drives do not support Audio Channel Manipulation:

- NEC CDR-35 (supports stereo & mute mode)

- Sony CDU-6111-01 (supports stereo mode only)

V. TECHNICAL SUPPORT:

To contact our Technical Support Department please call :

Future Domain Corporation

2801 McGaw Ave.

Irvine, Ca. 92714

TS HOTLINE #: (714) 253-0440

FAX HOTLINE #: (714) 253-0429

Monday - Friday, 8am - 5pm PST.