Future Domain Corporation (FDC)

Products: Future Domain SCSI 32-Bit Disk Access (FastDisk) Driver

Version:  1.1

1. General Product Overview

The Future Domain SCSI 32-Bit Disk Access (FastDisk) is a Windows

V3.1 Driver that supports SCSI disks with the Future Domain line of

adapters. This driver improves the performance of disk accesses

under Windows V3.1 Enhanced Mode.

This version supports the following Future Domain adapters;

Sixteen bit: TMC-1650/1670/1660/1680/1760/1790/1795 MCS-600/700

Eight bit: TMC-845/850/860/875/885/850M/860M/885M

This version supports the following Future Domain VLSI integrated circuits;

Sixteen bit:  TMC-1800/18C50

Eight bit: TMC-950/9C50A

2. Installation

Run the installation program INSTALL.EXE from the Program Manager File

menu. This will copy all the Future Domain files into a directory you

specify. It will also install the Future Domain FastDisk icon on

the Control Panel window.

To actually install the SCSI FastDisk interface, you need to

double-click on the FastDisk icon that appears on your Control Panel

window. This program should guide you through the installation of

the Future Domain 32-Bit Disk Access (FastDisk) driver and the other

drivers it depends on.

If you have a 950 VLSI adapter, the installation will also ask for

the Memory Base Address and the IRQ Level used by the adapter. Be

prepared with this information before running the installation of the

FastDisk interface.

3. FastDisk Support On More Than Two Fixed Disks

By default Windows V3.1 can support only a maximum of two fixed disks

with FastDisk. This support is usually enough with majority of the

users but in situations where more than 2 fixed disks are installed

it will be beneficial to have FastDisk (32-Bit Disk Access) support

for more than 2 fixed disks. If you have installed our Future Domain

driver FDDEVICE.SYS (included in our The Disk Maestro software

package) to support more than 2 fixed disks under DOS you

are greatly encouraged to install our modified INT13.386 driver on

your Windows V3.1 system. This INT13.386 driver replaces the one

provided by your Windows V3.1 Retail Product and enables FastDisk

support on all drives supported by FDDEVICE.SYS. This by default will

be installed by INSTALL.EXE and the item name is 'Enhanced INT 13H

Driver'.

4. Precautions

First, each of the Future Domain adapters require a valid IRQ number

assigned to it and this number does not include 0 (zero). We require

you to have a non-zero IRQ for each of the Future Domain adapters in

your system for the SCSI FastDisk to work.

Secondly, be careful in assigning the IRQ value to your Future Domain

adapters. Make sure that none of them will conflict with any other

hardware/peripheral in your system. The reason behind this is that if

one of your hardware/peripheral (like the mouse installed in COM port 1)

has the same IRQ as one of your Future Domain adapters, when Windows

starts up your mouse will not work.

Lastly, if you have Future Domain's CD-ROM Driver installed in your

system please see the "Compatiblity Issues" help topic in the

FastDisk Installation Program.

5. SMARTDRV V4.0 Incompatiblity with Disk Maestro V6.0, V5.X

The disk caching utility SMARTDRV.EXE V4.0 is incompatible with the

disk driver FDDEVICE.SYS shipped with Future Domain's The Disk

Maestro V6.0 or V5.X product. An updated FDDEVICE.SYS that resolves

this problem is available in release V6.1.  Call 1-800-879-7599 to

order a Disk Maestro V6.1 upgrade package.

6. One Final Note

The first time that you run INSTALL.EXE, it will store the following

lines in your WIN.INI file:

[SCSI FastDisk]

DriverPath=<path>

where <path> is the path you have indicated the Future Domain files

will be copied to. Please do not remove these lines.

7. Customer Service

Write or call:

Future Domain Corporation

Customer Service

2801 McGaw Avenue

Irvine, CA 92714

Monday - Friday, 8 AM to 5 PM (PST)

Software Upgrade Orderline (800) 879-7599

TS Hotline: (714) 253-0440

FAX Hotline: (714) 253-0429