June 1, 1994

_ _ _

/ / (_)

_\ \ \

(_)_/_/  Find-ATA v1.0 - An ATA interface identify drive utility.

Seagate Technology, Inc.

B-5 Technical Support

920 Disc Drive

Scotts Valley, CA 95066

Usage: FIND-ATA [P or S] [M or S] [optional: R or D]  or....

FIND-ATA /filename.ATA

switches:

P or S        -  Primary or Secondary Port Address (these default to

1F0h and 170h)

M or S        -  Master or Slave drive

The optional R and D switches will only work if the Port and Drive

switches are also set on the command line.  ex: Find-ATA P M D

R             -  allow command line redirection (this feature is

useful for redirecting the output to a printer or a

file)

D             -  dump the Identify binary data to <filename>.ATA (this

allows a copy to be saved that can be studied at a

later time. A "collection" of various drive files

might be useful for reference purposes.)

/filename.ATA -  load a previously saved Identify data dump.

(no switches) -  program will prompt for Port and Drive info

Find-ATA is designed for the support of users with Seagate disc drives.

A little background:

The ATA interface has evolved rapidly since its initial design by

Compaq Corporation.  After refining the basic ATA interface concepts

and circuitry, Compaq Corporation worked with Imprimis (now a part of

Seagate) to build the first ATA interface drive.  At this stage, the

interface was far from being an accepted standard.  However, it was a

natural extension of the ATA I/O bus, and gained industry-wide

acceptance because most of the necessary framework needed for the

implementation was already present in the host machine.

Initially, there were no industry-wide standards for implementing the

ATA interface, leaving manufacturers free to extend and improve upon

it.  In the latter part of 1988, a Common Access Method (CAM)

committee was established to develop such standards.  Their results

were adopted by the American National Standards Institute (ANSI) with

the intent of creating a common ATA command specification.

The ANSI standard for the ATA interface now provides specifications

for mandatory commands, signal conventions, register descriptions and

other information necessary for basic compatibility across

manufacturers and platforms.  The current ANSI specification includes

provisions for extended features such as caching and power management,

while also providing options for vendor-specific enhancements.

The "Identify Drive" command is defined for the ATA interface.  This

valuable command, when executed, will return to the host 512 bytes

which describe the drive and its available features.  It is very

similar to the original Identify drive developed for the older ESDI

interface and is analogous to the SCSI interface's Mode Sense command,

albeit with less detail.  Find-ATA is a simple utility which helps

organize the data in a readable form.

The following screen print shows the Identify Drive bytes that

correspond to the Find-ATA field names.

������������������������������͵ Find-ATA v1.0 �������������������������͸

�                                                                        �

�     Drive: 27-46                                      Port:            �

�  Serial #: 10-19                 CMOS Type:        Unit:               �

�  Firmware: 23-26                                                       �

�            Hardware       DOS            Current          Max ECC: 22  �

� Cylinders: 1                             54                            �

�     Heads: 3                             55                            �

�   Sectors: 6                             56                            �

�  Capacity:                               57-58                         �

�                                                                        �

�                     Available  Information                             �

� R/W Multiple Mode:  47.7-0                 Current: 59.7-0             �

�          LBA Mode:  49.9       total sectors 60-61                     �

�          DMA Mode:  49.8       Single: 62         Multi: 63            �

� Advanced PIO Mode:  64.7-0               +,-IORDY: 68,67               �

�                                                                        �

�  Default PIO Mode:  51.15-8                                            �

�                                                                        �

�  Buffer: 21   - Type: 20                                               �

�                                                                        �

�                                                                        �

�������͵ Copyright 1994 Seagate Technology, Inc. All rights reserved. �;

IDENTIFY DRIVE command

The Identify Drive command (command code ECh) transfers information

about the drive to the host.  When the command is issued, the drive

sets BSY, prepares to transfer a single 512-byte block of data, sets

DRQ, and generates an interrupt.  The host then reads the data from

the drive.  The data is organized as shown in the table below.  All

reserved bits or words are zero.

Words 53 to 49 and 62 to 63 change depending on the current operating

mode of the drive.  All other words are constant for a particular

drive.

An asterisk (*) indicates words for which a value of 0000h indicates

parameter not specified.

The following table is organized in words.  When a lower order of

detail is required, the bits under consideration are placed after a

period.  For example word 49 bit 8 would be expressed as 49.8.  If a

range of bits within a word is needed, a hyphen is placed between the

hi-lo range.  For example word 49 bits 7 to 0 is written as 49.7-0.

Word.Bit      Description

==========================

0.15      0 = reserved for non magnetic drives

0.14      1 = format speed tolerance gap required

0.13      1 = track offset option available

0.12      1 = data strobe offset option available

0.11      1 = rotational speed tolerance is > 0.5%

0.10      1 = disc transfer rate > 10 Mbit/sec

0.9       1 = disc transfer rate <= 10 Mbit/sec but > 5 Mbit/sec

0.8       1 = disc transfer rate <= 5 Mbit/sec

0.7       1 = removable cartridge drive

0.6       1 = fixed drive

0.5       1 = spindle motor control option implemented

0.4       1 = head switch time > 15 usec

0.3       1 = not MFM encoded

0.2       1 = soft sectored

0.1       1 = hard sectored

0.0       0 = ATA reserved (should be zero)

1         Number of user-addressable cylinders in the default

translation mode.  When the formatted capacity of the drive

exceeds 528 MB this field is set to 1024.

2         ATA reserved (should be zero)

3         Number of user-addressable heads in the default translation

mode.  When the formatted capacity of the drive exceeds 528

MB this field is set to 16.

4         Number of unformatted bytes per track

5         Number of unformatted bytes per sector

6         Number of user addressable sector in the default translation

mode.  When the formatted capacity of the drive exceeds 528

MB this field is set to 63.

7-9       ATA reserved (should be zero)

10-19  *  Serial number (20 ASCII characters and padded with spaces).

20     *  Buffer type.

0000h = not specified

0001h = a single-ported, single sector buffer which is not

capable of simultaneous data transfers to or from

the host and the disc.

0002h = a dual ported multisector buffer capable of

simultaneous data transfers to or from the host and

the disc.

0003h = a dual ported multisector buffer capable of

simultaneous transfers with a read caching

capability.

0004h through FFFFh are reserved.

21     *  Buffer size in 512-byte increments

22     *  Number of ECC bytes available on read/write long commands.

23-26  *  Firmware revision (8 ASCII characters and padded with spaces

[20h]).

27-46  *  Model number (40 ASCII characters and padded with spaces

[20h]).

47.15-8   Seagate reserved.

47.7-0    00h = Read/Write multiple commands not implemented;

xxh = Maximum number of sectors that can be transferred per

interrupt on read and write multiple commands.

48        0000h = cannot perform double-word I/O

0001h = can perform double-word I/O.  (included for

backwards compatible use)

49        Capabilities

49.15-13  ATA reserved (should be zero)

49.12     ATA reserved for advanced PIO mode support (should be one)

49.11     1 = IORDY supported (required for PIO modes > 2)

0 = IORDY may be supported

49.10     1 = IORDY can be disabled

49.9      1 = LBA mode supported

49.8      1 = DMA mode supported

49.7-0    Seagate reserved.

50        ATA reserved

51.15-8   PIO data transfer cycle timing mode

51.7-0    Seagate reserved.

52.15-8   DMA data transfer cycle timing mode

52.7-0    Seagate reserved.

53.15-2   ATA reserved

53.1      1 = the fields reported in words 64 - 70 are valid

(required for PIO modes > 2 or multiword DMA modes > 0)

0 = the fields reported in words 64 - 70 are not valid

53.0      1 = the fields reported in words 54 - 58 are valid

0 = the fields reported in words 54 - 58 are not valid

54        Number of current cylinders

55        Number of current heads

56        Number of current sectors per track

57        Current capacity in sectors HI-word

58        Current capacity in sectors LO-word

59.15-9   ATA reserved.

59.8      1 = Multiple sector setting is valid

59.7-0    xxh = Current setting for number of sectors that can be

transferred per interrupt with Read/Write multiple commands

60        Total number of user addressable sectors HI-word (LBA mode

only)

61        Total number of user addressable sectors LO-word (LBA mode

only)

62.15-8   Single word DMA transfer mode active

62.7-0    Single word DMA transfer modes supported

63.15-8   Multiword DMA transfer mode active

63.7-0    Multiword DMA transfer modes supported

64.15-8   ATA reserved.

64.7-0    Advanced PIO transfer modes supported (bit significant

field: 00000001 means PIO mode 3 supported, bits 7-1 are

reserved for future use)

65        Minimum multiword DMA transfer cycle time per word in

nanoseconds

66        Manufacturer's recommended  multiword DMA transfer cycle

time per word in nanoseconds during a single sector host

transfer while performing a multiple sector Read DMA or

Write DMA command over all locations on the media under

nominal conditions. If a host attempts to run at a faster

cycle rate by operating at a cycle time of less than this

value, the drive may be forced to negate DMARQ to prevent

data corruption.  The rate at which DMARQ is negated could

result in reduced throughput despite the faster cycle rate.

67        Minimum PIO transfer cycle time without flow control in

nanoseconds (required for PIO modes > 2)

68        Minimum PIO transfer cycle time with IORDY flow control in

nanoseconds (required for PIO modes > 2

69-70     ATA reserved for advanced PIO mode support

71-127    ATA reserved.

128-159   Seagate reserved.

160-255   ATA reserved.

Note: The definition of each of the Identify Drive bytes is subject to

change by the ATA review committee. Typically, once a definition is in

place it will not be changed.  Fields which are listed as "ATA

reserved" are the ones to watch for change.

=====Rev=Hist===============

Written with Borland Pascal v6.0 by B.Rudock, R.Dimick and R.Stacy.

Revision History:

6/94   v1.0    Public release.

=====Legal=Disclaimer=======

LICENSE AGREEMENT

Seagate provides the accompanying object code software ("Software")

and nonexclusively licenses its use on the following terms and

conditions. The Software is copyrighted by Seagate.  YOU ASSUME FULL

RESPONSIBILITY FOR THE SELECTION OF THE SOFTWARE TO ACHIEVE YOUR

INTENDED PURPOSES, FOR THE PROPER INSTALLATION AND USE.  SEAGATE DOES

NOT WARRANT THAT THE SOFTWARE WILL MEET YOUR REQUIREMENTS, THAT THE

SOFTWARE IS FIT FOR ANY PARTICULAR PURPOSE OR THAT THE USE OF THE

SOFTWARE WILL BE ERROR FREE.  SEAGATE EXPRESSLY DISCLAIMS ALL

WARRANTIES, WHETHER ORAL OR WRITTEN, EXPRESSED OR IMPLIED, INCLUDING

WITHOUT LIMITATION WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A

PARTICULAR PURPOSE.  IN NO EVENT WILL SEAGATE BE LIABLE TO YOU, YOUR

CUSTOMERS OR OTHER USERS FOR ANY INDIRECT, INCIDENTAL, CONSEQUENTIAL,

SPECIAL OR EXEMPLARY DAMAGES ARISING OUT OF OR IN CONNECTION WITH THE

USE OR INABILITY TO USE THE SOFTWARE.

End of License agreement.

=====Other==================

Available on the Seagate Tech Support BBS (408)438-8771 and many fine

and helpful BBS's around the world:

Specifications and jumper drawings for all Seagate Disc Drives and

Controllers.

Reprints of Installation Guides.

Troubleshooting essays.

FINDTYPE - Utility which displays bios drive type table and matches a

Seagate model to the best drive type.  Also prints complete

specifications lists and much more!

FINDINIT - Utility for Seagate controllers and host adapters that have

onboard bios. Queries the system to determine bios memory address and

initiates low-level format.

DESK REFERENCE - Hypertext data system for all Seagate products,

troubleshooting, other OEM phone numbers and much, much more. A must

for dealers who do a fair amount of support for Seagate products.

SEAGATE FORMAT - A lo-level and mid-level formatting utility for AT

systems (286+) which supports MFM, RLL, ESDI and ATA interface drives.

-=EOF: FIND-ATA.DOC=-