================================================================================

=                                                                              =

=                    8000 SERIES IDE ATAPI CD-ROM DRIVE                        =

=                            INSTALLATION GUIDE                                =

=                                  UNDER                                       =

=                              OS/2 (2.xx/3)                                   =

=                                                                              =

=                               (JAN-23-95)                                    =

=                                                                              =

================================================================================

============================================================

N.B.  - All 8000 SERIES IDE ATAPI CD-ROM are using the same

driver - OSIDEA01.FLT.

(ver 1.01)

============================================================

CONTENTS:

---------

PART 1: FIRST TIME OS/2 (2.xx/3) INSTALLATION FROM 8000 SERIES IDE ATAPI CD-ROM

DRIVES

I.  PROCEDURAL STEPS

II. FURTHER CHECKING

PART 2: INSTALLATION OF 8000 SERIES IDE ATAPI CD-ROM DRIVES AFTER OS/2 (2.xx/3)

IS INSTALLED

I.  PROCEDURAL STEPS

II. FURTHER CHECKING

PART 1: FIRST TIME OS/2 (2.xx/3) INSTALLATION FROM 8000 SERIES IDE ATAPI CD-ROM

-------------------------------------------------------------------------------

DRIVES

------

Key:

DISK 1 = "Diskette 1" if OS/2 (2.1/2.11) package is used.

= "Diskette for CD-ROM" if OS/2 3 WARP package is

used.

DISK 2 = A brand new 3.5inch-1.44MB formatted diskette.

I. PROCEDURAL STEPS

-------------------

1. Duplicate "DISK 1" into a newly formatted diskette and label

it "DISK 2";

2. Copy the 8000 series CD-ROM driver file (OSIDEA01.FLT) into

"DISK 2";

3. Add the statement below into the CONFIG.SYS file in "DISK 2":

BASEDEV=OSIDEA01.FLT

4. Switch off the PC power, connect up 8000 series IDE CD-ROM drive.

5. Boot up the PC up using the OS/2 (2.xx/3) installation booting

diskette. When the installation program requests for "DISK 1",

insert "DISK 2" instead. Then put in the OS/2 Installation CD-ROM

into 8000 series CD-ROM drive and press ENTER to continue to proceed

with OS/2 installation following the standard installation

instructions.

6. After OS/2 (2.xx/3) is fully installed, copy the OSIDEA01.FLT

into C:\OS2 directory (assuming that C drive is OS/2 booting

drive). For OS/2 3 WARP, the correct directory should be

C:\OS2\BOOT.

II. FURTHER CHECKING

--------------------

Make sure:-

1. The following files exist in the directory C:\OS2 for

OS/2 2.xx/3 (assuming that C drive is OS/2 booting drive):

CDFS.IFS

OS2CDROM.DMD

OSIDEA01.FLT

For OS/2 3 WARP, the correct directory should be C:\OS2\BOOT.

2. The following statements exist in C:\ (assuming that C drive is

OS/2 booting drive):

IFS=CDFS.IFS /Q

BASEDEV=OS2CDROM.DMD /Q

BASEDEV=OSIDEA01.FLT

For OS/2 3 WARP, the correct statements should be:

IFS=C:\OS2\BOOT\CDFS.IFS /Q

DEVICE=C:\OS2\BOOT\OS2CDROM.DMD /Q

BASEDEV=OSIDEA01.FLT

PART 2: INSTALLATION OF 8000 SERIES IDE ATAPI CD-ROM DRIVES AFTER OS/2 (2.xx/3)

-------------------------------------------------------------------------------

IS INSTALLED

------------

=============================================================

NOTE. - This guide is only suitable for users who intend to

install 8001 IDE/8222 IDE/8322 IDE ATAPI CD-ROM drive

_AFTER_ OS/2 (2.xx/3) is installed.

=============================================================

I. PROCEDURAL STEPS

-------------------

1. Insert "CD-ROM Installation Diskette" included in the package

into floppy drive A or B;

2. First, invoke OS/2's SYSTEM FOLDER, followed by SYSTEM SETUP

FOLDER, then execute DEVICE DRIVER INSTALL program to run driver

installation;

3. After the DEVICE DRIVER INSTALATION dialog appears, ensure that

the SOURCE DIRECTORY field is correct. i.e. the  SOURCE DIRECTORY

should correspond to where the Installation Diskette is (drive A

or B) with appropriate directory stated;

4. When SELECT DEVICE DRIVERS dialog appears, select "8001 IDE/

8222 IDE/8322 IDE ATAPI CD-ROM Device Driver" by single clicking

the field. Then click OK button, and then INSTALL button to install.

II. FURTHER CHECKING

--------------------

Please refer to PART 1, Section II's FURTHER CHECKING.

=============================== END OF DOCUMENT ================================