-------------------------------------------------------------------------

Description of CHINON05.SYS in CONFIG.SYS

-------------------------------------------------------------------------

CHINON05.SYS is the CD-ROM device driver used with the Chinon host

adapter cards.  It must be included in a DEVICE statement within

the MS-DOS CONFIG.SYS file:

DEVICE=\<path>\CHINON05.SYS /D:<device_name> /N:<drives> /A:<port_address>

<path>         : the pathname of the directory containing

the CHINON05.SYS file

<device_name>  : the name of device (up to eight characters)

<drives>       : the number of CD-ROM drives connected

<port_address> : I/O base address of the host adapter, described

in hexadecimal notation.

If /A:<port_address> is omitted, the port address will

be set to 300 (the factory default value) automatically.

Notes:

When installing more than one CD-ROM drive in your system, each drive

must have a different drive ID.

Remember that after changing CONFIG.SYS or AUTOEXEC.BAT, you

must reboot your system (by pressing Ctrl+Alt+Del) for these

changes to take affect.

-------------- Copyright(C) 1990-1992 CHINON Industries Inc. -----------