DigiWARE Release Notes   93000098N

for

DigiCHANNEL PC/X*, COM/Xi, MC/Xi, Xem, C/X Universal OS/2 Device Driver

XALL.SYS  V1.4.6

Microsoft OS/2          1.0, 1.1, 1.2, 1.21, 1.3

IBM OS/2                1.0, 1.1, 1.2, 1.3, 2.0, 2.1

IBM OS/2 Ext. Edition   1.1, 1.2, 1.3, 2.0

Diskettes               P/N's 40000593M (5) or 40000594M (3)

Diskettes               P/N's 40000943  (5) or 40000908  (3)

Software Manual         P/N 92000100C

3/2/94

Below are Enhancements and Bug fixes made to XALL.SYS V 1.4.5

resulting in the new XALL driver V 1.4.6

1) Enhancements

- Support has been added to the driver for the PC/8em and

PC/8emp modules.

- A command line switch, "/d:m" has been added to display

debug information during driver load.

- A command line switch, "/t:u" has been added to force the

computer type to MicroChannel. On some PS/2's, ABIOS calls

fail to identify the computer correctly and thus the driver

fails to initialize at boot time. Use of this switch eliminates

the problem.

- Adjusted flow control buffer levels for better performance.

- Added New Ioctl to block a thread until a modem or break event

occurs. See XALL.DOC

- Added a new user diagnostic program to the release diskette,

DGOS2UD.EXE. See DGUDREAD.ME

2) Bug Fixes

- A new XXFEP.BIN file fixes DSR reporting problems on the PC/2e

and MC/2e boards.

- Fixed SETBREAKON (fct 4Bh) & SETBREAKOFF (fct 45h) IOCTL

functions to work across entire product line.

- Fixed TX IMMEDIATE (fct 44h) IOCTL function.

- Fixed COM STATUS (fct 64h) function which would return erroneous

values.

- Fixed QUERY COM EVENT (fct 72h) bug that reported ring events on

both rising and trailing edges. Events should be reported only

on the trailing edge.

- Failed to check OS/2 PHYS_2_VIRT kernel calls for failure

returns and could thus use invalid addresses.

- Fixed bug in DevOpen that incorrectly set the Transmit Low Water

Mark, causing DosWrites to return immediately instead of waiting

for a timeout period to transpire.

- Wait longer in Close to let the transmitter to drain. Flushing

buffers too soon caused loss of print data to occured.

- Fixed SETDCBINFO (fct 53h) bug which could cause DTR would go

high when RTS Flow was enabled.

- Fixed COM/Xi bug on port 8. The driver thought it's rx buffer

was 8k longer than it really was.

- Fixed COM/Xi bug on 4 port cards; buffers exceeded 16k and thus

prevented the driver from reading all received data.

- Set default timeouts to 1 Second instead of 1 Min. There is

an error in the OS/2 specs.

3) Known Limitations

- Maximum 256 channels.

- The COM/Xi is limited to 9600 or at most 19200 baud.  All other

intelligent cards will support 38.4K baud on all channels, and

57K baud on a few channels.

4) Manual Errata