AutoBOOST

User's Manual

Version 5.0

Introduction

Congratulations! You	have purchased	one of the most powerful productivity

tools available  for AutoCAD	users.	In very little time you will be ready

for a new level of productivity.

AutoBOOST is	a set  of high	performance, full  feature real and protected

mode drivers	specifically designed  for AutoCAD  R10, AutoCAD  386 R10 and

R11.	 AutoBOOST is a proven product with more than 4,000 users world wide.

In  addition,  the  new  256-color  driver  will  also  support  AutoShade,

RenderMan, and 3D Studio.

How this manual is organized

This manual  is organized  in three  parts: Protected  Mode Installation  &

Configuration, Real  Mode Installation & Configuration, and Using AutoBOOST

Commands.

At the  end of the manual there are two appendices which will help you with

the configuration.   Appendix  A  contains  a  summary  of  menus  used  by

CONFIG.EXE  utility,	 and  Appendix	B  explains  how  to  configure  with

AutoSHADE, and 3D Studio.

I.  Protected Mode Installation and Configuration

Before using AutoBOOST display list drivers, three steps must be performed.

First the  relevant programs	must be  moved to  your hard  drive, then the

proper program  must be  configured, and  finally AutoCAD  (AutoShade or 3D

Studio) must be configured for operation with an ADI driver.

What's on the diskette

The diskette accompanying this manual contains programs and document files.

AutoBOOST has  been  organized  into	several  programs  according  to  the

capabilities of  your  graphics  board,  and	drawing  requirements.	  The

following programs are on the standard distribution diskette:

BOOST16.EXP	 16-color program for VGA

BOOST256.EXP	 256-color program for VGA

BOOSTxxxx.EXP	 Programs for other display boards

The following utility files are also included:

CONFIG.EXE 	 Configuration program for AutoBOOST.

README.DOC 	 Latest information on the programs

For a  complete list	of programs  provided on your diskette please see the

README.DOC file.

Copying AutoBOOST to your disk

The installation process consists of copying the desired files to your hard

disk. Only the executable files *.EXP,and the configuration utility CONFIG.EXE

are needed for the driver operation.  These files can reside in any directory

on your hard disk.  For example to copy all files from \GENOA\AUTOCAD to

the directory BOOST on hard drive C:, the following commands can be used:

C:

MD \BOOST

CD \BOOST

COPY \GENOA\AUTOCAD\*.EXP

COPY \GENOA\AUTOCAD\*.EXE

Configuring AutoBOOST

Configuration of AutoBOOST is a simple process using the CONFIG.EXE utility

program.   This program  must be  run before AutoBOOST can be used.  To run

the program type the following:

CONFIG

The program  will then  present you with a list of driver files and a brief

description of each driver available to you.	Simply use your arrow keys to

highlight the desired driver and press the <ENTER> key.  The driver file is

configured according	to selections made in subsequent menus.  At any point

you may press the <F1> key for help.

Software Registration Menu

This menu will be displayed the first time you configure a driver, the menu

will prompt  you for	your company  name.  Once you accept the name, it can

not be changed and will appear as part of all AutoBOOST identifications.

Main Configuration Menu

This menu  allows you to access all configurable options of AutoBOOST.  You

can access  an option  by using the cursor keys to highlight the menu title

and then  pressing the <ENTER> key to activate the selected menu.  When you

are finished	with all  your configuration changes, highlight the "Save New

Configuration &  Exit"  option and press the <ENTER> key.  The <F1> key can

be used for on-line help.  For more information on these menus see Appendix

A.

Configuring AutoCAD

In order for AutoCAD to be able to properly recognize the AutoBOOST display

driver, the  environmental variable DSPADI needs to be set to the drive and

directory where the display driver resides.  It is very important that this

setting is  done correctly or AutoCAD 386 will not recognize AutoBOOST.  It

is recommended  that the  command similar to the following is placed in the

AUTOEXEC.BAT file:

SET DSPADI=C:\BOOST\BOOST16.EXP

Note that  there are	no spaces  around the  equal sign.  The above command

assumes that	you have  already configured the 16-color driver BOOST16.EXP,

and that the driver has been copied into C:\BOOST directory.

The AutoCAD  386 program itself must also be properly configured to use the

P386 ADI   v4.0/4.1  video display.	This  is done  by using the following

steps:

Invoke AutoCAD

Select option (5), "Configure AutoCAD" from the "Main Menu".

Select option (3), "Configure video display"

Select option (1), "P386 ADI v4.0/4.1"

Select option (0), "Return to Main Menu"

Press "Y" to save the new configuration

You are now ready to use the AutoBOOST driver.

Trouble shooting

AutoCAD will not enter graphics mode

The following  is the  one most  commonly encountered  message  during  the

installation process:

"Please Correct Display Adapter / Configuration Inconsistency"

This message	usually means  that AutoBOOST  did  not  load  or  initialize

properly.   Please verify  that in  your AUTOEXEC.BAT file that "DSPADI" is

set to  the directory  path where AutoBOOST resides, and that the AutoBOOST

driver has been properly configured with the CONFIG.EXE utility program.

Excessive hard disk activity

When the  display-list gets large, the DOS-Extender will start paging least

used memory  to disk, causing a performance decrease.  If this occurs often

you should add more memory to your system.

Technical support

If you are experiencing problems please contact AutoBOOST technical support

at:

Graphics Software Labs, P.O. Box 6247, Huntington Beach, CA 92615, USA

(714) 960-6361

II.  Real Mode Installation and Configuration

What's on the diskette

The real  mode drivers  are easily  distinguished from  the protected  mode

driver by the extension *.EXE.  The following programs are available:

BOOST16.EXE	 16-color program for VGA

BOOST256.EXE	 256-color program for VGA

BOOSTxxxx.EXP	 Programs for other display boards

CONFIG.EXE 	 Configuration utility

BOOST.DOC		 Latest information on the programs

Copying AutoBOOST to your disk

The installation  process consists  of copying  desired files  to your hard

disk.   Only the executable files BOOST*.EXE, and the configuration utility

CONFIG.EXE are  needed for the driver operation.  For example the following

commands can be used to install the AutoBOOST:

MD BOOST

COPY  \GENOA\AUTOCAD\*.EXE  C:\BOOST\*.*

Configuring Real Mode AutoBOOST

The configuration  of AutoBOOST  is a very easy process with the CONFIG.EXE

utility program.   The  steps are  similar to those used for protected mode

versions of  the driver  (see section  titled Configuring AutoBOOST, at the

start  of   this  manual),  except  that  several  additional  options  are

available.

Interrupt Vector (in hex)

Real mode version of AutoBOOST is an AutoCAD ADI real mode driver using ADI

4.0 interface  via a	software interrupt.   By  default AutoBOOST  utilizes

interrupt 7Ah.

Display list drive

AutoBOOST uses a display-list to store the description of a drawing sent by

AutoCAD to  the driver.   This  display-list is  stored by  the driver in a

file.  If you are using multiple viewports, there will be multiple display-

list files  in use.	AutoBOOST  allows to  specify which  drive is used to

store the  display list  files.   For best performance it is recommended to

use a RAM based drive (such as one used by a VDISK).

Pop-up save area drive

A virtual  disk option  is also  available for the pop-up menu data storage

file.   The drive  specified in this option will be used to save the region

under the pop-up menus.

Changing CONFIG.SYS File

Since AutoBOOST  may use several files to store its display lists, you will

need to  make sure  that both FILES and BUFFERS variables in our CONFIG.SYS

file are set to values over 30. For example:

FILES=30

BUFFERS=30

Running AutoBOOST

Removing Existing ADI Drivers

AutoBOOST is	your new  ADI driver.  If you were previously running AutoCAD

with a different ADI driver, you must remove the old driver.	These drivers

are often  loaded from  the AUTOEXEC.BAT  file or from a batch file used to

launch AutoCAD.   It	is recommended	that after  you remove	all other ADI

display drivers  from your  *.BAT files  you also  reboot the system before

AutoBOOST is loaded.

Loading AutoBOOST

You must  load AutoBOOST  before you run AutoCAD.  To load AutoBOOST, go to

the directory  where AutoBOOST  was installed and type BOOST16 (or the name

of the  driver you  have chosen).   You  should then	see a  sign-on screen

showing the  license	number,  the  distributor,  and  your  company	name.

Pressing any key will complete the loading of the driver.

If instead  of the  sign-on screen you see a blank or distorted screen, you

may have  selected the  wrong graphics  card during  your configuration  of

AutoBOOST.  If, after reviewing your AutoBOOST configuration, you are still

having difficulties, call our technical support line for assistance.

If you  see a  message from  AutoBOOST saying that it is already in memory,

you may have to unload AutoBOOST (see section titled Unloading AutoBOOST).

If you  see a  statement that says interrupt vector for which AutoBOOST has

been configured  is in use, this means that you may still be loading an old

ADI driver,	in  which case	you should unload the old driver (see section

Removing Existing  ADI drivers).   If you are not loading any other display

drivers by  accident, then you may have another software product using that

interrupt vector.   To  solve this problem you should reconfigure AutoBOOST

and AutoCAD  to use  a different  interrupt vector  (see section  AutoBOOST

Interrupt Vector, and section Configuring AutoCAD).

No License Screen Pause /N

The sign-on  screen and the wait for a key during the AutoBOOST loading can

be disabled  by using the /N command line option.  This can be helpful when

a batch  file is  used to  load AutoBOOST.  You should use this option only

after you  are sure  that AutoBOOST can load successfully with your current

graphics card setting.  To load AutoBOOST without pause type:

BOOST16 /N

Unloading AutoBOOST /U

The command  line option  /U is  used to  unload AutoBOOST from memory.  To

unload AutoBOOST,  go to  the directory that contains the AutoBOOST program

and type:

BOOST16 /U

Configuring AutoCAD

To use  AutoBOOST, AutoCAD must be configured for ADI v4.0 display. This is

done from AutoCAD main menu as follows:

Select option 5 - Configure AutoCAD in Main menu

Select option 3 - Configure video display

Select ADI v4.0

Select interrupt vector (default is 7A).

Using a VDISK or RAMDRIVE

For increased  performance, we  suggest that you set up a VDISK to hold the

display-list.  This will result in faster drawing speed.

The size  of VDISK  to use  will depend on the maximum size of the display-

list which depends on the type of entities in your drawings. On average the

display-list should  be 60%  of the  drawing size. However, drawings with a

large number	of circles,  arcs, and	text will produce display-list larger

than average.   To  determine the  size of VDISK, we suggest you experiment

with a  variety  of  drawings  to  see  their  requirements.	  Should  you

underestimate the  display-list size,  you will  get a "Disk Full" message,

the display list will then be disabled, and you will be returned to regular

AutoCAD speed.   If needed, you can increase your VDISK size or reconfigure

AutoBOOST to use the hard disk for its display-list storage.

Trouble Shooting

Memory Messages

As AutoCAD comes up, you may see the following messages:

Autolisp disabled

or

AutoCAD needs xxxx K of memory to use the last

yyyy K of extended memory ...

This probably means that there are too many resident (TSR) programs loaded.

You may  be able  to remedy the problem by a)  Checking your CONFIG.SYS and

AUTOEXEC.BAT files and removing those TSRs that are not needed for AutoCAD;

or b)   If  you have	a 386  system, you may be able to load some TSRs into

High Memory  with a  memory manager  like QEMM or  386MAX. or c)  You could

try to  reduce ACADFREERAM  to 20,  and set LISPHEAP and LISPSTACK to lower

values.

Display Adapter Inconsistency message

If AutoCAD displays the following message:

Please Correct Display Adapter / Configuration Inconsistency

then AutoBOOST  did not initialize properly. This problem occurs when FILES

or BUFFERS values are too low, or when the disk drive you selected to store

display-list does  not exist.    Please  verify  CONFIG.SYS  and  AutoBOOST

configuration.

No ADI Driver

When you  attempt to	bring up  an AutoCAD drawing, you may get the message

that there  is no  ADI driver loaded at a specified interrupt vector.  This

means that  AutoBOOST may  not have  been loaded.   If  AutoBOOST has  been

loaded  successfully,  then  the  ADI  interrupt  vector  for  AutoCAD  and

AutoBOOST do not agree.  You should configure both AutoCAD and AutoBOOST to

use the  same interrupt  vector  (see  sections  Configuring	AutoCAD,  and

AutoBOOST Interrupt Vector).

AutoBOOST Logo missing from the top of Menu

If AutoBOOST	is loaded  properly, you  will see  the AutoBOOST logo in the

upper-right hand corner of the screen, and the display-list size underneath

it.	You can  use the this size to approximate the amount of memory needed

by VDISK to store the  display-list.

When a  drawing appears to load OK, but AutoCAD does not seem to understand

any AutoBOOST  commands, and	there is no AutoBOOST Logo in the upper right

hand corner,	then AutoCAD  was not configured for AutoBOOST.  In this case

you should  reconfigure AutoCAD  (see section  Configuring  AutoCAD).    If

AutoCAD is  configured for  an ADI  driver, then check to make sure that an

old ADI  driver is  not being  used  (see  section  Removing	Existing  ADI

drivers).

AutoBOOST Loaded But Commands Don't Work.

If AutoBOOST	logo appears  in the  upper right  hand corner	but AutoBOOST

commands are not recognized, you may be redefining AutoCAD commands with an

old BOOST.LSP program used in previous releases.  This program is no longer

necessary (use  the CONFIG.EXE  utility instead).  Remove the re-definition

and try again.

III. Using AutoBOOST Commands

Introduction

The base  version of	AutoBOOST, version  without the  optional Birdseye or

EasyPIK commands,  contains only  the display list clean-up command CD, and

anti-alisasing related  commands.   The standard  REDRAW command  has  been

modified to  automatically perform  a display-list  clean-up whenever there

are erased  vectors in  the display-list  (see  AutoBOOST  command  CD  for

further details).

All AutoBOOST commands can be entered from the keyboard or assigned to your

tablet or buttons for quicker access.

All command names can be changed through the CONFIG.EXE program.

All AutoBOOST  commands can  be made transparent to any AutoCAD commands by

preceding it with a quote character <'>.

The <ENTER> key will repeat the last AutoBOOST command entered, except when

you are in the middle of an AutoCAD command.

Command Summary

CD or 'CD (Clean Display-list)

How Editing Modifies the Display-list

_____________________________________

Changes in  the drawing  which cause	graphics to  be undrawn and drawn are

captured in  the display-list and appended in the order they are made.  For

example, suppose  an object  in the  drawing is  moved from one location to

another.   On completion  of selection,  the active graphics are dragged to

the new  location and  their new  position is fixed.	AutoCAD will draw the

graphics in  the background  color at their initial position, then draw the

graphics in  their proper  color  in	their  final  position.    Both  draw

sequences cause  graphics to	be added  to the  AutoBOOST display-list, and

both can  be observed when an AutoBOOST viewing command is used to draw the

modified display-list;  the object  which was  moved will  be drawn in it's

proper color at it's original position, then be undrawn, (i.e. drawn in the

background color)  at it's  original position,  then drawn  in it's  proper

color at the new position.

Thus, the  appearance of  the AutoBOOST  display-list captures a "window of

history" of  the drawing  through several incremental edits.  The window is

"replayed" every  time an  AutoBOOST command  is used  to view the display-

list.

Cleaning the Display-List

_________________________

Because the  undrawing of  graphics still causes data to be appended to the

display-list, indefinite editing  activity will fill up the available space

maintained by AutoBOOST.

To purge  the display-list  of unneeded editing history, you can use the CD

command or  invoke an  AutoCAD REDRAW.    This  creates  a  completely  new

display-list which  exactly represents  the current  state  of  the  active

drawing, without any redundant undrawn objects.

AP (Anti-Aliasing On)

Removes the  "jaggies" from  your diagonal  lines, circles,  and  polygons.

Performs anti-aliasing  on all  lines  over  any  background,  and  on  all

polygons.

Anti-aliasing is not supported by all boards.

AO (Anti-Aliasing On)

Removes the  "jaggies" from you diagonal lines and circles.  Performs anti-

aliasing only  on lines  over black background.  This method is much faster

than the one used for AP command, but is limited only to line drawings.

AF (Anti-Aliasing Off)

Disables anti-aliasing,  all drawing	is done  in  a	conventional  manner.

Cancels both AP and AO commands

Appendix A   CONFIG.EXE Summary

In this  section is  a description  of the  menus used in the configuration

utility CONFIG.EXE.

Set Board Type and Resolution

This option allows you to tell AutoBOOST which graphics card you are using,

and at  what resolution.   The  current graphics option is indicated by the

highlighted line.   To  select a  different card,  or resolution,  use  the

cursor keys to highlight the desired line and then press the <ENTER> key.

If you don't see a listing of the card that you have, you can try a generic

version or a similar card type.  Sometimes different manufacturers will use

the same graphics chip sets.	If you are still having trouble, after trying

several configurations,  call for  technical assistance  (please see	cover

sheet for our phone number and address).

Set Screen, Interrupt, Disk..

BirdsEye Box Position.

Not used in versions without Birdseye.

BirdsEye Size

Not used in versions without Birdseye.

Button To Activate BirdsEye

Not used in versions without Birdseye.

Digitizer Tablet Mapping

Not used in versions without Birdseye.

Interrupt Vector (in hex)

Not used  in the  protected mode  driver.   See section  titled  Real  Mode

Installation & Configuration for more details on use in real mode.

Display list drive

Not used  in the  protected mode  driver.   See section  titled  Real  Mode

Installation & Configuration for more details on use in real mode.

Text Character Box Size

AutoBOOST can display up to three different size text fonts in your AutoCAD

banner and  command areas.   Based  on your  graphics card  resolution  and

graphics area  needs, you  may choose which size text font AutoBOOST should

use.	This menu will display possible choices and list the current selected

font size.  Use arrow keys to toggle choices, press <ENTER> key to choose.

Scroll Area Height

When running	AutoCAD, in single screen mode, the command area is below the

graphics region  and is  set to  a scroll height of three lines.  This menu

allows you  to tell  AutoBOOST to either increase scroll area for a greater

command history  or to  decrease it in order to provide more graphics area.

Use arrow keys to toggle choices, press <ENTER> key to choose.

Flip Screen Save

AutoBOOST attempts to save the graphics area when you flip to text mode and

to restore the information when you return to graphics mode.	Some graphics

cards may  corrupt the  portion of  the image that AutoBOOST tried to save.

This menu  gives you	the opportunity  to have  the graphics region redrawn

after flip  screens, should  you need  to do	so.  Use arrow keys to toggle

choices, press <ENTER> key to choose.

Dual Screen Enable/Disable

If your  computer system has both a monochrome (MDA) and a VGA/EGA display,

AutoBOOST will  automatically go  into dual  screen mode  if AutoCAD is run

from the  monochrome screen.	 This  menu provides  you with	the option of

disabling dual  screen mode  if you  desire to  do so.   Use	arrow keys to

toggle choices, press <ENTER> key to choose.

Layer Indicator color

The "LAYER"  indicator in  the upper left corner of the graphics screen can

be displayed	in either  the default	text color  or in the current drawing

color.   This option	gives you  the ability to choose which mode you would

like to  use.   Use arrow  keys to  toggle choices,  press <ENTER>  key  to

choose.

Set Text and Menu Colors

This option  presents you with a list of display items whose colors you can

modify by  highlighting the  desired option  and using  the arrow  keys  to

toggle color	choices.  For example, you can change the graphics background

color between light grey and black.  You can also change the background and

foreground colors  of text,  alert boxes, the menu bar, pull-downs, and the

dialogue box.

If the  colors on the graphics screen do not appear to be the colors chosen

from the CONFIG.EXE file check your monitor connections.

Set Custom Command Names

This option  allows you  to change  the names   of  the AutoBOOST  commands

should they conflict with any of your other customized operations.  This is

done by using the cursor keys to highlight the command whose name you would

like to  change, typing in the new name, and then pressing the <ENTER> key.

After entering  the change,  the command name will be updated.  The default

command names are:

Description		 Default Command Name

Clean Display List	      CD

Anti-alias lines	      AO

Anti-alias polygons	      AP

Disable anti-aliasing       AF

Set Command Names Sent to Foreign AutoCAD

AutoBOOST performs  several functions  by passing  an AutoCAD  command name

string back  to AutoCAD.  Since versions of AutoCAD for different languages

may use  different names  to perform these commands, it may be necessary to

inform AutoBOOST  of the  new names.	 This menu item is used to define the

new names.  Currently the following three commands are used by AutoBOOST:

ZOOM

ZOOM PREVIOUS

REDRAW

Appendix B - Configuring AutoShade and 3D Studio

Configuring Autoshade and RenderMan

First set  the environment variables using the DOS SET command to where the

AutoBOOST driver  resides.   For example  if	AutoBOOST  is  in  drive  C:,

directory BOOST, then the following commands would be used:

SET DSPADI=C:\BOOST\BOOST256.EXP

SET RCPADI=C:\BOOST\BOOST256.EXP

SET RDPADI=C:\BOOST\BOOST256.EXP

Next, start AutoShade with  the /R command line option:

SHADE /R

AutoShade program  will then	ask several questions about the devices being

used.  The following are related to the AutoBOOST driver:

Select Display  Device:   Select the	P386 Display Device Interface display

driver.

Select Rendering  Display Driver:  Select the P386 Display Device Interface

rendering driver

Do the  display and  AutoShade rendering  devices share  a  single  screen?

Answer Y.

Do the display and RenderMan devices share a single screen:

Answer Y.

Does flip screen require redraw? Answer Y.

The rest  of the  questions are  answered according to type of the pointing

and hardcopy devices being used.

It should be noted that only the 256-color, and the 32k-color, drivers work

with AutoShade and RenderMan.  The 16-color drivers do not.

Configuring 3D Studio

Configuration

Configuration of 3D Studio is done in three steps.

First, delete the configuration file 3DADI.CFG.

Second, edit	the file  3DS.SET, so  that device  name after	the  DEFAULT-

DISPLAY and MAIN DISPLAY definitions are set to the following:

DEFAULT-DISPLAY = "rcpadi"

...

MAIN DISPLAY = RCPADI

Note that  in the  first case  there are quotes around the name rcpadi, but

not in the second.

In the third step, set the environment variable RCPADI, using the DOS SET

command, to where the AutoBOOST driver resides. For example, if AutoBOOST is

in drive C:, directory BOOST, then the following command would be used:

SET RCPADI=C:\BOOST\BOOST256.EXP

It should be noted that only the 256-color, and the 32k-color, drivers work

with 3D Studio.  The 16-color drivers do not.

Known problems

On some boards, e.g. Everex Viewpoint, it is often difficult to return from

rendering to	the display  screen.   In such	cases the  line, in  the file

3DS.SET, containing MAIN-DISPLAY should be deleted or commented out.

3D Studio  often starts  excessive disk  activity and  often crashes	after

rendering several  times in a row.  You need to exit 3D Studio periodically

to release allocated memory.