****************************************************************************

NCR VGA Drivers

SoftSet.Exe and SoftMode.Sys

Versions 2.xI and 2.xP

Copyright (c) NCR Corporation, 1992

****************************************************************************

Contents:

---------

- Why Should You Use SoftMode?

- Setting up SoftMode with SoftSet.exe

- Setting up SoftMode to support VESA calls

- Dot Clock Frequencies of SoftMode 2.xI and 2.xP

- Setting up 132x25, 132x50, 80x25, and 80x50 text.

- Using SoftMode.Sys without SoftSet.exe.

- CONFIG.SYS Example

- Setting a Mode

- Setting up SoftMode for 8-bit wide memory

- Problem List

Why Should You Use SoftMode?

---------------------------------------

The reasons you should use SoftMode:

- Extended Modes:

There is no support in ROM BIOS for modes beyond

1024x768 256 color (mode 62h).  SoftMode provides

the ability to set modes beyond mode 62h including

the new HiColor 32k and 64k color modes.  The NCR

Windows drivers rely on SoftMode to properly set these

extended modes.  In addition, the NCR AutoCad 11 drivers

require the use of SoftMode.

- Mode Timing Selection:

There is no way you can tell ROM BIOS which timing to use

for a given mode number.  SoftMode supports a variety of

timing sets for certain modes.  This enables you to select

the timing which best suits your monitor.

- Image Centering:

There is no way you can center the video image using

BIOS.  By running SoftSet to initially select the mode

timings for SoftMode, you can also center each mode for

your monitor.  This way you won't need to touch the monitor

each time a different mode is set by applications.

- VESA BIOS Extensions:

There are no VESA extensions built into the ROM BIOS.

SoftMode supports VESA version 1.2 BIOS extensions.

Setting up SoftMode with SoftSet.exe

------------------------------------

It is recommended that you run SOFTSET to setup SoftMode.

SoftSet.exe provides the user with an easy way to setup

SoftMode.Sys.  To Run SoftSet make sure that both files,

softmode.sys and softset.exe, are in the same directory.

Go to that directory and type SOFTSET <return>.

Softset allows you to test and observe each of the mode settings

provided in SoftMode.  You may then "Use" or "Not Use" each setting.

All of the used settings are assembled in a list which is added

to a command line in the CONFIG.SYS file.  Users may edit the

CONFIG.SYS file themselves or allow SoftSet to add the command

line automatically.

Setting up SoftMode to support VESA calls

-----------------------------------------

SoftMode 1.6x supports VESA BIOS extensions.  To enable VESA

functions, include the /V switch on the command line.  This

can be done with SoftSet.exe version 1.55 or later.

Dot Clock Frequencies of SoftMode

---------------------------------

There are two versions of SoftMode.

SoftMode.sys 2.xI assumes that the dot clocks of Table 1a are selectable.

SoftMode.sys 2.xP assumes that the dot clocks of Table 1b are selectable.

Table 1a: Dot Clock Frequencies for the ICS2494-253 and ICS1394-073.

Hex Address      Frequency in Mhz

0              25.175

1              28.322

2              40.000

3              65.000

4              44.900

5              50.000

6              56.000

7              75.000

8              25.175

9              28.322

A              00.000

B              00.000

C              60.000

D              80.000

E              00.000

F              00.000

Table 1b: Dot Clock Frequencies for the AV9194-46 and ICS2494-277.

Hex Address      Frequency in Mhz

0               25.175

1               28.322

2               36.000

3               65.000

4               44.900

5               50.000

6               80.000

7               75.000

8               56.644

9               63.000

A               72.000

B              130.000

C               90.000

D              100.000

E              110.000

F              120.000

Setting up 132x25, 132x50, 80x25, and 80x50 text.

------------------------------------------------

There is a new version of SETMODE.EXE to replace the old SETMODE.COM.

The old setmode.com was a TSR which provided high resolution text.

Now, this feature is built into SoftMode.  Use SETMODE.EXE to set up

the resolution required the same way the old TSR was used.

See SETMODE.DOC for details.

Using SoftMode.Sys Without SoftSet.exe

--------------------------------------

SoftMode's purpose is to allow access to extended graphics modes

without up-grading BIOS.  In addition, the user may specify

which timings apply when a specific mode is called for by an

application. This usually depends on the monitor.

For example, if the monitor is an NEC 4d, then the user would

want this driver to allow a non-interlaced timing for the

1024x768 256 color mode. At the same time, the 1280x1024 mode

must be limited to an interlaced timing.

To accomplish the above, it is necessary to supply command

line parameters following the DEVICE=softmode.sys statement.

The following is a list of rules required to set up the parameters:

- The parameter list must be preceded by the /M: switch.

- Each parameter must be a decimal number taken from the

list of supported setting numbers of Table 2 below.

- Multiple parameters must be separated by commas.

Table 2: Graphics Modes with Frequency Info and Setting Numbers.

Frequencies:

Vertical   Horizonatal    Dot          Type           Setting Number

--------   -----------    ---          ----           --------------

12h: 640x480,16 colors

72.874  Hz  37.967 KHz  32.500 MHz  Non-Interlaced   ( Setting # 13 )

60.004  Hz  35.403 KHz  28.322 MHz  Non-Interlaced   ( Setting # 17 )

5fh: 640x480,256 colors,  VESA Mode 101

72.814  Hz  37.791 KHz  32.500 MHz  Non-Interlaced   ( Setting # 12 )

59.940  Hz  31.469 KHz  25.175 MHz  Non-Interlaced   ( Setting # 2 )

58h: 800x600,16 colors,  VESA Mode 102

71.971  Hz  48.077 KHz  50.000 MHz  Non-Interlaced   ( Setting # 8 )

60.317  Hz  37.879 KHz  40.000 MHz  Non-Interlaced   ( Setting # 0 )

56.021  Hz  35.461 KHz  40.000 MHz  Non-Interlaced   ( Setting # 15 )

5ch: 800x600,256 colors,  VESA Mode 103

71.971  Hz  48.077 KHz  50.000 MHz  Non-Interlaced   ( Setting # 10 )

60.317  Hz  37.879 KHz  40.000 MHz  Non-Interlaced   ( Setting # 4 )

56.021  Hz  35.461 KHz  40.000 MHz  Non-Interlaced   ( Setting # 16 )

5dh: 1024x768,16 colors,  VESA Mode 104

70.069  Hz  56.476 KHz  75.000 MHz  Non-Interlaced   ( Setting # 14 )

70.059  Hz  56.818 KHz  75.000 MHz  Non-Interlaced   ( Setting # 1 )

60.530  Hz  48.363 KHz  65.000 MHz  Non-Interlaced   ( Setting # 9 )

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 3 )

62h: 1024x768,256 colors,  VESA Mode 105

70.099  Hz  56.991 KHz  75.000 MHz  Non-Interlaced   ( Setting # 11 )

60.004  Hz  48.363 KHz  65.000 MHz  Non-Interlaced   ( Setting # 6 )

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 5 )

6ah: 1280x1024,256 colors,  VESA Mode 107

60.024  Hz  63.025 KHz  90.000 MHz  Non-Interlaced   ( Setting # 107 )

86.938  Hz  48.077 KHz  75.000 MHz  Interlaced       ( Setting # 7 )

5eh: 640x400,256 colors,  VESA Mode 100

60.055  Hz  31.469 KHz  28.322 MHz  Non-Interlaced   ( Setting # 20 )

6ch: 1280x1024,16 colors,  VESA Mode 106

86.938  Hz  48.077 KHz  75.000 MHz  Interlaced       ( Setting # 67 )

5ah: 1024x768,2 colors

87.064  Hz  35.522 KHz  44.900 MHz  Interlaced       ( Setting # 19 )

70h: 640x480,32k colors,  VESA Mode 110

59.974  Hz  31.486 KHz  50.000 MHz  Non-Interlaced   ( Setting # 100 )

78h: 640x480,64k colors,  VESA Mode 111

59.974  Hz  31.486 KHz  50.000 MHz  Non-Interlaced   ( Setting # 110 )

71h: 800x600,32k colors,  VESA Mode 113

59.979  Hz  38.986 KHz  80.000 MHz  Non-Interlaced   ( Setting # 102 )

56.230  Hz  36.550 KHz  75.000 MHz  Non-Interlaced   ( Setting # 101 )

79h: 800x600,64k colors,  VESA Mode 114

59.979  Hz  38.986 KHz  80.000 MHz  Non-Interlaced   ( Setting # 112 )

56.230  Hz  36.550 KHz  75.000 MHz  Non-Interlaced   ( Setting # 111 )

72h: 1024x768,32k colors,  VESA Mode 116

84.902  Hz  35.489 KHz  90.000 MHz  Interlaced       ( Setting # 108 )

75.468  Hz  31.546 KHz  80.000 MHz  Interlaced       ( Setting # 105 )

7ah: 1024x768,64k colors,  VESA Mode 117

84.902  Hz  35.489 KHz  90.000 MHz  Interlaced       ( Setting # 109 )

75.468  Hz  31.546 KHz  80.000 MHz  Interlaced       ( Setting # 106 )

50h: 640x480,16M colors,  VESA Mode 112

60.024  Hz  31.513 KHz  75.000 MHz  Non-Interlaced   ( Setting # 103 )

51h: 800x600,16M colors,  VESA Mode 115

97.777  Hz  33.733 KHz  90.000 MHz  Interlaced       ( Setting # 114 )

86.913  Hz  29.985 KHz  80.000 MHz  Interlaced       ( Setting # 113 )

The following text modes are also included in SoftMode:

setting#   mode

30      3

54     54

55     55

CONFIG.SYS Example

------------------

To tell SoftMode to enable non-interlaced timings for mode 5d,

but interlaced timings for mode 62, the user must create the following line

in his or her config.sys file:

DEVICE = path\SOFTMODE.SYS /M:1,5

where path is the path to the directory location of the Softmode.sys file.

After modifying the config.sys file, the system must be re-booted.

Upon loading at boot time, SoftMode will respond by displaying:

Modes and settings enabled:

5d.....1

62.....5

Note that the above example produces the following results:

- Modes 5d and 62 will be set by SoftMode.sys.

- Any standard modes (57 hex or below) will be set by BIOS.

- Any modes above 57 hex, other than 5d and 62, will not be set.

If the setting number 30 was specified with the /M:30 switch, then

mode 3 would be set by SoftMode and not by BIOS.

If no parameters are given, SoftMode will not be loaded and an error

message will appear on the screen.

Setting a Mode

--------------

Once the system is re-booted and SoftMode is loaded successfully, the

video mode will be, as always, mode 3 set by BIOS. To activate one of the

modes that you have enabled in SoftMode, you can use the SETVID.EXE

utility as follows:

In the above example, modes 5d and 62 are the only possible mode choices

for SoftMode to set.

To activate mode 5d, simply type:

SETVID 5d  <return>

Then, mode 5d will be selected using the timings of number one, i.e. non-

interlaced 70Hz timings.

To return to mode 3 set by BIOS, type SETVID 3  <return>.

Setting up SoftMode for 8-bit wide memory

-----------------------------------------

A conflict with some adapter cards may require the VGA to

use 8-bit wide memory.  To force the VGA into 8-bit mode, include

the /8 switch on the command line.

The Softset program may not allow the user to add the /8

switch automatically.  Use an editor to add the /8 switch after

using SoftSet.

Problem List

------------

- SoftMode.Sys does not currently support any functions other

than the setting of a mode.  Therefore, all other INT 10 calls

are routed to BIOS.  If the user has BIOS which

lacks, for example, the write tty function for the mode

selected,  the screen may not be legible when the mode is

set with SETVID.  The mode is, however, useful when set

by an application driver, such as a Windows 3.0 driver.