This disk contains the Beta Version (0.92) DynaLink program for installing the

flicker free parameters for your ProDesigner/e and ProDesigner II.  The

DynaLink can also be used to center the screen in high resolution modes to

tailor for your monitor.

To install the Dynalink on your hard disk, copy the necessary files from the

floppy diskette to the hard disk.  Use the DOS COPY command to copy the

following files:

STD_MODE.DAT

DynaLink.COM

If you have a ProDesigner/e with Bios Version 1.x, copy the following file

from the floppy drive to your Hard drive:

EXT_MOD1.DAT

For the ProDesigner/e with BIOS Version 2.x, copy the following file from the

floppy drive to your Hard drive:

EXT_MOD2.DAT

If you have a ProDesigner II with Bios Version 3.x, copy the following file

from drive A to your Hard drive:

EXT_MOD3.DAT

For the ProDesigner II with BIOS Version 4.x, copy the following file from

drive A to your Hard drive:

EXT_MOD4.DAT

NOTE: The BIOS Version is displayed on the ORCHID LOGO boot-up message.

You can then install the DynaLink by typing the command:

DYNALINK -i <drive>

Please refer to section below for details explanation of the parameters.

REQUIREMENT:

1. For ProDesigner II you will need the BIOS version 3.1 or above.

2. To run the flicker free 800x600 in 70Hz with the ProDesigner/e, you must

have 512k of video card memory.

DynaLink Screen Parameters:

----------------------------------

DynaLink.Com can be called with these parameters:

-i <drive> : Install the DynaLink.Com and modify the autoexec.bat file in

the specified drive.

-u <drive> : Uninstall DynaLink.Com from the Autoexec.bat file

(automatically delete the "DynaLink -on" line from the

Autoexec.bat file).

-on        :  Load the DynaLink flicker free Parameter into memory.

Remember that DynaLink must be installed prior to using this

command (DynaLink -i) to insure that the proper Ext_Modx.Dat

file is accessed.

-off        : Reset to the standard ProDesigner (II or /e) BIOS Parameter.

If, for some reason, you have problems running a particular

program you may want to use the original ProDesigner (II or

/e) BIOS parameter. The "DynaLink -off" parameter will

turn off the flicker free parameters with parameters in the

BIOS.

-s          : This is the DynaLink status flag. It will report any optional

flicker free parameters that have been installed.