OS/2 V2.0 Presentation Manager Driver for ET4000/Clock 2 chip

���������������������������������������������������������������

��������������Ŀ

� Beta Release �

����������������

This OS/2 driver extends the resolution and color of OS/2 V2.0 for VGA cards

based on Tseng ET4000 Chip and TsengLabs International's Clock 2 clock chip

or compatible clock chips. Due to OS/2 requirements, clock chips with

different video frequency table may not work with this driver. Drivers for

other clock frequency specifications will be available in the future.

This driver supports the following:

640x480           256  colors: 60Hz/72Hz

800x600        16/256  colors: 56Hz/60Hz/72Hz

1024x768        16/256  colors: 43.5Hz/60Hz/70Hz

Installation Procedure:

1. Backup DISPLAY.DLL of \OS2\DLL directory.

(DISPLAY.DLL is the 16-color driver of standard OS/2).

2. Boot up your PC with the OS/2 V2.0 diskette.

3. Copy the appropriate driver to your hard disk under

\OS2\DLL directory as follows:

A. Copy ET4000PM.DLL of this diskette to \OS2\DLL

directory.

B. Copy "*.DLL" file to \OS2\DLL directory and

rename it to "DISPLAY.DLL"; "*.DLL" stands for

one of the following files:

- 2E.DLL ( 640x480, 256 colors)

- 29.DLL ( 800x600,  16 colors)

- 30.DLL ( 800x600, 256 colors)

- 37.DLL (1024x768,  16 colors)

- 38.DLL (1024x768, 256 colors)

C. Copy "*.CFG" to \OS2\DLL directory and rename

it to "DISPLAY.CFG"; where "*.CFG" stands for

one of the following files:

- 2E60.CFG ( 640X480-256 COLORS/60Hz)

- 2E72.CFG (                   /72Hz)

- 2956.CFG ( 800X600- 16 COLORS/56Hz)

- 2960.CFG (                   /60Hz)

- 2972.CFG (                   /72Hz)

- 3056.CFG ( 800X600-256 COLORS/56Hz)

- 3060.CFG (                   /60Hz)

- 3072.CFG (                   /72Hz)

- 3743.CFG (1024X768- 16 COLORS/43.5Hz)

- 3760.CFG (                   /60Hz)

- 3770.CFG (                   /70Hz)

- 3843.CFG (1024X768-256 COLORS/43.5Hz)

- 3860.CFG (                   /60Hz)

- 3870.CFG (                   /70Hz)

Example: to select 1024x768-256 color/43.5Hz

1. Copy C:\OS2\DLL\DISPLAY.DLL  C:\640_16.DLL

(Assumeing that your \OS2\DLL\DISPLAY.DLL is located

in drive C.  This backup procdure needs only to be

done once).

2. Insert driver diskette into Drive A

Copy A:\ET4000PM.DLL  C:\OS2\DLL\ET4000PM.DLL

3. Copy A:\38.DLL  C:\OS2\DLL\DISPLAY.DLL

4. Copy A:\3843.CFG  C:\OS2\DLL\DISPLAY.CFG