AHA- 1540A/1542A USER'S MANUAL

Copyright 1989 Adaptec, Inc. All rights reserved.

No part of this publication may be reproduced, stored in a retrieval system, or

transmitted, in any form or by any means, electronic, mechanical, photocopying,

recording, or otherwise, without the prior written consent of Adaptec, Inc. 691

South Milpitas Blvd., Milpitas, CA 95035.

TRADEMARKS

IBM is a registered trademark, PS/2, PC AT, DOS, MS-DOS, OS/2 and Micro Channel

are trademarks of International Business Machines Corporation. SCO and SCO

XENIX are trademarks of The Santa Cruz Operation. Unix is a registered

trademark of AT & T Bell Laboratories.

CHANGES

The material in this manual is for information only and is subject to change

without notice.

While reasonable efforts have been taken in the preparation of this manual to

assure its accuracy, Adaptec, Inc. assumes no liability resulting from any

errors or omissions in this manual, or from the use of the information

contained herein.

Adaptec reserves the right to make changes in the product design without

reservation and without notification to its users.

Additional information may be obtained from:

Adaptec Literature Department 691 South Milpitas Blvd. Milpitas, CA 95035

This equipment generates and uses radio frequency energy and if not installed

and used properly, that is, in strict accordance with the manufacturer's

instructions, may cause interference to radio and television reception. It has

been type tested and found to comply with the limits for a Class B computing

device in accordance with the specifications of Subpart J of Part 15 of FCC

Rules, which are designed to provide reasonable protection against such

interference in residential installation. However, there is no guarantee that

interference will not occur in a particular installation. If this equipment

does cause interference to radio or television equipment reception, which can

be determined by turning the equipment off and on, the user is encouraged to

try to correct the interference by one or more of the following measures:

Reorient the receiving antenna 	Relocate the computer with respect to the

receiver 	Move the computer away from the receiver 	Plug the computer into a

different outlet so that computer and receiver are on different branch circuits

If necessary, the user should consult the dealer or an experienced

radio/television technician for additional suggestions. The user may find the

following booklet prepared by the Federal Communications Commission helpful:

"How to Identify and Solve Radio-TV Interference Problems" This booklet is

available from the U.S. Government Printing Office, Washington, D.C. 20402,

Stock No. 004-000-00345-4.

Warning: This equipment has been certified to comply with the limits for Class

B computing device, pursuant to Subpart J of Part 15 of FCC Rules. Only

peripherals (computer input/output devices, terminals, printers, etc.)

certified to comply with the Class B limits may be attached to this computer.

Operation with non-certified peripherals is likely to result in interference to

radio and TV reception. Also this equipment must be used with shielded power

cables and shielded I/O cables.

AHA-1540A and AHA-1542A Industry PC/AT-to-SCSI Host Adapter User's Manual and

Specification Copyright 1989  Adaptec, Inc. All Rights Reserved

Reproduction or use, without express written permission from Adaptec, Inc.

and/or its licensor, of any portion of this manual is prohibited. While

reasonable efforts have been taken in the preparation of this manual to assure

its accuracy, Adaptec, Inc. assumes no liability resulting from any errors or

omissions in this manual, or from the use of the information contained herein.

1.0  Introduction........................................................1-1

1.1  Document Scope......................................................1-1

1.2  Purpose.............................................................1-1

1.3  Features............................................................1-1

1.4  Product Specifications..............................................1-2

1.5  Reference Documents.................................................1-4

2.0  Host Adapter Architecture...........................................2-1

2.1    Hardware..........................................................2-1

2.1.1  Bus Master DMA....................................................2-1

2.1.2  SCSI Protocol Chip................................................2-1

2.1.3  8-Bit Memory......................................................2-1

2.1.4  Block Diagram.....................................................2-2

2.2    Firmware..........................................................2-4

2.2.1  Multithreaded Operation...........................................2-4

2.2.1.1  Mailboxes.......................................................2-4

2.2.1.2  Command Control Block...........................................2-5

2.2.1.3  Command Descriptor Block........................................2-6

2.2.1.4  Principles of Operation.........................................2-6

2.2.1.5  Task Queueing...................................................2-7

2.2.2    Single Threaded Operation.......................................2-7

3.0  Installation........................................................3-1

3.1  Unpacking and Inspecting............................................3-1

3.2  Installation........................................................3-1

3.2.1  System Configuration..............................................3-1

3.2.2  Hardware Setup....................................................3-2

3.2.3  Checklist.........................................................3-4

3.2.4  Operation Using the BIOS under DOS................................3-4

3.3  Jumper Configuration................................................3-5

3.3.1  Jumper Block J1...................................................3-6

3.3.1.1  SCSI Synchronous Negotiation....................................3-6

3.3.1.2  Diagnostic Jumper...............................................3-6

3.3.1.3  SCSI Parity.....................................................3-6

3.3.2  SCSI Address......................................................3-7

3.3.3  DMA Channel.......................................................3-7

3.3.4  AT Interrupt Channel..............................................3-9

3.3.5  DMA Transfer Speeds...............................................3-10

3.3.6  AT BIOS Wait State................................................3-10

3.3.7  AT Port Address...................................................3-11

3.3.8  AT BIOS Address...................................................3-11

3.3.9  Front Panel LED...................................................3-11

3.3.10  LA Enable Jumper.................................................3-12

3.3.11  Auxiliary Jumpers................................................3-12

3.3.12  BIOS Enable/Disable..............................................3-12

3.3.13  SCSI Terminators.................................................3-13

3.3.14  SCSI Terminator Power............................................3-13

3.4.1  Flexible Diskette Controller Secondary Address....................3-13

3.4.2  Flexible Diskette Controller Enable...............................3-13

3.4.3  Flexible Diskette Controller Interrupt Channel....................3-13

3.4.4  Flexible Diskette Controller DMA Request Channel..................3-14

3.4.5  Flexible Diskette Controller DMA Acknowledge Channel..............3-14

3.4  Flexible Diskette Controller Jumpers (AHA-1542A Only)...............3-13

4.0  Hardware Functional Description.....................................4-1

4.1  Overview............................................................4-1

4.2 I/O  Port Interface..................................................4-1

4.2.1  Control and Status Port...........................................4-2

4.2.2  Command and Data Port.............................................4-4

4.2.3  Interrupt Flag Port...............................................4-5

4.3  Reset Overview......................................................4-7

4.3.1  Hard Reset Operations.............................................4-7

4.3.2  SCSI Reset Operations.............................................4-7

4.3.2.1  SCSI "Soft" Reset Option........................................4-8

4.3.2.2  SCSI "Hard" Reset Option........................................4-9

5.0   Firmware Functional Description....................................5-1

5.1  Host Adapter Command Overview.......................................5-1

5.1.1  Host Adapter Operation Codes......................................5-1

5.1.1.1  No Operation....................................................5-2

5.1.1.2  Mail Box Initialization.........................................5-2

5.1.1.3  Start SCSI Command..............................................5-3

5.1.1.4  Start PC/AT BIOS Command........................................5-3

5.1.1.5  Host Adapter Inquiry............................................5-3

5.1.1.6  Enable Mailbox Out Available Interrupt..........................5-4

5.1.1.7  Set Selection Time-out..........................................5-4

5.1.1.8  Set Bus-On Time.................................................5-5

5.1.1.9  Set Bus-Off Time................................................5-5

5.1.1.10  Set Transfer Speed	............................................5-6

5.1.1.11  Return Installed Device........................................5-7

5.1.1.12  Return Configuration Data......................................5-8

5.1.1.13  Enable Target Mode Command.....................................5-8

5.1.1.14  Return Setup Data..............................................5-9

5.1.1.15  Write Adapter Channel 2 Buffer.................................5-11

5.1.1.16  Read Adapter Channel 2 Buffer..................................5-12

5.1.1.17  Write Adapter FIFO Buffer......................................5-12

5.1.1.18  Read Adapter FIFO Buffer.......................................5-12

5.1.1.19  Echo Command Data..............................................5-13

5.2  Mailbox Command Overview............................................5-13

5.2.1  Mailbox-Out Definition............................................5-14

5.2.2  Mailbox-In Definition.............................................5-15

5.3  Command Block Definition............................................5-19

5.3.1  Scatter Gather Definition.........................................5-25

5.4  Description of Operation............................................5-26

5.4.1  Execution of Initiator Mode Operations............................5-27

5.4.2  Execution of Target Mode Operations...............................5-28

5.4.2.1  Target Mode With a Prepared CCB.................................5-28

5.4.2.2  Target Mode Without a Prepared CCB..............................5-28

5.5  DMA Channel Initialization With the Host Adapter BIOS Disabled......5-29

5.6  Interrupt Initialization............................................5-29

6.0  Single-Threaded Software Interface..................................6-1

6.1  Single Threaded BIOS Command Interface..............................6-1

6.2  BIOS Command Execution Using Interrupt 13...........................6-2

6.2.1  Physical to Logical  Translation..................................6-2

6.2.2  BIOS Command Return Codes.........................................6-3

6.3  BIOS Disk Commands..................................................6-4

6.3.1  Reset Disk Subsystem..............................................6-5

6.3.2  Read Status of Last Operation.....................................6-5

6.3.3  Read Desired Sectors Into Memory..................................6-6

6.3.4  Write Desired Sectors From Memory.................................6-6

6.3.5  Verify Desired Sectors............................................6-6

6.3.6  Identify SCSI Devices.............................................6-7

6.3.7  Read Drive Parameters.............................................6-7

6.3.8  Initialize Drive Pair Characteristics.............................6-8

6.3.9  Seek..............................................................6-8

6.3.10  Alternate Disk Reset.............................................6-8

6.3.11  Test Drive Ready.................................................6-9

6.3.12  Recalibrate......................................................6-9

6.3.13  Read DASD Type...................................................6-9

7.0  Device Drivers......................................................7-1

7.1  PC/AT BIOS..........................................................7-1

7.1.1  Initialization....................................................7-1

7.1.2  System Boot.......................................................7-2

7.1.3  Hard Disk I/O.....................................................7-2

7.2  XENIX/UNIX..........................................................7-2

7.3  DOS Driver..........................................................7-2

7.3.1  DOS Driver Support of Virtual Mode................................7-3

7.4  AHA-1540 On-board Utilities.........................................7-4

8.0  SCSI Features.......................................................8-1

8.1  SCSI Description Initiator Mode.....................................8-1

8.1.1  Linked SCSI Commands..............................................8-2

8.1.2  Zero Latency Read Operation.......................................8-2

8.1.3  SCSI Messages.....................................................8-3

8.2  SCSI Description, Target Mode.......................................8-3

8.2.1  Initiator Conformance Level Requirements..........................8-3

8.2.2  Support of Synchronous Transfer...................................8-3

8.2.3  SCSI Target Operation in Processor Target Mode....................8-4

8.2.3.1  Test Unit Ready.................................................8-5

8.2.3.2  Request Sense...................................................8-5

8.2.3.3  Inquiry.........................................................8-7

8.2.3.4  Send............................................................8-8

8.2.3.5  Receive.........................................................8-9

8.2.4 Incorrect Length Management........................................8-9

9.0  Floppy Disk Interface...............................................9-1

10.0  Problem Determination..............................................10-1

10.1  Self Diagnostic Capability.........................................10-1

10.2  Indicators.........................................................10-2

10.3  Problems Detected During Operation.................................10-2

10.3.1  HA Status Error Indications and Corrective Actions...............10-3

10.3.2  SCSI Error Indications and Corrective Actions....................10-5

10.4  Problems Detected During Installation..............................10-7

11.0  Glossary...........................................................11-1

Appendix A  AHA-1540 System Timing Diagram................................A-1

Appendix B  Connector Pin Out.............................................B-1

Appendix C  Product Comparison and Compatibility..........................C-1

1.1 DOCUMENT SCOPE

This manual provides the information required to install and program the AHA-

1540A and the AHA-1542A  Intelligent Host Adapters in  AT bus-based systems.

1.2 PURPOSE

The Adaptec AHA-1540A and AHA-1542A provide a powerful  multitasking interface

between the Industry Standard Architecture (PC/AT compatible) bus and the Small

Computer System Interface (SCSI) bus. The AHA-1540A/1542A are high performance

intelligent host adapters supporting a maximum asynchronous SCSI rate of 2.0

MBytes/second and a synchronous transfer rate of 5 MBytes/second. The AHA-

1540A/1542A support multithreaded I/O operations, allowing simultaneous

operations on multiple targets/LUNs. Disconnect/Reconnect support maximizes bus

utilization for multiple target systems. Target mode operation allow the AHA-

1540A and AHA-1542A to receive information from other host adapters. Scatter-

gather allows high-performance even in systems with fragmented memory buffers.

The Adaptec AHA-1540A/1542A provides a solution for system applications

requiring very high performance, configuration flexibility and multithreaded

I/O capability, and system redundancy. The Adaptec BIOS also allows the AHA-

1540A/1542A to be used in place of a standard hard disk controller.  The AHA-

1540A host adapter provides only the high performance host adapter circuitry.

The AHA-1542A host adapter provides identical host adapter circuitry, but adds

an IBM compatible flexible diskette controller to the circuit board.

1.3 AHA-1540A/1542A PRODUCT FEATURES

High performance Bus Master DMA with selectable or programmable data rates of

up to 10

MBytes/second.

16- and 8-bit transfers 	Odd and Even starting address

transfers and odd or even data lengths

Maximum synchronous SCSI transfer rate of up to 5.0 MBytes/second

Asynchronous and synchronous peripherals supported simultaneously

Programmable AT bus burst transfer on and off time

Jumper selectable DMA channel for bus arbitration (channels 0,5,6,7)

Jumper selectable interrupt channels (IRQ 9, 10, 11, 12, 14, 15)

True multithreaded operation supporting up to 255 tasks simultaneously

Programmable mailbox architecture �	Adaptec BIOS for standard hard disk

emulation

Jumper selectable I/O port address

Internal and external SCSI connectors

Initiator and Target modes of operation fully supported

On-board flexible diskette controller (AHA-1542A only) �	Scatter-gather

1.4 PRODUCT SPECIFICATIONS

Physical Dimensions:

Length : 13.25 "  Width : 4.125"   Height : 0.5"

Standard PC/AT� compatible form factor

Power Requirements:

+5.0 +/- 0.25 Volts at 2.1 Amps maximum.

Environmental Requirements:

Temperature 0-60 degrees C (operating or storage)

Reliability Information

Mean Time Between Failures:	15000 hours 			(calculated

per Mil Handbook 217E, ground benign, 40 �C)

Mean Time Between Failures:	TBD hours 			(experimental)

Mean Time to Repair:				30 minutes

Industry Standard Architecture Interface

Standard PC/AT bus electronic and physical interface.

Driver output:

VOL: min:0 volts max.  0.4 volts IOL:	24 ma.

VOH: min:2.4 volt max. 5.25 volts

IOH:  8 ma.

Receiver Input

VIL: 0.8 volts

VIH:	2.0 volts

Connector configuration as specified by IBM PC/AT Technical Reference Manual or

manual of Industry Standard Architecture host computer.

SCSI Interface:

Electrical Interface:

As specified by ANSI X3.131-1986 for single-ended operation.

Minimum driver output:48 milliamps, sinking @ 0.5 volts

Signal true:min: 0 volts max:   0.8 volts

Signal false:min:2.0 volts max:5.25 volts

Input hysteresis:min: 0.2 volts

Terminator Power voltage: min:4.25 volts  max:5.25 volts

Terminator Power current: min:800 ma.  max:1.5 amp (fused)

Internal connector: Unshrouded 50-pin header, compatible with unshielded

alternative 1 connector as specified in ANSI X3.131-1986

Figure 4-1 For connector pin out, see Appendix B. Partial list of

compatible connector plugs (for reference only): Manufacturer Model Part Number

3-M	N.A.	3425-6000 T&B Ansley	N.A.	609-5000M Molex	N.A.	15-29-8505

Cable for internal SCSI connector should be good quality 50 conductor flat

cable with 26 or 28 gauge conductors and a characteristic impedance (Z0) of 100

10. Cable shielding is necessary if extremely noisy circuitry or extremely

noise sensitive circuitry is present inside the host computer frame.

External connector:

Shielded 50-pin connector as specified in ANSI X3.131-1986, Figure D-4.

For connector pin out, see Appendix B.

Partial list of compatible connector plugs or cable assemblies  (for reference

only):Manufacturer Model Part No. Amphenol N.A. 200-1M115-02003(Mates with 57-

BC50B-AM200) AMP Champ Plug 554953-1 Cover 554946-x(1-2) Ferrule	554725-x(1-6)

Cable for external SCSI connector should be good quality 100% shielded round

cable with 25 twisted pairs.  Each pair should have a characteristic impedance

(Z0) between 90 and 135 �. Wire gauge may be 26 or 28 AWG.  All pairs should

have the same impedance and should have the same delay per length of cable.

Cables meeting these requirements will normally operate correctly in any SCSI

configuration and should normally meet all FCC requirements.

Floppy Disk Interface

Standard Floppy Disk electronic and physical interface:

Driver output:  VOL: min: 0 volts max. 0.5 volts

IOL: 60 ma.

VOH: open collector  max. 5.25 volts

IOH:.1 ma.

Receiver Input

Tied to +5 volt supply through 150 ohm resistors. Schmidt Trigger with 0.8

volts hysteresis VT-:max 1.0 volts  VT+:		min	1.4 volts

Connector: Unshrouded 34-pin header. For connector pin out, see Section 9.1

Partial list of compatible connector plugs (for reference only):

Manufacturer  Model  Part Number

3-M           N.A.   3414-6000

T&B Ansley    N.A.   609-3400M

Cable for floppy connector should be good quality 34 conductor flat cable with

28 gauge conductors. Addressing of the second drive may be generated by

twisting connector signals 10 through 16 or by changing jumpers in the floppy

disk drives. Meets radiation limits specified for a Class B computing device in

accordance with the specifications in Subpart J of Part 15 of FCC Rules. See

FCC Compliance notes and recommendations in preface of this document.

1.5 REFERENCE DOCUMENTS IBM PC/AT Technical Reference Manual

Small Computer System

Interface, ANSI X3.131-1986, American National Standards Institute

Adaptec AHA-1540A/AHA-1542A Host Adapter Installation Guide

Adaptec AHA-1540 Programming Guide

2.1 HARDWARE

2.1.1 Bus Master DMA

The AHA-1540A/1542A controls the host AT bus as a master and transfers data

directly to and from main system memory. This implementation is known as Bus

Master DMA. Bus Master DMA greatly reduces the host software overhead because

the host CPU is no longer required to maintain the DMA channel's address

pointers and word counts. Bus Master DMA also reduces the number of interrupts

generated per I/O command. The Adaptec AIC-560L is the DMA controller.

Adaptec's implementation of Bus Master DMA can achieve a 10 MB/second burst

data rate. This speed is especially valuable in multitasking systems where the

tasks execute on a time shared basis. Appendix A shows a diagram of the timing

required to achieve the DMA rates that are supported by the AHA-1540A/1542A.

The AHA-1540A/1542A DMA hardware will handle both odd-byte and odd-memory

address data transfers with no performance degradation.

2.1.2 SCSI Protocol Chip (AIC-6250)

The AHA-1540A/1542A utilizes the AIC-6250 SCSI protocol chip to maximize the

SCSI bus utilization. The AIC-6250 is an Adaptec VLSI device which allows the

AHA-1540A/1542A to achieve greater than 2.0 MBytes/Sec asynchronous SCSI data

transfer rates, and up to 5.0 MBytes/second synchronous transfer rates. The

AIC-6250 will also enable the AHA-1540A/1542A to operate as both an initiator

and as a target device. Through a 16-bit host interface, the AIC-6250 reduces

bus busy time during data transfer by bursting data across the AT bus at up to

10 MBytes/second. The AIC-6250 has separate data busses for the local

microprocessor and for the system data bus. This further increases the

performance of the AHA-1540A/1542A by reducing the overhead associated with

SCSI commands.

2.1.3  8 Bit Memory

During normal DMA operations, nearly all transfers to and from memory are 16

bit transfers. At the very end, or the very beginning of an odd address

boundary, an 8 bit transfer on the upper data bits (D8-D15) will occur

according to the AT bus architecture. Some memory in the I/O space, such as

video RAM, is 8 bits only and always transfers data only on the lower data bits

(D0-D7). The AHA-1540A/1542A will transfer 16 bit or 8 bit memory in the

address space between 0A0000 hex and 0BFFFF hex depending on the signal line

MEM16 on the AT bus. If this signal is active, 16 bit memory is assumed, and if

inactive 8 bit memory is assumed. Outside of this address space 16 bit memory

is always assumed.

2.1.4 Hardware Block Diagram(cannot be included because of ASC transmission)

BLOCK DIAGRAM OF AHA-1540A

BLOCK DIAGRAM OF AHA-1542A

2.2 FIRMWARE

The AHA-1540A/1542A supports multithreaded SCSI initiator operation through a

simple mailbox protocol. The firmware accepts as many Command Control Blocks

(CCB's) as required and executes them from its local RAM. The firmware controls

all of the SCSI activity that a task may require. This includes: Arbitration,

Selection, Disconnection, Reconnection, and Command completion.

Using the same mailbox protocol, the AHA-1540A/1542A can operate as a Processor

type device serving as a multitasking target to other initiators. This feature

allows high bandwidth communication between multiple hosts.

In addition, the AHA-1540A/1542A firmware cooperates with the BIOS installed on

the host adapter to emulate the standard DOS BIOS calls. This allows booting

operations and the execution of standard DOS operations from attached SCSI

disks, allowing the SCSI subsystem to completely replace the usual internal

disk functions.

2.2.1 Multithreaded Operation

A multiuser, multitasking operating system issues a large number of I/O tasks

in a rapid sequence. The architecture of the AHA-1540A/1542A makes management

of this activity very easy and straight forward for the operating system and

its associated I/O drivers. This section briefly explains the interaction

between the system and the AHA-1540A/1542A required to accomplish an I/O task.

2.2.1.1 Mailboxes

The AHA-1540A/1542A uses a mailbox architecture for task communication between

the host and host adapter. This allows the host adapter to perform

multithreaded operations with a minimum of host intervention.The mailboxes are

located in main system memory . Each mailbox entry is four bytes long. After

power up sequencing, host initialization procedures, and the boot procedure are

completed, the host issues an initialization command to inform the host adapter

of the mailbox location. There are always an equal number of Outgoing Mailboxes

(MBO) and Incoming Mailboxes (MBI). MBIs are located immediately after the

MBOs. A typical mailbox structure is shown below:

Base address

+0-CMD----CCB 4 POINTER----MBO 0

+4-CMD----CCB 2 POINTER----MBO 1

+8-00-----FREE ENTRY-------MBO 2

+12-CMD----CCB 3 POINTER----MBO 3

+16-00-----FREE ENTRY-------MBI 0

+20-STATUS-CCB 1 POINTER----MBI 1

+24-00-----FREE ENTRY-------MBI 2

+28-00-----FREE ENTRY-------MBI 3

In this example there are four MBOs and four MBIs. The first byte of each MBO

contains the MBO Command byte. The remaining three bytes point to a Command

Control Block (CCB). The CCB provides all the rest of the information needed to

complete a task. A MBO is available to accept a new entry if the first byte is

zero.

The first byte of each MBI contains the status of a completed task. The

remaining three bytes point to the CCB of the completed task. An MBI is free if

the Status byte is zero. Mailboxes may point to CCB's controlling initiator

tasks, controlling target tasks, or controlling error recovery tasks.

2.2.1.2 Command Control Block

A Command Control Block provides the information required to control a SCSI

command sequence. The block contains pointers to the data area to be used by

the command. It contains areas for presenting status of both the host adapter

and the addressed SCSI device. In addition, it contains the SCSI Command

Descriptor Block defining the action to be taken by the addressed SCSI device.

An error information buffer area is also provided.

A Command Control Block is also used to service an operation requested by

another initiator when the AHA-1540A/1542A is being addressed as a SCSI

Processor device.

The CCB is defined completely in Section 5.

A typical CCB is shown below:

2.2.1.3 Command Descriptor Block

The Command Descriptor Block (CDB), a part of the Command Control Block, is a

standard format command packet that is transmitted to the addressed SCSI

device. It contains all the command information required by the SCSI device to

perform the desired operation. The Command Descriptor Block contains the

command Operation Code followed by a Logical Unit Number (LUN), command

parameters if required, and a control byte. A typical Group 0 six byte CDB is

shown below:         BIT����������������������������������������

BYTE  7     6     5     4     3     2     1     0

0    ��O�P�E�R�A�T�I�O�N�����C��O��D��E����������Ĵ

1    �L���U���N����Ĵ��LOGICAL�BLOCK�ADDRESSS(MSBĴ

2    ���������LOGICAL BLOCK ADDRESS��������������Ĵ

3    ���������LOGICAL BLOCK ASDDRESS(LSB)��������Ĵ

4    ���������TRANSFER LENTH���������������������Ĵ

5    �VENDOR�������RESERVED����������Ĵ�FLAG��LINK�

Please refer to the SCSI specification  ANSI X3.131, the Common Command Set

(CCS) revision 4B, and the SCSI 2 draft for additional information on Command

Descriptor Blocks.

2.2.1.4 Principles of Operation

At power-up, the host must inform the host adapter of the location and number

of mailboxes. To start a task, the host builds a CCB and stores its memory

address into a free mailbox. A non-zero Mailbox Out command byte is then

written to indicate that the mailbox entry is full and valid. The host then

writes to an I/O port (defined in section 4) to indicate that the host adapter

should scan the MBO area. When a full MBO is found, the host adapter copies the

mailbox's CCB pointer into its internal RAM and clears the Mailbox entry by

writing a zero to the MBO command byte. This frees the MBO so that it can be

used to start another task.

After completing a task, the host adapter scans the MBI area for a free

mailbox. When one is found, it is updated with the task's completion status and

CCB pointer. The CCB pointer identifies the completed task. A MBI Stored

interrupt is generated to notify the host that a task has been completed. The

host scans the MBI area searching for a non-zero Status byte. When one is

located, the host obtains the CCB pointer and frees the MBI by writing a zero

into the Status byte. The host then examines the contents of the CCB to

determine that the command was successfully completed. The freed MBI can now be

used to indicate the completion of another task. The host adapter fills the MBI

area and scans the MBO area in a round robin fashion. If the host saves the

position of the last active MBI entry, it can determine the MBI of a new entry

immediately without searching, since a new entry will be in the next MBI

location.

The host adapter transmits a new MBO Available or MBI Full interrupt to the

host whenever all non-Mailbox interrupts have been cleared and serviced by the

host. The host should analyze the interrupts and clear them as soon as possible

so that the host adapter can post any new interrupts quickly. The host adapter

will not wait until an interrupt can be transmitted to the host before

processing an MBO entry or creating a new MBI entry. Thus, in processing a

single MBI interrupt, the host may find several MBI entries waiting by the time

the interrupt processing is finished. Similarly, a later MBI interrupt for the

last of the later MBI entries may find nothing to service because the MBI entry

was examined and processed as a result of the first MBI Full interrupt. If the

interrupts are reset quickly by the host, the probability of an interrupt

occurring when no MBI entry is available is much lower, providing an important

performance improvement. If the MBI entries are emptied by the host in a round-

robin order, the scan for the next full entry is very simple, since it is

always the next MBI entry in the Mailbox area.

2.2.1.5 Task Queueing

Multiple tasks may be started against a target/LUN or against multiple

targets/Logical Units. Since only one task can be active against any one given

LUN at a time, all other tasks for the same LUN are queued. Other LUN's may

have active tasks at the same time.

The host adapter dequeues on a first in, first out  (FIFO) basis for each

target/LUN combination. However, due to the optimization algorithm used by the

host adapter, a task may sometimes be started earlier in spite of it's late

entrance in the queue on multiple target/LUN systems. Task queueing should not

be used where changes in the order of command execution may cause data

integrity failures.

2.2.2 Single-Threaded Operation

A host adapter BIOS is provided to emulate the standard  hard disk BIOS and

boot functions. With this BIOS, the host adapter can be used in lieu of a

standard hard disk controller on any ISA compatible system.

The BIOS is compatible with the standard hard disk BIOS. This allows DOS to

access up to two hard disk devices on the SCSI bus without a driver. All normal

I/O functions are supported including system booting. Single threaded operation

and multithreaded operation do not operate simultaneously. Single threaded

operation cannot be requested until all multithreaded operations are completed.

Similarly, all multithreaded operations must be complete before a single

threaded operation can be requested. For most multitasking operating systems,

such as Xenix and OS/2, single threaded operation is normally used only for the

early part of the boot operation, after which multithreaded drivers take over

all control of the SCSI operations.

3.1 UNPACKING AND INSPECTION

The carrier is responsible for damage incurred during shipment. In case of

damage, have the carrier note the damage on both the delivery receipt and the

freight bill, then notify your freight company representative so that the

necessary insurance claims can be initiated.

After opening the shipping container, use the packing slip to verify receipt of

the individual items listed on the slip. Retain the  shipping container and

packing material for possible later reuse should return of the equipment to the

factory  necessary.

CAUTION: The AHA-154XA is carefully designed to resist the effects of static

electricity. However, like all electronic equipment , it can be damaged or its

life can be shortened by unusual static discharges. Please take the proper

precautions when handling the board. Keep the board in its conductive wrapping

until it is ready to be installed in your system. Be sure that  the  host

computer and the personnel handling the board are properly grounded while

installing the board.

3.2 INSTALLATION

The following section details the installation procedure for the Adaptec AHA-

1540A/1542A AT to SCSI host adapter. The installation of the board consists of

setting the various on-board jumpers, preparing the SCSI devices, installing

the correct terminations, inserting the board into a full-length AT compatible

connector, and connecting a SCSI cable from the on-board connector to a SCSI

target.

NOTE: The system must be turned off during the installation procedure.

3.2.1 System Configuration

The DOS operating system and the standard AT BIOS support two hard disk drives,

drive C: and drive D:. If two standard hard disk drives are installed, they are

the only hard disk drives accessible from the operating system. If one standard

hard disk drive is installed, the AHA-154XA BIOS allows DOS to access the SCSI

drive with the Target:LUN address of 0:0 as the second of the two supported

drives (drive D:). If no standard hard disk drives are installed, the AHA-154XA

BIOS allows DOS to access the SCSI drive with the address of 0:0 as the first

of two supported drives (drive C:) and the SCSI drive with the address of 0:1

or 1:0 as the second of the drives (drive D:). System booting is performed from

the floppy drive if a floppy diskette is installed. If no floppy is installed,

system booting is attempted from the drive chosen as drive C: through the above

process, whether the drive is a standard hard disk or a SCSI hard disk. The

AHA-154XA BIOS fully supports the extended partitioning capabilities of DOS 3.3

for up to two drives. The AHA-154XA BIOS only supports the AHA-154XA if its I/O

Port Address is set to 330h.

Adaptec provides a special DOS driver for the AHA-154XA that allows the support

of up to 24 physical or logical devices under DOS. Many other operating

systems, including SCO Xenix and Unix, do not have this limitation and will

allow the access of any number of attached SCSI devices.

The term "standard hard disk" refers to the disks attached to the system by a

standard ISA disk controller. These standard hard disks can be set to the

"Installed" or "Not Installed" state by the SETUP program that is supplied with

each ISA host computer. The SETUP program allows the user to select the number

of standard hard disks that are recognized by the system regardless of whether

or not they are physically installed.

3.2.2 Hardware Setup

Several preparatory steps must be taken to install the host adapter in the host

computer. The initialization jumpers must be correctly placed in the AHA154XA.

The SCSI Bus terminators must be installed in the correct SCSI devices.

Finally, the correct SCSI addresses must be assigned to each peripheral device.

The Adaptec AHA-1540A/1542A 16-Bit AT bus to SCSI bus Host Adapter has been

designed to operate as shipped in the majority of AT class computers. The host

adapter's jumper settings should almost always remain in their original default

positions. Refer to section 3.3 if conflicting memory or port assignments

require that the host adapter jumpers be reconfigured. The AHA-1540A/1542A is

shipped with the following default settings:

SCSI Address-7

SCSI Parity-Enabled

Terminators-Installed

Terminator Power-Supplying

Synchronous Negotiation-Disabled

DMA Channel-5

Interrupt Channel-11

AT Port Address-330H

AT BIOS Address-ODCOOOH

LA Enable Jumper-Installed

Termination

The SCSI bus must be terminated correctly to assure proper operation. The first

and last physical SCSI devices on the SCSI cable must have terminators

installed. All other SCSI devices must have terminators removed. The host

adapter is shipped with terminators already installed at locations RN3, RN4,

and RN5.   The internal and external connectors connect to the same SCSI bus,

so both internal and external cabling must be considered in determining where

terminators are installed.

If only one cable (either internal or external) is connected to the host

adapter, the terminators must remain installed in the host adapter. Terminators

must also be installed on the device at the farthest end of the cable from the

host adapter. Terminators must be removed from all other attached SCSI devices.

If both an internal and an external cable are connected to the host adapter,

remove the terminators on the host adapter and install terminators on the

devices at the farthest end of each cable. Terminators must be removed from all

devices except the device at the end of each cable. The instruction manuals for

each SCSI device will indicate how the terminators can be removed or replaced.

Addressing

The SCSI target address for each SCSI device to be attached must be selected by

setting the proper jumpers or switches on the device. If the SCSI device is a

hard disk that is to be used as the boot disk, it must have the Target

Identifier (SCSI Address) set to zero and the Logical Unit Number set to zero.

SCSI Addresses 0 and 1 should be reserved for SCSI hard disk drives.  SCSI

Address 2 is often used for tape devices. Each installed peripheral must have a

different Target Identifier. The host adapter's default address is 7. Duplicate

SCSI Addresses will cause errors that are extremely difficult to identify. Any

jumpers that control operating modes must also be properly set. If there is a

jumper that enables synchronous transfer negotiation, the jumper should be set

to enable negotiation. Check the SCSI drive or controller to ensure that Parity

Checking is enabled. If Parity checking on the device is disabled or not

supported, the jumper at location J8 of the host adapter should be removed.

Parity checking should only be enabled if all SCSI devices support it.

Installation Procedure

Now that the host adapter and the devices to be installed have been properly

prepared, the devices should be installed in the following manner:

1) Turn OFF the power to the computer system.

2) Remove the system cover according to the directions of the computer

manufacturer.

3) If only an external SCSI subsystem is to be used, no internal cabling is

required. If an internal SCSI peripheral is to be used, install a 50 pin SCSI

ribbon cable to the host adapter. This cable must be oriented correctly. Pin 1

of the SCSI cable is designated by a red stripe. Multi-color 50 pin ribbon

cables signify pin 1 with a brown color. Pin 1 on the host adapter 50 pin SCSI

header is located on the left hand side, farthest from the installation bracket

and is designated by the words PIN 1 on the board adjacent to the header. After

locating pin 1 on the host adapter and on the SCSI cable, carefully insert the

connector located at the end of the long end of the cable into the host adapter

connector. After ensuring that all pins are lined up and that the pin 1

orientation is correct, firmly seat the connector to the board. If it is ever

necessary to remove the cable, gentle prying with a small thin-bladed

screwdriver may be required.

4) After installing the SCSI cable, the host adapter can be installed in any

one of the  adapter slots in the host computer.

5) If an internal SCSI device is to be used, it should be installed in the

drive bays in accordance with the directions on the peripheral.  The proper

power supply must be connected to the SCSI peripheral device.

6) The 50 pin SCSI ribbon cable can now be attached to each SCSI device. Refer

to the device's installation instructions to ensure proper pin 1 orientation.

Pin 1 orientation must be consistent throughout the system. Keep the ribbon

cable neatly dressed away from the ventilation slots in the computer system.

Keep the ribbon cable dressed away from possible electrical noise sources or

noise sensitive components, particularly large microprocessors, memory boards,

switching power supplies, and analog data acquisition boards. If the internal

configuration requires the cable to come near noise sensitive circuits , make

sure that the cable crosses the boards at right angles and is near the noise

sensitive circuits for the shortest distance possible.

7) Carefully reinstall the cover of the computer.

8) If an external SCSI subsystem is to be installed, it can now be cabled to

the External SCSI Connector projecting from the shielding bracket on the back

of the AHA-154XA Host Adapter. The proper shielded SCSI cable must be used for

proper operation. The external connector on the AHA-1540A/1542A is a D shell

connector that ensures correct pin 1 orientation on the host adapter. The

subsystem, cables, and SCSI terminators must be installed in accordance with

the directions provided with the external SCSI subsystem. The addresses

selected for external SCSI devices must not overlap with the addresses of the

host adapter or any other SCSI devices attached internally.

3.2.3 Checklist

Before applying power to your system, the following items should be completed

and checked:

1. 50 pin SCSI ribbon cable is connected to the host adapter with proper pin 1

orientation.

2. The host adapter is firmly seated in the host computer's adapter

slot.

3. The correct SCSI addresses are selected on all attached SCSI devices.

Address 0 is reserved for the boot hard disk and address 1 is used for a second

hard disk.

4. The correct operating modes are selected on all attached SCSI devices.

5. Terminators are installed or removed on the drives and host adapter as

required.

6. External SCSI devices are properly installed and cabled.

3.2.4 Operation Using the BIOS Under DOS

To bring up the system for the first time with the host adapter installed,

insert a bootable system diskette in drive A:, and close the door. After the

system initialization and power-up diagnostics have completed, system error

messages, such as "SYSTEM CONFIGURATION CHANGED, RUN SETUP" and prompts on how

to continue, may be displayed. After following the instructions displayed on

the screen, the system will boot from the flexible diskette drive.

Run your SETUP program to configure the system as required. Remember that a

SCSI drive can only be used as a boot device if both standard hard disks are

mapped out. When the SETUP parameters have been defined, follow the directions

on the screen to re-initialize the system.

Most SCSI drives are shipped from the factory with a complete low-level format.

A few SCSI drives and most bridge controllers, including the CCS compatible

Adaptec ACB-4525Z, require a low-level SCSI format operation before they will

operate. Section 7.4 describes how the low-level format utility that is

available in the Adaptec BIOS Utilities should be used to format such SCSI

devices.

Run the DOS FDISK program to partition the disk for the number of cylinders to

be used by DOS. When using DOS 3.2 or under, it is generally a good idea to

select 1 less cylinder than the maximum allowable per partition according to

DOS. This eliminates the possibility of exceeding the 32 MB limit. Activate the

first DOS partition before leaving the FDISK program if it is to be the boot

partition. If you are not sure if a DOS partition exists, use the menu entry in

FDISK to display partition data.

After the DOS partition has been created and activated, the drive is ready for

a DOS FORMAT. Refer to the DOS manual for the format options that are

available. If the system is going to boot from a SCSI disk, it is usually

easiest to transfer the hidden system files to that disk during the FORMAT.

CAUTION: If the drive you are using was previously formatted or partitioned

with a different host adapter or disk controller, a DOS partition may already

exist. If such a partition does exist it should be deleted and re-created using

this host adapter. If this precaution is not followed, erratic system operation

may result.

The host adapter and SCSI disks are now ready for normal operation. The host

adapter's BIOS performs the necessary initialization procedures during the boot

operation. This procedure is described in section 6.1.1

3.3 Jumper Configuration

All of the jumpers have been preset at the factory to ensure proper system

operation with the majority of AT bus systems. This  information is provided to

help the OEM or system integrator to configure the system properly if other

option boards present conflicts.

The following diagram shows the approximate location of the various customer

configurable items on the AHA-1540A/1542A. Listed after the diagram are the

functions, defaults, and set-up options for each jumper.

Jumper locations:���������������������������������������������������������

�    j5    j4                       �scsi conn.�      �

��Ŀ     �diskette header�                            ��Ŀ

�                                 j6    j7         �  �

�                                           j10    �  �

�                                                  �  �

�      j9                        j8   j12          �  �

�          j11                           j18       �  �

� j13             j14j15 j16    j17      j19       ����

��������������������������Ŀ     �Ŀ          ������

������� ������������

3.3.1  Jumper Block J1

Jumper block J1 is shown below:

�����Ŀ

pin 1�  � 0 0 � Syncronous transfer

� 0 0 � Diagnostics

� 0 0 � SCSI parity

� 0 0 � SCSI address

� 0 0 � SCSI address

� 0 0 � SCSI address

� 0 0 � DMA channel�����������Ŀ j14 and j15 must also be set.

� 0 0 � DMA channel�������������

� 0 0 � Interrupt channel�����Ŀ

� 0 0 � Interrupt channel      ���j16 must also be set.

� 0 0 � Interrupt channel�������

� 0 0 � DMA transfer speed

� 0 0 � DMA transfer speed           x= jumper installed

�������

Jumper block J1 is the large vertically oriented block of jumpers located near

the center of the host adapter. Pin pair 1 is the topmost pair of pins.

3.3.1.1 SCSI Synchronous Negotiation

Pin-pair 1 of jumper block J1 is the synchronous negotiation enable jumper. The

AHA-1540A/1542A will initiate SCSI synchronous negotiation during

initialization, or after a SCSI reset if this jumper is installed. If the

jumper is not installed, the AHA-1540A/1542A will still support synchronous

SCSI transfers, but a different SCSI device must initiate the negotiation.

Default is jumper removed, synchronous negotiation initiation disabled. If any

attached SCSI devices support synchronous transfer, the initiation of

negotiation should be enabled by both the AHA-154XA and the attached devices to

fully implement the SCSI synchronous negotiation protocol.

3.3.1.2 Diagnostic Jumper

Pin-pair 2 of jumper block J1 is a factory diagnostic jumper. This jumper is

reserved for Adaptec use and must not have a jumper shunt installed. If this

jumper is installed, the firmware will loop through the power on diagnostics,

preventing normal operation. Default is jumper removed.

3.3.1.3 SCSI Parity

Pin-pair 3 of jumper block J1 is the parity enable/disable jumper. The SCSI

parity checking is disabled if this jumper is installed. The default is parity

checking enabled.

3.3.2 SCSI Address

Pin-pairs 4, 5, and 6 of jumper block J1 define the SCSI address. The SCSI

address is selected according to the following table. The default address is 7.

J1      �����Ŀ

pin 1�  � 0 0 �                        pin pair  scsi

� 0 0 �                        4 5 6     address

� 0 0 �                        0 0 0       7      default

pin-pair 4 � 0 0 � SCSI address�0         x 0 0       6

pin-pair 5 � 0 0 � SCSI address�1         0 x 0       5

pin-pair 6 � 0 0 � SCSI address�2         x x 0       4

� 0 0 �                        0 0 x       3

� 0 0 �                        x 0 x       2

� 0 0 �                        0 x x       1

� 0 0 �                        x x x       0

� 0 0 �

� 0 0 �

� 0 0 �

�������    x=jumper installed

3.3.3 DMA Channel

There are three jumper blocks involved in selecting the DMA channel. These are

J1, J14, and J15. The DMA channel selection jumpers consist of pin pairs 7 and

8 located in the large block of jumper pins J1. The DMA channel reported to the

AT during the Return Configuration command is set by these jumpers according to

the following table. Default is DMA channel 5.

Jumper set J14 selects the DMA REQ signal to be used by the AHA-1540A/1542A

according to the following table. This jumper set is located near the bottom

center of the host adapter. Pin pair 1 is the right most pair of pins. The

default is DMA Request 5.

Jumper  set J15 selects the DMA ACK signal to be used by the AHA-1540A/1542A

according to the following table. This jumper set is located near the bottom

center of the host adapter. Pin pair 1 is the right most set of pins. The

default is DMA Acknowledge 5.

There are four DMA channels that may be chosen for use by the AHA-1540A/AHA-

1542A, channels 0, 5, 6, and 7. The DMA channel is set up by using the three

jumper blocks and pin-pairs previously described. The jumper settings for each

channel are shown below:      �����������������������Ŀ

�����Ŀ            �j1 DMA channel jumpers �

pin 1�  � 0 0 �            �pin-pair  DMA          �

� 0 0 �            � 7 8      channel      �

� 0 0 �            � 0 0        7          �

� 0 0 �            � x 0        6          �

� 0 0 �            � 0 x        5          �

� 0 0 �            � x x        0          �

pin-pair 7 � 0 0 � DMA channel�������������������������

pin pair 8 � 0 0 � DMA channel

� 0 0 � ����������������������Ŀ    ����������������������Ŀ

� 0 0 � � j14-2                �    � j15-2                �

� 0 0 � � 0 0 x 0��pin 1       �    � 0 0 x 0��pin 1       �

� 0 0 � � 0 0 x 0              �    � 0 0 x 0              �

� 0 0 � � � � � ����channel 0  �    � � � � ����channel 0  �

������� � � � ������channel 5  �    � � � ������channel 5  �

� � ��������channel 6  �    � � ��������channel 6  �

x=jumper installed � ����������channle 7  �    � ����������channle 7  �

������������������������    ������������������������

3.3.4 AT Interrupt Channel

There are two jumper blocks involved in selecting the AT interrupt channel.

These are J1 and J16. The AT interrupt channel jumpers consist of pin pairs 9,

10, and 11 in the large block of jumper pin pairs J1. The interrupt channel

reported to the AT during the Return Configuration Command is set by these

jumpers according to the following table. The default is interrupt channel 11.

Jumper set J16 selects the AT interrupt channel to be used by the AHA-

1540A/1542A. This jumper set is located near the bottom center of the host

adapter. Pin pair 1 is the right most pair of pins. The interrupt channel used

is set according to the following table. The default interrupt channel is 11.

�����������������������Ŀ

j1      �����Ŀ                   � j1 interrupt channels �

pin 1�  � 0 0 �                   � pin-pair    interrupt �

� 0 0 �                   � 9 10  11   channel    �

� 0 0 �                   � 0  0   0       9      �

� 0 0 �                   � x  0   0       10     �

� 0 0 �                   � 0  x   0       11     �

� 0 0 �                   � x  x   0       12     �

� 0 0 �                   � 0  0   x       14     �

� 0 0 �                   � x  0   x       15     �

pin-pair 9 � 0 0 � Interrupt channel �������������������������

pin-pair 10� 0 0 � Interrupt channel     ��������������������Ŀ

pin pair 11� 0 0 � Interrupt channel     � j16   3            �

� 0 0 �                       � 0 0 0 x 0 0��pin 1 �

� 0 0 � x=jumper installed    � 0 0 0 x 0 0        �

�������                       � � � � � � ���IRQ 9 �

� � � � � �����IRQ 10�

� � � � �������IRQ 11�

� � � ���������IRQ 12�

� � �����������IRQ 14�

� �������������IRQ 15�

����������������������

3.3.5 DMA Transfer Speed Default

Pin-pairs 12 and 13 of jumper block J1 set the default DMA transfer speed.

Installing jumper shunts as shown below will set any of 4  default DMA transfer

speeds. (See Appendix A for system timing requirements.) The default speed is

selected after power on or after a hard reset occurs. This speed may also be

tuned to allow for the fastest possible transfer (minimum bus on time) for any

particular system by using the Host Adapter command Set Transfer Speed. The

Host Adapter Command overrides the jumper settings. This command is detailed in

section 5. The default is no jumper installed.

J1      �����Ŀ                      �����������������Ŀ

pin 1�  � 0 0 �                      � pin pair   DMA  �

� 0 0 �                      � 12  13     Speed�

� 0 0 �                      �  0   0      5.0 � default

� 0 0 �                      �  X   0      5.7 �

� 0 0 �                      �  0   X      6.7 �

� 0 0 �                      �  X   X      8.0 �

� 0 0 �                      �������������������

� 0 0 �

� 0 0 �

� 0 0 �

� 0 0 �

pin-pair 12� 0 0 �DMA Transfer speed

pin-pair 13� 0 0 �DMA Transfer speed

�������                        x=jumper installed

3.3.6 AT BIOS Wait State

Jumper block J7 allows IOCHRDY on the AT bus to be driven not active during

BIOS reads. The time may be set to 100ns, 200ns, 300ns, or the function may be

disabled. The default is Wait State Disabled.

�J7����Ŀ

�0 0 0 X�pin 1

�0 0 0 X�

x=jumper installed  ������ ������disable wait states

� � ���������100 ns

� �����������200 ns

�������������300 ns

3.3.7 AT Port Address

The starting address of the block of four I/O ports required by the AHA-

1540A/1542A is selected by the jumpers in jumper block J6. Pin pair 1 is the

right most pair of pins. The default address is 0330H. That port address is

required by the BIOS. Other valid port addresses are 334H, 330H, 234H, 230H,

134H, and 130. These port addresses may be used by device drivers and operating

systems that support multiple host adapters.  The default address is 330H.

�J6��Ŀ                 AT Port Address

�0 0 X�pin 1            A9  A8  A2   ADRESS

�0 0 X�                  0   0   0    334H

���� ����A2              0   0   X    330H

� ������A8              0   X   0    234H

��������A9              0   X   X    230H

x=jumper installed        X   0   0    134H

X   0   X    130H

3.3.8 AT BIOS Address

�����Ŀ                  BIOS Address

J10 � 0  0�                  Jumpers   BIOS

� 0  0���Pin 1            0   1    ADRESS

�ĳ�ĳ�                   0   0    0DC000H  DEFAULT ADDRESS

�  ���BIOS Address 0    X   0    0CC000H

�������BIOS Address 1   0   X    0D8000H

x=jumper installed            X   X    OC8000H

The starting address of the block of address space reserved for the Adaptec

BIOS is selected by the jumper pins located in J10. The default address is

ODCOOOH. This address must not conflict with any other BIOS or decoded address

space in the system.

3.3.9 Front Panel LED Connector

J5 �����������Ŀ

�0  0  0  0 ��PIN 1

�������������

J5 is the front panel LED header. Pins 1 and 4 are connected to the same 220

ohm pull up resistor and pins 2 and 3 are pulled low by the same LED driver

whenever the on board LED is on. This allows the Front Panel LED cable to be

plugged in with either orientation. The driver is specified to pull 24

milliamps through the LED. J5 is located near the top left of the host adapter.

3.3.10 LA Enable Jumper (AHA-W1542A Only)

Note: This jumper exists only on the early production units. These are easily

identified by a label on the board that has a model number of AHA-W1542A.

Standard production AHA-1540A and AHA-1542A boards do not have this jumper.

When J8 is installed, address drivers LA17 - LA19 are enabled. When not

installed these address lines are not driven. Some computers, especially 80386

based machines, short address lines LA17 and SA17, LA18 and SA18, and LA19 and

SA19 together. If your computer system shorts these lines then remove jumper

J8. Default is jumper installed.

CAUTION:   Incorrect installation of this jumper may not result in immediate

system problems. If it is necessary to change the state of this jumper after

the system has been configured, the SCSI hard disk must also be re-formatted.

Contact the computer system manufacturer if there is any doubt on how the

system implements these address lines.

3.3.11 Auxiliary Jumpers

Installation of a jumper shunt on pin pair 4 of jumper block J9 will disable

the automatic request sense function on the AHA-1540A/1542A. If the automatic

Request Sense function is enabled, it may additionally be disabled on a command

by command basis using the Request Sense Allocation Length Field of the CCB

(see Section 5.3). All other pins-pairs on jumper block J9 are reserved for

future use.

1        4

����������Ŀ

J9 �0  0  0  0�

�0  0  0  0�

��ĳ�ĳ�ĳ�

�  �  �  ���DISABLE AUTO SENSE

�  �  ������RESERVED

�  ���������RESERVED

������������RESERVED

3.3.12 BIOS Enable/Disable

Only one host adapter BIOS should be enabled in any host system. That BIOS

allows DOS to access up to two SCSI drives on the host adapter having a port

address of 330H. Any other host adapters installed in the system must have

their BIOS disabled.  In addition, certain operating systems may require that

the BIOS be disabled. The enabling of the BIOS is controlled by jumper J11,

near the bottom left of the host adapter. When the jumper is installed on J11

the BIOS is enabled. When the jumper is removed, the BIOS is disabled and will

not respond to host reads. The default is jumper installed.

3.3.13 SCSI Terminators

RN3, RN4, and RN5 are the SCSI terminators. If the AHA-1540A/1542A is not the

first or the last SCSI device in a string of SCSI devices, or if in-line

terminators are used, then all of these resistor networks must be removed.

More information is provided in section 3.2.2 about the proper use of the SCSI

terminators. The default is terminators installed.

3.3.14 SCSI Terminator Power

Fuse F1 controls the terminator power. If another SCSI device is supplying

terminator power, then F1 may optionally be removed. No more than 5 SCSI

devices should be configured to supply terminator power to a single SCSI bus.

The default is F1 installed with the AHA-1540A/1542A supplying terminator

power. The terminator power is protected with a 1.5 amp fuse to prevent short

circuits from burning SCSI cables. The terminator power is additionally

protected from terminator power back-flow with a low voltage-drop diode so that

the SCSI bus will continue to operate even if one or more SCSI host adapters is

powered down.

3.4 Flexible Diskette Controller Configuration Jumpers (AHA-1542A Only)

3.4.1 Flexible Diskette Controller Secondary Address

Jumper J12, located near the center right of the host adapter, controls the

selection of the flexible diskette controller secondary address. When J12 is

installed, the flexible diskette controller will respond to I/O addresses 370-

377. When not installed, the flexible diskette controller will respond to I/O

addresses 3F0-3F7. The default is no jumper installed.

3.4.2 Flexible Diskette Controller Enable 	Jumper J13, located near the bottom

left of the host adapter, enables or disables the flexible diskette controller.

When this jumper is installed, the flexible diskette controller will respond to

host reads and writes. When not installed, the flexible diskette controller

will not respond to any signal. The default is jumper installed.

3.4.3 Flexible Diskette Controller Interrupt Channel

INTERRUPT CHANNEL 6

�������Ŀ                        �������Ŀ

J17 �0  X  X��PIN 1             J17  �X  X  0��PIN 1

���������                        ���������

Jumper block J17, located near the bottom right of the host adapter, sets the

interrupt channel for the flexible diskette controller. The default is

interrupt channel 6.

3.4.4 Flexible Diskette Controller DMA Request Channel

DMA REQ CHANNEL 2                DMA REQ CHANNEL 3

�������Ŀ                        �������Ŀ

J18 �0  X  X��PIN 1             J17  �X  X  0��PIN 1

���������                        ���������

Jumper J18, located near the bottom right of the host adapter, selects the DMA

Request channel for the flexible diskette controller. Jumper J19, the DMA

Acknowledge jumper, must be set to the same position. Default is DMA channel 2.

3.4.5 Flexible Diskette Controller DMA Acknowledge Channel

DMA ACK CHANNEL 2                DMA ACK CHANNEL 3

�������Ŀ                        �������Ŀ

J19 �0  X  X��PIN 1             J19  �X  X  0��PIN 1

���������                        ���������

Jumper J19, located near the bottom right of the host adapter, selects the DMA

acknowledge channel for the flexible diskette controller. Jumper J18, the DMA

Request jumper, must be set to the same position. Default is DMA channel 2.

4.0 HARDWARE FUNCTIONAL DESCRIPTION

4.1 HARDWARE OVERVIEW

This section provides a description of the AHA-1540A/1542A hardware functional

interface to the PC/AT host software.

The hardware consists of a custom SCSI protocol chip, the AIC-6250, a FIFO

buffer, a set of I/O ports, and a controlling microprocessor.

The DMA control logic on the host adapter controls the AT bus arbitration and

data transfer hand shaking. During DMA data transfers, the AHA-1540A/1542A

becomes a bus master. The DMA logic supports both odd and even starting

addresses. For odd starting addresses, the first transfer will be an 8-bit

transfer. Thereafter, 16 bit transfers will be used to complete the data

transfer. Odd-byte transfers, and the the last transfer of an even number of

bytes to an odd address are treated in a similar manner.

The SCSI port is controlled by the AIC-6250, an Adaptec SCSI protocol device

which supports arbitration, selection, and reselection with a minimum of

processor intervention. This VSLI device also supports target mode and

synchronous SCSI transfers.

4.2 I/O PORT INTERFACE

The I/O port interface consists of three address locations. These three port

addresses are decoded in the AT I/O address space. They form the primary

communications channel between the host and the host adapter. The I/O ports are

8 bits wide. The base port is for control and status, the second port for

command and data transfer, and the third port for interrupt flags.

I/O PORT INTERFACE BIT DEFINITION

PORT ADDRESS = BASE + 0

WRITE:  Control Register        READ:   Status Register

Bit 7 Hard Reset (HRST)         Bit 7 Self Test in Progress (STST)

Bit 6 Soft Reset (SRST)         Bit 6 Internal Diagnostic Failure (DIAGF)

Bit 5 Interrupt Reset (IRST)    Bit 5 Mailbox Initialization Required (INIT)

Bit 4 SCSI Bus Reset (SCRST)    Bit 4 SCSI Host Adapter Idle (IDLE)

Bit 3 Reserved  (0)             Bit 3 Command/Data Out Port Full(CDF)

Bit 2 Reserved  (0)             Bit 2 Data In Port Full (DF)

Bit 1 Reserved  (0)             Bit 1 Reserved ( Undefined )

Bit 0 Reserved  (0)             Bit 0 Invalid H A  Command (INVDCMD)

PORT ADDRESS = BASE + 1

WRITE:  Command / Data Out       READ:  Data In

Bit 7 Command/Data Out Bit 7     Bit 7 Data In Bit 7

Bit 6           "      Bit 6     Bit 6     "   Bit 6

Bit 5           "      Bit 5     Bit 5     "   Bit 5

Bit 4           "      Bit 4     Bit 4     "   Bit 4

Bit 3           "      Bit 3     Bit 3     "   Bit 3

Bit 2           "      Bit 2     Bit 2     "   Bit 2

Bit 1           "      Bit 1     Bit 1     "   Bit 1

Bit 0           "      Bit 0     Bit 0     "   Bit 0

PORT ADDRESS = BASE + 2

WRITE:  Reserved, do not write			READ:	Interrupt Flags

Bit 7                            Bit 7 Any Interrupt

Bit 6                            Bit 6 Reserved

Bit 5                            Bit 5 Reserved

Bit 4                            Bit 4 Reserved

Bit 3                            Bit 3 SCSI Reset Detected (SCRD)

Bit 2                            Bit 2 HA Command Complete (HACC)

Bit 1                            Bit 1 MBO Empty (MBOA)

Bit 0                            Bit 0 MBI Full (MBIF)

4.2.1 Control and Status Port

Writing a one to the bits of the Control Port initiates certain special host

adapter operations.   There is no requirement to return the bits to the zero

state, since they are reset automatically after the requested operation is

initiated. Read operations to the Status port address return host adapter

status information.

Base+0 Port, Write:  Host Adapter Control Port.

Bit 7-Hard Reset  (HRST): The setting of the Hard Reset bit to one forces the

host adapter into a state identical to a normal power on state. Diagnostic

functions are executed and all status for ongoing SCSI operations is lost. A

Reset Condition is generated on the SCSI bus. While the reset is being

processed, the Self Testing in Progress bit (Host Adapter Status Port bit 7) is

set. When the reset is complete, that bit is reset and the Mailbox

Initialization Required bit (Host Adapter Status Port bit 5) and the SCSI Host

Adapter Idle bit (Host Adapter Status Port bit 4) are set, indicating that the

AHA-154XA mailbox structure must be re-initialized and that no other operations

are active on the host adapter. See section 4.3, describing the overall reset

structure.

Bit 6-Soft Reset (SRST): The Soft Reset bit clears all ongoing SCSI and host

adapter commands. All Command Control Blocks are abandoned and all queued

commands are abandoned. Mailbox In and Mailbox Out entries must be cleared by

the host. No diagnostic functions are executed. No Reset Condition is generated

on the SCSI bus.  The Mailbox Initialization Required bit (Host Adapter Status

Port bit 5) and the SCSI Host Adapter Idle bit (Host Adapter Status Port bit 4)

are set when the reset processing is completed. This indicates that the AHA-

154XA Mailbox structure must be re-initialized and that no other operations are

active on the Host Adapter. See section 4.3, describing the overall reset

structure.

Bit 5-Interrupt Reset (IRST): The setting of this bit clears the interrupt port

of all bits that have been set and resets the interrupt line. The host adapter

manages the interrupt presentation to minimize the possibility of incorrectly

resetting an interrupt. MBOE and MBIF interrupts are presented immediately

unless an SCRD or HACC interrupt has not yet been cleared. An SCRD or HACC will

only be presented after any interrupt bit has been cleared and DF is zero,

indicating an operation is fully completed. The prompt resetting of MBOE and

MBIF interrupts minimizes the chance of a reset of one also resetting the

other. Host programs should, however, be aware that there is a small chance of

falsely resetting a new MBIF reset while clearing an MBOE interrupt. This can

be resolved by periodically scanning the MBI entries when activity is expected

on the host adapter or by not enabling the MBOE interrupt.

Bit 4-SCSI Bus Reset (SCRST):The setting of this bit causes a SCSI Bus Reset to

be generated on the SCSI bus. The SCSI Bus Reset is triggered at the time the

SCRST bit is set to one and raises the RST line on the SCSI Bus for the

architected 25 microsecond period. The reset is managed as a SCSI Soft Reset

and will allow partially completed operations to continue after the reset

occurs. See section 4.3, describing the overall reset structure.

Bits 0-3 -Reserved: Reserved bits must be set to zero to avoid compatibility

problems with future extensions of the Control Register.

Base+0 Port, Read: Host Adapter Status Port

Bit 7-Self Testing in progress (STST): This bit, when one, indicates that the

host adapter is performing self-initialization and internal diagnostics. The

bit is asserted after a power-on or hard reset (Control Port Bit 7 HRST = 1).

When diagnostic operation is complete,  the STST bit is set to zero and bit 5

or bit 6 is set to indicate the successful or unsuccessful completion of the

diagnostics. If bit 7 remains on, it indicates that the initialization or

diagnostic could not be completed. The error condition can be determined as

described in section 10. In most cases, bit 6 (DIAGF) will be set to indicate

that an internal diagnostic failure occurred.

Bit 6-Internal Diagnostic Failure (DIAGF): This bit, when one, indicates that

the self-testing process has completed and that an error was detected. The host

adapter must be reset by setting the Hard Reset bit (bit 7 of the Control Port)

to clear the error. If the AHA-154XA again detects an error, troubleshooting

procedures must be performed to identify and correct the error condition. The

diagnostic LED will usually present a flash code that indicates the nature of

the failure. Section 10 describes the corrective procedures.

Bit 5-Mailbox Initialization Required (INIT):This bit, when one, indicates that

the self-testing process has completed successfully and that the AHA-154XA is

ready for mailbox initialization to be performed. The base memory address of

the Mailbox area must be established by execution of the Mailbox Initialization

command. After execution of the Mailbox Initialization command and any other

desired initialization operations, the AHA-154XA is ready for full operation.

Bit 4-SCSI Host Adapter Idle (IDLE):This bit, when one, indicates that the host

adapter is in the idle state. The host adapter has no outstanding Adapter

Commands or SCSI commands. The host processor must wait for the idle state

before executing any Adapter Command except the Start SCSI (02) and Enable

Mailbox Out Interrupt (05) commands.

Bit 3-Command / Data Out Port Full (CDF): The host uses the CDF bit to

synchronize command and data transfers to the host adapter. An Adapter Command

byte or an outbound parameter byte can be placed in the Command / Data Out Port

when the port is empty, indicated by the CDF bit being zero. When a byte is

placed in the Command / Data Out Port, the CDF bit is set to one and remains

one until the host adapter has obtained and processed the byte. When the CDF

bit returns to zero, the next Command or parameter byte can be placed in the

port.

Bit 2-Data In Port Full (DF): The host uses the DF bit to synchronize transfers

of data from the host adapter to the host. When the DF bit is set to one, the

host adapter has placed a byte in the Data In Port for the host to remove and

process. When the host performs a read to the Data In Port address, the DF bit

is set back to zero automatically and not set to one again until a new data

byte has been placed in the Data In Port by the host adapter for the host.

Bit 1-Reserved: This bit is zero.

Bit 0-Invalid Host Adapter Command (INVDCMD): The Invalid Host Adapter Command

bit is set to one if an invalid command or parameter byte was received in the

Command / Data Out Port . After sending a command byte or data byte, the host

software determines that the next byte is ready to send by waiting for the CDF

bit to be reset.  If the command byte or parameter byte is not valid, the

command sequence will instead be terminated by the host adapter. The host

adapter always terminates a command by raising the Host Adapter Command

Complete (HACC) interrupt. If the HACC interrupt is set to one and the INVDCMD

bit is not  set, the command terminated normally. If the INVDCMD bit is also

set to one, the command or parameter bytes were determined to be invalid and

the command terminated abnormally. The INVDCMD bit is only valid from the time

the HACC interrupt is set until the HACC interrupt  is reset. The bit's value

is not predictable until a new HACC interrupt is set for a new Adapter Command.

4.2.2 Command/Data Out and Data  In Port

The second I/O port address is used by the host to write Adapter Command bytes

and accompanying parameter data bytes to the host adapter. It is also used by

the host adapter to send parameters back to the host to be read. The Command /

Data Out Port is used by the host to send host adapter initialization and

management commands and parameters that cannot be sent by the standard Mailbox

protocol. Information requested by the Adapter Commands placed in the Command /

Data Out Port is returned through the Data In Port. The host should understand

the format and number of bytes to be transmitted for each command so that extra

invalid bytes are not passed across the interface.  Bytes in addition to those

required by the particular command are likely to be interpreted as invalid,

although they may instead cause the execution of valid commands that were not

supposed to be performed.

The host should only write to the Command / Data Out Port when CDF is zero.

This allows time for the host adapter to process a previously written command

or parameter byte. CDF is automatically set to one when the host writes to the

port and is reset to zero after the host adapter removes the byte from the

port.

If an Adapter Command needs additional data bytes, the host waits until CDF is

zero before writing the additional bytes to the Command / Data Out Port. Just

as in the command transfer case, each parameter byte written will set the CDF.

The host can write additional data bytes only after CDF is again zero. The HACC

interrupt will indicate when the command has terminated, normally or

abnormally. If INVDCMD is also set, the host adapter found either the command

or data bytes to be invalid and terminated abnormally. The use of CDF as a

hand-shaking bit is required to prevent the transfer of invalid data.

If an Adapter Command requires data transfer from the host adapter, the host

adapter will place the data bytes in the Data In Port and set the DF bit

(Status Port bit 2) to indicate that the requested parameter is ready for the

host to read. When the host reads the Data port, DF is automatically reset. The

host should wait until DF is again set before attempting to transfer the next

parameter byte. The use of the DF bit to control the hand-shaking process is

required to prevent the transfer of invalid data. After the last data byte has

been transferred, the HACC interrupt bit will be set indicating command

completion. If the Adapter Command was invalid, the HACC interrupt will occur

before all data bytes have been transmitted and the INVDCMD bit will be set.

4.2.3 Interrupt Flag Port

The Interrupt Flag Port contains bits that indicate the reason that an

interrupt was provided to the host from the host adapter. The host adapter uses

the interrupt to notify the host that the host adapter is ready for immediate

service from the host. The Interrupt Flag Port is a read-only port. When an

interrupt bit is set by the host adapter to indicate that the host should

respond, the Any Interrupt bit and the interrupt line are both also set. When

the host begins to examine the returned registers and mailboxes to determine

the cause of the interrupt and to perform the operations needed to service the

interrupt, the host will first read the Interrupt Flag Port to record which

interrupts must be serviced. The host will then clear the interrupts by setting

the IRST bit (Host Adapter Control Port bit 5). The host adapter presents MBOA

and MBIF interrupts immediately unless there is already an SCRD or HACC

interrupt present. If the SCRD or HACC is present, the MBOA and/or MBIF

interrupt will be posted after the SCRD or HACC interrupt is cleared. An SCRD

or HACC interrupt will only be presented if the Any Interrupt signal is zero

and the DF signal is zero, indicating the completion of all pending interrupt

presentation. It is recommended that the MBOA interrupt be enabled only when

required by the host. This prevents the possible presentation and resetting of

MBIF interrupts before they are processed. Other reset operations will also

reset the Interrupt Flag Port and the interrupt line, including the Hard Reset

bit (HRST), the Soft Reset bit (SRST), and the power on reset issued by the

motherboard.

Base+2 Port, Read only: Interrupt Flag Port

Bit 7-Any Interrupt: This bit, when one, indicates that  the interrupt to the

host has been established. The actual identify of the interrupting condition is

provided in bits 0 to 3.

Bit 6-Reserved: Returned as zero.

Bit 5-Reserved: Returned as zero.

Bit 4-Reserved: Returned as zero.

Bit 3-SCSI Reset Detected (SCRD): This bit, when one, indicates that a SCSI

Reset has been received on the SCSI bus. The Any Interrupt bit and the AT

interrupt signal will also be set. The host adapter supports SCSI Soft Reset

(see section 4.3). Any ongoing target or initiator activities will continue

normally after first clearing the SCSI bus. In some rare cases, host

intervention will be required to restart a SCSI command that was aborted by the

reset operation. The host can convert the SCSI Soft Reset to a SCSI Hard Reset

by setting the Control Register Soft Reset (Bit 6) to one, clearing all ongoing

operations in the host adapter. In this case, the host must recognize that any

operations not yet completed will never be completed and must perform

appropriate error recovery operations. See section 4.3, describing the overall

reset protocol. The SCRD bit is not set for host initiated SCSI Reset

conditions caused by the setting of the HRST bit  or the SCRST bit, since the

host is already aware of the actions it has requested. If the Any Interrupt

signal or DF signal is present, the SCRD interrupt will not be presented until

the interrupts already present are cleared.

Bit 2-Host Adapter Command Complete (HACC): This bit, when one, indicates that

an Adapter command has been completed, normally or abnormally. The Any

Interrupt bit and the AT interrupt signal will also be set. If the command was

completed normally, only the HACC bit will be on. If the command was completed

abnormally or was aborted before it was completed, the HACC bit will be one and

the Invalid Command Bit (Status Register bit 5) will also be one. During

parameter transfers to or from the host adapter, the HACC bit should be

examined to verify  that  the command is still being processed and has not been

ended abnormally. If the Any Interrupt signal or DF signal is set, the HACC

interrupt will not be presented until the interrupts already presented are

cleared.

Bit 1-Mailbox Out Available (MBOA): This bit, when one, indicates that an

outbound Mailbox entry is now available for use by the host. The Any Interrupt

bit and the AT interrupt signal will also be set. Most operating systems will

choose to leave this interrupt disabled to avoid the generation of extra

interrupts. The host adapter will normally empty Mailbox Out entries to its

local RAM so rapidly that round robin filling of the Mailbox Out entries will

assure that a Mailbox Out entry will already be empty by the time that the host

is ready to fill it again.

If the host finds that all Mailbox Out entries are full, it can enable the

Mailbox Out Available interrupt by executing an Enable Mailbox Out Interrupt

command through the I/O Command Port. The Enable Mailbox Out Interrupt command

is one of the two commands that can be executed without waiting for the IDLE

state of the host adapter. As soon as any Mailbox Out entry is cleared by the

host adapter, an MBOA interrupt will be generated to indicate to the host that

an MBO entry is available. An MBOA interrupt is generated after that each time

a Mailbox Out entry is cleared by the host adapter until an Enable Mailbox Out

Interrupt command is executed to force the reporting of MBOA interrupts to be

disabled. If the SCRD or HACC interrupts are present, the MBOA interrupt is not

presented until they are cleared. At all other times, MBOA is presented

immediately.

Bit 0-Mailbox In Full (MBIF): This bit, when one, indicates that an entry has

been placed by the host adapter in the Mailbox In.  The interrupt should be

reset as soon as possible so that any subsequent interrupts can be detected.

The host adapter may return information in other Mailbox In entries, so the

host should check the next entry to determine if more than one set of

information has been provided. The MBI entries are filled in round-robin order,

so the host should simply check the next MBI entry after the last one that was

found when a new MBIF interrupt occurs.

If an MBIF interrupt is set and other Mailbox In entries are made before the

interrupt is cleared, then all the entries can be scanned as found. The new

MBIF interrupt will be presented if the SCRD and HACC interrupts are cleared. A

new MBIF interrupt will be presented regardless of the state of the MBOA

interrupt bit. It is important to clear and record each interrupt as soon as

possible to avoid the possible accidental resetting of a valid interrupt. In

addition, it is desirable to enable the MBOA interrupt as rarely as possible.

The host system software must be ready to scan for MBI entries even if no MBIF

interrupt occurred.

4.3 RESET OVERVIEW

The reset functions provided by the AHA-154XA are extensive to allow the

fullest flexibility and architectural consistency, both with SCSI and with the

Industry Standard Architecture.

4.3.1 Hard Reset Operations

Resets may be generated by the hardware through two mechanisms. The RESET

signal from the ISA socket is generated by the system board to reset or

initialize all installed adapters upon power on or during a low line voltage

condition. The system can also activate this signal under host program control.

In addition to the RESET signal, the setting of the HRST bit (bit 7 of the Host

Adapter Control Port) will force a hardware reset to the AHA-154XA. Regardless

of the source, a Hard Reset forces the following actions on the AHA-154XA.

All  internal registers of the AHA-154XA are returned to their reset

condition.

The host adapter's microprocessor returns all internal information

and parameters to their initial state.

The host adapter performs all required internal diagnostics.

A standard SCSI Reset condition is generated to all other attached SCSI

devices.

During the Hard Reset process, the AHA-154XA will indicate to the attached host

that self-testing is in process by raising the STST bit in the Host Adapter

Status Port. After the Hard Reset process is complete, the AHA-154XA will

indicate that initialization parameters are required from the host by raising

the INIT bit in the Host Adapter Status Port.

4.3.2 SCSI Reset Operations

The SCSI Reset condition is defined in the SCSI Standard, X3.131-1986, section

5.2.2. A SCSI Reset condition may be forced by any SCSI device on the bus. The

condition is forced by the assertion of the SCSI Reset signal.

The AHA-154XA has four mechanisms which may force a SCSI Reset condition. The

SCSI Reset condition may be invoked from the host software if the software sets

the SCRST bit (Bit 4 of the Host Adapter Control Port). In this case, the

normal SCSI Reset operations will be performed by the Host Adapter and the SCSI

Reset signal will be asserted on the SCSI bus. In this case, the SCRD bit

(Interrupt Flag Port bit 3) will not be set, since the host itself caused the

reset.

The SCSI Reset is invoked if a hard reset is generated, either by the system

board RESET signal or by the setting of the HRST bit. This reset is described

in Section 4.3.1.

The SCSI Reset condition may be invoked by the AHA-154XA as part of the

recovery mechanism for a bus phase error. Bus phase errors may include

detection of an invalid information transfer phase or detection of an

impossible phase sequence (Command Phase after a Data Phase in the same

command). In this case, the normal SCSI Reset operations will be performed and

the SCSI Reset signal will be asserted on the SCSI bus. In addition,  the SCRD

bit (Interrupt Flag Port bit 3) will be set to indicate to the host computer

that a SCSI Reset condition occurred. The setting of the SCRD bit also causes

the Any Interrupt bit (Interrupt Flag Port bit 7) and the appropriate interrupt

signal  to be presented.

The SCSI Reset condition may be invoked by another SCSI device attached to the

AHA-154XA as part of the other device's recovery mechanism or initialization

procedure. In this case, the normal SCSI Reset operations will be performed. In

addition,  the SCRD bit (Interrupt Flag Port bit 3) will be set to indicate to

the host computer that a SCSI Reset condition occurred. The setting of the SCRD

bit also causes the Any Interrupt bit (Interrupt Flag Port bit 7) and the

appropriate ISA interrupt  signal to be presented.

4.3.2.1 SCSI "Soft" Reset Option

The SCSI standard indicates two optional methods of handling the normal SCSI

Reset operations. The AHA-154XA implements the "Soft" Reset Option, described

in section 5.2.2.2 of the SCSI specification. The "Soft" Reset Option is

designed to allow a SCSI Reset signal to correctly clear the SCSI bus, but to

allow ongoing system operations to continue without any major interruptions.

When the SCSI Reset occurs, any activity on the SCSI bus is immediately halted

and all bus lines are cleared from the bus. After the Reset condition ends, any

operations in progress are again allowed to start up in the normal manner. No

status or pointer information is destroyed. All disconnected commands are

allowed to reselect and continue in the normal manner. This "Soft" Reset

function allows a multiple initiator system to use reset to clear certain types

of bus failures without damaging ongoing tasks from any initiator.

The SCSI "Soft" Reset option is useful in multitasking systems that cannot

tolerate the overhead of a complex reconfiguration and reinitialization after a

normal reset operation. For the SCSI "Soft" Reset option to operate correctly,

all SCSI devices that communicate on the SCSI bus must support the "Soft" Reset

option. If any SCSI devices support the "Hard" Reset option, it is likely that

operations will be terminated without warning and the system will have to time

out and monitor the requirement to restart some activities.

4.3.2.2 SCSI "Hard" Reset Option

The "Hard" Reset Option is designed to restore all attached SCSI devices,

including both hosts and peripheral devices, to their power on reset state. All

system activities that have not been recorded on a non-volatile memory device

or through another SCSI path are completely lost and must be restarted. The

system must be completely re-initialized. For certain types of systems that

frequently do back-up or check point their transactions, that re-initialize

quickly and easily, or that infrequently do resets, the "Hard" Reset Option is

appropriate. The AHA-154XA responds to a SCSI Reset Condition by executing only

the "Soft" Reset Option, but it notifies the host whenever a SCSI Reset

condition has been established by causing an interrupt to the host. The host

then has the option of converting the "Soft" Reset to a "Hard" Reset by forcing

the host adapter to clear all the ongoing operations and return to its initial

state. The host requests this by raising the SRST bit (bit 6 of the Host

Adapter Control Port). The host must raise the SRST bit within 300 microseconds

to disable the restarting of operations according to the rules of "Soft" Reset.

The raising of the SRST bit causes the host adapter to abandon all CCB's and

prepare itself to begin accepting new instructions from the host. No secondary

SCSI Reset signals are activated. The Mailbox initialization and all normal

SCSI initial conditions are reset by the SRST bit, so that reinitialization is

required to restart the system. Of course the system still has the right at any

time that the IDLE bit (bit 4 of the Host Adapter Status Port) is on to execute

any of the Adapter Commands and modify the Mailbox Address or the SCSI initial

conditions.

If the host requires that the SCSI be reset according to the SCSI "Hard" Reset

option, the host raises the HRST bit. The host adapter will then set a SCSI

Reset Condition on the SCSI Bus and clear all its CCB and status information,

thus performing a SCSI "Hard" Reset with a single load to the Host Adapter

Control Port. Reinitialization will be required.

5.1 HOST ADAPTER COMMAND OVERVIEW

The AHA-1540A/1542A supports two types of commands: SCSI and Adapter Commands.

SCSI commands are issued using the mail box protocol and a Command Control

Block. When SCSI commands are used, the AHA-1540A/1542A is operating in true

multithreading mode. In this mode of operation, the AHA-1540A/1542A is capable

of executing multiple commands for multiple targets concurrently. The AHA-

1540A/1542A maximizes the I/O transaction throughput by managing SCSI

disconnection and reconnection.

Adapter Commands are issued by writing to the Command / Data Out port. Most

Adapter commands can not be issued when there are outstanding SCSI commands.

Adapter commands are used to initialize the host adapter and to establish

control conditions within the host adapter. Adapter commands are also used to

transmit the special parameters for communication between the BIOS and the host

adapter for the execution interrupt 13 operations.

5.1.1 Adapter Command Operation Codes

Below is a list of the Adapter Command operation codes.

00 -- No Operation

01 -- Mailbox Initialization

02 -- Start SCSI Command *

03 -- Start PC/AT BIOS Command

04 -- Adapter Inquiry

05 -- Enable Mailbox Out Available Interrupt*

06 -- Set Selection Time-out

07 -- Set Bus-On Time

08 -- Set Bus-Off Time

09 -- Set Transfer Speed

0A -- Return Installed Devices

0B -- Return Configuration Data

0C -- Enable Target Mode **

0D -- Return Setup Data **

1A -- Write Adapter Channel 2 Buffer **

1B -- Read Adapter Channel 2 Buffer **

1C -- Write Adapter FIFO Buffer

1D -- Read Adapter FIFO Buffer

1F -- ECHO Command Data

*Note: This command can be issued when the host adapter is executing a SCSI

command.

** These commands are not supported by the AHA-1540.

All Adapter Commands except Start SCSI (02) and Enable Mailbox-Out Available

Interrupt (05) must be executed only when the IDLE bit (Status bit 4) is one.

Many commands require additional parameter bytes which are then written to the

Command / Data Out I/O port (base + 1). Before each byte is written by the host

to the host adapter, the host must verify that the CDF bit (Status bit 3) is

zero, indicating that the command port is ready for another byte of

information. The host adapter usually clears the Command / Data Out Port within

100 microseconds. Some commands require information bytes to be returned from

the host adapter to the host. In this case, the host monitors the DF bit

(Status bit 2) to determine when the host adapter has placed a byte in the Data

In I/O port for the host to read. The DF bit is reset automatically when the

host reads the byte. The format of each Adapter Command is strictly defined, so

the host adapter and host system can always agree upon the correct number of

parameter bytes to be transferred during a command.

All  Adapter Commands except Return Installed devices, Start SCSI, and Start

PC/AT BIOS typically require less than 200�s to complete. Return installed

devices will typically complete in less than 3 seconds. Start SCSI and Start

PC/AT commands completion times will vary with the SCSI device and the command

issued.

5.1.1.1 No Operation  (Operation Code 00)

No host adapter action is taken, but HACC is set indicating command completion.

No additional information bytes are exchanged.

5.1.1.2 Mailbox Initialization (Operation Code 01)

This command is used to specify the number of mailbox locations used by the

host adapter and to specify the base memory location of the mailbox area. The

host adapter requires that four bytes of outbound data  follow the command

byte. The definition of those four bytes is shown below.

00  Mailbox count  (>0)

01  Mailbox address (MSB)

02  Mailbox address

03  Mailbox address (LSB)

Byte 0 Mailbox Count: The Mailbox count must be greater than zero.

Byte 1,2,3 Mailbox Address: The Mailbox address specifies the location of the

first byte of the Mailbox area. Byte 1 is the most

significant byte.

When the Mailbox Initialization command and parameters are received,  the host

adapter will then assume that the specified number of Mailbox Out entries and

the same number of Mailbox In entries will be assigned beginning at the Mailbox

address. The total number of bytes reserved for the Mailbox area will be eight

times the Mailbox count. If the Mailbox Count is zero, the INVDCMD bit will be

set with HACC to indicate that the parameter is invalid.

At command completion, HACC will be set to one and INIT will be reset to zero.

HACC will be reset as specified by section 4.2.3.

5.1.1.3 Start SCSI Command (Operation Code 02)

This command indicates that the host has made at least one Mailbox Out entry

and that the host adapter should begin to scan for active MBO entries. Once

scanning has been started, it continues until all MBO entries have been

serviced, either by beginning the requested operations or by queueing the

activities for later execution.  Since it is not easy for the host to determine

that scanning is still taking place, the host should normally issue this

command every time a Mailbox Out is filled. This command does not require

additional data bytes. To avoid unnecessary interrupts, HACC is NOT set after

command completion. HACC will be set with INVDCMD if the mailbox was not yet

initialized.

5.1.1.4 Start PC/AT BIOS Command (Operation Code 03)

This command is used by the Adaptec BIOS to communicate with the host adapter

firmware. The command is not available for use by programs other than the

Adaptec BIOS.

5.1.1.5 Adapter Inquiry (Operation Code 04)

After receiving this command, the host adapter returns four bytes of data

describing the host adapter firmware revision level. After completing the

transfer of the four bytes of inbound data, the HACC interrupt is set

indicating normal command completion. The bytes contain the following

information:

00  Board Identification

01  Special Options Identification

02  Hardware Revision Level

03  Firmware Revision (ASCII)

Byte 0 Board ID: The value in this byte allows software supported on both the

PC/AT and on the Microchannel(tm) to distinguish the type of

supporting host adapter.

Value             Meaning

00H- board is an AHA-1540 with 16 Head BIOS

30H- ('0' ASCII) Board is an AHA-1540 with 64 Head BIOS

41H- ('A' ASCII) Board is an AHA-1540A/1542A, 64 Head BIOS

42H- ('B' ASCII)	Board is an AHA-1640, 64 Head BIOS

All Others- Reserved

Byte 1  Special Options ID: The value in this byte indicates what special

options are supported on the AHA-154XA host

adapter. Other host adapters use other values in

this byte.

Value              Meaning

41H- ('A' ASCII)	Board is standard model

All Others- Reserved

Byte 2 Firmware Revision: This value indicates an ASCII value from 0-9,A-Z that

indicates the firmware revision installed in the AHA-

154XA.

Byte 3 Firmware Revision: This value indicates an ASCII value from 0-9,A-Z that

indicates the firmware revision installed in the AHA-

154XA.

After completing this command, HACC will be asserted, indicating normal

completion.

5.1.1.6 Enable Mailbox-Out Available Interrupt (Operation Code 05)

The Enable Mailbox Out Available Interrupt command specifies that a Mailbox Out

Available Interrupt should be generated whenever a Mailbox Out entry has been

cleared by the host adapter. One byte of outbound data is transmitted to

indicate whether the interrupt should be enabled or disabled.

Byte 0 Enable/Disable Parameter: The Enable/Disable parameter byte must be

either 00H or 01H. If the Enable/Disable

parameter byte is 00H, the Mailbox Out

Available interrupt  is not returned. If the

Enable/Disable parameter byte is 01H, the

Mailbox Out Available interrupt is returned as

soon as a Mailbox Out has been cleared by the

host adapter.

After completing this command, HACC will NOT be asserted to avoid generating

additional interrupts. If the data byte is neither 00 nor 01, INVDCMD will be

asserted indicating an invalid command. In this case, HACC will also be

asserted.

The Mailbox Out Available interrupt is normally intended to be used by the host

as an indicator that a Mailbox Out entry is available. This function should

only be enabled if all Mailbox Out entries have been found full by the host. It

should be disabled soon afterward. If used in other ways, the interrupt may

generate a large number of relatively useless interrupt handling activities.

5.1.1.7 Set Selection Time-out  (Operation Code 06)

This command sets the SCSI selection time out value. The SCSI Selection time

out value is used to determine whether or not a SCSI selection was successful.

If the SCSI BSY signal is not returned within the specified time out value, the

selection will be terminated and an appropriate error message posted with the

returned CCB. This command expects four outbound data bytes to be provided as

defined below:

00 Enable/Disable Selection Time Out

01 Reserved  (00)

02 Time Out Value (MSB)

03 Time Out Value (LSB)

Byte 0- Enable/Disable Selection Time Out: This parameter byte specifies whether

or not the Selection Time-out will be used. If the byte is set to 00H,   no

time-out will be performed. If the byte is set to 01H, the time specified in

bytes 02 and 03 will be used as the Selection Time Out by the SCSI. The default

value established by the reset process is 01H, indicating the time out is

enabled.

Byte 1-Reserved:	This byte must be zero.

Byte 2,3-Time Out Value:	This two-byte value specifies the time in milliseconds

that will be used for the Selection Time Out. The default value is 250

milliseconds.

After completing this command, HACC will be asserted, indicating normal

completion. INVDCMD will be asserted if the data byte 0 is neither 00 nor 01 or

if byte 1 is not zero, indicating an invalid command.

5.1.1.8 Set Bus-On Time (Operation Code 07)

This command specifies the time that the host adapter spends on the bus when

transferring data. The Bus-On duration is adjustable from 2 to 15 microseconds.

The default setting is 11us. One data byte is passed out to the host adapter to

indicate the Bus-On duration in microseconds.

Data Byte 0 -- Bus On-Time (2 to 15 us)

After completing this command, HACC will be asserted indicating normal

completion. INVDCMD will be asserted, indicating an invalid command, if the

data byte is greater than 15. The valid range is 2 to 15 decimal.

5.1.1.9 Set Bus-Off Time (Operation Code 08)

This command sets the time that  the host adapter will spend off the bus during

a data transfer. The Bus-On duration is adjustable from 1 to 64 microseconds.

The default setting is 4us. After receiving this command, the host adapter

expects one byte of data which specifies the bus off time in microseconds.

Data Byte 0 -- Bus-Off Time ( 1 to 64 us)

After completing this command, HACC will be asserted indicating normal

completion. INVDCMD will be asserted, indicating an invalid command, if the

data byte is greater than 64. The valid range is 1 to 64 decimal. The actual

time implemented by the host adapter is rounded down to the next four

microsecond step at or below the specified value. The minimum value is

approximately one microsecond.

5.1.1.10 Set Transfer Speed (Operation Code 09)

This command adjusts the Bus Master DMA circuitry to a specified maximum AT bus

transfer speed to and from the host memory. Several speeds may be selected by

setting the jumper configuration as explained in section 3.3. The selected

jumper speeds are overridden if the Set Transfer Speed command is executed. The

single data byte transmitted after the command byte either sets the read and

write speed together, or establishes separate values for each. Appendix A gives

the timings that result from the selected values. The default setting is the

value set by the jumpers. The I/O Channel Ready signal automatically slows the

system further if required by the host memory.

Data Byte 0 	-- AT Bus Transfer Speed

00 -- 5.0 MB/second

01 -- 6.7 MB/second

02 -- 8.0 MB/second

03 -- 10 MB/second

04-- 5.7 MB/second

80-FF-- See Table

After completing this command HACC is set indicating normal completion.

5.1.1.11 Return Installed Devices (Operation Code 0A)

This command returns information about which SCSI Targets and Logical Units

(LU's) are installed on the SCSI bus. The host adapter issues the SCSI Test

Unit Ready command to each target/LUN combination and reports the results using

eight bytes of data returned through the Data In Register. A bit having a value

of one indicates that the associated LU is installed.  Each byte is associated

with the  corresponding target. Each bit within a target byte is associated

with a particular Logical Unit, bit 7 indicating the presence of LU 7 and so

forth. The state of the target is analyzed using the SCSI TEST UNIT READY

command, analyzing the returned data to determine if the addressed LU is

available.

After receiving this command, the host adapter returns eight bytes of

information which specify the installed configuration  as shown below:

Byte 0          Target 0 Configuration:

Bit 7 LU 7 Installed

Bit 6 LU 6 Installed

Bit 5 LU 5 Installed

Bit 4 LU 4 Installed

Bit 3 LU 3 Installed

Bit 2 LU 2 Installed

Bit 1 LU 1 Installed

Bit 0 LU 0 Installed

Byte 1 Target 1 Configuration

Byte 2 Target 2 Configuration

Byte 3 Target 3 Configuration

Byte 4 Target 4 Configuration

Byte 5 Target 5 Configuration

Byte 6 Target 6 Configuration

Byte 7 Target 7 Configuration

The byte associated with the SCSI Device Identifier of the host adapter will

always be zero. After completing the information transfer, the HACC bit will be

set to indicate normal completion.

5.1.1.12 Return Configuration Data (Operation Code 0B)

The DMA arbitration priority, the Interrupt channel, and the SCSI ID of the

adapter are returned by this command. The three bytes of information are

returned in the following format:

Byte 0 DMA Arbitration Priority:

Bit 7 Channel 7

Bit 6 Channel 6

Bit 5 Channel 5

Bits 4-1 Reserved (0)

Bit 0 Channel 0

Byte 1 Interrupt Channel:

Bit 7 Reserved (0)

Bit 6 Interrupt channel 15

Bit 5 Interrupt channel 14

Bit 4 Reserved (0)

Bit 3 Interrupt channel 12

Bit 2 Interrupt channel 11

Bit 1 Interrupt channel 10

Bit 0 Interrupt channel 9

Byte 2	SCSI Identifier

Bits 7-3 Reserved (0)

Bits 2-0 Binary value of SCSI Identifier

After completing this command, HACC will be set indicating normal completion.

5.1.1.13 Enable Target Mode Command (Operation Code 0C)

A special host adapter command  (0C)  enables and disables target mode. The

host adapter requires that two bytes of outbound information follow the command

byte. The information bytes contain the following information:

Byte 0-  Enable/Disable Target Mode:	This parameter specifies which operating

modes the host adapter will use. If the byte is set to 00H, the AHA-1540A/1542A

will operate only in Initiator Mode. If the byte is set to 01H, the host

adapter will operate both as an initiator and as a Processor type Target SCSI

Device. Any other value is invalid. The default value after  a Hard Reset, Soft

Reset, or Power On Reset is 00H (Initiator only).

Byte 1-Logical Unit Mask: This parameter byte contains an eight bit bit-

significant mask indicating the Logical Units which will respond in Target

Mode.  Bit 7 corresponds to LU 7, and so forth. If the bit is one, the

corresponding LU will be treated as installed. If the bit is zero, the LU will

be treated as not installed.

If the AHA-1540A/1542A does not have the Target Mode feature installed, the

host adapter will indicate an invalid host adapter command. If an attempt is

made to change from target mode while there are still target mode CCB's being

processed by the host adapter, the host adapter will post an invalid host

adapter command indication.

If the command disables Target Mode, the Logical Unit Mask byte will be

ignored. If the command enables Target Mode, the Logical Unit Mask byte must

contain at least one bit, indicating the presence of at least one Logical Unit.

The SCSI INQUIRY command will provide an indication to other initiators that

the Logical Unit is installed or not installed  in byte 1 of the returned

inquiry data.

If target mode is not enabled, the host adapter will behave on the SCSI

interface as if it were an ordinary SCSI initiator. Any attempt to select the

host adapter will result in a SCSI selection time out. Most Reset Operation,

including Soft Reset, Hard Reset, and Power-On Reset will return the AHA-

1540A/1542A Target Mode to the disabled state. SCSI Resets, generated either by

the host or by other SCSI Devices, will not change the previously established

enabled or disabled state of Target Mode.

5.1.1.14 Return Setup Data (Operation Code 0D)

This command returns information describing the setup of the host adapter. The

information returned reflects either the values supplied by previous host

adapter commands or default values. The command is followed by an outbound data

transfer and an inbound data transfer. The outbound transfer is a one byte

parameter indicating the length of the required inbound data transfer. The

inbound data transfer contains from zero to 255 bytes of information describing

the setup of the host adapter. The inbound information normally transferred

will be truncated or padded with zeros as necessary to transfer the requested

number of bytes. The number of bytes normally transferred by the AHA-154XA is

16.

Outbound Data Byte:

Byte 0     Data In Length: The number of data bytes requested can be from 0 to

255. A value of zero will be accepted and 256 bytes

will be returned.

Inbound Data Summary:

00     SDT and Parity Status

01     Transfer Speed

02     Bus On Time

03     Bus Off Time

04     Number of Mailboxes

05     Mailbox Address (MSB)

06     Mailbox Address

07     Mailbox Address (LSB)

08-0F  Synchronous Transfer Agreements

10-FF  Reserved (00)

Inbound Data Bit Assignments:

Byte 0   SDT and Parity Status:

Bit      Meaning

0       If this bit is zero, Synchronous Data Transfer negotiation

will not be initiated by the host adapter, but will be

responded to if requested by an attached SCSI Device. If the

bit is one, Synchronous Data Transfer negotiation will be

initiated by the host adapter under those conditions which

require such negotiation. The state is set from jumper J1.

1       If this bit is zero, parity checking on inbound SCSI transfers

has been disabled. If this bit is one, parity checking on

inbound SCSI transfers is enabled. The state is set from

jumper J1.

2-7      Reserved (0)

Byte 1 Transfer Speed-This byte returns the value passed in to the host adapter

by the Set Transfer Speed command (See Section  5.1.1.10)

Byte 2 Bus On Time:Indicates the Bus On Time in microseconds

(See Section 5.1.1.8)

Byte 3 Bus Off Time:Indicates the Bus Off Time specified by the Bus Off Time

Value in microseconds. (See Section 5.1.1.9)

Byte 4- Number of Mailboxes:The number of Mailboxes established by  a previous

Mailbox Initialization command will be returned in this byte. This number will

be 00H if the Mailbox Initialization command has not yet been successfully

completed.

Byte 5-7 -Mailbox Address:The base address of the Mailbox area established by a

previous Mailbox Initialization command will be returned in these bytes. The

Most Significant byte is byte 5. These bytes have no meaning if Mailbox

Initialization has not yet been successfully completed.

Byte 8-Sync Neg, Target 0 Returns information about the synchronous negotiation

with Target 0. The byte will be 00H for the address of the host adapter.

Bit 7-Set to one if synchronous transfer is negotiated. Set to zero if

asynchronous transfer is being performed.

Bits 6-4 -These bits contain a value between 0 and 7 that defines the

synchronous transfer period according to the following equation. Period =200 +

50 (value) nanoseconds

Bits 3-0 -These bits contain the negotiated offset value. The value will

normally be between 1 and 7.

Byte 9 Sync Neg, Target 1	Same as byte 8, for target 1

Byte 10 Sync Neg, Target 2	Same as byte 8, for target 2

Byte 11 Sync Neg, Target 3	Same as byte 8, for target 3

Byte 12 Sync Neg, Target 4	Same as byte 8, for target 4

Byte 13 Sync Neg, Target 5	Same as byte 8, for target 5

Byte 14 Sync Neg, Target 6	Same as byte 8, for target 6

Byte 15 Sync Neg, Target 7	Same as byte 8, for target 7

If the command completes normally, the HACC interrupt will be set to one. If

the Mailbox area has not been properly initialized, all 16 bytes are still

requested by the host. If byte 4, the Number of Mailboxes, is zero, bytes 5-7

must be ignored.

5.1.1.15  Write Adapter Channel 2 Buffer (Operation Code 1A)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 64 bytes in

system RAM. The area pointed to will be transferred to the host adapter's

Channel 2 Buffer using the host adapter's DMA circuitry.  After completing the

transfer of the 64 bytes from the indicated buffer area to the host adapter,

the HACC interrupt will be set indicating normal completion. This command is

used in conjunction with the Read Channel 2 Buffer command for host  adapter

diagnostics. The Channel 2 Buffer is used for transmission of all information

except the actual data field between the host adapter and the host system.

Byte 0       Buffer area address, Most significant byte

Byte 1       Buffer area address

Byte 2       Buffer area address, Least significant byte

5.1.1.16  Read Adapter Channel 2 Buffer (Operation Code 1B)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 64 bytes in

system RAM.  The area pointed to will be used as a buffer to receive 64 bytes

of information transferred from the host adapter's Channel 2 Buffer to the

host's memory using the host adapter's DMA circuitry. After completing the

transfer of the 64 bytes from the Channel 2 Buffer to the indicated host memory

area, the HACC interrupt will be set indicating normal completion. This command

is used in conjunction with the Write Channel 2 Buffer command for host adapter

diagnostics.

Byte 0   Buffer area address, Most significant byte

Byte 1   Buffer area address

Byte 2   Buffer area address, Least significant byte

5.1.1.17 Write Adapter FIFO Buffer (Operation Code 1C)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 54 bytes in

system RAM. The area pointed to will be transferred to the host adapter's FIFO

buffer using the host adapter's DMA circuitry.  After completing the transfer

of the 54 bytes from the indicated buffer area to the host adapter, the HACC

interrupt will be set indicating normal completion. This command is used in

conjunction with the Read Adapter FIFO Buffer command for host  adapter

diagnostics.

Byte 0   Buffer area address, Most significant byte

Byte 1   Buffer area address

Byte 2   Buffer area address, Least significant byte

5.1.1.18 Read Adapter FIFO Buffer (Operation Code 1D)

After receiving this command, the host adapter expects three outbound

information bytes to be transferred which point to an area of 54 bytes in

system RAM.  The area pointed to will be used as a buffer to receive 54 bytes

of information transferred from the host adapter's FIFO to the host's memory

using the host adapter's DMA circuitry. After completing the transfer of the 54

bytes from the FIFO to the indicated buffer area, the HACC interrupt will be

set indicating normal completion. This command is used in conjunction with the

Write Adapter FIFO Buffer command for host adapter diagnostics.

Byte 0      Buffer area address, Most significant byte

Byte 1      Buffer area address

Byte 2      Buffer area address, Least significant byte

5.1.1.19 ECHO Command Data (Operation Code 1F)

This command is used to test the Command / Data Out Port, the Data In Port, and

the associated control bits in the other I/O Ports.  After receiving this

command, the host adapter expects one byte of outbound information to be

transferred through the Command / Data Out Port. The host adapter then sends

(Echos) the same data value back to the host through the Data In Port. After

the host has read the data value provided on the Data In Port, the host adapter

will generate the HACC interrupt to indicate normal command completion.

Byte 0, Out   Outbound Echo Value

Byte 0, In    Returned Echo Value

5.2 MAILBOX OVERVIEW

The AHA-1540A/1542A uses a mailbox architecture for task communication between

the host and the host adapter when executing SCSI commands. This allows the

host adapter to perform multithreaded operations with a minimum of host

intervention. The mailboxes are located in main system memory. Each mailbox

entry is four bytes long. At power up, the host issues an initialization

command to inform the host of the mailbox location. There are always an equal

number of Outgoing Mailboxes (MBO) and Incoming Mailboxes (MBI). The MBIs are

located immediately after the MBOs. Initiator operations and target operations

use the same mailboxes. Both initiator and target operations may be in process

at the same time. A typical mailbox structure is shown below.

In this example, there are four MBOs and four MBIs. The Mailbox Count in the

Mailbox Initialization command was set to four. The base address is the address

specified by the Mailbox Address field.

Note: The Adaptec BIOS initializes the AT DMA controller to accommodate Bus

Master DMA. If the host adapter BIOS is removed or disabled, the host DMA

controller must be initialized, via software, for Bus Master DMA operation. See

section 5.5.

5.2.1 Mailbox Out Definition

The first byte of each MBO contains the Mailbox status byte. The remaining

three bytes contain an address pointer to the first byte of a Command Control

Block (CCB). The CCB provides additional task control information. A MBO is

free if the first byte is zero. The host can make an entry in any free MBO and

indicate that it is filled out or completed by placing the proper MBO command

in the first byte of the MBO. After the MBO has been examined by the host

adapter and all relevant information obtained by the host adapter, the host

adapter sets the MBO command byte back to zero to allow the host to fill it

again. For a multitasking operating system, it is desirable that the number of

mailboxes be sufficient to allow at least one mailbox for each active

independent task or activity.

The MBO format is described in detail below:

Byte 0 MBO Command  This byte specifies the state of the MBO entry.

Value         Definition

00H        Mailbox Out is free

01H        SCSI or Host Adapter command is to be started. CCB pointer

indicates location of CCB to be processed.

02H        SCSI or Host Adapter command is to be aborted. CCB pointer indicates

location of CCB to be terminated.

Byte 1     CCB Pointer (MSB)

Byte 2     CCB Pointer

Byte 3     CCB Pointer (LSB)

The use of Mailbox Out (MBO) entries to pass pointers to CCB's is identical for

target and initiator modes. The appropriate target mode CCB may be prepared

early and posted to the host adapter in preparation for an operation that is

expected to happen. If a SCSI operation occurs before the CCB is prepared, the

host adapter processes as much of the transaction as possible, then requests a

CCB from the host through the MBI. The host is fully responsible for preparing

the correct CCB from the information provided through the MBI.

In target mode, one CCB may be presented for each unique combination of LUN,

Initiator, and direction. If a second CCB to the same LUN and initiator with

the same direction bit is sent to the AHA-1540A/1542A, the CCB will be returned

with a host status of 19H, Duplicate CCB Received.

Initiator type CCB's may be queued for an LUN and Target. The host adapter will

always search for new MBO entries in a round-robin order, beginning with the

entry after the last MBO entry that was processed. By always placing the MBO

entries in  the MBO area consecutively, the host can assure that the SCSI

commands will be started with a minimum scan overhead. Initiator CCB queueing

must be used with caution, since under some circumstances it is possible for

CCB's to be executed out of order.

If the Mailbox Out Available Interrupt is enabled by execution of Adapter

Command 05 (Enable Mailbox Out Available Interrupt), the host adapter will take

one of the following actions after each MBO entry is freed by the host adapter:

1)If the Any Interrupt bit in the Interrupt Flag Port is zero, indicating that

there are no interrupts pending, the host adapter will set the Mailbox Out

Available Interrupt and the Any Interrupt bit and indicate a hardware interrupt

to the host.

2)If the Mailbox Out Available Interrupt has been set to one by the previous

clearing of a Mailbox Out entry and the interrupt has not yet been cleared by

the host, the host adapter will not change the MBOA interrupt bit and will

continue to scan for other stored MBO entries.

3)If interrupts other than the Mailbox Out Available Interrupt are pending, the

host adapter will wait for the pending interrupts to be cleared before setting

the Mailbox Out Available Interrupt. This guarantees that the MBO Available

Interrupt will not be cleared accidentally by clearing another interrupt.

5.2.2 Mailbox In Definition

The Mailbox In entries are used to pass completion information concerning a

task from the host adapter to the host. In addition, requests for Target Mode

CCB's are passed to the host using MBI entries. The first byte of each MBI

contains the MBI Status byte, summarizing the type of information being passed

from the host adapter to the host. The remaining three bytes contain

specialized information that provides more detail about the information. In the

case of a CCB Completed MBI, the bytes contain a pointer to the completed CCB.

In the case of a CCB Required MBI, the bytes contain the information necessary

for the host to prepare an appropriate CCB. Only those MBI's with a MBI Status

that is non-zero have information for the host. When the host returns the MBI

Status byte to zero, the MBI Free state, the host adapter is allowed to place a

new set of information in the MBI entry.

When a SCSI command completes or if a new CCB is required, the host adapter

scans the first byte of an MBI entry to find a free mailbox. If one is found,

the host adapter will update the MBI's status byte with a non zero value and

update the following bytes with the appropriate required pointers or

parameters. The valid MBI formats are defined below:

Mailbox In Format for CCB Complete

Byte 0        MBI Status-This byte specifies the state of the MBI entry.

Value         Definition

00H           Mailbox In is free

01H           CCB completed without error. CCB pointer indicates location of

successfully completed CCB.

02H           CCB aborted by host. CCB pointer indicates location of CCB that

was aborted.

03H           Aborted CCB not found. CCB pointer indicates the supposed location

of the CCB that was to have been aborted. It is likely that the

CCB was already presented to the host before the Abort  CCB MBO

entry was completed.

04H           CCB Completed with Error. The CCB fields indicate the details of

the error condition. This code allows normal CCB completion to be

processed without bothering to examine the completion codes in

the CCB.

Byte 1        CCB Pointer (MSB)

Byte 2        CCB Pointer

Byte 3        CCB Pointer (LSB)

Mailbox In Format for CCB Required

Byte 0       MBI Status	This byte specifies the state of the MBI entry.

Value        Definition

10H          SCSI Target Command received with no CCB available. The host must

prepare an appropriate CCB and place it in the MBO to complete the

SCSI command. The remaining three bytes of the MBI specify the

information necessary to prepare a SCSI command.

Byte 1       Initiator and LUN

Bit 7-5      Binary address of initiator that selected the host adapter in

Target Mode.

Bit 4        The SCSI Target Command received was a RECEIVE command. A CCB must

be prepared to transmit data to the Initiator.

Bit 3        The SCSI Target Command received was a SEND command. A CCB must be

prepared to transmit data from the Initiator.

Bits 2-0     Binary address of the logical unit of the Target Mode host adapter

that was addressed by the Initiator.

Byte 2-3     Data Length The high order two bytes of the data length specified

in the SEND  or the RECEIVE command. A CCB must be prepared to

transmit this amount of data plus up to 256 more bytes. It is

assumed that the data fields are self-defining or of a known length

in such protocols.

The CCB Required Mailbox In entries are only generated when a command has been

received by the host adapter in target mode. For the CCB Required Mailbox In

entries, byte 0 takes on a distinctive value of 10H, indicating both that the

MBI entry is filled and that the MBI entry is being presented to request a CCB

appropriate to service a SCSI transaction for which no CCB was available. The

host adapter will have disconnected on the SCSI after receiving the initiator

and LU addresses and after having received either a SCSI SEND or a RECEIVE

command. If the LU address is enabled by the Logical Unit Mask, the MBI will

provide the addresses in bits 7-5 and bits 2-0 and will indicate by setting

either bit 3 or bit 4 that a SCSI SEND or RECEIVE command was accepted from the

initiator. The host system is then expected to prepare a CCB suitable for

processing the Send or Receive command. The information in bytes 2 and 3 is a

copy of bytes 2 and 3 of the Send or Receive command CDB. This provides the

host  an approximation of the length of the requested data transfer. This

length is used by the host to allocate the correct amount of buffer to accept

data in a SEND command or to control the maximum amount of data that can be

transferred in a RECEIVE command. If the LU address received is disabled,  no

MBI will be presented and a check condition will be posted to the Initiator. A

subsequent REQUEST SENSE command will recover an Invalid LUN error indication.

After each Mailbox In entry is stored by the host adapter, it will indicate

that Mailbox In scanning must be performed in the following manner:

1)-If the Any Interrupt bit is not set to one in the Interrupt Flag port,

indicating that no interrupts are pending, the host adapter will set the

Mailbox In Full interrupt bit, the Any Interrupt bit, and raise the hardware

interrupt line to indicate that there is at least one Mailbox In entry to be

examined by the host system.

2)-If the Mailbox In Full interrupt bit has already been set to indicate that

an entry was made in the Mailbox In area and if that interrupt has not yet been

cleared by the host, no further notification is provided by the host adapter.

3)-If interrupts other than the Mailbox In Full interrupt are pending, the host

adapter will wait for all pending interrupts to be cleared before setting the

Mailbox In Full interrupt.

Note that careful host Target Mode software design is necessary to prevent

ambiguity in the notification process. If a CCB was just prepared but the MBO

has not yet been searched by the host adapter at the time the MBI entry is made

to the host, the host may choose to examine the MBI entry and not act on it,

having already provided the needed CCB. The host should also examine the entire

MBI space to be sure that a previously supplied CCB which should have been able

to service the notification MBI entry was not already used for a previous

operation that had been posted by the host adapter but not yet serviced by the

host. Proper care in host system design will prevent these overlapping

operations from becoming a problem.

5.3 COMMAND BLOCK DEFINITION

The CCB specifies detailed information about a SCSI command. The format of a

CCB is shown below. Each of the fields is separately explained following the

table.

COMMAND CONTROL BLOCK FORMAT

Byte 0    Command Control Block Operation Code

Value       Definition

00H       SCSI Initiator Command

01H       Target Mode Command

02H       SCSI Initiator Command with Scatter/Gather

81H       Bus Device Reset

Byte 1   Address and Direction Control

Bits 7-5   CCB Op Code = 00, 02:  SCSI Target Id

CCB Op Code = 01:     SCSI Initiator Id

Bit   4    Outbound data transfer, length is checked.

Bit   3    Inbound data transfer, length is checked.

Bit  2-0   Logical Unit Number

Byte 2     SCSI Command Length

Byte 3     Request Sense Allocation Length/Disable Auto Sense

Byte 4-7   Data Length (Byte 4 most significant)

Byte 7-9   Data Pointer (Byte 7 most significant)

Byte 10-12 Link Pointer (Byte 10 most significant)

Byte 13    Command Linking Identifier

Byte 14    Host Adapter Status  (HASTAT)

Byte 15    Target Device Status  (TARSTAT)

Byte 16-17 Reserved (00)

Byte 18-n  SCSI Command Descriptor Block

(Length specified by byte 2)

Byte n-m   Reserved for REQUEST SENSE information bytes

(Length of reserved space in byte 3)

The standard format of the Command Control Block is shown in the following

diagram:

The bytes of the CCB are defined further below:

Byte 0: Command Control Block Operation Codes

The AHA-1540A/1542A supports four CCB Operation Codes. The valid command values

are shown below:

00  SCSI Initiator Command Control Block

01  SCSI Target  Command Control Block

02  SCSI Initiator Command Control Block with Scatter/Gather

81  SCSI Bus Device Reset

If the Operation Code value is 00H, the SCSI command specified in the Command

Descriptor Block field of the CCB is executed against the addressed Target  /

LUN. The other fields of the CCB support the required Initiator functions of

the AHA-1540A/1542A.

If the Operation Code value is 01H, the CCB is intended to service a SEND or

RECEIVE command sent to the host adapter as a target from another initiator.

The values in the other fields are used to service the Target Mode operation.

If an Operation Code of 01H is specified to a host adapter that has not had its

Target Mode enabled, the host adapter returns a Host Status indication of 18H,

Invalid CCB Parameter.

If the Operation Code value is 02H, the SCSI command specified in the Command

Descriptor Block field of the CCB is executed against the addressed Target /

LUN. The definition of the Data Length and the Data Pointer is modified to

support the Scatter/Gather function. Refer to section 5.2.4.

If the value is 81(hex) a Bus Device Reset message will be sent to the

addressed target. This command forces the host adapter to abort all outstanding

tasks against the selected target. All remaining CCB Bytes are ignored. The

host adapter will generate a Bus Device Reset message out to the specified

target.

Any other command value generates a Host Adapter Detected Error (Host Status

byte of 16H).

Byte 1: Address and Control Byte

This byte identifies the address of the devices that will be serviced and

provides information about the expected direction of data flow.

If the CCB is an Initiator CCB, this byte identifies the target SCSI device in

bits 7,6, and 5. If the CCB is a Target CCB, the byte identifies the initiator

which the CCB will serve.

Bits 4 and 3 are set to determine the direction of data transfer. For an

Initiator CCB, if neither bit is set, the direction of data transfer will be

established by the SCSI command being executed.  For a Target CCB, if neither

bit is set or both bits are set, the CCB will be returned with an indication of

Invalid CCB Parameters (18H) in the Host Status field, since each Target CCB

must be identified as to whether it will service a SEND or a RECEIVE command.

If both bits are set for an initiator CCB,  the command must perform no data

transfer. If only bit 3 is active, the data transfer will be to the host

adapter and from the external SCSI device. If the CCB is a target CCB, the data

transfer will be required to be a Data Out phase, while if the CCB is an

initiator CCB, the data transfer will be required to be a Data In phase. If

only bit 4 is active, the data transfer will be from the host adapter and to

the external SCSI device. If the CCB is a target CCB, the data transfer must be

a Data In phase, while if the CCB is an initiator CCB, the data transfer will

be a Data Out phase.

If bits 3 or 4 are set for an Initiator CCB, the data length will be checked.

If the amount of data transferred exceeds the specified amount, the CCB Host

Status field will contain an indication of Data Over/Under Run (12H). For a

Processor Target mode CCB, the handling of incorrect lengths is described in

Section 8.2.4.

If a data underrun/overrun condition occurs for an operation that accesses the

drive's media (READ/WRITE, EXTENDED READ/WRITE, WRITE AND VERIFY) and the

direction bits are set to zero, the host adapter will complete the operation

without error. However, some or all of the data specified by the host may not

be transferred.

CAUTION:  THE APPROPRIATE DIRECTION BIT SHOULD BE SET FOR ALL OPERATIONS THAT

ACCESS THE DRIVE'S MEDIA.

If the CCB is an initiator CCB, bits 2,1, and 0 define the target Logical Unit

which will be addressed. If the target accepts an IDENTIFY message out, the

value in bits 2,1, and 0 will be provided in the LUN field of the message byte.

The LUN field in the SCSI Command Descriptor Block (CDB) is expected to be

zero. If the target does not accept an IDENTIFY message out, the LUN field in

the SCSI CDB must contain the correct Logical Unit address. SCSI devices with

conformance level 2, including Common Command Set (CCS) disk drives and all

SCSI-2 devices will always accept the IDENTIFY message out. The few SCSI

devices not meeting those requirements must be examined on a case by case basis

to determine whether the Logical Unit Address should be placed in CCB Byte 1 or

in the CDB.

This enables the host adapter to check the length of the data transfer and if a

data underrun/overrun condition occurs, the CCB will be returned with an

indication of Data Underrun/Overrun (12H) in the Host Status field. For

operations that do not access the drive's media, the direction bits should be

set to zero unless transfer length checking is desired. Setting both direction

bits to one should be used only when no data transfer is expected or

suppression of data transfer is desired for READ operations.

Byte 2: SCSI Command Length

This byte establishes the length, in bytes, of the SCSI Command Descriptor

Block.

Byte 3: Request Sense Allocation Length

When a SCSI device terminates an operation with CHECK CONDITION status, it

means that the device has error or status information as a result of execution

of the operation. The SCSI specification indicates that a REQUEST SENSE command

must be executed before any other command is executed in order for the host

initiator to be sure of obtaining the error information. Since the AHA-154XA

host adapter has the capability of queueing commands for execution, the host

adapter itself must take charge of generating the Request Sense command. The

automatic generation of Request Sense is inconvenient for some specialized

operating systems, so the function can be disabled by installation of a jumper

in J9 (See section 3.3.11). If the jumper is not installed, the function can

also be disabled by specifying a value of 01H for the Request Sense Allocation

Length.

This byte indicates the length, in bytes, of the area reserved for information

that may be obtained by a REQUEST SENSE command. A value of 00H indicates that

an allocation length of 14 bytes is to be used, sufficient to capture the sense

key and error code of all normal extended sense type devices. A value of 01H

requests that no automatic REQUEST SENSE be executed. The values from 02H to

07H are reserved. Values from 08H to FFH are valid allocation lengths. The

value is used to notify the host adapter that the specified number of bytes

have been reserved at the end of the CCB to receive possible Request Sense data

bytes. The REQUEST SENSE also uses the indicated allocation length as its byte

count in the Command Descriptor Block generated by the host adapter.

If an operation that treats the AHA-1540A/1542A as a Target Mode device fails

and presents a Check Condition status byte,  the Initiator should return a

REQUEST SENSE command. The AHA-1540A/1542A will return appropriate sense

information in response to the command. If the command that originally failed

was a SEND or a RECEIVE command, the same REQUEST SENSE information bytes that

will later be sent to the Initiator are also sent to the host when the CCB is

returned. The sense information is placed in the specified Request Sense

Allocation area with a length not exceeding the Request Sense Allocation

Length.

Bytes 4, 5, 6: Data Length

These bytes determine the length, in bytes, of the data transfer. CCB host

adapter error 12(hex) is posted if a data over run occurs.

If the CCB specifies a Scatter/Gather operation, the Data Length field contains

the total number of bytes in the Data Segment List.

Bytes 7, 8, 9 : Data Pointer

These bytes specify the real address of the first byte of the data area to be

used during the data phase of the SCSI command.

If the CCB specifies a Scatter/Gather operation, the Data Pointer field

contains the pointer to the first byte of the Data Segment List.

Bytes 10, 11, 12: Link Pointer

These bytes are used when a SCSI command contains a LINK or LINK WITH TAG bit

in the command. When a Linked command is completed, the host adapter will use

the contents of this field as a pointer to the next CCB to execute. If the

Linked Flag bit is set, an interrupt will be generated before the next command

is started. A completed CCB is always reported back in an MBI, but MBIF

interrupts are only reported if the linked set of commands is finished or if a

Link with Flag message is presented. There must be enough MBI entries to

receive the entire set of linked commands.

Target Mode does not support the linking function.

Byte 13 : Command Link ID

This byte is used in conjunction with linked commands. It is set by the host to

identify commands in a command chain.

Linking is not supported in Target Mode.

Byte 14 : Host Status

This byte is used to report the host adapter status (HASTAT).

HASTAT has the following definitions:

00 H No host adapter detected error-The CCB was completed normally.

0A H Linked command complete without error-The SCSI command

completed and linked normally.

0B H Linked command complete without error, interrupt generated.-The SCSI

command completed and linked with a LINKED COMMAND COMPLETE WITH FLAG message.

11 H Selection time out-The initiator selection or target reselection was not

complete within the Set SCSI Selection Time-out period.

12 H Data over run/Under run-The target attempted to transfer more data than

was allocated by the Data Length field or the sum of the Scatter / Gather

Data Length fields.

13 H Unexpected bus free-The target dropped the SCSI BSY at an unexpected time.

14 H Target bus phase sequence failure-An invalid bus phase or bus phase

sequence was requested by the target. The host adapter will generate a SCSI

Reset Condition, notifying the host with a SCRD interrupt.

15 H MBO command was not 00, 01, or 02-The first byte of the MBO command was

invalid. This usually indicates a software failure.

16 H Invalid CCB Operation Code-The first byte of the CCB was invalid. This

usually indicates a software failure.

17 H Linked CCB does not have the same LUN-A subsequent CCB of a set of linked

CCB's does not specify the same logical unit number as the first.

18 H Invalid Target Direction received from Host-The direction of a Target Mode

CCB was invalid.

19 H Duplicate CCB Received in Target Mode-More than once CCB was received to

service data transfer between the same target LUN and initiator SCSI ID in

the same direction.

1A H Invalid CCB or Segment List Parameter-A segment list with a zero length

segment or invalid segment list boundaries was received. A CCB parameter was

invalid.

Byte 15: Target Status

For an initiator CCB, this byte is used to return the SCSI status byte sent to

the host adapter from the initiator. If a SCSI command returns with BUSY

status, the normal recovery process is to execute the command again. The AHA-

154XA takes that burden off the host and periodically restarts the command

automatically until the command completes with a status other than Busy.

For a target mode CCB, this byte is used to indicate to the host what status

the host adapter returned to the initiator.

Target Status may have the following values in target mode

00 -- Good Status

02 -- Check Status (See REQUEST SENSE byte area)

08 -- LUN Busy

Byte 16: Reserved (must be 0)

Byte 17: Reserved (must be 0)

Byte 18: n: SCSI Command Descriptor Block

This field holds the SCSI Command Descriptor Block (CDB) as described in the

SCSI specification. The length of this command is described in byte 02. For

initiator mode CCB's, the CDB provided by the host is transmitted to the

target. For target mode CCB's, the CDB provided from the initiator is returned

to the host in this space.

Byte 18+n - 18+n+m : Allocated for Sense Data

If a Check Status condition is detected by the AHA-154XA as it completes an

operation on the SCSI bus,  the host adapter automatically executes a REQUEST

SENSE command with the data length specified by Request Sense Allocation

Length. The actual bytes returned, up to the maximum indicated by the Request

Sense Allocation Length, are placed in the area Allocated for Sense Data. If

the Request Sense Allocation Length was 01h, no REQUEST SENSE command is

executed.

If a check condition is detected by the AHA-154XA while it is operating in

Processor Target Mode, the same information that will later be recovered by the

initiator that received the Check Status is also placed in the area Allocated

for Sense Data so that the host processor is also aware of the failure.

5.3.1 SCATTER/GATHER LIST DEFINITION

Using the normal CCB Operation Codes of 00 and 01 (SCSI Initiator and SCSI

Target CCB's), the CCB itself contains a pointer to the first byte of a

contiguous area of data of a specified length. The direction of transfer with

respect to the data area and the checking of the length of the data transfer to

the data area are both managed by the AHA-1540A/1542A according to the

requirements of the particular mode and control bit set-up.

Using the Scatter/gather Operation Code of 02, the CCB instead contains a

pointer to a list of data segments and an indication of the length of the list

of data segments. The data segment list contains pointers to the actual

location in host memory of the data segments to be transferred as well as a

precise indication of the length of each data segment. Each data segment list

entry contains a three-byte pointer to the location of a data segment and

three-byte length indicator telling how long that particular data segment shall

be. The data segment list is arranged in the order in which data will be

gathered or distributed, the first pointer in the list being used first.

A typical data segment list is shown in the following table. The data segment

list describes the distribution of 4096 bytes distributed in four separate

locations within the memory. The beginning of the data segment list is

indicated by the data segment list pointer. The length of the data segment list

(24 bytes) will be contained in the data segment list length field of the CCB.

A data segment list can have from one to sixteen segments. A list with 0

segments or a list with more than 16 segments causes an Invalid Segment List

error to be posted in the host status field.

The AHA-154XA has certain constraints in the data segment address boundaries

and lengths that are allowed. If these boundaries are violated, an Invalid

Segment List error will be posted in the host status field of the CCB. The

simplest way to assure that these boundaries are not violated is to require all

boundaries between segments to be on even word boundaries. The first segment

may begin on any boundary and the last segment may end on any boundary. All

segments but the first and last should have even byte counts.

The actual boundary limitation is somewhat less restrictive. If the binary

values of the starting address of a segment, the byte count of that segment,

and the starting address of the next segment are all exclusive OR'd together,

and the result of that exclusive or is an even number, the boundary between

those two segments is valid. Another way to express the limitation is to

require that the ending boundary of one segment must be the same as the

beginning boundary of the next segment. If a segment ends on a word boundary,

the next segment must begin on a word boundary. Similarly, if a segment ends on

an odd-byte boundary, the next segment must begin on an odd-byte boundary.

5.4 DESCRIPTION OF OPERATION

This section describes the interface operations required to invoke the desired

SCSI behavior. The AHA-1540A/1542A must be properly initialized before any of

these operations can be performed.

After system initialization is done, the I/O Command Port initialization

commands must be executed. The Mailbox Initialization command must be executed

to assign the Mailbox area. The Enable Target Mode command may optionally be

executed if Target Mode is to be allowed.

5.4.1 Execution of Initiator Mode operations

To begin an Initiator Mode SCSI command, the host first allocates a data buffer

area. A CCB is then created to perform the desired operation to the correct

peripheral device and a pointer to the data buffer area is placed in the CCB.

Once the CCB is completely defined, the host places a pointer to the CCB in an

empty MBO location, places an MBO full status in the MBO status byte, and

transmits a Start SCSI Command to the I/O Command Port. If it is not already

scanning the MBO for an active MBO entry, the Start SCSI Command to the I/O

Port causes the host adapter to begin scanning for such an entry.

After finding an active MBO entry, the AHA-1540A/1542A copies the MBO Command

field contents and CCB pointer into its internal RAM and clears the MBO Command

byte, freeing the mailbox. Up to 16 initiator CCBs and 16 target CCBs can be

stored in the host adapter's internal RAM concurrently. The AHA-1540A/1542A

maximizes the SCSI bus utilization by starting the next available CCB as soon

as the bus is free. Disconnection  and reconnection on the bus are

automatically taken care of by the AHA-1540A/1542A. As the host adapter starts

the SCSI operation, it  will first determine if the addressed target/LUN is

busy. If the target/LUN is busy, then the command will be placed in the task

queue to be tried again later.

Commands are removed from the queue in the order that they were received,

establishing a (FIFO) first in first out command execution order. Of course,

the order of task completion may vary due to the different amounts of time

required to process and complete different commands. If the target/LUN is not

busy, the new command is started at the next bus free phase. If BUSY status is

received by the AHA-1540A/1542A, the CCB is placed at the end of the FIFO queue

to be restarted later.

If a CHECK CONDITION status is received from the target, the AHA-1540A/1542A

will issue a Request Sense command to get the sense data. The sense data is

stored in the CCB after the SCSI command data. The driver software must reserve

the allocated number of bytes at the end of the CCB to hold the sense data

which is returned as result of a receiving a check status. Automatic Request

Sense can be optionally disabled by jumpers or by using the CCB.

If the MBO Command is to abort a SCSI command, the host adapter first searches

the active and queued CCBs. If the CCB is found, the task is aborted at the

earliest possible moment and an MBI entry is made to indicate that the CCBs

execution was successfully terminated. If the CCB is not found among the active

or queued CCBs, the AHA-1540A/1542A completes the command and reports that fact

in the MBI Status byte.  The CCB may not be found because it has previously

been aborted, because an intervening reset occurred, or because the CCB was

already finished normally and returned.

The AHA-1540A/1542A scans the MBOs in a round robin fashion. This is to ensure

that all of the mailboxes will be scanned with equal probability. A host can

minimize the AHA-1540A/1542A MBO scan time by using MBO's sequentially.

When executing a START SCSI command, the AHA-1540A/1542A does not verify that

the new CCB resides in an unused memory area. Therefore, the host must not

reuse a CCB location until it has been returned through a Mailbox In entry and

the host has examined all the pertinent information in the CCB.  CCB addresses

are used by the AHA-1540A/1542A as task identifiers.

5.4.2 Execution of Target Mode Operations

5.4.2.1 Target Mode with a Prepared CCB

Typical target operation is managed in the following way by the host software.

After power-on initialization is complete, the host sends an Enable Target Mode

command to the host adapter to enable Processor Target mode. After that, the

host, from previous configuration knowledge, generates a pair of CCB's for each

likely initiator and for each LUN supported by the host. One CCB is for

outbound data from the initiator using the SEND command, while the other is for

inbound data from the target to the initiator using the RECEIVE command. The

SEND command CCB defines a data buffer for the expected network type command

from the initiator. The RECEIVE command CCB defines a set of data which is

known and expected by the initiator, typically a welcome or configuration type

message packet. The host adapter tests to be sure that duplicate CCBs are not

provided by the host.

As the initiator finishes its initialization procedure, it requests

availability and configuration information using the TEST UNIT READY, REQUEST

SENSE, and INQUIRY commands. Finally, it may choose to address SEND and RECEIVE

commands to the targets it has located to transmit or request  information

packets.

When the initiator executes a SEND or a RECEIVE command, the target accepts the

selection on the SCSI bus, accepts the command from the initiator, and executes

the proper data transfer to or from the area specified by the proper CCB. When

all SCSI activity is finished successfully, the CCB is posted back to the host

program by an entry in the MBI. The host adapter updates the CCB byte count and

CDB field contents to correctly reflect the operation performed. The host is

notified that there is an entry in the MBI by an interrupt from the host

adapter.  The host then prepares a new CCB to control the target's next

activity expected from the initiator.

5.4.2.2 Target Mode without a Prepared CCB

Alternatively, the processor type device may be addressed by an initiator when

no CCB has yet been prepared for use by the host adapter. If the host adapter

command enabling target mode has not yet been executed, the host adapter will

act like any initiator. It will not accept any selection sequence and any

attempts to select it by its target ID will cause the initiator to detect a

selection time-out.

If the host adapter has been enabled as a Processor Target by a host adapter

command, any selection to the host adapter's target address will be accepted.

The availability commands and identification commands (TEST UNIT READY, REQUEST

SENSE, and INQUIRY) will be executed completely and normally. If a SEND or

RECEIVE command is received and if a CCB is not active, either because the host

has not completed its program initialization or because the host has not

finished analyzing the results of previous data transfers and has not generated

a new CCB, the Processor Target will accept the selection, obtain the IDENTIFY

Message Out to determine what Logical Unit has been selected, request the CDB

from the SCSI, and then disconnect. The CDB will be partially interpreted so

that the proper MBI can be generated to notify the host that a new CCB with a

certain address, direction, and data length must be generated. When the new CCB

is passed to the host adapter by an MBO entry, the CCB is filled up with the

command information, the SCSI is reconnected, and the command is completed as

described above.

5.5 DMA Channel Initialization With the Host Adapter BIOS Disabled

The DMA circuitry must be set to a special state to allow the Bus Master

operation of the AHA-154XA to operate correctly. This state is normally

established by the BIOS during initialization so that no other activity is

required. If a DMA channel other than the default channel is being used or if

the DMA channel is manipulated by other programs, the circuitry must be

initialized in the following way before the AHA-154XA can be used.

For the DMA channel being used, 2 bytes of data must be written to the DMA

controller port specified to initialize the Host DMA controller for Bus Master

DMA operation. The following table specifies these values in HEX.

DMA Channel       DMA Controller Port    Data

0                       0B               0C

0A               00

5                       D6               C1

D4               01

6                       D6               C2

D4               02

7                       D6               C3

D4               03

5.6 INTERRUPT INITIALIZATION

This setup procedure is normally completed by the BIOS during initialization so

that no other activity is required. If modifications to the interrupt handler

are required, this information together with the programming information

provided by the host system should be sufficient to properly set up the

interrupt vectors.

The host adapter will drive one of several interrupts in the AT system. The

particular interrupt used must be set up on power up initialization and be

properly managed during usage. The AT interrupts of interest to the host

adapter driver along with their corresponding vector locations are summarized

below. All these interrupst are handled by a slave interrupt controller. The

master controller handles all system interrupts such as keyboard, timer, etc.

and is assumed to be correctly initialized to allow interrupts by the slave

controller. Upon receiving an interrupt, the processor will be vectored to the

contents of the corresponding vector location.

Hardware   Software interrupt

interrupt  vector location (hex)

int 9    --     INT71

int 10   --     INT72

int 11   --     INT73

int 12   --     INT74

int 13   --     INT76

int 14   --     INT77

int 15   --     INT78

The interrupt is initialized by clearing the corresponding interrupt mask bit

in the slave controller. The mask register is a read/write register, and only

the bit of interest should be cleared. The port address is A1 hex, and bit

definitions follow.

Interrupt mask bit definition (port address A1 hex)

bit 0 -- int 8

bit 1 -- int 9

bit 2 -- int 10

bit 3 -- int 11

bit 4 -- int 12

bit 5 -- int 13

bit 6 -- int 14

bit 7 -- int 15

6.1 SINGLE THREADED BIOS COMMAND INTERFACE

The AHA-1540A/1542A BIOS resides on the host adapter board. It provides support

for up to two SCSI Common Command Set disk drives under DOS. If SCSI devices

other than CCS disk drives or if support for more than two disk drives is

required under DOS, the Adaptec DOS Driver, ASW-1110 must be used. The BIOS

provides a very simple single threaded capability that does not make use of the

Mailbox Interface used by more sophisticated programs. This capability allows

the host system to boot from a SCSI disk drive and to support standard DOS

calls from any standard program.

The BIOS communicates with the host adapter through a special set of commands

passed under the I/O Command Port command 02H (Start Bios Command). These

commands are not available to any programs except the Adaptec BIOS. The Adaptec

BIOS provides a standard BIOS interface. The DOS interface to the standard BIOS

is described in this section.

On an AT compatible machine, up to two hard disks are supported by the BIOS

resident on the mother board. Any other drives must be managed through the

Mailbox Interface using an appropriate driver. The operation of the AHA-

1540A/1542A BIOS on attached SCSI devices depends on the number of standard

hard disks installed directly on the system.

No standard hard disks installed:

In this case, two SCSI drives can be supported by the AHA-1540A/1542A BIOS.

SCSI Device 0 Logical Unit Number 0 is designated as drive 0 (Drive C:). Either

SCSI Device 0 LUN 1 or SCSI Device 1 LUN 0 may be designated as drive 1 (Drive

D:). This set of addresses allows the use of two drives with either embedded

controllers or bridge controllers. Booting is only performed from SCSI Device 0

LUN 0.  When no standard hard disks are installed, the AHA-1540A/1542A BIOS is

operating in non-concurrent mode.

One standard hard disk installed:

In this case, the internally installed hard disk is designated as drive 0

(Drive C:). SCSI Device 0 LUN 0 is designated as drive 1 (Drive D:). Booting is

performed from the internally installed hard disk. This mode of operation is

called concurrent operation, indicating that the internally installed drive and

one SCSI drive can operate together under the BIOS.

Two standard internal hard disks installed:

In this case, all the disks that can be supported by the BIOS are directly

installed. The AHA-1540A/1542A BIOS will not support any SCSI drives. Therefore

a device driver will need to be installed to access the SCSI drives.

6.2 BIOS COMMAND EXECUTION USING INTERRUPT 13

On IBM compatible systems, disk I/O requests are passed from the operating

system to the BIOS through software interrupt 13H. CPU registers are used to

pass information about the desired operation and the associated parameters.

The AHA-1540A/1542A BIOS provides SCSI support by intercepting each interrupt

and managing the request according to the drive address. If the drive is a

floppy or internally installed fixed disk, the call is managed by the mother

board BIOS with no changes. If the selected drive is a SCSI disk drive, the

request is handled by the Adaptec BIOS resident on the AHA-1540A/1542A. For

concurrent operation, only calls to drive 81H (D:) are handled by the AHA-

1540A/1542A BIOS. For nonconcurrent operation, calls to both drive 80H (C:) and

81H (D:) are managed by the AHA-1540A/1542A BIOS.

On each Interrupt 13 call, the correct host microprocessor registers are set by

the operating system program to provide the required parameters to both the

Adaptec BIOS and the mother board BIOS. Most commands use the registers as

indicated in the following table.

Register Function

AH       BIOS Function Code

AL       Number of Sectors

CH       Low Order Byte of Cylinder Number

CL       Cylinder and Sector Numbers

Bits 7,6 High Order Cylinder Bits

Bits 5-0 Sector Number

DH       Head Number

DL       Drive Number

ES:BX    Data Buffer Address

6.2.1 Physical to Logical Address Translation

As shown above, the starting sector address for read, write and verify requests

is passed as a physical address including a 10 bit Cylinder Number, an 8 bit

Head Number, and a 6 bit Sector Number. SCSI devices are addressed by logical

block address without regard to the physical layout of the drive. For this

reason, all SCSI devices are defined as drives with 64 heads and 32 sectors per

track and with the appropriate number of cylinders to provide the full capacity

of the drive using this physical to logical translation. The SCSI logical

address is formed by the BIOS by concatenating the values as show below:

Cylinder(10 bits) || Head(6 bits) || Sector(5 bits)=SCSI Logical Block Address

6.2.2 BIOS Command Return Codes

The BIOS command, when complete, returns control to the requesting program at

the next instruction after the software interrupt. A return code is set in the

Carry Flag (CF) and a status code is placed in register AH for examination by

the requesting program.  The expected return codes placed in the Carry Flag are

shown below.

CF Contents at Return    Return Code

CF=1                     Status is non-zero. Unusual condition or error

detected.

CF=0                     Status is zero. Normal command completion 	occurred.

AH Contents at Return    Status of Operation

AH = 00H                 No error:The operation completed successfully.

AH = 01H                 Invalid Function Request:	The operation code or  an

associated parameter provided by the Interrupt 13

operation was not valid.

AH = 02H                 Unable to Read Address Mark: One of the following

additional sense codes (SCSI ASC) was presented in the

sense information returned by the REQUEST SENSE

command:

12H (No AM Found on Data Field)

21H (Illegal Logical Block Address)

AH = 03H                  Write Protect Error:	Returned SCSI ASC was:

27H  (Data Protect)

AH = 04H                Read Error:Returned SCSI ASC during a read operation

was:

14H  (No Record Found)

16H  (Data Sync Error)

AH = 10H                Uncorrectable ECC Error in Data:  Returned SCSI ASC

during a read operation was:

10H  (ID ECC Error)

11H  (Unrecovered Read Error)

AH = 11H                ECC Corrected Data Error:Returned SCSI ASC during a

read operation was:

17H  (Recovered Read Error w/o ECC)

18H  (Recovered Read Error w/ ECC)

AH = 20H                General Controller Failure:A host adapter detected

failure occurred or one of the following SCSI ASC's

was returned:

01H 08H 40H 46H

03H 09H 41H 47H

05H 1BH 42H 48H

06H 1CH 43H 49H

07H 1DH 44H

AH = 40H               Seek Operation Failed:  Returned SCSI ASC was:

15H  (Seek Positioning Error)

02H  (No Seek Complete)

AH = 80H               Selection Time-out: A host adapter does not respond to

BIOS.

AH = AAH               Device Not Ready:Returned SCSI ASC was:

04H  (LUN Not Ready)

28H  (Unit Attention, Ready)

29H  (Unit Attention, Power On)

2AH  (Unit Attention, Mode Select Change)

AH = BBH               Unknown Target Sense Error Occurred: A SCSI ASC other

than those specified above was returned from the

selected SCSI device.

AH = CCH               Write Error:   Not Returned

H = FFH                Sense Operation Failed:BIOS unable to retrieve sense

information from target.

6.3 BIOS DISK COMMANDS

The commands that can be accepted from the operating system by the BIOS include

all those commands required for normal DOS operation, for booting of the DOS

operating system, for the booting of other operating systems, and for basic

maintenance and verification of normal disk operation. The command set is

summarized in the following table and each command is described in more detail

in the sections following.

AH Register Meaning

00 H Reset Disk System

01 H Read Status of Last Operation

02 H Read Desired Sectors into Memory

03 H Write Desired Sectors from Memory

04 H Verify Desired Sectors

06 H Identify SCSI Devices

08 H Read Drive Parameters

09 H Initialize Drive Pair Characteristics

0C H Seek

0D H Alternate Disk Reset

10 H Test Drive Ready

11 H Recalibrate

15 H Read DASD Type

The Format commands (AH = 05, 06, 07, and 1AH) are not supported by the AHA-

1540A/1542A BIOS. The format operation is performed under a special format

utility installed in ROM and initiated through the DEBUG program.

The Diagnostic Reserved commands (AH = 0A, 0B, 0E, 0F, 12, and 14H) are not

supported by the AHA-1540A/1542A BIOS.

The Park Heads command  (AH = 19H) is not supported by the AHA-1540A/1542A

BIOS.

The Reserved for Diskette commands (AH = 16, 17, 18) and the commands from 1BH

up to FFH are not supported by the AHA-1540A/1542A BIOS. Any attempt to execute

any of these unsupported commands will be ended by the BIOS and an error code

of  01H (Invalid Function Request) will be returned in the AH register.

6.3.1 Reset Disk System

The BIOS is requested to reset the disk subsystem. A SCSI Reset operation is

performed on the SCSI bus if the bus is busy at the time the Reset Disk System

command is received by the host adapter. No other SCSI or host adapter activity

occurs. The BIOS command is then passed on to the standard BIOS so that any

internally installed hard disks and floppy disk devices can also be reset. If

the BIOS is operating concurrently with an internally installed hard disk, the

drive number is decremented by the AHA-1540A/1542A BIOS so that the standard

BIOS does not attempt to reset the drive that is not installed.

Input  Parameters:      AH = 00H

DL = Drive Number (80H or 81H)

Output Parameters:      AH = Status of Operation

CF =  Return Code

6.3.2 Read Status of Last Operation

The status of the last operation performed to the specified disk is returned.

No SCSI activity occurs. The disk status is reset to zero.

Input  Parameters:     AH = 01H

DL = Drive Number (80H or 81H)

Output Parameters:     AH = Status of Operation

CF =  Return Code

6.3.3 Read Desired Sectors Into Memory

The sectors requested by the parameters are read from the disk to the system memory. The SCSI command is executed as a READ (EXTENDED) command (SCSI Operation Code 28H). If the operation fails and an error is reported through the BIOS  status byte, the operation should be retried one time. If a 11 Error (ECC Corrected Data Error) is returned,  the data returned has been corrected and may be used with confidence.

Input  Parameters:	AH = 02H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Read

ES:BX = Address of buffer area

Output Parameters:  AH = Status of Operation

CF =  Return Code

6.3.4 Write Desired Sectors from Memory

The sectors requested by the parameters are written from the system memory to

the indicated disk. The SCSI command is executed as a WRITE (EXTENDED) command

(SCSI Operation Code 2AH). If the operation fails and an error is reported

through the BIOS  status byte, the operation should be retried one time.

Input  Parameters: AH = 03H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Write

ES:BX = Address of buffer area

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.5  Verify Desired Sectors

The sectors defined by the parameters are verified to be correctly written on

the SCSI disk. The SCSI command is executed as a VERIFY command (SCSI Operation

2F) with the Byte Check bit set to zero. If the VERIFY command is not supported

by the selected disk, the Verify Desired Sectors will perform a SCSI READ

command (SCSI Operation Code 28H) and throw away the received data. If the

operation fails and an error is reported through the BIOS  status byte, the

operation should be retried one time.

Input  Parameters: AH = 04H

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder and Sector

AL = Number of Sectors to Verify

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.6 Identify SCSI Devices

This is a special AHA-1540A/1542A BIOS call that is used to return the number

of the first supported SCSI drive. In the  nonconcurrent case (2 SCSI drives),

the returned value will be 80H. If only one SCSI drive is installed, the

returned value will be 81H.

Input  Parameters:	AH = 06H

Output Parameters: AH = Status of Operation

AL = First Drive Supported

80H Nonconcurrent operation

81H Concurrent operation

CF =  Return Code

6.3.7 Read Drive Parameters

A SCSI READ CAPACITY command is used to determine the maximum logical block

of the selected drive. This information is then used to calculate the proper

number of cylinders to be returned to the host system. The number of heads

returned will always be 64 and the number of sectors per track will always

be 32. The number of drives returned will include both internally installed

drives and SCSI drives.

Input  Parameters:AH = 08H

DL = Drive Number (80H or 81H)

Output Parameters:AH = Status of Operation

DL =  Number of BIOS accessible drives attached (1 or 2)

DH =  Maximum value for head number (3FH)

CH =  Maximum value for Cylinder Range (Low Byte)

CL =  Maximum value for Sector, High Cylinder Bits

Bits 7,6  	High Order Cylinder Bits

Bits 5-0 Sector Number Maximum Value ( 20H)

CF =  Return Code

6.3.8 Initialize Drive Pair Characteristics

This command performs no operation, since SCSI CCS drives are self

configuring.

Input  Parameters: AH = 09H

DL = Drive Number (80H or 81H)

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.9 Seek

A seek to the logical block address defined by the physical parameters is

performed. The seek is performed using the SEEK (EXTENDED) command (SCSI

Operation 2BH). The SEEK commands are not mandatory CCS commands and are not

required for proper functioning of SCSI devices. If the addressed disk drive

reports an error indicating that the SEEK (EXTENDED) command is not

supported,  the BIOS command will complete normally anyway. Since the seek

operation is performed automatically by a Read or a Write operation, there

is no need to generate a separate seek operation to access data.

Input  Parameters: AH = 0CH

DL = Drive Number (80H or 81H)

DH = Head

CH = Cylinder

CL = High Cylinder. (Sector bits = 0)

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.10 Alternate Disk Reset

The BIOS is requested to reset the disk subsystem. A SCSI Reset operation is performed on the SCSI bus if the bus is busy at the time the Reset Disk System command is received by the host adapter. No other SCSI or host adapter activity occurs. The BIOS command is then passed on to the standard BIOS so that any internally installed hard disks and floppy disk devices can also be reset. If the BIOS is operating concurrently with an internally installed hard disk, the drive number is decremented by the AHA-1540A/1542A BIOS so that the standard BIOS does not attempt to reset the drive that is not installed.

Input  Parameters: AH = 00H

DL = Drive Number (80H or 81H)

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.11 Test Drive Ready

The BIOS command determines that the specified drive is available and ready

by executing a TEST UNIT READY command (SCSI Operation Code 00H) to the SCSI

device. The command may have to be executed a second time if a Unit

Attention condition was detected in the first execution of the TEST UNIT

READY.

Input  Parameters: AH = 10H

DL = Drive Number (80H or 81H)

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.12 Recalibrate

The BIOS command transmits a REZER0 UNIT command (SCSI Operation Code 01H)

to the specified drive. Since the REZERO UNIT command is not a mandatory

command and is not required for proper functioning of the SCSI device, the

BIOS command will complete without indicating an error even if the SCSI

device indicates that the command is invalid.

Input  Parameters: AH = 11H

DL = Drive Number (80H or 81H)

Output Parameters: AH = Status of Operation

CF =  Return Code

6.3.13 Read DASD Type

The AHA-1540 BIOS executes this command by performing an INQUIRY  (SCSI

Operation Code 12H) and a READ CAPACITY (SCSI Operation Code 25H) to the

selected SCSI drive. The INQUIRY command is used to verify that the device

is a Direct Access Storage Device. The READ CAPACITY is used to determine

the number of logical blocks available. A special return format is used to

obtain the required information.

Input  Parameters: AH = 15H

DL = Drive Number (80H or 81H)

Output Parameters:AH = Status of Operation (Special Format)

00H  Drive not present or DL invalid

01H Reserved

02H Reserved

03H Fixed Disk installed

CX,DX  Number of 512 byte blocks available on disk

CF =  Return Code

7.1 PC/AT BIOS

The PC/AT BIOS is provided to allow the user to use the AHA-1540A/1542A in

lieu of, or in addition to, a standard PC/AT hard disk controller. This

compatibility includes booting from a SCSI device. Hard disk requests are

sent to the BIOS via an INT 13 software interrupt. CPU register contents

specify the desired I/O function and associated parameters. The BIOS then

uses host adapter commands to emulate PC/AT functions. Again, since the

mailbox architecture is circumvented, PC/AT BIOS functions run in a single-

threaded mode.

The BIOS consists of three major functional components. These are:

1. Initialization Routine

2. System Boot Routine

3. Hard Disk I/O Routine

7.1.1 Initialization

The Initialization routine is called at power-up. This routine performs all

of the necessary initialization functions and will display the following

message if the system successfully recognizes the AHA-1540A/1542A BIOS.

ADAPTEC AHA-1540A/1542A HOST ADAPTER BIOS

COPYRIGHT 1987 ADAPTEC, INC.

ALL RIGHTS RESERVED.

If any subsequent initialization of the AHA-1540A/1542A fails, the following

message is displayed:

HOST ADAPTER NOT FOUND AT PORT 330H.

otherwise, the AHA-1540A/1542A BIOS chooses a course of action based upon

the number and types of hard disks already installed. In each case, the

number of hard disks already installed is displayed along with the ID

numbers of any SCSI devices that are successfully used by the BIOS.

Possible Messages:

HARD DISK #0 ALREADY INSTALLED.

HARD DISK #1 ALREADY INSTALLED.

HOST ADAPTER DIAGNOSTIC FAILURE.

SCSI DEVICE 0 : 0 NOT FOUND.

USING SCSI DEVICE 0 : 0 AS HARD DISK #0.

USING SCSI DEVICE 0 : 0 AS HARD DISK #1.

USING SCSI DEVICE 0 : 1 AS HARD DISK #1.

USING SCSI DEVICE 1 : 0 AS HARD DISK #1.

NOTE: No error message is displayed if a second SCSI drive is not available.

If the system reports an initial number of hard drives greater than 2, the

following message is displayed:

SYSTEM CONFIGURATION ERROR.

The AHA-1540A/1542A BIOS allows the use of 0, 1, or 2 hard disks on the SCSI

bus under DOS without a device driver.

If 2 hard disks are attached to an internal hard disk controller, and are

already recognized by the system, the BIOS does nothing. A device driver

will be needed to access SCSI devices.

If 1 hard disk is attached to an internal hard disk controller, and is

already recognized by the system, the BIOS attempts to assign the SCSI

device (Target : LUN) 0 : 0 as the second drive. (drive D:) If this is

unsuccessful, an initialization error is reported to the system.

If there are 0 hard disks attached to an internal controller, the BIOS

attempts to assign the first SCSI device (Target : LUN) 0 : 0 as the first

drive. (drive C:) If this is unsuccessful an initialization error is

reported to the system. If successful, the BIOS attempts to assign another

SCSI device as the second drive. (drive D:) Device 0 : 1 is tried first, and

if that fails, device 1 : 0 is tried. There is no initialization error

reported if a second drive is not available.

7.1.2 System Boot

System boot is invoked by execution of an Interrupt 19. No interception of

this interrupt is necessary, as the PC/AT BIOS manages the boot process.

When the boot procedure attempts to read the boot track from drive C:, the

AHA-154XA BIOS will respond if drive C: is a SCSI Drive. Only SCSI drive 0:0

can be assigned as a SCSI C: drive. Mother board BIOS's that use interrupt

13 to perform booting should boot normally from SCSI drives using the AHA-

154XA.

7.1.3 Hard Disk I/O

The BIOS enables up to two SCSI drives to be accessed via interrupt 13 as

described in Section 6.

7.2 XENIX/UNIX Drivers

SCO Xenix and Unix, Interactive Systems Unix, and Microport Unix all support

the AHA-1540A/1542A at the time of this publication. Please consult with

these Operating System companies for more information on their products.

7.3 DOS Driver

An optional DOS Driver is available that enables the AHA-1540A/1542A to

access additional SCSI devices on the SCSI bus. When operating under DOS, a

device driver is needed to access more than two hard disks, tape devices or

other types of SCSI peripherals. In addition, DOS applications that use the

virtual memory mode of the 80386 processor require this driver. If you are

not running DOS programs that operate in the virtual 80386 mode, or if all

DOS operations will be confined to two SCSI disks or one internal and one

SCSI disk, a device driver is not needed because of the on-board BIOS. The

DOS driver is installed by creating a DOS configuration (CONFIG.SYS) file

that calls the device driver at boot time. Please refer to your DOS manual

for information on creating a CONFIG.SYS file. At initial system loading,

the AHA-1540A/1542A device driver will determine the number of hard disks

attached to the system. The device driver is normally only installed if one

of the following cases is true.

1) There are two internal hard disks and one or more SCSI disks.

2) There is one internal hard disk and two or more SCSI disks.

3) There are no internal hard disks and more than two SCSI disks.

The following table shows how hard disks in a system are addressed:

configuration                 c:           d:           device driver

������������������������������������������������������������������������Ŀ

�two internal hard drives�  1st int. disk � 2nd int. disk �  SCSI        �

�                        �                �               �  devices     �

������������������������������������������������������������������������Ĵ

�one internal hard drive �  int. hard disk�  SCSI 0/0     � other SCSI   �

�                        �                �               �  devices     �

������������������������������������������������������������������������Ĵ

�no internal hard drive  �  SCSI 0/0 disk �  SCSI 0/1     � other SCSI   �

�                        �                �   or 1/0      �  devices     �

��������������������������������������������������������������������������

The SCSI disk drives that are accessed by the device driver are divided into

32 MB logical disks. There can be multiple logical disks on one physical

SCSI disk drive. A utility program is also provided that performs the

functions of the DOS FDISK and FORMAT utilities. This utility will perform

FDISK and FORMAT to SCSI disks that can only be accessed through the device

driver. These utility programs partition the disks and create file

directories so that DOS files may be copied and accessed from SCSI drives.

Please refer to your DOS manual for details of FDISK and FORMAT.

SCSI 2 compatible and QIC 104 command set  1/4" tape drives are also be

supported by the DOS device driver. The user accesses a SCSI tape drive by

building a CCB as described in Section Five. The device driver passes the

CCB to the AHA-1540A/1542A and instructs the AHA-1540A/1542A to complete the

command. Software products such as Sytron Corporations SYTOS Tape Operating

System automatically interface to the Adaptec AHA-1540A/1542A DOS driver.

To install the driver into your boot routine, you must create, or edit the

DOS configuration file named CONFIG.SYS. You can use any file editor to do

this, or you can create it through the "COPY CON: CONFIG.SYS" DOS command.

Refer to your  DOS manual for more information on this command. Once

installed, the driver operates transparently to the user.

7.3.1 DOS Driver Support of Virtual Mode

The use of high performance 16-bit Bus Master DMA on the AHA-1540A/1542A

SCSI host adapter may result in incompatible operation with application

programs that use the 80386 virtual memory mode of operation.

In real mode, the host adapter BIOS is passed a physical address. This is

necessary since the host adapter controls all SCSI I/O without CPU

intervention. This physical address may not be passed to the host adapter

when the system is in the V86 mode. Programs that use the V86 mode can only

be run on 80386 machines.

The AHA-1540A/1542A DOS driver also allows this type of application program

to run under DOS.

7.4 AHA-1540A/1542A On-board Utilities

The AHA-154XA BIOS contains a utility program that allows the user to

perform the several important functions. The utility programs are accessed

using a self explanatory menu.

The utilities are accessed using the DOS Debug function. To access the

utilities, boot using a standard DOS system disk or diskette. When the DOS

prompt symbol appears on the screen, enter the debug program by typing debug

and pressing Enter. The debug program will load and present a new cursor,

the - symbol. Enter the following line and press enter to invoke the On-

board Utility program.

G=dc00:6

If a BIOS base address other than DC000H is being used on the adapter to be

tested, replace the dc00 parameter with the actual BIOS base segment. The

segment is the upper 16 bits of the 20 bit BIOS base address.

The menu options include the following items which are selected by pressing

the index number of the menu function.

(1)  A list of all devices installed on the SCSI bus may be requested. This

information is retrieved via a RETURN INSTALLED DEVICES host IOCP command,

followed by INQUIRY commands to all available targets.

(2) A device may be selected for execution of the following two functions.

Pressing the '2' key toggles the selection from one installed device to the

next.

(3) A low level, or primary, format of the selected Target/LUN can be

requested. A MODE SELECT is issued first to set the block size at 512 bytes

per sector. Then a FORMAT command is issued which will format the drive with

factory as well as grown defects.  Only one SCSI drive can be formatted at a

time.

(4) Integrity of a target can be checked by verifying an entire drive. Any

correctable errors found will automatically be handled through the SCSI

Reassign Blocks Command. If any uncorrectable errors are found the user will

first be asked via a screen prompt if reassignment of the effected block

should be performed.

(5) The user can leave the AHA-154XA On-board Utility functions and return

to normal DOS operation.

In addition to the On-Board Utilities, a DMA channel test also resides on

the AHA-154XA BIOS. The test verifies that all DOS-available memory can be

accessed correctly from the host adapter using first party DMA. The test

takes from 2 to 10 minutes to execute, depending on the CPU speed and the

amount of memory available. The test is invoked by entering the debug

program as above and entering: G=DC00:9

The program then is activated and verifies each segment of the memory,

indicating with screen messages as each segment is verified. The test runs

indefinitely or until an error is detected. In most cases of error, the

error information cannot be presented since the host operating system is

likely to be compromised. The error symptom is usually a system crash. The

test is halted by rebooting.

8.0 SCSI FEATURES

8.1 SCSI DESCRIPTION, INITIATOR MODE

The AHA-1540A/1542A provides a very high performance SCSI interface

connection. The host adapter meets the ANSI Standard X3.131-1986, describing

the SCSI. The host adapter additionally meets conformance level 2 of the

specification by implementing the following SCSI options:

Accepts or manages the following messages:

Command Complete

Disconnect

Message Reject

Identify

Save Data Pointer

Performs arbitration

The following alternatives are selected for the AHA-1540A/1542A SCSI

interface connection from those described by X3.131-1986.

Single-ended SCSI driver/receivers are used.

The Termination Power is supplied through a removable fuse, allowing the

user to select whether or not termination power is provided by the host

adapter at installation time.

Parity is always generated. Parity checking can be disabled through the

jumper selection.

The "Soft" Reset option is always performed by the AHA-1540A/1542A. If the

user desires the "Hard" Reset, an interrupt servicing program that detects

the SCSI Reset Detected interrupt in the Interrupt Flags Port must set the

Soft Reset bit in the Control Port. This bit forces the host adapter to

clear all SCSI related operations, but does not require execution of the

diagnostic functions. If the Soft Reset bit has been set, the host software

must re-initialize the AHA-1540A/1542A.

Synchronous data transfer is supported. Negotiation for the synchronous

transfer initiated by another SCSI device will be accepted by the AHA-

1540A/1542A at any time. If the jumper has been set to allow it, the AHA-

1540A/1542A will also initiate synchronous data transfer negotiation when it

has detected that  such negotiation may be required.

Multitasking is fully supported.

Modify Data Pointers is supported to allow Zero Latency Read operations.

In addition to these SCSI functions, the SCSI Common Command Set at level 4B

is also supported. While this document was never made a standard, it

describes a widely available set of disk drive functions which are supported

by the host adapter. The Host Adapter BIOS commands are all mapped in to

SCSI CCS commands to allow the proper support of all the most common SCSI

disk drives. The Adapter Command "Return Installed Devices" also uses CCS

commands to determine which devices are available.

The draft ANSI standard for SCSI-2 has been used as a reference for the

implementation of all SCSI functions with the expectation that the host

adapter will be fully compatible with the final version of the SCSI-2

standard. In particular, the Processor type device command set has been

selected from the SCSI-2 manual for Target Mode operation.

8.1.1 Linked SCSI Commands

The AHA-1540A/1542A supports linking of SCSI commands in initiator mode.

When the link bit in the SCSI command control byte is set, the target will

present either a 0A or 0B message at command completion. The AHA-1540A/1542A

uses the the link pointer in a CCB to fetch another CCB. At the same time,

the completed CCB status and address are stored in an MBI. If the target

returns a 0B or 00 message, the AHA-1540A/1542A generates an interrupt to

inform the host of the full MBI. If the target returns a 0A message, the

MBIF interrupt is not posted until all linked commands are completed. The

linked CCBs must address the same target and LUN since the target is not re-

selected.

8.1.2 Zero Latency Read Operation

The AHA-1540A/1542A implements Zero Latency operation through the use of

modify data pointer messages. Zero Latency can eliminate rotational latency,

depending on the length of the data transfer, by supporting out of order

data transfers. This advanced feature is currently only implemented on the

ACB-4525Z SCSI to ESDI controller, and is supported transparently to the

user.

Since the Adaptec host adapter family is the first to support Zero Latency

Read operation, this section is intended to briefly explain the required

target support. Please refer to the Adaptec ACB-4525Z SCSI to ESDI

controller manual for complete details of Zero Latency Read operation.

After seeking to the target track, the ACB-4525Z (ACB-4525Z is assumed since

it is the first SCSI controller to implement this feature) will begin

reading block IDs. If the first block ID is within range of the data

transfer, but not the last block of the the data transfer, the ACB-4525Z

will begin reading the subsequent blocks into its buffer. Before

transferring data the ACB-4525Z will issue a MODIFY DATA POINTER message to

the AHA-1540A/1542A. This supplies a positive argument that is added to the

value of the current data pointer. The ACB-4525Z will now send this portion

of the data transfer to the host. The ACB-4525Z will resume reading data

into its buffer as soon as the first block of the data transfer is detected.

Before sending this data to the host, the ACB-4525Z will issue a second

MODIFY DATA POINTER message to the AHA-1540A/1542A which supplies a negative

argument. This returns the data pointer to its original position. This

guarantees that a data transfer of one track or less will never require more

than a single revolution since data can now be transferred out of order.

8.1.3 SCSI Messages

The AHA-154XA host adapter supports a number of special messages in addition

to the messages required by meeting conformance level 2. Those messages are

described in detail in the SCSI specification, X3.131-1986, and in this

section where they are used. The messages are summarized in this table:

FUNCTION                MESSAGE                   CAUSED BY

Standard Messages      Command Complete          Normal Sequencing

Error Management       Message Reject            Invalid Messages

Bus Device Reset          Special CCB

Abort                     Special MBO

Disconnect / Reconnect  Identify                  Normal Sequencing

Disconnect                Normal Sequencing

Save Data Pointer         Normal Sequencing

Restore Pointers          Special Sequencing/ZLR

Synchronous Transfer    Synchronous Data Transfer Initialization Sequencing

Request

Zero Latency Operation  Modify Data Pointers      ZLR Sequencing

Linked Commands         Linked Command Complete   Command Linking

Linked Command Complete   Command Linking

With Flag

8.2 SCSI DESCRIPTION, TARGET MODE

8.2.1 Initiator Conformance Level Requirements

Initiators that  execute commands against an  AHA-1540A/1542A operating in

Target Mode are required to have the following conformance levels, as

described in Appendix E of the SCSI  Specification, ANSI  X3.131-1986.

Conformance must be present  with respect to each of the following items:

The  initiator must use single-ended drivers.

Termination power may optionally be provided by the initiator, but must meet

the SCSI specification  in both its over-current protection and its  reverse

current diode protection.  The terminators may be  installed on the AHA-

1540A/1542A board or  installed as in-line terminators at the cable

connectors.

The implementation of parity is optional, but desirable. The  initiator may

support either "hard" reset or "soft" reset. All attached devices must

support the same type of reset.

The initiator must meet the requirements of conformance level two. In

particular, all LUN addressing must be performed by the IDENTIFY message,

not by the  LUN field in the CDB. Disconnection and reconnection must be

supported.

The initiator and target functions have the same SCSI ID.

8.2.2 Support of Synchronous Transfer

Synchronous transfer is supported by the AHA-1540A/1542A in target mode

without any instruction or support from the system processor. If an

initiator  invokes a synchronous transfer negotiation, the AHA-1540A/1542A

will complete the negotiation of the required transfer offset and period. If

the proper jumper has been set, the AHA-1540A/1542A will also attempt to

negotiate synchronous transfer during the initial selection period of the

first command after an initialization or after a SCSI reset.

8.2.3 SCSI Target Operation in Processor Target Mode

When the AHA-1540A/1542A has been set to respond in Processor Target Mode,

the host adapter appears on the SCSI bus as a normal processor type device

as defined by the SCSI specification. From one to eight LUN's may be

supported, depending on the LUN mask byte in the Enable Target Mode command.

Five SCSI commands are accepted:

TEST UNIT READY

REQUEST SENSE

INQUIRY

SEND

RECEIVE

All other commands are rejected with CHECK CONDITION status. The sense

information will indicate a Sense Key of 05H (Illegal Request) with a Sense

Code of  20H (Invalid Command Operation Code).

The commands that do not perform data transfer to or from the host are

handled completely by the AHA-1540A/1542A with no CCB communication with the

host system. Those commands are the TEST UNIT READY, REQUEST SENSE, and

INQUIRY commands. The SEND and RECEIVE commands must have a CCB from the

host with the proper direction bits, the proper initiator address, and the

proper LUN number to complete the SCSI operation. The SEND and RECEIVE CCBs

may be provided to the host adapter before a command is received on the SCSI

or may be requested after the command is received.

Each time an initiator activates a command to the AHA-1540A/1542A, an

internal sub-channel is activated to manage the command. The sub-channel is

dedicated to that particular LUN-Initiator transaction until all operations

associated with the command are completed. Such operations include

disconnection to obtain a CCB,  pending error conditions, and linked

operations. If all sub-channels are busy, a selection to the AHA-1540A/1542A

will result in the AHA-1540A/1542A accepting the command and then generating

BUSY status immediately. If this occurs, the initiator must re-issue the

command later.

8.2.3.1 TEST UNIT READY

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �test unit ready code(00)�����������������Ĵ�

����������������������������������������������Ĵ

1  �  �LUN(unused=0)�Ĵ�������reserved(00)������Ĵ�

����������������������������������������������Ĵ

2  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

3  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

4  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

5  �  �����������reserved(00)����������flag��link��

������������������������������������������������

The TEST UNIT READY command follows the SCSI specification in all respects.

If the AHA-1540A/1542A has been initialized by the Enable Target Mode

command to the Processor Target Mode, the command will finish normally with

GOOD status and a COMMAND COMPLETE message. The host adapter supports the

normal definition of UNIT ATTENTION on the first operation after power on,

after a SCSI Reset, or after a BUS DEVICE RESET.

If a TEST UNIT READY command is executed against an LUN which was not

allowed by the Enable Target Mode command, then CHECK CONDITION status will

be presented with sense data of Sense Key 5 (Invalid Request) and an Error

Code of 25H (Invalid LUN).

8.2.3.2 REQUEST SENSE

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �request sense operation code (03h)�������Ĵ�

����������������������������������������������Ĵ

1  �  �LUN(unused(00)Ĵ�������reserved(00)������Ĵ�

����������������������������������������������Ĵ

2  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

3  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

4  �  �����������������reserved(00)�������������Ĵ�

����������������������������������������������Ĵ

5  �  ����������reserved(00)�����������flag��link��

������������������������������������������������

If the AHA-1540A/1542A has returned CHECK CONDITION status to a previous

command, the REQUEST SENSE command will obtain the sense information

associated with the error. The sense information will be sent in the

extended sense format according to the SCSI standard. The data format is

given in the following figure:

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �error code(07h orf0h)��������������������Ĵ�

����������������������������������������������Ĵ

1  �  �������������reserved(00)�����������������Ĵ�

����������������������������������������������Ĵ

2  �  ����00���Ĵ�ilf����0�Ĵ���sense key�������Ĵ�

����������������������������������������������Ĵ

3- �  ������������information bytes�������������Ĵ�

�                                              �

6  �  ������������(residue)���������������������Ĵ�

����������������������������������������������Ĵ

7  �  ����������additional sense bits�����������Ĵ�

����������������������������������������������Ĵ

8- �  ����������reserved(00000000h))������������Ĵ�

�                                              �

11  �  ������������������������������������������Ĵ�

����������������������������������������������Ĵ

12  �  ����������additional sense code�����������Ĵ�

����������������������������������������������Ĵ

13  �  ������additional sense code qualifier(00h)Ĵ�

������������������������������������������������

The following errors are detected and presented by the AHA-1540A/1542A while

operating in Processor Target Mode:

Error                         Sense      Additional Sense

Key               Code

No Sense Data                   00                00

Invalid Command Operation Code  00                20

Invalid LUN                     05                25

Invalid Command Parameter       05                26

Power Up Attention              06                29

Reset  Attention                06                29

Interface Parity Error          0B                47

Initiator Detected Error        0B                48

Dumb Initiator                  05                2B

One set of error data may be buffered for each initiator - LU association

possible, up to a total of 56 sets of sense data. No Contingent Allegiance

or Extended Contingent Allegiance state is established.

The Error Code (Byte 0) will be F0h if the residue field is valid and 70h if

the residue field has no information.

The Incorrect Length Indicator (ILI) will be set if an incorrect data

transfer length is executed as described in Section 8.2.4.

The residue is set equal to the transfer length requested in the initiator

CDB minus the target host's specified data length specified as a four-byte

two's complement number.

8.2.3.3 INQUIRY

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �Inquiry Operation code(12h)��������������Ĵ�

����������������������������������������������Ĵ

1  �  �LUN(unused=00)Ĵ���reserved(00)����������Ĵ�

����������������������������������������������Ĵ

2  �  ������������reserved(00)������������������Ĵ�

����������������������������������������������Ĵ

3  �  ������������reserved(00)������������������Ĵ�

����������������������������������������������Ĵ

4  �  ������������Allocation lenth��������������Ĵ�

����������������������������������������������Ĵ

5  �  ������������reserved(00)����������flag��link�

������������������������������������������������

The INQUIRY command provides the information necessary to uniquely identify

the Adaptec AHA-1540A/1542A as a Processor-type device. The information is

returned in the SCSI-2 format. The following information is returned to any

selecting initiator from any selected AHA-1540A/1542A Logical Unit:

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �Peripheral qual'er��processor device type(03h��

����������������������������������������������Ĵ

1  �  ������������reserved(00)������������������Ĵ�

����������������������������������������������Ĵ

2  �  ������������reserved(00)��Ĵ� ansi ver(02)Ĵ�

����������������������������������������������Ĵ

3  �  ���������Response data format(02h)��������Ĵ�

����������������������������������������������Ĵ

4  �  ������������Additional lenth(1Dh)���������Ĵ�

����������������������������������������������Ĵ

5  �  ������������reserved(00)������������������Ĵ�

����������������������������������������������Ĵ

6  �  ������������reserved(00)������������������Ĵ�

����������������������������������������������Ĵ

7  � ��0����0�����0����syn=1�lnk=1��0����0�����0Ĵ�

����������������������������������������������Ĵ

8- �  ����Vendor Identification(ascII)����������Ĵ�

�                                              �

15  �  ����������Adaptec bbb���������������������Ĵ�

����������������������������������������������Ĵ

16- �  �����Product identification(asc11)��������Ĵ�

�                                              �

31  �  ���������AHA-1540bbbbbbbb�����������������Ĵ�

����������������������������������������������Ĵ

32- �  �����Product Revision level(asc11)��������Ĵ�

�                                              �

35  �  ������������������������������������������Ĵ�

������������������������������������������������

If the INQUIRY command is attempted against a Logical Unit that has not been

enabled as a target, byte 0 is returned as 23h, indicating that the LUN is

not installed, but would be a Processor Device if it were installed. The

remaining bytes are returned normally.

If a length shorter than the required 36 bytes is specified by the INQUIRY

command, the number of bytes specified by the command is transferred. If a

length longer than 36 bytes is specified, the command will only transmit 36

bytes.

8.2.3.4 SEND

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �����Send Operation code 0A)��������������Ĵ�

����������������������������������������������Ĵ

1  �  �LUN(unused=00)Ĵ���reserved(00)����������Ĵ�

����������������������������������������������Ĵ

2  �  �������Transfer lenth(msb)����������������Ĵ�

����������������������������������������������Ĵ

3  �  �������Transfer lenth���������������������Ĵ�

����������������������������������������������Ĵ

4  �  �������Transfer lenth(lsb)����������������Ĵ�

����������������������������������������������Ĵ

5  �  ������������reserved(00)����������flag��link�

������������������������������������������������

The SEND command has the format specified by the SCSI standard.

The SEND command transfers data from the initiator to the target. The

information is placed in the area specified by the appropriate CCB. If an

appropriate CCB has not already been provided to the host adapter by the

host software, an MBI entry requesting the appropriate CCB is sent to the

host from the host adapter. In this case, the target host adapter

disconnects from the SCSI until the CCB is made available to the host

adapter through the MBO protocol. An appropriate CCB must have the same

initiator address, target LUN, and direction as is required to complete the

command.

The transfer length  in the SEND command specifies the length in bytes of

data that is sent during the DATA OUT phase. A transfer length of zero

indicates that no data is sent.  Management of incorrect length transfers is

described in Section 8.3.2.6.

The CDB information is included in the returned CCB so that the receiving

host programming can determine whether or not the information transmitted by

the SEND command was application data or asynchronous event notification

data.

8.2.3.5 RECEIVE

bit 7     6     5     4     3    2    1    0

byte����������������������������������������������Ŀ

0  �  �����Receive Operation code(08h)����������Ĵ�

����������������������������������������������Ĵ

1  �  �LUN(unused=00)Ĵ���reserved(00)����������Ĵ�

����������������������������������������������Ĵ

2  �  �������Allocation lenth(msb)��������������Ĵ�

����������������������������������������������Ĵ

3  �  �������Allocation lenth�������������������Ĵ�

����������������������������������������������Ĵ

4  �  �������Allocation lenth(lsb)��������������Ĵ�

����������������������������������������������Ĵ

5  �  ������������reserved(00)����������flag��link�

������������������������������������������������

The RECEIVE command has the format specified by the SCSI standard

The RECEIVE command transfers data from the target to the initiator. The

information is taken from the area specified by the appropriate CCB. If an

appropriate CCB has not already been provided to the host adapter host

software, an MBI entry requesting the appropriate CCB is sent to the host

from the host adapter. In this case, the target host adapter disconnects

from the SCSI until the CCB is made available to the host adapter through

the MBO protocol. An appropriate CCB must have the same initiator address,

target LUN, and direction as is required to complete the command.

The transfer length  in the RECEIVE command specifies the length in bytes of

data that is sent during the DATA IN phase. A transfer length of zero

indicates that no data is sent. Management of incorrect length transfers is

described in section 8.3.2.6.

8.2.4 Incorrect Length Management for Target Mode Operation

The messages transmitted using the SEND and RECEIVE commands are normally

expected to have a length that has previously been agreed to by the

initiator system software and by the target system software. If the transfer

length specified by the command is equal to the transfer length specified by

the CCB,  normal operation takes place and no errors are posted.

If the transfer length specified by the initiator's command is shorter than

the space defined by the target CCB, all data bytes expected and required by

the initiator will be transmitted. The target AHA-1540A/1542A will indicate

GOOD status on the SCSI at the end of the transfer. The target system's

software however must be notified that the entire area of data defined by

the CCB was not transmitted. A  Target Status of GOOD will be presented in

the returned CCB. At the same time,  the Incorrect Length Indication bit

(bit 5 of byte 2) will be set in the CCB Request Sense data area. Bytes  3

through 6 of the Request Sense data area will contain the residue in two's

complement notation of the length requested in the initiator command minus

the length of the data area defined by the CCB. In this case, the residue

will be a negative number, since the requested length was less than the area

specified by the CCB. The HA status stored in the CCB will be 12h,

indicating a Data Over/Under Run. The MBI Status Byte will be set to 04 to

indicate that the CCB was completed with an error. If the transfer length

specified by the initiator's command is longer than the space defined by the

target CCB, only those bytes contained within the CCB's data transfer area

will be transmitted. The target AHA-1540A/1542A must indicate with an error

condition to the initiator that not all the requested bytes could be

transferred. The target AHA-1540A/1542A presents a SCSI status of CHECK

CONDITION at the end of the data transfer. The Request Sense information

transmitted to the initiator as a result of an immediately following REQUEST

SENSE command indicates that an Incorrect Length Indication is present by

setting bit 5 of byte 2. Bytes 3 through 6 of the Request Sense information

transmitted to the initiator will contain the residue in two's complement

notation of the length requested in the initiator command minus the length

of the data area defined by the target CCB. In this case, the residue will

be a positive number, since the requested length was greater than the

available area. The target system's software must also be notified that the

transfer length requested by the initiator exceeded the assigned buffer

area. A Target Status of CHECK CONDITION will be presented in the returned

target CCB. A Host Status of '12'h will be returned indicating a Data

Over/Under Run. At the same time, the information that will later be posted

to the initiator by the REQUEST SENSE command will be posted to the target

system in the CCB Request Sense data area. This includes both the Incorrect

Length Indicator and the Residue. The MBI Status Byte will be set to 04 to

indicate that the CCB was completed with an error.

9.1 FLOPPY DISK INTERFACE

The floppy disk controller allows the attachment of any standard IBM

compatible floppy disk drive to the AHA-1542A. The floppy disk controller is

accessed by the standard IBM compatible BIOS through the standard IBM

compatible floppy diskette program interface. The floppy disk controller

section of the host adapter is completely independent of the host adapter

function. The floppy disk controller section can be disabled by removing

jumper J13 as described in section 3.4.2. The base address and interrupt and

DMA channels can be modified by jumpers as described in section 3.4.

A floppy disk is attached to the host adapter using a 34 pin ribbon cable

connector as specified in section 1.4. The pin out of the cable is described

here, although that is usually not necessary, since all standard floppy disk

drives have the same pin out. Some systems choose to use the same address

for all floppy disk drives and to twist the cable, pins 10-16, to switch the

address lines at the drive.

Signal Name       Pin      Pin      Signal Name

Ground             1        2           -LD

Ground             3        4        Reserved

Ground             5        6        Reserved

Ground             7        8        -INDEX

Ground             9       10        MOTOR ENB DRIVE A

Ground             11      12        DRIVE SELECT B

Ground             13      14        DRIVE SELECT A

Ground             15      16        MOTORO ENB DRIVE B

Ground             17      18        -DIRECTION

Ground             19      20        -STEP

Ground             21      22        -WRITE DATA

Ground             23      24        -WRITE ENABLE

Ground             25      26        -TRACK 0

Ground             27      28        -WRITE PROTECT

Ground             29      30        -READ DATA

Ground             31      32        -HEAD SELECT

Ground             33      34        -DSKCHNG

10.0 PROBLEM DETERMINATION

10.1 Self Diagnostic Capability

The AHA-1540A/1542A executes self diagnostics upon power up or after a hard

reset. These diagnostics test the CPU operation, perform a sum check on the

EPROM, and check the data transfer paths on the board. The host system has

the option of exercising more extensive diagnostics involving reads and

writes to memory.

The red light emitting diode (LED) on the host adapter indicates the result

of the self diagnostic process.  When power is first applied to the board,

the LED turns on. If the board is operating normally, the light will soon go

off and stay off until SCSI or I/O port activity is requested by the host.

If the board is not operating correctly,  a flash code number is flashed on

the LED to indicate which test failed. The flash code number is indicated by

a series of 1 to 6 closely spaced flashes followed by a longer pause. The

flash code is repeated continuously until the board is powered off, reset,

or repaired.  The SCSI interface should be disconnected if these diagnostic

tests are being run for fault isolation purposes. At least one set of

terminators must remain installed or the LED will stay on, indicating that

the AHA-154XA is receiving an active RST signal. Continuous execution of the

diagnostics can also be forced by inserting the diagnostic jumper (pin-pair

2 in jumper J1). The flash code and associated failure modes are indicated

in the following table.

TABLE OF FLASH CODES

FLASH CODE    POSSIBLE MEANINGS OF FLASH CODE

LED           Remains On Host Adapter Control Processor

inoperative/terminators missing or not powered.

1 Flash       Firmware EPROM check sum failed.

2 Flashes     RAM test failed.

3 Flashes     AIC-6250 SCSI protocol chip verification failed

4 Flashes     FIFO read data path test failed.

5 Flashes     FIF  write data path test failed.

6 Flashes     Channel 2 test failed.

If any of these failure conditions is identified, the host computer should

be powered down, the host adapter removed, and the host adapter inspected

for physical damage. Such damage can include EPROM's that are not correctly

installed or not firmly seated, broken wires, missing or damaged components,

or conductive debris on the board. If no such physical damage is found, the

AHA-154XA should be returned for repair.

All boards are fully tested, burned in,  cleaned, and inspected before they

are shipped. Care should be taken to keep the board in its protective

conductive wrapping until it is installed. With these simple precautions,

mechanical damage can normally be avoided.

10.2 INDICATORS

The red light emitting diode (LED) on the AHA-154XA is used to provide the

fault isolation information described above. In addition, the LED indicates

when the host adapter is performing activities on the host interface and the

SCSI interface. The LED is such a useful activity indicator that a connector

is made available on the host adapter to allow the cable to an externally

visible LED to be attached. (See Section 3.3.9).

As an activity indicator the LED is turned on from the time that an Adapter

Command is transmitted to the host adapter until the HACC interrupt is

generated to indicate that the Adapter Command is complete. In addition, the

LED will be turned on whenever SCSI bus activity is occurring. This is

roughly the same period of time that the SCSI BSY signal is present on the

SCSI bus. If the light stays on when no activity is expected to be present

on the bus, it is possible that the bus is hung, the processor has failed,

that unexpected activity is occurring, or that the SCSI cables are

incorrectly installed. If the SCSI cables are installed reversed, the host

adapter is forced in to a solid SCSI reset state which halts normal

operation.

10.3 PROBLEMS DETECTED DURING OPERATION

The information in section 10.3 is not intended for routine users of the

AHA-1502A/1542A. It is intended to provide a reference for programmers

preparing device drivers, error recovery procedures, and error information

presentation programs.

Operation of the I/O Port interface is controlled and monitored by host

system software. Two bits in the Status Port (Base + 0), described in

section 4.2.1, are provided to indicate unusual conditions in the host

adapter.

Internal Diagnostic Failure (Status Port, bit 6): This bit indicates that

the self testing process after either a hard reset or a power on operation

was terminated by an error condition. The error indicates that a critical

failure was found in the AHA-154XA control circuitry or data paths.   The

error should be presented to the host video display to indicate that

diagnostic actions are required. The diagnostic action uses the host adapter

diagnostic flash codes to determine if the AHA-154XA is failing. The host

system should be powered off, then opened so that the LED can be observed.

All SCSI cables, both external and internal, should be removed from the host

adapter. The system should then be powered on. If the LED flashes once and

turns off, the host adapter has passed its diagnostics and should be

accessible again. If the LED flashes one of the error flash codes, the AHA-

154XA  should be replaced. If the LED remains on and does not turn off, the

host adapter is not able to begin operation at all. The host system may be

holding the AHA-154XA in the reset state or the host adapter may have

failed. To distinguish these two cases, the AHA-154XA should be replaced and

the new host adapter operation should be verified. Be sure to power down the

host system when removing or replacing the host adapter. If the diagnostics

indicate that the AHA-154XA must be replaced, it should be sent to an

authorized service facility for repair.

Invalid Host Adapter Command (Status Port, bit 0):  This bit is set to

indicate that a command or parameter transmitted to the host adapter was

invalid.  This is an indication that the host software, usually an operating

system, special application program, or device driver, has generated an

invalid request. The responsible programming organization should be notified

so that the invalid sequence or parameter can be corrected.

10.3.1 HA Status Error Indications and Corrective Actions

The Host Adapter Status indications, in addition to indicating several

normal completion states, indicate three general groups of errors. One set

describes software errors in the requests made to the host adapter. Even

though most of these errors are caused by software design problems, the AHA-

154XA may need to be replaced as an isolation step, since there exists a

small chance that hardware failures in the AHA-154XA cause the appearance of

a software error.

A second set describes errors detected by the host adapter firmware in the

host adapter hardware. The most likely failing component in this case is the

AHA-154XA, although certain system or cable failures may generate the

appearance of a host adapter hardware error.

A third set describes the appearance of unexpected or incorrect sequences

executed by the attached SCSI devices. In this case, the cables are the most

likely failure point, with the peripheral device second and the AHA-154XA

third. The host adapter status code should be returned to the host video

display to indicate to the operator what errors have occurred and under what

conditions the errors occurred. The following section describes those Host

Adapter Status indications that indicate an error.

NUMBER TYPE ERROR   DESCRIPTION AND CORRECTIVE ACTION

11     SCSI NO      Selection Time Out: The SCSI attempted to select a

device that was not installed or that did not respond to

selection due to a power, parity or addressing failure.

Verify correct address values set to SCSI devices.

Verify that SCSI cable routing includes the required

devices. Verify  SCSI cable integrity by replacement of

cables. Verify that SCSI Selection Time Out value has

been correctly established by Adapter Command.

12     HOST NO      Data Over Run / Under Run: Data length or direction

specified by CCB did not agree with the data length

actually provided by the attached peripheral device.

Often a normal error or accompanied by a check condition

indicating transfer truncation. Verify program requested

correct length or direction. Verify peripheral provided

expected data length, number of blocks, or block length.

NUMBER TYPE ERROR   DESCRIPTION AND CORRECTIVE ACTION

13     SCSI  YES    Unexpected Bus Free: The target dropped BSY without

executing the proper messages first. This normally

indicates that the BSY and/or other portions of the SCSI

bus failed or that the target encountered such an

invalid sequence that no recovery was possible. Some

targets may have sense information available to qualify

the error condition. Verify SCSI cable integrity. Verify

SCSI cables are all properly connected. Replace SCSI

cables. Replace peripheral. Replace AHA-154XA.

14     SCSI  YES    Target bus phase sequence failure:	The initiator detected

an invalid phase or an invalid phase sequence. If this

occurs, it is likely that the host adapter forces a SCSI

reset on the bus as the first phase of the recovery

process. Verify SCSI cable integrity. Verify SCSI cables

are all properly connected. Replace SCSI cables. Replace

peripheral. Replace AHA-154XA

15     HOST  YES    MBO Command Byte invalid:	This indicates a software

failure or bug in the host's development of the MBO

entry. Generally, this indication will only occur during

initial debug processes of new operating systems.

Replace the AHA-154XA. Replace the host computer system.

Contact the software developer for design support.

16     HOST  YES    Invalid CCB Operation Code:	This indicates a software

failure or bug in the host's development of the CCB.

Generally, this indication will only occur during

initial debug processes of new operating systems.

Replace the AHA-154XA. Replace the host computer system.

Contact the software developer for design support.

17     HOST  YES    Linked CCB does not have same LUN:  This indicates that

the host software generated an invalid combination of

link commands. Generally, this indication will only

occur during initial debug processes of new operating

systems. Replace the AHA-154XA. Replace the host

computer system. Contact the software developer for

design support.

NUMBER TYPE ERROR    DESCRIPTION AND CORRECTIVE ACTION

18     HOST  YES    Invalid Target Direction Parameters received from Host:

This indicates that the host software generated an

invalid Target Direction Parameter. Generally,  this

indication will only occur during initial debug

processes of new operating systems. Replace the AHA-

154XA. Replace the host computer system. Contact the

software developer for design support.

19     HOST YES     Duplicate CCB Received in Target Mode: This indicates

that the host system was not keeping correct management

information for target mode operation and incorrectly

generated a second CCB identical to one already active.

Generally,  this indication will only occur during

initial debug processes of new operating systems.

Replace the AHA-154XA. Replace the host computer system.

Contact the software developer for design support.

1A     HOST YES     Invalid CCB Parameter or Segment List: A segment list

was presented to the host adapter with a zero length

segment or invalid segment boundaries. An invalid CCB

parameter was presented to the host adapter.

Generally,this indication will only occur during initial

debug processes of new operating systems. Replace the

AHA-154XA. Replace the host computer system. Contact the

software developer for design support.

10.3.2 SCSI Error Indications and Corrective Actions

Error conditions detected by SCSI peripherals usually cause a CHECK

CONDITION Status byte to be presented. When this is presented, the host

adapter automatically retrieves the sense information from the SCSI

peripheral by executing a REQUEST SENSE command according to the SCSI

standard. The information returned is mapped for single-threaded BIOS

operation into the categories of error conditions described in section

5.2.2. For multitasking (mailbox) operation, the information returned is

made available in the area allocated for Sense Data. In either case, an

intelligent attempt is made by most operating systems to retry the operation

at least one time. Such retry operations may require the management of

queued operations that have already started execution. This attempt to retry

the operation is rarely successful, since SCSI peripheral devices have very

extensive automatic retry and correction mechanisms designed to be executed

before the error information is returned in the first place.

Since it is rare that such an operation is successful, the host software and

operating systems should make every attempt to make available key

information about the error which can be analyzed by the system operator or

customer engineer. In small systems, it may be sufficient to present the

important Sense Data together with the command that created the error. Some

simpler systems may perform a preliminary analysis of the error information

and generate a code or descriptive text that describes the error and

indicates the corrective action. In very complex systems, a logging process

may take place, allowing a customer engineer to analyze the data at some

future time. In all systems, such information is very important for host

program verification and for system integrity verification, even if the

designers choose not to make the information available to any system users

other than the design and maintenance engineers.

The SCSI Sense Data is generally self descriptive. The errors can be mapped

in to four major categories, each category with its own diagnostic

procedure.

The first category includes those errors caused by incorrect command or

parameter bytes or by incorrect sequences of commands. These errors

typically are presented only during the early development stages of an

operating system or device driver, since a properly operating program will

not generate invalid commands.

The second category includes those sense codes associated with peripheral

status presentation. Such  sense codes are not truly errors, in that they

inform the host program of an unusual but not unexpected condition. Such

sense codes include indications that a device is not ready, that the device

has just become ready, that a device has reached the end of its media (End

of Tape or Blank Check),  or that the expected data length and actual data

length differed. These sense codes are normally used by the device driver to

execute the correct operation in response to the condition. Such sense codes

are not normally presented to the system user except as text that requests

some action. As one example, an End Of Tape condition may require the system

user to replace the tape cartridge.

The third category of errors points to a particular hardware failure in the

peripheral device or its supporting electronics. Such error conditions

usually require the adjustment, repair, or replacement of the peripheral

device or some of its components. In some cases, the error condition may

also indicate possible cable or host adapter failures. These errors must be

exposed to the system user so that the proper actions can be taken.

The fourth category  of errors points to a media failure in the peripheral

device. Such errors include bits that cannot be recovered from magnetic

media and imperfections in the surface of optical media. Most such errors

are recovered using the extensive retry and correction algorithms programmed

into the peripheral device. In some peripheral device technologies, the

errors may be caused by noise conditions or by marginal electronic failures

in the read or write path. Those rare errors that cannot be recovered

usually indicate that some data important to the operating system or

application has been lost. The system must make this error information

available to the system user so that the system user can replace the media

or recover the data from a back up copy as required. In addition, logging of

errors that were successfully recovered by the peripheral device is often

useful as an indicator of the overall reliability of the device or of the

requirement for periodic maintenance.

10.4 PROBLEMS DETECTED DURING INSTALLATION

This section may be useful to correct problems related to installation. The

information in this section is included in the Host Adapter Installation

Guide.

If the system will not boot from the flexible diskette drive after initial

hardware installation the following items should be checked:

AHA-1540A/1542A internal diagnostics: The LED on the AHA-1540A/1542A should

come on briefly at  system power up. If the LED begins to blink at regular

intervals then the host adapter has detected an internal failure and should

be returned for repair or replacement to the place of purchase. A message

may also be posted to the screen.

If the AHA-1540A/1542A LED and the SCSI drive LED are always on, the SCSI

cable's pin 1 orientation has probably been reversed between the host

adapter and the drive. See section 3.2.

Is the AHA-1540A/1542A BIOS message displayed on the screen? If not, the

AHA-1540A/1542A BIOS is not being recognized by the system.

a) Check for BIOS address conflicts between the AHA-1540A/1542A and other

option boards.

b) Try a different BIOS address. See section 3.3.

c) Change the BIOS wait state jumper. See section 3.3.

If the HOST ADAPTER NOT FOUND AT PORT 330H message is displayed check the

AT port address jumper setting. Also verify that the SCSI cable is correctly

installed. A forced SCSI reset caused by an inverted or displaced internal

SCSI cable may cause the problem. See Section 3.3.

Problems Booting the System from a SCSI drive:

Make sure that both standard hard disks are mapped out of the system by

using the AT SETUP program.

Make sure that the SCSI boot drive address is set to SCSI ID 0:0. Check the

drive installation manual for information about setting the SCSI ID for that

device. The Return Installed Devices utility in the Onboard Utilities can

also be used to determine the SCSI addresses of peripherals on the SCSI bus.

Make sure that SCSI parity is consistently enabled or disabled on all

devices on the SCSI bus.

Verify that the host adapter and the SCSI devices are properly configured

and installed by referencing Section 3.

Power should be cycled off and on after changing any values on a HA, Setup

program, or SCSI device to be sure that a DOS format operation has been

successfully completed.

Make sure that the SCSI bus is properly terminated.

Make sure that the intended boot disk has an active DOS partition and a DOS

format. See section 3.0 and the DOS manual for more information.

Problems using a SCSI drive as drive D: with a standard hard disk as drive C:

Make sure that the second hard disk is mapped out of the system with the AT

SETUP program.

Make sure that the SCSI drive to be used as drive D:  is set to SCSI ID

0:0. Check the drive manual for information on setting the SCSI ID for that

device. The Return Installed Devices utility in the Onboard Utilities can

also be used to determine the SCSI addresses of peripherals on the SCSI bus.

Make sure that SCSI parity is consistently enabled or disabled on all

devices on the SCSI bus.

Verify that the host adapter and the SCSI devices are properly configured

and installed by referencing Section 3.

Power should be cycled off and on after changing any values on an HA, Setup

program or SCSI device to be sure that the new initial values are loaded.

Make sure that the SCSI bus is properly terminated. See Section 3.

Make sure that the disk has a  DOS partition and a DOS format. See section

5.0 and the system DOS manual for more information.

Problems using a SCSI drive as drive D: with another SCSI drive as drive C:

Make sure that both standard hard disks are mapped out of the system with

the AT SETUP program.

Make sure that the SCSI drive to be used as drive D:  is set to SCSI ID 0:1

or 1:0. Check the drive manual for information on setting the SCSI ID for

that device. The Return Installed Devices utility in the Onboard Utilities

can also be used to determine the SCSI addresses of peripherals on the SCSI

bus.

Power should be cycled off and on after changing any values on an HA, Setup

program or SCSI device to be sure that the new initial values are loaded.

Make sure that SCSI parity is consistently enabled or disabled on all

devices on the SCSI bus.

Verify that the host adapter and the SCSI devices are properly configured

and installed by referencing section 3.

Make sure that the SCSI bus is properly terminated. See section 3.

Make sure that the disk has a  DOS partition and a DOS format. See section

3.0 and the DOS manual for more information.

System works erratically. Hangs or the host adapter can't always find the

drives.

Check SCSI parity for consistency.

Check Termination.

Check cable length and integrity.

If host adapter and drive light remain on during a hang condition, make sure

that the SCSI  drive conforms to the Common Command Set revision 4B. (CCS

4B)

If only the host adapter LED remains on during a hang, it is probably a host

adapter, system interface problem. The system may not be capable of First

Party DMA transfers. Check with the System manufacturer for information.

Adapter Command-A command transmitted to the host adapter using the

Command/Data Out Port and the Data In Port. The commands are sequenced using

the Control Port, the Status Port, and the Interrupt Flag Port. Abbreviated

as IOCP command.

AEN-See Asynchronous Event Notification

AHA-AHA-1540A-Adaptec Host Adapter for connecting SCSI devices to the PC/AT

backplane bus. The AHA-154XA is an enhancement of the AHA-1540 (see Appendix

C)

AHA-1540A-The enhanced high performance Adaptec Host Adapter for connecting

SCSI devices to the PC/AT backplane bus.

AHA-1542A-The enhanced high performance Adaptec Host Adapter for connecting

SCSI devices and standard IBM compatible floppy disk devices to the PC/AT

backplane bus.

AHA-154XA-Either the AHA-1540A or the AHA-1542A or both.

AHA-1540A/1542A-Either the AHA-1540A or the AHA-1542A or both.

Asynchronous Event Notification-A process by which a target can send

unsolicited sense information to an initiator using the SEND command in

order to inform the initiator about the occurrence of an important unusual

occurrence.

Asynchronous Data Transfer-Data transfer performed by the SCSI interface

involving the interlocking of a signal to the initiator (REQ) and a signal

to the target (ACK) such that each step of the data transfer protocol must

occur before the next step can begin. Characterized by a low data rate and

independence of external timing constraints, including cable length and

circuit response times.

AT Bus-The Industry Standard Architecture bus.

Bus Device Reset-A SCSI message that clears all activity in the target to

which it is addressed.

Byte-An 8 bit unit of data. An octet. A byte is normally the smallest

addressable unit of a memory and the unit of transfer on the SCSI.

CCB-See Command Control Block

CCS-See Common Command Set

CDB-See Command Descriptor Block

Command Control Block-A software object prepared by the host microcomputer

software for the host adapter to provide it all the control information it

needs to execute a SCSI command.  Abbreviated CCB.

Command Descriptor Block-A block of information passed across the SCSI bus to

provide the command, parameter, and address information necessary for the

target to execute the desired functions. Prepared by the host software and

placed in the CCB to be passed to the target by the host adapter.

Abbreviated CDB.

Common Command Set-A defacto standard SCSI command set for communication with

hard disk drives. The Common Command Set (CCS) is the basis for the SCSI-2

command set for all types of peripheral devices.

Control Microprocessor	An integrated circuit computer used to execute the

software that controls the host adapter's operation.

Device Driver-A program that is linked with or attached to an operating

system to map the software interface of the operating system to the

requirements of attached peripheral devices and host adapters.

DMA-See Direct Memory Access

Direct Memory Access-A mechanism that allows hardware control of the

transfer of streams of data to or from the main memory of a computing

system. The mechanism may require set up the host software. After

initialization, it automatically sequences the required data transfer and

provides the necessary address information.

EPROM-Erasable Programmable Read Only Memory.  An integrated circuit used to

store the host adapter firmware and the host adapter BIOS.

FIFO-First In / First Out.-

A queueing order in which items are removed from the queue for execution in

the same order in which they are placed in the queue.

An integrated circuit that buffers data in such a manner that the each byte

placed in the buffer is removed from the buffer in the same order.

Firmware-The software that controls and manages the host adapter. It is

'firm' as opposed to 'soft' because it  is designed in to the host adapter

and cannot be modified by the user.

IBM PC/AT Compatible-Any computer system that emulates exactly the IBM PC/AT

and that uses an ISA backplane bus.

ISA-The Industry Standard Architecture

Industry Standard Architecture-The IBM PC/AT functions have been duplicated

by a number of manufacturers. All the IBM PC/AT� compatible machines use a

backplane bus that very closely emulates the function of the backplane bus

of the PC/AT. Because of the broad usage of this bus structure, it has

become known as the Industry Standard Architecture bus, even though there is

no presently accepted standard for the bus.

initiator-A SCSI device that requests an operation to be performed by another

SCSI device (the target). The initiator provides all the command information

and parameters required to perform the operation, but the details of the

operation are actually sequenced by the target.

host-A microcomputer in which a host adapter is installed. The host uses

software to request the services of the host adapter in transferring

information to and from peripheral devices attached to the SCSI bus

connector of the host adapter.

host adapter-A hardware printed circuit board that installs in a standard

microcomputer backplane and provides a SCSI bus connection so that SCSI

devices can be connected to the microcomputer. A host adapter is

"intelligent" if it has a simple high-level software interface to the

microcomputer. A host adapter is "dumb" if the microcomputer must directly

manage the SCSI protocol using the microcomputer processor.

Logical Unit-A physical or virtual device addressed through a target.

Logical Unit Number-An encoded three-bit identifier for a logical unit.

LU-See Logical Unit

LUN-See Logical Unit Number

Mailbox In-An area in main memory assigned by the host microcomputer software

for communication with the host adapter. The host adapter places status and

pointer information in entries in the Mailbox In (MBI) to indicate to the

host microcomputer what operations have been completed or what information

must be obtained from the host microcomputer.

Mailbox Out-An area in main memory assigned by the host microcomputer

software for communication with the host adapter. The microcomputer software

places commands and pointer information in entries in the Mailbox Out (MBO)

to indicate what operations should be started by the host adapter.

MBI-See Mailbox In.

MBO-See Mailbox Out.

Multi-tasking Operation-The execution of commands in such a way that more

than one command is in progress at the same time, allowing the system to

take advantage of overlapping activities by using resources that are

temporarily not required for other operations. More than one program or more

than one portion of a program may be operating in parallel.

PC/AT-A family of small computers sold by IBM, also called the Personal

Computer /AT family of computers. The name is trademarked by IBM.

RAM-Random Access Memory. Memory of which any byte can be accessed directly

in a single memory cycle. Information can be read from and written to the

memory.

SCSI-Small Computer System Interface.

SCSI ASC-SCSI Additional Sense Code. Byte 12 of the extended sense

information. Provides a standardized description of the condition described

by the sense information.

SCSI Device-A device attached to a Small Computer System Interface bus

cable. The device may be an initiator, a target, or capable of both types of

operation. The device may be a peripheral device, a host device, or a device

mixing both roles.

Single Threaded Operation-Operation of the computing system such that only

one program can be operating or active at a time. The computing system must

wait until all resources are available before starting an operation and

cannot start another operation until the first one is completed. No

overlapping of latencies or program operation occurs.

Synchronous Data Transfer-A method of data transfer on the SCSI bus

involving clocking data on to the bus with a fixed-length fixed-frequency

strobe pulses. The acknowledgements may be delayed several clock periods

from the data requests. Synchronous Data Transfer can be used only for data

transmission on the SCSI bus. It is prohibited for command, message, and

status transmission.

Synchronous Data Transfer Negotiation-The message exchange between the

initiator and the target  that allows the negotiation of the data transfer

frequency and delay between requests and acknowledgements required for

synchronous data transfer. Once negotiated, synchronous data transfer

parameters remain unchanged until certain reinitialization activities occur.

Target-A SCSI device that performs an operation requested by an initiator.

The target may be  a peripheral device performing a service for an

initiator. The target may also be a host adapter performing a Processor type

device service for an initiator.

Word-A two byte (16 bit) unit of data.

AHA-1540A/1542A MEMORY CYCLE TIMING

to see this diagram send for manual.

STANDARD   tRR   TIMING

SPEEDS     tWW

5.0 MB     250     A      tARS Address Setup to MEMR Leading Edge

5.7 MB     200     A      tARH Address Hold from MEMR Trailing Edge

6.7 MB     200     A      tBRS SBHE Setup to MEMR Leading Edge

8.0 MB     150     A      tDRS Data Setup to Trailing Edge of MEMR

10.0 MB    100     B      tDRH Data Hold from MEMR Trailing Edge

tRW	Read Pulsewidth

SPEED   TIMING A    TIMING B  tAWS Address Setup to MEMW Leading Edge

tAWH Address Hold from MEMW Trailing Edge

tARS    73           23       tBWS SBHE Setup to MEMW Leading Edge

tARH    55           55       tDWS Data Setup to MEMW Leading Edge

tBRS    73           23       tDWH Data Hold from MEMW Trailing Edge

tWW  Write Pulsewidth

tDRS    32           32

tDRH    26           26       Note: I/O Channel Ready may slow the actual

tAWS    73           23             transfer rate

tAWH    55           55

tBWS    73           23

tDWS    13           13

tDWH    60           60

Appendix B CONNECTOR PINOUT

Internal Connector Pin Assignments

Signal Name   Pin   Pin  Signal Name

Ground         1     2    -DB(0)

Ground         3     4    -DB(1)

Ground         5     6    -DB(2)

Ground         7     8    -DB(3)

Ground         9    10    -DB(4)

Ground        11    12    -DB(5)

Ground        13    14    -DB(6)

Ground        15    16    -DB(7)

Ground        17    18    -DB(P)

Ground        19    20    Ground

Ground        21    22    Ground

Ground        23    24    Ground

Open          25    26    Term Power (Fused)

Ground        27    28    Ground

Ground        29    30    Ground

Ground        31    32    -ATN

Ground        33    34    Ground

Ground        35    36    -BSY

Ground        37    38    -ACK

Ground        39    40    -RST

Ground        41    42    -MSG

Ground        43    44    -SEL

Ground        45    46    -C/D

Ground        47    48    -REQ

Ground        49    50    -I/O

External Connector Pin Assignments

Signal Name   Pin   Pin   Signal Name

Ground         1     26    -DB(0)

Ground         2     27    -DB(1)

Ground         3     28    -DB(2)

Ground         4     29    -DB(3)

Ground         5     30    -DB(4)

Ground         6     31    -DB(5)

Ground         7     32    -DB(6)

Ground         8     33    -DB(7)

Ground         9     34    -DB(P)

Ground        10     35    Ground

Ground        11     36    Ground

Ground        12     37    Ground

Open          13     38    Term Power (Fused)

Ground        14     39    Ground

Ground        15     40    Ground

Ground        16     41    -ATN

Ground        17     42    Ground

Ground        18     43    -BSY

Ground        19     44    -ACK

Ground        20     45    -RST

Ground        21     46    -MSG

Ground        22     47    -SEL

Ground        23     48    -C/D

Ground        24     49    -REQ

Ground        25     50    -I/O

Appendix C	AHA-1540/1640 FAMILY COMPATIBILITY INFORMATION

Brief summary of board characteristics:

AHA-1540:

High performance SCSI to ISA Bus Master host adapter. Most revisions support

synchronous data transfer. Separate user's manual. Subset of AHA-154XA

Adapter Commands available.

W AHA-1542A:

High performance SCSI to ISA Bus Master host adapter. Supports synchronous

data transfer. Supports floppy disk drives. Higher level of host adapter

integration.

AHA-1542A:

High performance SCSI to ISA Bus Master host adapter. Supports synchronous

data transfer. Supports floppy disk drives. Higher level of host adapter

integration. Improved SCSI connection pattern and jumper controls. Supports

Target Mode operation.

AHA-1540A:

High performance SCSI to ISA Bus Master host adapter. Supports synchronous

data transfer. Does not support floppy disk drives. Higher level of host

adapter integration. Improved SCSI connection pattern and jumper controls.

Supports Target Mode operation.

AHA-1640:

Very high-performance SCSI-to-Micro Channel Bus Master host adapter.

Supports synchronous data transfer. Does not support floppy disk drives.

High level of host adapter integration. Supports Target Mode Operation. See

Adaptec's AHA-1640 User's Manual for details, including POS management,

differences in interrupt presentation, differences in reset management and

improvements in Adapter commands.

COMPATIBILITY CHART

Function                  1540      W1542A      1542A      1540A      1640

Floppy Disk Controller      No        Yes        Yes         No         No

Synchronous Transfer        No        Yes        Yes        Yes        Yes

Selectable DMA Speeds       No        Yes        Yes        Yes        POS

Programmable DMA Speeds    Yes        Yes        Yes        Yes        Yes

Internal SCSI Connectors Straight    Rt Angle   Rt Angle   Rt Angle   Straight

Scatter Gather              No        Yes        Yes        Yes        Yes

64 Head BIOS              Option      Yes        Yes        Yes        Yes

16 Head BIOS               Yes         No         No         No         No

LED Connector               No        Yes        Yes        Yes        Yes

Target Mode                 No         No        Yes        Yes        Yes

LA 17-19 Jumper            Yes        Yes         No         No         NA

Micro Channel               No         No         No         No        Yes

PC/AT                      Yes        Yes        Yes        Yes         No