adaptec

AHA-1540B/1542B

Addendum

to the

User's Manual

and

Installation Guide

Using Disk Drives Over 1

Gigabyte Under DOS 5

Extended Translation

Adaptec host adapters have always supported the full

range of disk drive sizes under all major operating systems.

As disk drives have recently grown beyond

1 Gigabyte in formatted capacity, they have run up

against the DOS 1024 cylinder limit.

To continue to provide support of all SCSI disk drive

capacities under DOS, Adaptec has introduced

extended translation. This new feature bypasses the

DOS disk capacity limit and supports disk drives up to

8 Gigabytes in size under DOS, providing room for

years of disk drive evolution.

The DOS One Gigabyte Limit

All versions of DOS are limited to 1024 cylinders of capacity

per drive. The standard translation scheme for

SCSI host adapters, using 64 heads and 32 sectors, provides

a maximum accessible capacity of 1 Gigabyte.

To eliminate the 1 Gigabyte limit, Adaptec's new extended

translation feature uses 255 heads and 63 sectors,

extending the disk drive capacity limit under

DOS to 8 Gigabytes.

When to Use Extended Translation

With DOS Version 5 Only

NetWare 386 and the newer versions of UNIX (such as ATT/USL all versions,

SCO v3.2.4 or later, ISC v3.0 or later) do not

share the 1024 cylinder limit of DOS and do not require

extended translation to support large disk

drives.  0S/2 does not currently support extended

PtPHPranslation, but IBM OS/2 v2.0 has support for full capacity with drives

greater than one Gigabyte using HPFS partitions (exception:  the boot partition

is limited to one Gigabyte or less).  NetWare 386 has full support for drives

greater than a gigabyte with the standard translation scheme.

With Disks Supported by the Host

Adapter BIOS Only

In a system that has only SCSI disk drives, the host

adapter BIOS controls only those drives at SCSI IDs 0

and 1. For systems that have a mix of SCSI and non-

SCSI disk drives, see the section titled Questions and Answers about extended

translation in this addendum.

Drives With Mixed Partitions

Do not use extended translation on drives formatted

with two or more partitions for different operating systems.

Use standard translation. The sum of the DOS

partitions will be less than 1 Gigabyte. Partitions for

UNIX and NetWare can be larger than 1 Gigabyte

when using standard translation.

How to Use Extended Translation

Back Up All Data

Changing translation schemes ,on a disk drive erases

all data on the drive Back up all drives on the SCSI

bus prior to making the change,

CAUTION

If you have already partitioned a large disk drive

with one translation method, conversion to another

method will erase your data. Be sure to back up

your disk drive prior to any change in the transla-

tion method used.

Install Jumper on J6

Install a jumper on pin pair 2 on jumper block J6.

Refer to Figure 2-1 on page 2-2 of the User's Manual or

Default Settings and Reference Diagram in the

Installation Guide for the location of jumper block J6.

Refer to the figure of jumper block J6 on page 2-9 of

the User's Manual or the Installation Guide for the location

of pin pair 2 (Spare 1).

Installation of this jumper enables extended translation

for disk drives controlled by the BIOS exceeding

1 Gigabyte in capacity. Other disk drives are not eligible

for extended translation and will continue to use

standard translation.

Using fdisk

To install a new disk, or to re-partition an existing

disk, use the fdisk DOS utility as you normally would.

The cylinder size increases to 8 MBytes when you enable

extended translation. The size of the partition you

request must therefore be a multiple of 8 MBytes. If

you request a partition size that is not a multiple of 8

MBytes, fdisk will round up to the nearest whole multiple of 8 MBytes.

Load ASW-1410 v3.0a

Load ASW-1410 v3.0a for ASPI support of more than

two disk drives or for non-disk peripheral devices.

ASW-1410 is also often used for compatibility with 386

memory managers.  You can upgrade for free to ASPI4DOS.SYS v3.0a by using the

UPGRADE program on the Adaptec BBS.

To protect your data, extended translation is not downward

compatible with earlier versions of software drivers.

If you intend to use ASPI-compatible software

with your host adapter for support of more than two

disk drives or for non-disk devices, install ASW-1410

v3.0a (or later).

Questions and Answers About

Extended Translation

What Happens if I enable extended translation

with SCSI drives that have less than 1 Gigabyte in

capacity?

Drives handled by the BIOS will use extended translation

provided they are over 1 Gigabyte in formatted capacity.

Drives with less than 1 Gigabyte of formatted

capacity will use standard translation regardless of

whether extended translation is enabled. Drives handled

by the disk driver ASPIDISK.SYS will continue to use

standard translation and not be capable of DOS partitions

over 1 Gigabyte.

How do I know which hard disk drives the BIOS

controls?

Your system allows up to two hard disk drives under

BIOS control; first SCSI ID O, then SCSI ID 1. Your

system may have either two non-SCSI, one non-SCSI

and one SCSI, or two SCSI hard disk drives.

What if I use older versions of software drivers

with extended translation enabled?

To protect your data, the host adapter will lock out out

driver that does not identify itself as capable of extended

translation. If you need to use software that is

not compatible with extended translation, do not enable

extended translation.

Adaptec's ASW-1410 v3.0a (and later) is the appropriate

ASPI manager software to use with extended translation

enabled.