======================== How to launch your CDROM =============================

1). Config.sys :

rem ----- ASPI -----

device=c:\btdosm.sys  /d  /p330  /n11  /f04

<driver specific switch>:

/d -- Displays driver on load

/p -- Specifies I/O port address

/l -- Provides support for Logical Unit Numbers

/n -- Specifies bus on time (default = 11 Microseconds ;range = 02-15 M/sec)

/f -- Specifies bus off time (default = 04 Microseconds ;range = 01-64 M/sec)

/i -- Specifies INT 13 module to not load (default  = INT 13 is redirected)

/m -- Specifies mailbox structure be reserved for Novell 386 install by CDROM

/e -- Specifies DOS manager rescan SCSI bus for added SCSI devices after boot

/x -- Specifies AT bus master transfer rate (default = 5.7 Mbytes)

Option          AT Transfer Speed

00                5.0 MBytes/sec

01                6.7 MBytes/sec

02                8.0 MBytes/sec

03               10.0 MBytes/sec

04                5.7 MBytes/sec

rem ----- CD ROM -----

device=c:\btcdrom.sys  /D:MSCD002

<driver specific switch> :

/D:device name  ( 8 Characters at most )

/L -- enable to support LUN

rem ----- Not Necessary ----- for Expanded memory

device=c:\qemm\qemm386.sys

or

device=C:\HIMEM.SYS

device=C:\EMM386.EXE

or

device=c:\386max.sys pro=c:\386max\386max.pro NOLOW

rem ----- end of Not Necessary -----

2). Autoexec.bat :

c:\MSCDEX.EXE /D:MSCD002 /L:E /M:10 /V /E

<driver specific switch> :

/D:device name -- must be same as the name which btcdrom.sys specifies.

/L:drive letter -- assign a drive unit for CD-ROM

/M:<value> -- how many sector buffers MSCDEX allocates when it installs itself

/V -- display memeory usage

/E -- use expanded memory if available

* Note - If the BIOS on the SCSI host adapter is enabled,  make sure the

SCSI-ID of the CDROM drive is between 2-6.  With the BIOS on the host adapter

enabled,  it will reserve the SCSI -ID's  0  &  1  for hard drives.

* Note - Device drivers can be loaded high with current version of the driver

V2.00 and current versions of Firmware and BIOS.  The Firmware must be at

least  V 3.35,  and BIOS must be at least V4.70.

*Note - On Some earlier manuals, a type-O was discovered.  The extention on

the BTCDROM.SYS driver was mistyped.  A colon is needed in the  "/D: "

parameter.  The correct syntax is shown above.

Support CD-ROM drives :

1. SONY CDU-541, CDU-561, CDU-6211

2. CHINON CDS-431, CDX-431, CDS-535, CDX-535

3. TOSHIBA XM-3301B, XM-3401B

4. HITACHI CR-3750

5. TEXEL DM-3024, DM-5024

6. NEC CDR-38, CDR-73, CDR-74, CDR-83, CDR-84, CDR-741, CDR-841

7. PANASONIC CDR-532

8. IBM

9. PIONEER DRM-600, DRM-604

10. Other CD-ROM drives which support SCSI-2 command sets

Multisession Photo CD-ROM drive :

1. SONY CDU-6211

2. CHINON CDS-535, CDX-535

3. TOSHIBA XM-3401B

4. TEXEL DM-5024

5. NEC CDR-38, CDR-741, CDR-841

#