How to Set the PCI-0640x for ATA PIO w/Flow Control Timing Modes

---------------------------------------------------------------

From the Small Form Factor Committee's latest proposals and ATA-2 (as of

January 1994), the minimum times in ns for PIO w/flow control modes are:

t1      t2      t2i     t0

addr    active  recover cycle

setup   pulse   pulse   time

(min)   (min)   (min)   (min)

Mode 0  70      165     *       600

Mode 1  50      125     *       383

Mode 2  30      100     *       240

Mode 3  30      80      70      180

Mode 4  25      70      40      125

Mode 5  20      50      25      100

X address valid

|        ___________            _____

DIOW-/DIOR-     |_______/           \__________/

<--t1--> <---t2----> <---t2i--->

<----------t0--------->

NOTE:  t0 = t2 + t2i; however, the minimum values for t2 and t2i sum to

less than t0.  Therefore, t2 and/or t2i must be extended as necessary

to meet the minimum t0 requirement.

Below are the PCI-0640x register settings needed to achieve the various

PIO w/flow control modes at various local bus clock frequencies.  Note that

we have played it conservatively where the timings did not fit nicely into

integer multiples of the clock period.  Therefore, you can do further

tweaking via the PCI640X.EXE utility.

20Mhz Local Bus -> 50 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 40h     47h   |100(2)   200(4)  400(8)  600(12)

Mode 1| 40h     34h   |100(2)   150(3)  250(5)  400(8)

Mode 2| 40h     22h   |100(2)   100(2)  150(3)  250(5)

Mode 3| 40h     21h   |100(2)   100(2)  100(2)  200(4)

min   | 40h     11h   |100(2)   100(2)  100(2)  200(4)

--------------------------------------------------------

25Mhz Local Bus -> 40 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 40h     59h   | 80(2)   200(5)  400(10) 600(15)

Mode 1| 40h     45h   | 80(2)   160(4)  240(6)  400(10)

Mode 2| 40h     32h   | 80(2)   120(3)  120(3)  240(6)

Mode 3| 40h     22h   | 80(2)    80(2)  120(3)  200(5)

min   | 40h     11h   | 80(2)    80(2)   80(2)  160(4)

--------------------------------------------------------

33Mhz Local Bus -> 30 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 80h     6dh   | 90(3)   180(6)  420(14) 600(20)

Mode 1| 40h     57h   | 60(2)   150(5)  240(8)  390(13)

Mode 2| 40h     43h   | 60(2)   120(4)  120(4)  240(8)

Mode 3| 40h     32h   | 60(2)    90(3)   90(3)  180(6)

Mode 4| 40h     31h   | 60(2)    90(3)   60(2)  150(5)

min   | 40h     11h   | 60(2)    60(2)   60(2)  120(4)

--------------------------------------------------------

40Mhz Local Bus -> 25 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 80h     70h   | 75(3)   175(7)  425(17) 600(24)

Mode 1| 40h     5ah   | 50(2)   125(5)  275(11) 400(16)

Mode 2| 40h     45h   | 50(2)   100(4)  150(6)  250(10)

Mode 3| 40h     43h   | 50(2)   100(4)  100(4)  200(8)

Mode 4| 40h     31h   | 50(2)    75(3)   50(2)  125(5)

Mode 5| 40h     21h   | 50(2)    50(2)   50(2)  100(4)

min   | 40h     11h   | 50(2)    50(2)   50(2)  100(4)

--------------------------------------------------------

50Mhz Local Bus -> 20 ns/clock

--------------------------------------------------------

|    REGISTER   | t1      t2      t2i     t0

| ARTTIM  DWRTIM| ns(clks)ns(clks)ns(clks)ns(clks)

--------------------------------------------------------

Mode 0| 00h     d0h   | 80(4)   260(13) 340(17) 600(30)

Mode 1| 80h     7ch   | 60(3)   140(7)  260(13) 400(20)

Mode 2| 40h     56h   | 40(2)   100(5)  140(7)  240(12)

Mode 3| 40h     44h   | 40(2)    80(4)  100(5)  180(9)

Mode 4| 40h     42h   | 40(2)    80(4)   60(3)  140(7)

Mode 5| 40h     31h   | 40(2)    60(3)   40(2)  100(5)

min   | 40h     11h   | 40(2)    40(2)   40(2)   80(4)

--------------------------------------------------------

NOTE:  To achieve maximum throughput, it is also beneficial to increase the

speed on the PCI-bus side.  Setting CNTRL (index 51) to 27h will

1) disable host write fifo reg longer data hold time 2) enable fast

DEVSEL timing 3) enable fast host write timing 4) enable fast host

read timing.  If you want to tweak the IDE side timings, use

PCI640X.EXE.

Example:  To set Mode 3 on the master drive on the primary IDE port on

a 33MHz local bus:

pci640x r 51 23  <- optional - specifies fastest host-side timing

pci640x r 52 32

pci640x r 53 40

pci640x r 54 32

or, simply

pci640x t 0 3

pci640x r 51 23  <- optional - specifies fastest host-side timing

Don't forget to set your drive for Mode 3 operation via:

pci640x d 0 3