11-07-94

Installation Guide of DTC3x80/3x70/3x50/3150V Device Driver

for IBM OS/2 version 2.x

-------------------------------------------------------------------------

1.	Version : 1.1

2.	Release diskette :

- One DOS format diskette with the following files :

. DTC3280.ADD : DTC3x80/3x70/3x50/3150V adaptor device

driver for IBM OS/2 2.x

. README.DOC	: This release note file

Note! If above files are downloaded compressed file from DTC BBS,

the first step is to create an Install Device Support Diskette.

(1) Format a blank diskette. (No label required)

(2) Make a directory \OS2X\3280 on the diskette.

(3) De-compress the files and copy them to directory \OS2X\3280

on the diskette.

3.	Installation:

SPECIAL NOTE:

Before you start the installation, make sure that the two sets

of the "Interrupt" jumpers are set to the same interrupt level.

For example, on DTC3280, if the Interrupt "A" is set to level

11, the Interrupt "B" must be also set to level 11. Also if the

interrupt on the controller is used by other device in the

system, the user should change either of them to avoid interrupt

conflict. This situation may easily happen in DTC3X50 or DTC3150V.

For DTC3150V, the "BASEDEV" statement must include a "Memory

Mapped Address" option. For example is the "Memory Mapped Address"

is set to "D800H", the "CONFIG.SYS" file must have a statement

"BASEDEV=DTC3280.ADD /M:D800". Other controllers do not require

"/M:D800" option, because the address can be detected from the

on-board BIOS.

Also, the DTC3150V is a secondary host adapter only. There must

be a primary controller in the system to boot the computer.

The user can use the DTC3150V connecting a CDROM drive to

install OS2 2.x into an IDE disk. This driver will then run

the SCSI hard disks and CDROM drives connected to DTC3150V.

3.1 If OS/2 2.x system has not been installed.

(1) Make a copy of OS/2 system Diskette 1.

(2) Copy the file DTC3280.ADD from DTC distribution diskette

to the root directory of copied Diskette 1.

If the copy of Diskette 1 space was insufficient, delete

the file README.INS.

(3) Except 3150V, add one line "BASEDEV=DTC3280.ADD" as the

last line of the file CONFIG.SYS on the Diskette 1 copy.

`	       For 3150V add  one line "BASEDEV=DTC3280.ADD /M:xxxx" where

xxxx is the "Memory Mapped Address" configured on the board.

(4) Use this copy as diskette 1, follow the OS/2 installation

guide to do system installation.

3.2 If OS/2 2.x,J2.1 system has been installed:

(1) Boot the OS/2 system.

(2) After system boot up complete, insert DTC distribution

diskette to floppy drive A(or B).

(3) Copy the file DTC3280.ADD from DTC distribution diskette

to the \OS2 sub-directory of the hard disk drive.

(4) Except 3150V, add one line "BASEDEV=DTC3280.ADD" as the

last line of the file CONFIG.SYS on the Diskette 1 copy.

`	       For 3150V add  one line "BASEDEV=DTC3280.ADD /M:xxxx" where

xxxx is the "Memory Mapped Address" configured on the board.

(5) Shutdown and reboot OS/2 system.