README.TXT

Author:    Richard Jolliffe / Micropolis UK Office. + 44 (0) 118 9231554

Heino Heusmann / Micropolis Germany  +49 (0) 89 899393 33

Date:        20 June 1997.

Subject:    X43H Firmware, tables and DSP upgrade.

Attached are the files and the utility necessary to download the latest revision of firmware and DSP onto your Micropolis disk drive.

CAUTION

PLEASE ENSURE YOU HAVE A CURRENT BACKUP OF ALL DATA CONTAINED ON THE DRIVE BEFORE ATTEMPTING ANY DOWNLOAD.

1)   Ensure you have the ASPI drivers for DOS loaded.

2)   Create a directory under DOS.

3)   Load the attached files into the directory.

4)   Execute X43H.EXE, (self extracting file).

5)   Connect the drive or drives to be downloaded to the SCSI bus.

6)   Ensure the last device on the bus is actively terminated.

7)   Ensure each SCSI device has a unique ID.

8)   Re-boot the system - the driver should find all the drives.

9)   Type, “Tomadown”.

10)  Select download menu.

11)  Select Firmware Download.  A list of drives and the revision awaiting download will be shown.

Check that the current firmware is P419 or higher.

If the Inquiry shows P409 then please call a Tech. support for more comprehensive instructions.

12)  Hit Return to start the utility downloading the Flash part of the firmware.

The drive or drives will spin down and up one by one.

13)  A message will now appear telling you to power up and down the drives. Wait until

the last drive has completely spun back up and has stopped flashing before power cycling.

14)  Press any key and then hit ESC twice to come out of the program.

15)  Type, “Tomadow2”.

16)  Select Download menu and then Firmware download. A list of drives awaiting download will be

shown.

17)  Hit Return to start the utility downloading the Media part of the firmware.

The drive or drives will spin down and up one by one.

18)  When the download is complete a message confirming this will appear

19)  Finally, a list of drives and the revision will be shown.

Check that all drives have the new  firmware revision (X43h).

Firmware history:  All Tomahawk drives.

Standard                                                                    AV

P409                                                                           AV10    latest 20 June 97

P419

P429

XN29

X43H     latest 20 June 97

-------------- END --------------