DRIVE ROCKET   VERSION 1.01

SEPTEMBER 24, 1993

Welcome to Drive Rocket Software

Installation of Drive Rocket Software

Fast Installation ................................. 1

How do I Register Drive Rocket?

Product Registration .............................. 1

What is Drive Rocket?

Drive Rocket Overview ............................. 2

About ROCKET.BIN

ROCKET.BIN ........................................ 3

About DH.COM

DH.COM ............................................ 4

DH.COM Installation ............................... 4

DH.COM Command Line Switches ...................... 4

README

The README File ................................... 5

Breaking out of Drive Rocket

Pressing the <ESC> Key ............................ 5

Drive Rocket Installation

Easy Mode Installation

Easy Mode Analyze ................................. 6

Is Easy Mode Safe? ................................ 6

Easy Mode Install Process ......................... 7

Rocket Summary Screen ............................. 7

Rocket Rating Table ............................... 8

Drive Rocket Summary Negative ..................... 8

Drive Rocket Advanced Installation

Advanced Installation Overview .................... 9

View IDE Identify ................................. 10

Product Registration .............................. 10

View or Print Manual .............................. 11

Install Drive Rocket .............................. 12

Remove Drive Rocket ............................... 12

Performance Benchmarks

Advanced Mode Benchmarks

Benchmarks Overview ............................... 13

Multiple Block Size Benchmark ..................... 13

Repetitive I/O Benchmark .......................... 14

Sequential I/O Benchmark .......................... 14

Advanced Drive Rocket Options

Altering Drive Rocket Options

Advanced Options Overview ......................... 15

Advanced Configuration Options .................... 15

Choose I/O Modes .................................. 15

READ-ONLY ......................................... 15

WRITE-ONLY ........................................ 16

WRITE-READ ........................................ 16

Choose I/O Size ................................... 17

Enter Starting Record ............................. 17

Enter Ending Record ............................... 17

Benchmark Sampling Duration ....................... 18

Advanced Drive Rocket Benchmarks

Advanced Benchmarks Overview ...................... 18

Repetitive I/O Benchmark .......................... 18

Forward Sequential I/O Benchmark .................. 18

Reverse Sequential I/O Benchmark .................. 19

Random Access I/O Benchmark ....................... 19

Combination I/O Benchmark ......................... 19

Data Integrity Tests

Data Integrity Tests Overview ..................... 19

Data Integrity Tests Menu ......................... 20

Data Integrity Test Options ....................... 20

Data Integrity Test One ........................... 21

Data Integrity Test Two ........................... 21

Data Integrity Test Three ......................... 21

Data Integrity Test Four .......................... 21

Data Integrity Test Five .......................... 21

Command Line Switches

Command Line Switches for ROCKET.EXE .............. 22

Command Line switches for ROCKET.BIN .............. 22

Error Messages

Error Messages for ROCKET.EXE

Can't run under Windows ........................... 24

Can't Access ROCKET.OCS ........................... 24

Cannot Run Easy Mode .............................. 25

Test memory shortage .............................. 25

MBS Not Supported ................................. 25

Error Setting New MBS ............................. 26

Can't Run DIT ..................................... 26

Advanced Drive Error Detected ..................... 26

Easy Drive Error Detected ......................... 27

Error Installing Rocket Software .................. 27

Can't Find Drive Letter ........................... 27

Can't Exist With FASTDISK ......................... 27

Can't Remove FASTDISK ............................. 28

Can't Read CONFIG.SYS ............................. 28

Can't Update Config.sys ........................... 28

Can't Access File ................................. 29

Can't Update File ................................. 29

No Hard Drives found .............................. 29

No Drives Support MBS ............................. 30

Error Launching Rocket ............................ 30

Insufficient Free Space ........................... 31

Destructive Test Warning .......................... 31

Error Messages for ROCKET.BIN

No Drives Found Requiring Use of Driver ........... 32

Syntax Error Detected in Command Line ............. 32

Invalid MBS Value Detected for Drive .............. 32

Hardware / Software Support

Hardware Support

Hardware Supported by Drive Rocket ................ 33

Software Support

Supported DOS versions ............................ 33

Microsoft Windows ................................. 33

Windows V3.1 FASTDISK ............................. 33

SMARTDRV 3.3 ...................................... 34

SMARTDRV 4.0 ...................................... 34

DISK CACHES ....................................... 34

Definition of Terms

Glossary :

AT-COMPATIBLE COMPUTER ............................ 35

BIOS .............................................. 35

CONFIG.SYS ........................................ 35

DATA TRANSFER RATE ................................ 35

HARD DISK ......................................... 35

I/O ............................................... 35

IDE ............................................... 36

INTERRUPT ......................................... 36

MBS ............................................... 36

RAM ............................................... 36

READ/WRITE MULTIPLE ............................... 36

ROM ............................................... 36

SECTOR ............................................ 36

Ontrack Computer Systems

About Ontrack Computer Systems

Ontrack Computer Systems Overview ................. 38

How to Contact Ontrack Computer Systems ........... 38

About Ontrack Data Recovery Inc.

Ontrack Data Recovery Overview .................... 38

How to Contact Ontrack Data Recovery .............. 39

Ontrack Products and Services

Products for Novell NetWare

NetUtils 4 ........................................ 40

NetUtils 3 ........................................ 40

NetOptimizer 2 .................................... 40

NetUtils-2 ........................................ 41

Disk Manager -N ................................... 41

Cognito ........................................... 41

NetWare Essentials Kit ............................ 42

Products For DOS

Disk Manager ...................................... 42

DOSUtils .......................................... 42

Dr. Solomon's Anti-Virus Toolkit .................. 43

Superproms ........................................ 43

Products For Macintosh

Disk Manager Mac .................................. 43

Products For Unix

RecoverEase ....................................... 43

Technical Support Infobase

Technical Support Infobase Information ............ 44

TechSource ........................................ 44

DiskSource ........................................ 45

SystemSource ...................................... 45

NetSource ......................................... 45

Drive Rocket License Agreement

ONTRACK COMPUTER SYSTEMS, INC.

License Agreement ................................. 46

Welcome to Drive Rocket Software

Installation of Drive Rocket Software

Fast Installation

Welcome to Ontrack Computer Systems Drive Rocket Software!

To install your Drive Rocket Software, follow these basic steps:

1) Insert your Drive Rocket diskette into drive A: and type

A: ROCKET.  Help is available for every option by pressing the

F1 key.

2) Select Easy Installation and press Enter.

3) When the benchmark has completed, and Drive Rocket Software

shows an increase in performance, press Y to install Drive

Rocket Software.

4) When the installation has completed, remove the Drive Rocket

diskette from drive A: and reboot from the hard drive.

For the latest information about updates, revisions, and products

from ONTRACK, call Ontrack's Technical Support bulletin board at

612-937-0860.

How do I Register Drive Rocket?

Product Registration

Register this product by using the form included in the Drive Rocket

Software package or the following one and mail or fax it to Ontrack.

Or just call 1-800-752-1333 to register your Drive Rocket software over

the phone for faster service.

Ontrack Computer Systems, Inc.

6321 Bury Drive

Suites 15-19

Eden Prairie, MN 55346 U.S.A.

Hours: 8:00 a.m. to 5:00 p.m.

Monday through Friday CT

Sales             1-800-752-1333

International     1-612-937-1107

Fax               1-612-937-5815

Technical Support 1-612-937-2121

24 Hr BBS         1-612-937-0860

Page 1

FAX TRANSMISSION

________________

TO:            Ontrack Computer Systems

FAX NUMBER:    612-937-5815

FROM:          ___________________________

SUBJECT:       Drive Rocket Registration

# of PAGES (INCLUDING THIS PAGE) _________

______________________________________________________________

PRODUCT REGISTRATION FORM

Upgrade     New Product    (circle one)

Please complete and fax (or mail) this product registration.

We will keep our registered customers informed of updated

versions and new product releases.

Product: DRIVE ROCKET  Ver: 1.00  Serial Number_____________

Name________________________________________________________

Company____________________________Title____________________

Address_____________________________________________________

City_____________________________State______Zip_____________

Telephone __________________________________________________

Where did you hear about the product?

___Computer store ___Computer show ___User recommended

___Prodigy  ___Advertisement in_____________________________

___Article in_______________________________________________

Comments:

______________________________________________________

______________________________________________________

______________________________________________________

______________________________________________________

What is Drive Rocket?

Drive Rocket Overview

The Drive Rocket Software package consist's of drive benchmark software,

driver installation and a device driver that makes use of new technology

Page 2

in most new IDE (AT interface) hard drives called read-and-write-

multiple mode (also known as multiple block size or MBS). By using

this new technique overall data transfer rate (measured in

kilo-bytes per second) can be increased.

When using MBS mode the drive can transfer multiple sectors of

data at a time between the host and the hard drive, rather than

transferring one sector of data at a time (see example below). In

AT and compatible computers, an interrupt needs to occur after each

sector transfer, but in an AT or compatible computer with a IDE

hard drive that uses multiple mode an interrupt occurs only

after a block transfer. Processing interrupts takes time, so

the more data that can be transferred per interrupt, the greater

the transfer rate.

The Drive Rocket Installation utility will determine if your hard

drive can support read-and-write-multiple mode, and the MBS

value (number of sectors per interrupt) that is best suited

for your system. The best MBS depends on many variables such

as the drive, controller, PC, BIOS, and BIOS cache.

If your system had a drive in it that supported read-and-write-

multiple mode, and the drive had a sectors/interrupt of 16 (see

View IDE Identify Data), that would mean that your hard drive

could read or write up to 16 sectors per interrupt.

Example: 512 bytes (1 sector) X 1 = 512 bytes / Interrupt

Drive without Multiple Block support

512 bytes (1 sector) X 16 = 8192 bytes / Interrupt

Drive with Multiple Block support

Drive Rocket Software may increase your performance even if the

drive does not support Multiple Block Size, especially if your system

BIOS does not support shadowing. If shadowing is not enabled, each I/O

request is handled by the system ROM BIOS. When Drive Rocket Software

is installed, its device driver is handling each I/O in RAM which is

which is faster then ROM.

About ROCKET.BIN

ROCKET.BIN

ROCKET.BIN is a DOS device driver which increases data transfer

rates on IDE drives that support the read-and-write-multiple commands.

Drive Rocket Software takes over INT 13, (DOS Interrupt 13) and from

that point on all drive access is directed through the device driver,

instead of the system BIOS.

Page 3

About DH.COM

DH.COM

DH.COM (Drive History) is a Terminate-Stay-Resident (TSR) utility

which intercepts INT 13 (DOS Interrupt 13) and records a history

of all drive access.

Specifically, DH.COM records the I/O size and number of reads and

writes and whether they required a repetitive, forward, or random

seek. This information is useful to understand how your computer

communicates to your hard drive(s) under normal day-to-day

operations.

For example, if when using DH.COM you see that you are doing most of

your work with files between 2.0K and 8.0K, you may want to set up

the Advanced Configuration Options to only test those I/O sizes.

Then run the benchmarks again, looking for the best performance

at those I/O sizes. This may give you even more of an increase to

your overall data transfer rate.

DH.COM Installation

Insert your DRIVE ROCKET diskette into drive A: and type A: DH and

press Enter. DH is now active and recording your drive's access.

Type A:DH and press Enter again anytime at the DOS prompt and the

recorded drive history will be displayed.

DH.COM Command Line Switches

/C Clear the disk history buffer

/2 Switch to a second, or third hard drive.

(i.e., DH /2 - Makes DH.COM track the SECOND hard

drive, /3 would track the THIRD hard drive and so

on.) Default looks at the first hard drive (no switch

needed). DH.COM can only track one drive at a time.

Page 4

README

The README File

The Drive Rocket diskette contains a file named "README". It

provides additional operating instructions and important update

information for this program. This file also describes other

utility products available from Ontrack Computer Systems, Inc.

To view the README file, from the DOS prompt enter this command:

TYPE README (Press Enter)

Ctrl-S stops and restarts the scrolling process. You may also

view or print the README file with your text (or word)

processor.

Breaking out of Drive Rocket

Pressing the <ESC> Key

Pressing the Esc Key will cancel any benchmark or test. Press YES

and Drive Rocket will go to the previous menu. Press NO and Drive

Rocket will continue with the benchmark or test.

Page 5

Drive Rocket Installation

Easy Mode Installation

Easy Mode Analyze

The EASY Installation Option has three basic parts:

1) In the EASY mode ROCKET.EXE will analyze your system and

determine if your hard drive(s) can support read-and-write-

multiple mode.

2) Drive Rocket will then run a series of benchmarks using all

available multiple block size (MBS) values and I/O sizes from

.5K to 64K for three seconds each to determine which MBS

value will give your hard drive(s) the best overall data

transfer rate in Kilo-bytes per second.  When the benchmarks have

completed, a graph showing a before-and-after comparison of your

hard drive's performance will be displayed in the Drive Rocket

Summary screen.

The top bar will indicate the data transfer rate of your hard

drive without the Drive Rocket installed.

The bottom bar will indicate the data transfer rate of your

hard drive if Drive Rocket Software were installed.

Data transfer rates are always shown in kilo-bytes per second.

3) Drive Rocket will then ask if you wish to install ROCKET.BIN

into your system, and you can answer (Y), (N) or Esc. If you

have a second drive in your system and answer (Y), Drive

Rocket Software will install the first drive and automatically

begin analyzing the second drive.

If you answer (N) to installing Drive Rocket after the first drive

has been analyzed, Drive Rocket will automatically begin analyzing

the second drive. If after the second drive analysis is complete and

you answer (Y) the first drive will be installed with a MBS of 0, and

the second drive with the best MBS value.

If you press the <ESC> key after installing the first drive, the

second drive will be installed using an MBS value of 0. In a two

drive system ROCKET.BIN must control both drives. A MBS value

of 0 tells ROCKET.BIN to control the drive but not to run read-

and-write-multiple mode.

Is Easy Mode Safe?

Page 6

Easy mode uses a non-destructive test and will not cause harm to

your data.  You must have a valid boot partition on your hard

drive for Drive Rocket Software to complete the installation (If

your system boots to DOS or WINDOWS you have a valid boot

partition).

The Drive Rocket installation utility assumes that your computer

boots to DOS from the C: partition. If you boot to DOS from a

partition other than C: or if you boot your computer to a floppy

disk, then you must manually edit or create the CONFIG.SYS file

on that boot device to include the ROCKET.BIN statement. You must

also copy the file ROCKET.BIN to the boot device as well.

Easy Mode Install Process

When you install Drive Rocket Software it will automatically copy

ROCKET.BIN to the root directory of your C: drive (C:\) and

update your CONFIG.SYS file with the multiple block size (MBS)

setting that is best suited to your configuration.

Example: Device = C:\ROCKET.BIN /M=32

And for a system with two drives:

Example: DEVICE = C:\ROCKET.BIN /M=32,16

C:\ is the path to find Drive Rocket's device driver ROCKET.BIN.

The /M=32,16 statement specifies an MBS value of 32 for drive

1, and 16 for drive 2. When you reboot your system Drive Rocket

will be enabled.

If Drive Rocket finds a hard drive that does not support

read-and-write-multiple it will execute the benchmarks

using an MBS value of 0. Drive Rocket would then report any

performance increase in the Rocket Summary screen.

Rocket Summary Screen

A Rocket Summary screen will be displayed after the benchmark

has completed. The screen will give you information on the

percentage of increase that Drive Rocket Software can produce for

your hard drive(s).

Easy Drive Rocket installation has calculated an increase in

performance if Drive Rocket were to be installed in your system.

Page 7

Drive Rocket Installation program has performed a sequential

read with several I/O sizes using different MBS values.

Drive Rocket has chosen the MBS value that gives the system

the best overall performance increase.

Do you want to install Drive Rocket (Y/N)?

If you press Y it will automatically update CONFIG.SYS (by adding

the ROCKET.BIN statement to your CONFIG.SYS file) and copy

ROCKET.BIN to your C: drive.

The Performance Analysis Screen also has a feature called the

Rocket Rating Table displayed in the lower left corner. After the

benchmarks are completed a check mark will appear next to

the rating which would apply to your system. The definitions of

the ratings are given in the Rocket Rating Table.

Rocket Rating Table

0% - 4%  Marginal: Drive Rocket Software has calculated only a

marginal increase in performance; this could be caused by

several different conditions.  The hardware may not support

or perform well in the read-and-write-multiple mode, or

there may be drive cache programs enabled.  Disable any drive

cache programs and run the benchmark again.

5% - 14% Average: Drive Rocket has calculated an average increase

in performance. Press Y to install ROCKET.BIN on your

system.

15%-29%  Excellent: Drive Rocket has calculated an excellent

increase in performance. Press Y to install ROCKET.BIN on

your system.

30%-49%  Super Sonic: Drive Rocket has calculated an increase in

performance relative to flying in an F-18. Press Y to

install ROCKET.BIN on your system.

50%-XX%  Warp Speed: Drive Rocket has calculated an increase in

performance relative to flying on the Starship

Enterprise...  More power, Scotty!  Press Y to install

ROCKET.BIN on your system.

Drive Rocket Summary Negative

Page 8

Drive Rocket Software has calculated a decrease in performance

if Drive Rocket were to be installed in your system.

Drive Rocket Installation program has performed a sequential read

with several I/O sizes using different MBS values. The hardware

does not support or perform well in the multiple read and write

mode, or there may be drive cache programs enabled. Disable any drive

cache programs and test again.  To see if the hardware supports

read-and-write-multiple, go into Advanced Drive Rocket Installation

and select View IDE Identify Data, then select the Formatted option.

Sectors/Interrupt is the maximum number of sectors the drive can

transfer per interrupt.  If Sectors/Interrupt is equal to zero or

one, this drive does not support read-and-write-multiple mode.

Drive Rocket Advanced Installation

Advanced Installation Overview

Drive Rocket Advanced Installation option has several benchmarks

which can provide a before-and-after (Drive Rocket) comparison

of your hard drive(s) data transfer rates.

The benchmarks will display two columns indicating the data

transfer rates, and one column showing a percentage-based

calculation of the increase or decrease your hard drive has

achieved.

Column one shows the hard drive's data transfer rate without Drive

Rocket in memory.

Column two shows your hard drive's data transfer rate with Drive

Rocket loaded into memory.

The benchmark, drive number, drive size, and drive model number

are displayed at the top of the page for each Drive Rocket

Performance Analysis report generated by the benchmark programs.

All benchmarks are done using several different I/O sizes (from .5K

to 64K), and the data transfer rates will be reported in kilo-bytes

per second.  Each of these benchmarks can be run in one of three

modes: READ, WRITE, & WRITE-READ. The testing time on each I/O

size is defaulted to 5 seconds, and can be user-defined from 1-60

seconds. To modify these options and others, see Advanced

Configuration Options.

Page 9

View IDE Identify

View IDE Identify Data will show you all the details about your

IDE drive(s). You will be offered two options for examining this

data, Unformatted IDE Identify  and Formatted IDE Identify.

Formatted: This method will break down the first 100 bytes of data

from the IDE Identify Data, and put it into a format that defines

what the numbers are (i.e. model number, firmware revision, serial

number, ...etc). Any data after the first 100 bytes is the

manufacturer unique code, used only by the hard drive manufacturer.

Unformatted: This method will show the IDE Identify Data as used

by hard drive manufacturers. This data is sometimes retrieved

from the firmware (a programmed EPROM on the hard drive itself),

or from the media.

Sectors/Interrupt is the significant entry in this display screen.

It is the maximum number of sectors the drive can transfer per

interrupt.  If Sectors/Interrupt is equal to zero or one, this

drive does not support read-and-write-multiple mode.

NOTE: Some of the first built IDE drives may not support IDE

Identify command.

Product Registration

Register this product by using the form included in Drive Rocket

Software package or the following one and mail, fax it to Ontrack.

For faster service register the software by phone using Ontrack Computer

Systems toll free number listed below.

Ontrack Computer Systems, Inc.

6321 Bury Drive

Suites 15-19

Eden Prairie, MN 55346 U.S.A.

Hours: 8:00 a.m. to 5:00 p.m.

Monday through Friday CT

Sales             1-800-752-1333

International     1-612-937-1107

Fax               1-612-937-5815

Technical Support 1-612-937-2121

24 Hr BBS         1-612-937-0860

Page 10

FAX TRANSMISSION

________________

TO:            Ontrack Computer Systems

FAX NUMBER:    612-937-5815

FROM:          ___________________________

SUBJECT:       Drive Rocket Registration

# of PAGES (INCLUDING THIS PAGE) _________

______________________________________________________________

PRODUCT REGISTRATION FORM

Upgrade     New Product    (circle one)

Please complete and fax (or mail) this product registration.

We will keep our registered customers informed of updated

versions and new product releases.

Product: DRIVE ROCKET  Ver: 1.00  Serial Number_____________

Name________________________________________________________

Company____________________________Title____________________

Address_____________________________________________________

City_____________________________State______Zip_____________

Telephone __________________________________________________

Where did you hear about the product?

___Computer store ___Computer show ___User recommended

___Prodigy  ___Advertisement in_____________________________

___Article in_______________________________________________

Comments:

______________________________________________________

______________________________________________________

______________________________________________________

______________________________________________________

View or Print Manual

(V)iew/Print Manual will let you view and print the Drive

Rocket manual.

Page 11

Install Drive Rocket

(I)nstall Drive Rocket. Installation of Drive Rocket Software in

the Advanced Mode will allow creating and editing CONFIG.SYS file

before copying ROCKET.BIN. If you run the Multiple Block Size

Benchmark before installing Drive Rocket Software, ROCKET.EXE

will update CONFIG.SYS with the best MBS value for the drive.

Remove Drive Rocket

(R)emove Drive Rocket renames the ROCKET.BIN file to ROCKET.BAK

and removes the line that loads ROCKET.BIN from the CONFIG.SYS

file.

Page 12

Performance Benchmarks

Advanced Mode Benchmarks

Benchmarks Overview

You can change the settings of Drive Rocket's benchmarks by

going into the Advanced Rocket Options, and then select Rocket

Configuration Options to alter the options available.  For more

information see the section on ROCKET CONFIGURATION OPTIONS.

All benchmark results have a margin of error of approximately +-2%.

For best analysis, run each benchmark multiple times and take the

average of all results.

Multiple Block Size Benchmark

The Multiple Block Size Benchmark finds the best multiple block

size (MBS) for your system by testing each value at a power of 2

(i.e., 2, 4, 8, 16, 32, 64) while doing a sequential read test

for 5 seconds at different I/O sizes (.5, 1, 2, 4, 8, 16, 32, &

64k).

What is displayed is the average of the sequential read at all I/O

sizes with the best MBS value for your system. Run this benchmark

before any others to select the best MBS value. The best MBS for

your system will have a checkmark next to it. To install Drive

Rocket Software on your system with this MBS value press Esc and

select the Install Drive Rocket option.

Use this as a guideline for improvement.

0% - 4%  Marginal: Drive Rocket Software has calculated only a

marginal increase in performance; this could be caused by

several different conditions.  The hardware may not support

or perform well in the read-and-write-multiple mode, or

there may be drive cache programs enabled.  Disable any drive

cache programs and test again.

5% - 14% Average: Drive Rocket has calculated an average increase

in performance.

15%-29%  Excellent: Drive Rocket has calculated an excellent

increase in performance.

30%-49%  Super Sonic: Drive Rocket has calculated an increase in

performance relative to flying in an F-18.

Page 13

50%-XX%  Warp Speed: Drive Rocket has calculated an increase in

performance relative to flying on the Starship

Enterprise...  More power, Scotty!

Verify that your system BIOS has shadow and cache enabled.

Repetitive I/O Benchmark

The Repetitive I/O Benchmark calculates data transfer rate while

keeping the Read/Write heads on your hard drive(s) in the same place

(the test stays at one cylinder).  This is a theoretical calculation of

the data transfer rate of your configuration.

Some benchmark utilities use a similar technique to test data

transfer rates by using the first 64K only. Use this benchmark for

comparison purposes only, NOT to find the best MBS value.

Sequential I/O Benchmark

The Sequential I/O Benchmark calculates data transfer rate while

doing a forward sequential read/write of consecutive records (a

record being 512 bytes of data). This is the test that the Multiple

Block Size Benchmark is using to determine the best MBS value.

Page 14

Advanced Drive Rocket Options

Altering Drive Rocket Options

Advanced Options Overview

The (A)dvanced Drive Rocket Options will allow selection of

different MBS values, I/O modes, I/O size, starting and ending

records, or sampling rates, as well as more benchmarks and data

integrity tests, allowing the true rocket scientist to test his/her

configuration in many different ways.

Advanced Configuration Options

The Choose MBS Value option lets the user select the default or

a user-defined Multiple Block Size (MBS).

The default MBS for Drive Rocket is determined in several ways.

Drive Rocket first looks to see if the Multiple Block Size

benchmark has been run. If so, Drive Rocket will use the best MBS

found. If not, Drive Rocket will look to see if an MBS was loaded by

ROCKET.BIN. If no MBS is found, Drive Rocket defaults to the maximum

multiple block size divided by two.  A user-definable MBS can be

entered at the prompt upon completion of the first benchmark pass

without Drive Rocket and prior to the pass with Drive Rocket.

All benchmarks operate in two passes in order to calculate

performance before and after the use of Drive Rocket Software.  In

this mode, pass number two (with Rocket) will run multiple times

allowing a new MBS value to be inserted each time. MBS values that

are a power of 2 like  2, 4, 8, 16, 32, 64 should be used.

The prompt will display the maximum MBS that the drive will support.

Choose I/O Modes

The Choose I/O Modes option allows selection between READ-ONLY,

WRITE-ONLY or WRITE-READ operations.

READ-ONLY

The READ-ONLY option will run all benchmarks in a read-only mode

Page 15

This is a non-destructive test. When a benchmark is run in this

mode it starts at the first record.

WRITE-ONLY

The WRITE-ONLY option will run all benchmarks in a write-only mode.

This is a DESTRUCTIVE test which writes a unique pattern to the drive.

A warning message will appear if trying to write on a drive with

valid partitions. We strongly recommend that your data be backed up

before running this option.

When a benchmark is run in this mode it defaults to the first

available one megabyte of contiguous free space on the drive,

unless the starting or ending record have been user-defined. If an

area of sufficient size is not located, an error message will come

up and instruct you to either delete unneeded files, or optimize

your hard drive. Hard drive optimization would make all of your

files contiguous and free up the empty space needed on your hard

drive(s). Utilities for disk optimization are commonly available.

If you have multiple partitions in your system the search for

the available one megabyte would start on the C: drive and

continue through your partitions.

** WARNING ** If you have changed the starting or ending record

to include areas of valid data you can overwrite your data.

Once this has been done you can not get your data back.

WRITE-READ

The WRITE-READ option will run all benchmarks in a write-read mode.

This is a destructive test that will write unique data pattern to

each record for the sampling rate and then reads the same data back

for set sampling rate. We strongly recommend that your data be

backed up before running this option.

When a benchmark is run in this mode it defaults to the first

available one megabyte of contiguous free space on the drive,

unless the starting or ending record have been user-defined. If an

area of sufficient size is not located, an error message will come

up and instruct you to either delete unneeded files, or optimize

your hard drive. Hard drive optimization would make all of your

files contiguous and free up the empty space needed on your hard

drive(s). Utilities for disk optimization are commonly available.

Page 16

If you have multiple partitions in your system the search for

the available one megabyte would start on the C: drive and

continue through your partitions.

** WARNING ** If you have changed the starting or ending record

to include areas of valid data you can overwrite your data.

Once this has been done you can not get your data back.

Choose I/O Size

The Choose I/O Size option allows selection of one or more I/O sizes

for benchmark testing.

Simply press the space bar to tag or untag the desired I/O size.

Only those that are tagged will be run on the next benchmark.  I/O

sizes tagged will display a check next to them. The default has

all I/O sizes turned on.

This option was designed to be used with DH.COM to find the most

used I/O size and match it to the best MBS value to fine tune your

system.

Enter Starting Record

The Enter Starting Record option will start benchmark testing on

the selected record.  Use this to test different areas on the drive.

Reads will default to record zero and writes will default to the

first available one megabyte of contiguous free space on the drive,

unless the starting or ending record have been user defined.

The starting record will be translated to physical parameters

before starting.

** WARNING ** If you have changed the starting or ending record

to include areas of valid data you can overwrite your data if a

write option is chosen. Once this has been done you can not get

your data back.

Enter Ending Record

Page 17

The Enter Ending Record option puts a limit on how far each

benchmark can run.

Benchmark Sampling Duration

The Benchmark Sampling Duration option can be changed in Advanced

Rocket Installation mode between 1-60 seconds. The default is five

seconds.  This is the time each I/O size will be run during each

benchmark. The longer the sampling rate the more accurate the

benchmark, but the user should still weigh accuracy with the amount

of time required to run the tests.

Advanced Drive Rocket Benchmarks

Advanced Benchmarks Overview

The Advanced Benchmarks are a collection of benchmarks requested by

drive manufacturers to further test the drives.

Repetitive, Forward and Reverse sequential, Random and Combination

benchmarks are available. All these benchmarks run two passes, one

without Drive Rocket and one with Drive Rocket.

All the benchmarks support Advanced Drive Rocket Options.

Repetitive I/O Benchmark

The Repetitive I/O Benchmark calculates data transfer while staying

on the same cylinder.  This is a theoretical transfer of this

configuration. Use it for comparison purposes only, NOT to find

the best MBS value.

Forward Sequential I/O Benchmark

The Forward Sequential I/O Benchmark calculates data transfer rates

while doing a forward sequential read/write. This benchmark reads

the drive forward taking full advantage of the drives read-ahead

cache.

Page 18

Reverse Sequential I/O Benchmark

The Reverse Sequential I/O Benchmark calculates data transfer rates

while doing a reverse sequential read/write. Doing so negates any

read-ahead cache.

Drives do not work in this fashion under DOS. Do not be alarmed if

negative numbers are displayed during this benchmark.  The results

should be used for comparison only.

Random Access I/O Benchmark

The Random Access I/O Benchmark calculates the data transfer while

performing a random read/write.

Combination I/O Benchmark

The Combination I/O Benchmark calculates the data transfer rates

while performing 10% random and 90% forward sequential I/Os.

Data Integrity Tests

Data Integrity Tests Overview

The Data Integrity Test (DIT) option includes a group of five tests

designed to examine your hardware and Drive Rocket Software

compatibility. In the default mode the first four Data Integrity

tests will run for approximately ten minutes. The fifth DIT will

run for forty minutes at each Multiple Block Size (MBS).

DIT supports several Advanced Configuration Options. They are

Choose MBS Input, Starting Record, and Ending Record.

The DITs all run on the first available one megabyte of contiguous

free space on the drive, unless the starting or ending record has

been user-defined.  If an area of sufficient size is not located,

an error message will inform you to either delete unneeded files, or

optimize your hard drive.

Page 19

Hard drive optimization will make all of your files contiguous and

free up the empty space needed on your hard drive(s).

If you have multiple partitions in your system the search for the

available one megabyte will start on the C: drive and continue through

your partitions. To run on the entire drive set the starting record to

zero.

Data Integrity Tests Menu

The Data Integrity Tests menu has three choices: (S)equential,

(R)andom and (C)hoose option.

(S)equential is the default mode and will run using the default,

or user-defined MBS while transferring up to 128 sectors. The data

is written in a unique pattern and then read back and compared.

(R)andom will run using the default, or user-defined MBS, while

executing 50 random writes while transferring up to 128 sectors.

A unique data pattern is written at all 50 locations and each

location is read back and compared.

(C)hoose options will allow the user to choose MBS, Starting or

Ending Record and Pass count. The Choose MBS option will allow choice

between default and user-defined MBS value.

Data Integrity Test Options

Enter Starting Record

The Enter Starting Record option will start DIT testing on the

selected record.  Use this to test different areas on the drive.

The Data Integrity Test will default to the first available

one megabyte of contiguous free space on the drive, unless

the starting or ending record have been user defined.

The starting record will be translated to physical parameters

before starting.

** WARNING ** If you have changed the starting or ending record

to include areas of valid data you can overwrite your data if a

write option is chosen. Once this has been done you can not get

your data back.

Enter Ending Record

The Enter Ending Record option puts a limit on how far each

Page 20

Data Integrity test can run.

Pass count

Pass count will allow the user to run the DITs as often as

desired.

Data Integrity Test One

Data Integrity Test One does a write read and compare of data

without Drive Rocket. Use this DIT to verify that your hardware

is functional.

Data Integrity Test Two

Data Integrity Test Two does a write, read, and compare of data

with Drive Rocket. Use this DIT to verify that your hardware

is functional with Drive Rocket Software.

Data Integrity Test Three

Data Integrity Test Three writes a unique pattern without Drive

Rocket, reads it back with Drive Rocket, then checks for errors.  This

DIT verifies that data written without Drive Rocket Software

can be read with Drive Rocket Software.

Data Integrity Test Four

Data Integrity Test Four writes a unique pattern with Drive Rocket,

reads it back without Drive Rocket, then checks for errors.

This DIT verifies that data written with Drive Rocket Software

can be read without Drive Rocket Software.

Page 21

Data Integrity Test Five

Data Integrity Test Five performs all the DITs above while cycling

through all MBSs.

Command Line Switches

Command Line Switches for ROCKET.EXE

/B   ALTERNATE DISPLAY MODE (BIOS CALLS ONLY) Display is

handled in two ways, directly or through BIOS.

Direct display is quicker, and therefore the

default. If the display flickers or has lines

through it, this command switch will cause the

display to be handled through BIOS.

/C   NO COLOR Disables the color for monochrome systems.

/I   IDENTIFY Goes directly to the IDE Identify Screen.

/TM  Jumps directly to the Multiple Block Size Analysis.

/TR  Jumps directly to the Repetitive I/O Benchmark.

/TS  Jumps directly to the Sequential I/O Benchmark.

/M=xx MBS Value. (xx = 0 to 128)

/S=xx Sampling duration of xx seconds. (xx = 1 to 60)

Default = 5 seconds.

/X   Will run Drive Rocket without ROCKET.OVL being installed.

If ROCKET.BIN is installed it will not be removed.

Command Line switches for ROCKET.BIN

ROCKET.BIN can accept command line parameters in CONFIG.SYS.

(ie. device = rocket.bin /M=16)

/M=x,y  Specify MBS values for each drive.  For example, if drive 1

uses MBS=16 and drive 2 uses MBS=32, the command line would

be device = rocket.bin /M=16,32. If you only have 1 drive,

only specify one MBS value.

There are 3 possible values for the MBS switch.

1) /M=8 means that your drive supports MBS and gets

Page 22

maximum performance at MBS=8.

2) /M=0 means that your drive supports MBS but shows a

performance decrease at all non-zero MBS values.

3) No /M or /M=, means that your drive does not support MBS.

(NOTE: There can still be a performance increase)

/R      Disable read multiple.

/W      Disable write multiple.

Page 23

Error Messages

Error Messages for ROCKET.EXE

Can't run under Windows

There are several reasons why Drive Rocket cannot be installed

when Microsoft (MS) Windows is running:

1) Drive Rocket Software hooks into the system and replaces the

device driver controlling the hard drive. MS Windows cannot

tolerate any application which attempts to modify critical

system functionality.

2) Drive Rocket Software performs hard drive benchmarks to

determine the best way to install Drive Rocket. Benchmark

results can be inconsistent while running under MS Windows.

3) Drive Rocket Software modifies the CONFIG.SYS file and forces

you to reboot by pressing CTRL-ALT-DEL. MS Windows does not

allow an application to completely restart the system.

Drive Rocket Software is compatible with (MS) Windows once installed.

Because of the reasons listed above, you must exit MS Windows

to install Drive Rocket.  This means completely leaving MS

Windows, not just clicking on the MS-DOS icon.

From the DOS prompt, insert the Drive Rocket diskette in the

A: drive and type A:ROCKET

Can't Access ROCKET.OCS

Drive Rocket detected a problem while accessing the ROCKET.OCS

file.  This file is located on the Drive Rocket diskette.

ROCKET.OCS contains information which helps Drive Rocket install

the device driver ROCKET.BIN into the CONFIG.SYS file.  Without

the information in this file, ROCKET.BIN will be installed on the

first line of CONFIG.SYS.  This will always work but may not be

the best place for it, especially if you have a memory manager in

your system.

It is recommended that you solve this problem before continuing.

Try making a new working diskette by copying all files from the

original diskette to a newly formatted diskette.

Page 24

Cannot Run Easy Mode

Drive Rocket Software did not find any IDE drives on your system.

For this reason, Easy Installation is not possible.  Furthermore,

installation of Drive Rocket into a system with no IDE drives

usually will result in little or no performance gain.

To learn more about Drive Rocket's view of your hard drive(s),

press ESC to get back to the Main Menu.  From there, choose the

Advanced Installation Option, and select View IDE Information.

Test memory shortage

There is not enough available base memory in this machine to

continue. Base memory is the first one Mb of memory in the machine;

and this area is where Drive Rocket executes from. To continue with

Drive Rocket, you must free up some base memory. You can do this

by removing device drivers and TSRs from your CONFIG.SYS and

AUTOEXEC.BAT files, and then reboot your machine.

NOTE: Drive Rocket Software will not execute on a machine with

less than 512K of total base memory.

MBS Not Supported

This drive does not support Multiple Block Size (MBS) and

therefore Drive Rocket Software will not allow you to enter

user-definable MBS mode. MBS will remain in the default mode with MBS

always equal to zero.

NOTE: MBS mode only applies to IDE drives that support read-

and-write-multiple mode.  Drive Rocket Software did not recognize

your drive as meeting these requirements.

Drive Rocket Software may increase your performance even if the

drive does not support Multiple Block Size, especially if your system

BIOS does not support shadowing. If shadowing is not enabled, each I/O

request is handled by the system ROM BIOS. When Drive Rocket Software

is installed, its device driver is handling each I/O in RAM which is

faster then ROM. Run Drive Rocket Software even if your drive does not

support MBS; it may increase your data transfer rate.

Page 25

Error Setting New MBS

Drive Rocket could not send the MBS value to the drive.  Most

drives only support MBS values that are powers of 2  (ie.

2,4,8,16,32,etc). Try again with one of these values.  If setting

the MBS continues to fail, exit Drive Rocket, reboot, and try

again.

Can't Run DIT

This test can only run on IDE drives. This hard drive is not an IDE.

You can only run Data Integrity Test #1 on this drive.

Advanced Drive Error Detected

A hard drive error has been detected.  This error could be caused

by a media defect on the drive, an incompatibility with the Drive

Rocket software, or a system limitation.

Refer to the following troubleshooting steps:

1) If this error occurred while analyzing performance BEFORE

Drive Rocket, it is most likely a media defect on the drive.

Modify the starting record so that this area of the drive will

not be used during future testing.

2) If this error occurred while analyzing performance AFTER

Drive Rocket, a software incompatibility is likely.  This

could be an indication that your hard drive does not fully

support MBS. Run this test again with different MBS values

including MBS=0.

3) If neither of the suggestions above seem to help, this error

could be caused by a system limitation in drive I/O sizes. Go

to the Advanced Rocket Options menu and choose Rocket

Configuration Options.  From there select the Choose I/O

Sizes option, disable the largest I/O size, and run this test

again.

4) If all these troubleshooting options fail, try temporarily

removing all device drivers and TSRs from your CONFIG.SYS

Page 26

and AUTOEXEC.BAT.

Easy Drive Error Detected

An unusually high number of unreadable sectors have been detected

on this hard drive.  This can be caused by faulty hardware or a

software incompatibility with Drive Rocket.

You can approach this problem in two possible ways:

1) Go back to the Main Menu and choose the Advanced

Installation option.  From there choose the Performance

Benchmarks option.  Run any one of the benchmarks and

detailed information will be given about all drive errors

detected.

2) Exit Drive Rocket and run a hard drive diagnostic program. If

no errors are detected by the diagnostic, a software

incompatibility with the Drive Rocket software is likely.

Error Installing Rocket Software

An error has been detected while installing Drive Rocket Software.

You should go back to the Main Menu and choose the Advanced Rocket

Installation option.  From there choose the Install Drive Rocket

option and the specific error condition will be shown.

Can't Find Drive Letter

Drive Rocket has detected an invalid C: drive.  The C: drive

must be valid to allow Drive Rocket installation.  Exit Drive

Rocket and run CHKDSK on the C: drive to determine the problem.

NOTE: If you have performed any destructive Drive Rocket tests, and

defined the starting record number, it is possible that the test

wrote on top of existing partitions and data on this drive.

Page 27

Can't Exist With FASTDISK

FASTDISK is a 32-bit protected mode hard disk driver for Microsoft

Windows. It can improve performance under certain situations while

running MS Windows.  Refer to your Microsoft Windows manual for

details on Fastdisk.

Drive Rocket and FASTDISK cannot co-exist.  When FASTDISK is

loaded during Windows startup, it looks around in the system

for any device drivers that may have already taken over the

hard drive (e.g. Drive Rocket is a device driver that takes

over the hard drive).

Because FASTDISK cannot feel safe about taking over a hard drive

that has already been taken over, it displays an error message and

refuses to load.

You are now faced with the choice of Drive Rocket or FASTDISK.  It

has been our experience that data transfers per second are larger

with Drive Rocket Software than with FASTDISK.

Can't Remove FASTDISK

Drive Rocket has detected a problem while attempting to remove

Microsoft Windows FASTDISK from your system.  This is typically

caused by insufficient free space on the C: drive. Exit Drive Rocket

and run CHKDSK to determine if there is any free space on the C:

drive. If the C: drive appears to be full, delete some unwanted

files and run Drive Rocket again.

NOTE: If FASTDISK and Drive Rocket are both installed, you

will get an error message from FASTDISK the next time you run

Microsoft Windows.

Can't Read CONFIG.SYS

Drive Rocket detected an error while reading your CONFIG.SYS

file.  Exit Drive Rocket and run CHKDSK to check for errors

on C: drive.  Then run Drive Rocket Software again.

Can't Update Config.sys

Page 28

Drive Rocket has detected a problem while attempting to update

your CONFIG.SYS file.  This is typically caused by insufficient

free space on the C: drive. Exit Drive Rocket and run CHKDSK to

determine if there is any free space on the C: drive.  If the C:

drive appears to be full, delete some unwanted files and run Drive

Rocket again.

NOTE: If you have performed any destructive Drive Rocket tests,

and defined the starting record number, it is possible that the

test wrote on top of existing partitions and data on this

drive.

Can't Access File

Drive Rocket has detected an error while attempting to read a file

from the Drive Rocket diskette.  The file is either missing,

unreadable, or corrupted.  Try making a new working diskette and

run Drive Rocket Software again.

Can't Update File

Drive Rocket has detected a problem while attempting to copy

a file to your C: drive.  This is typically caused by insufficient

free space on the C: drive.

Exit Drive Rocket and run CHKDSK to determine if there is any free

space on the C: drive.  If the C: drive appears to be full, delete

some unwanted files and run Drive Rocket Software again.

NOTE: If you have performed any destructive Drive Rocket Software

tests, and defined the starting record number, it is possible

that the test you ran wrote on top of existing partitions and

data on this drive.

No Hard Drives found

Drive Rocket cannot find any hard drives on this machine. Remember

that Drive Rocket only works with IDE drives.  If there is a hard

drive in your machine and Drive Rocket cannot see it, it could be

due to a driver incompatibility, Driver problems fall into two

Page 29

categories:

1) Hardware Incompatibilities. Try slowing down the processor

speed and/or disabling processor cache. Remove other hard drive

controller cards.

2) Software Incompatibilities. Try temporarily removing device

drivers from your CONFIG.SYS. Also remove TSRs from your

AUTOEXEC.BAT.

No Drives Support MBS

Drive Rocket is designed to work with IDE drives that support

the read-and-write-multiple commands. No drives were found on

this machine which satisfy that requirement.

NOTE: Drive Rocket may not be compatible with IDE controllers

that have a BIOS on them.  Specifically, Drive Rocket will

only see IDE drives attached to a controller that provides the

standard 1003 AT register set.  This means that IDE drives

installed in an XT-class machine will not be seen as IDE.

Drive Rocket Software may increase your performance even if the

drive does not support Multiple Block Size, especially if your system

BIOS does not support shadowing. If shadowing is not enabled, each I/O

request is handled by the system ROM BIOS. When Drive Rocket Software

is installed, its device driver is handling each I/O in RAM which is

which is faster then ROM. Run Drive Rocket Software even if your drive

does not support MBS; it may increase your data transfer rate.

Error Launching Rocket

Drive Rocket detected an error while attempting to start

ROCKET.OVL.  The following is a list of possible causes and

suggestions:

1) The ROCKET.OVL file is missing, unreadable, or corrupted.

This file exists on the Drive Rocket diskette and is loaded

as an overlay when the program starts.  Try making a new

working diskette and run Drive Rocket again.

2) There is not enough available base memory in the machine.

Remove device drivers and TSRs from your CONFIG.SYS and

Page 30

AUTOEXEC.BAT, reboot, and run Drive Rocket again.

It is recommended that you resolve this issue before

continuing.  Without this overlay, Drive Rocket cannot

calculate the optimum configuration for your system.

Insufficient Free Space

Drive Rocket was unable to find more than one megabyte of contiguous

free space on the drive to perform the write operations.

This could be caused by the following reasons:

1) The drive has less than one megabyte of contiguous free space

available. Exit Drive Rocket and delete some unneeded files,

and run Drive Rocket again.

2) The Drive is highly fragmented. Exit Drive Rocket, run a disk

optimizer utility or delete some unwanted files, and run

Drive Rocket again.

3) Drive Rocket may not be able to see all partitions on the drive

or a partition is not stable because of a File Allocation Table

mismatch.  Exit Drive Rocket and verify all DOS device drivers

are loaded, run CHKDSK to verify all drive letters are valid,

and run Drive Rocket again.

4) Drive Rocket may not be able to understand the partitions on

this drive. This is especially true if you are using data

compression or other operating systems besides DOS are installed.

5) Change the Starting and Ending Records to specify a testing

range that is known to free of valuable data.

Destructive Test Warning

Drive Rocket is about to start a test that writes to the hard

drive.  When a benchmark is run in this mode, it defaults to the

first available one megabyte of contiguous free space on the drive,

unless the starting or ending record have been user-defined.

If an area of sufficient size can not be located, an error message

will come up and inform you to either delete some unneeded files or

optimize your hard drive. Hard drive optimization would make all of

your files contiguous and free up the empty space needed on your hard

drive(s).

Page 31

If you have multiple partitions in your system, the search for the

available one megabyte will start on the C: drive and continue

through your partitions. We do not suggest that you run this option

unless the data is backed up and safe.

** WARNING ** If you have changed the starting and ending record

to included valid data areas, you can overwrite your data.

Once this has been done you will not get your data back.

Error Messages for ROCKET.BIN

No Drives Found Requiring Use of Driver

Either CMOS=0 for both drives or the hard drives are not

responding. Check CMOS and verify proper operation of your drives.

Syntax Error Detected in Command Line

The arguments you have entered on the command line are either not

correct or misspelled. Please check the syntax (characters) on the

command line.

Invalid MBS Value Detected for Drive

The MBS value entered on the command line was rejected by the hard

drive. Either the number on the command line was mis-typed or you

may have to run the benchmark tests again to get a supported

MBS value.

Page 32

Hardware / Software Support

Hardware Support

Hardware Supported by Drive Rocket

IBM AT, 386, 486, EISA, VESA or compatible personal computers.

Supports IDE (AT interface) hard drive configurations which

support the read-and-write-multiple commands.

Drive Rocket does not support drives attached to secondary

controllers installed by Disk Manager.  Drive Rocket does

not support IBM PS/2 MCA hardware.

Software Support

Supported DOS versions

PCDOS or MSDOS version 3.0 or higher, including MSDOS

version 5.0 and 6.0.

DRDOS version 4.0 and higher

Microsoft Windows

Drive Rocket's installation program ROCKET.EXE can not be run

from within the Microsoft Windows environment, and will give an

a warning message when this is attempted. Install Drive Rocket

Software from the DOS prompt. Once installed, Drive Rocket and

MS Windows are compatible.

Windows V3.1 FASTDISK

Drive Rocket's ROCKET.BIN and FASTDISK cannot be run at the same

time. Consult your Microsoft Windows Manual.

If you are currently using FASTDISK you may want to disable it

and try using ROCKET.BIN. In testing we have seen a significant

performance increase when using Drive Rocket rather then FASTDISK.

Drive Rocket will check to see if 32-bit access is turned on. If

it is enabled, you will be given a warning message with the option

Page 33

to turn FASTDISK off or to exit Drive Rocket.

SMARTDRV 3.3

SMARTDRV version 3.3 performs a disk cache on sectors per track.

Rocket.bin should be placed before SMARTDRV in your CONFIG.SYS.

There are no compatibility problems with Drive Rocket and SMARTDRV.

(SMARTDRV 3.3 was shipped with DOS 5.0.)

SMARTDRV 4.0

SMARTDRV version 4.0 performs disk cache on either eight or sixteen

thousand bytes. SMARTDRV 4.0 is run from AUTOEXEC.BAT.

There are no compatibility problems with Drive Rocket and SMARTDRV.

(SMARTDRV 4.0 was shipped with MS Windows 3.1 and MS DOS 6.0.)

DISK CACHES

Typically a disk cache will reserve memory where it stores

more data than requested each time a read to the drive is

performed.  This is commonly known as a read-ahead algorithm.

The disk cache then expects the next data that will be requested

will already be stored in memory, where response time is much

faster than going to the drive to do the I/O request.

This is known as a cache hit. Cache hits reduce the number

of I/O requests which go to the drive, thus speeding up the

overall performance of the system.

Drive Rocket Software helps to improve a disk cache by increasing

the transfer rate when a cache miss occurs and the I/O request

must go to the drive.

Page 34

Definition of Terms

Glossary :

AT-COMPATIBLE COMPUTER

Any computer that can run software programs written for an IBM AT

computer.

BIOS

Basic Input Output System. Refer to your computer documentation

for setup. Usually stored on a Read-Only Memory chip in the computer.

CONFIG.SYS

A special DOS file that you create and use to configure the DOS

operating system. The CONFIG.SYS file can only contain a specific

set of DOS configuration commands.  See your DOS reference manual.

DATA TRANSFER RATE

Data Transfer Rate is a measure of speed for a hard disk in a computer.

Typically this is measured in KILO-BYTES per SECOND (KBS), where

Kilo refers to a thousand. All measurements within drive rocket are

reported in Kilo-Bytes per Second.

Example: 640 KBS  = 640,000 bytes per second.

HARD DISK

Sometimes called a fixed disk, one that is built into the computer.

A hard disk can store much more information than a floppy disk, and

the computer can retrieve information from it much faster.

I/O

I/O (Input/Output) This terminology, when used in this manual,

pertains to size of the input or output, although it usually

just means Input/Output.

Page 35

IDE

IDE is an acronym for Integrated Drive Electronics. It refers

to a popular type of hard disk used in computers today.

INTERRUPT

The suspending of microprocessor program execution by a demand for

attention coming from a peripheral device, such as a hard disk.

After the interrupt has been serviced, the suspended microprocessor

task can be resumed where it was broken off by the interrupt.

MBS

MBS (Multiple Block Size) This is the function in an IDE drive

that enables multiple sector (block) transfers per interrupt.

MBS is the same number of sectors per interrupt.

RAM

Random Access Memory. Usually high speed memory used to process your

data programs.

READ/WRITE MULTIPLE

Read/Write Multiple is a capability in IDE drives to transfer more than

one sector of information per interrupt in an MS-DOS system.

ROM

Read only memory. Usually slower than RAM. Usually a chip in your

computer where the BIOS is stored.

SECTOR

A section of a hard disk shaped like pie slices on each disk platter.

As the disk platter rotates, each sector area travels past the read/

write heads, at which point data can either read from or written to

Page 36

that sector. A sector contains 512 bytes of data.

Page 37

Ontrack Computer Systems

About Ontrack Computer Systems

Ontrack Computer Systems Overview

Ontrack Computer Systems, Inc. revolutionized the computer

industry by automating the hard drive installation process with its

Disk Manager line of products.

The enormous success generated by the initial Disk Manager product

for the DOS environment led to an immediate demand for the same

product in the Novell and Macintosh environments.

Ontrack has expanded its product line to include data recovery and

organization utilities, chip sets, value-added device drivers,

hardware kits, and virus intervention software.

How to Contact Ontrack Computer Systems

Ontrack Computer Systems, Inc.

6321 Bury Drive

Suites 15-19

Eden Prairie, MN 55346 U.S.A.

Hours: 8:00 a.m. to 5:00 p.m.

Monday through Friday CT

Sales             1-800-752-1333

International     1-612-937-1107

Fax               1-612-937-5815

Technical Support 1-612-937-2121

24 Hr BBS         1-612-937-0860

About Ontrack Data Recovery Inc.

Ontrack Data Recovery Overview

Ontrack Data Recovery, Inc. specializes in recovering data from

any network, microcomputer hard drive, floppy diskette, for most

magnetic tapes suffering from mechanical failure or data

corruption.

Ontrack uses advanced custom software, clean room facilities, and

information gathered from unique OEM (original equipment

Page 38

manufacturer) relationships to insure the best chance of

recovering lost data.

Ontrack Data Recovery, Inc. offers the most advanced technology

for recovering data from computers with DOS, Novell NetWare 286

and 386, OS/2, Unix, Xenix, SUNOS, DEC, Wang, Banyan Vines,

Macintosh, and other operating systems.

In addition to hard disks, ODR will work on SyQuest, Bernoulli,

and other removable media.

How to Contact Ontrack Data Recovery

Ontrack Data Recovery, Inc.

6321 Bury Dr.

Suites 15-19

Eden Prairie, MN 55346

U.S.A Corporate Offices

Toll Free         1-800-872-2599

International     1-612-937-5161

Fax               1-612-937-5815

California Sales Office

2400 Main Street,

Suite 200

Irvine, CA, 92714

Toll Free         1-800-752-7557

International     1-714-263-9245

Fax               1-714-263-9246

Europe Ontrack Data Recovery Europe Ltd.

Surrey House, 34 Eden St.

Kingston upon Thames Surrey KT1 1ER United Kingdom

U.K. Toll Free    0800-24 39 96

Germany Toll Free 0130-815-198

Outside U.K.      44-81-974 5522

Telefax           44-81-974 5544

Page 39

Ontrack Products and Services

Products for Novell NetWare

NetUtils 4

Available Fall 1993

The NetUtils 4 software is a toolbox of expert hard disk utilities

for systems using Novell NetWare 3.x or 4.x   NetUtils 4 consists of

three programs: NetDisk4, NetFile4, and NetScan4.

NetDisk4 is a sector editor that lets you examine and modify data

anywhere on the hard drive.

NetFile4 is a file editor that lets you access and modify data

within files in a NetWare 3.x or 4.x volume. It also allows coping

of files between volumes, servers or any DOS drive.

NetScan4 is designed to examine and repair NetWare 386 and 486

structures, and verify the hard drive media for a NetWare volume

is error-free.

NetUtils 3

The NetUtils 3 software is a toolbox of expert hard disk utilities

for systems using Novell NetWare 3.x.  NetUtils 3 consists of

three programs: NetDisk3, NetFile3, and NetScan3.

NetDisk3 is a sector editor that lets you examine and modify data

anywhere on the hard disk.

NetFile3 is a file editor that lets you access and modify data

within files in a NetWare 3.x volume.

NetScan3 is designed to examine and repair NetWare 386 structures,

and verify the hard disk media for a NetWare volume is error-free.

NetOptimizer 2

The NetOptimizer 2 Data Organization Software can help enhance the

performance of your NetWare 2.x file server in two ways. It can

de-fragment any files found in NetWare volumes, rearranging the

files into sequential chains. It may also reorganize a NetWare

file server volume into user-specified zones.

Page 40

The NetOptimizer 2 package consists of five programs: NetOpt,

NetDisk, NetTree, NetScan, UtilGen.

NetOpt is the actual NetWare 286 optimizer, NetDisk is a sector

editor used to examine any sector of a hard disk, while NetTree

lets you display all the files in a NetWare volume.

NetScan is designed to examine NetWare 286 structures, and verify

hard disk media for a NetWare volume is error-free.  UtilGen

generates device drivers for the other NetOptimizer 2 utilities.

NetUtils-2

The NetUtils software is a toolbox of expert hard disk utilities

for systems using Novell NetWare 2.x. NetUtils consists of five

programs: NetDisk, NetTree, NetFile, NetScan, and UtilGen.

NetDisk is a sector editor that lets you examine and modify data

anywhere on the hard disk, while NetTree lets you display all of

the files in a NetWare volume.

NetFile is a file editor that lets you access and modify data

within files on a NetWare 2.x volume. NetScan is designed to

examine NetWare 286 structures, and verify the hard disk media for

a NetWare volume is error-free.

UtilGen generates any NetWare 2.x device drivers needed by the

NetUtils utilities.

Disk Manager -N

Disk Manager -N is a complete package that will initialize,

partition, and prepare virtually any hard disk for use in a Novell

NetWare 2.x file server, especially IDE drives.

Cognito

Cognito is a collection of VADDs, value-added device drivers, used

with many SCSI host adapters in a system running NetWare 2.x.

The supported SCSI host adapters are: Adaptec AHA-1540/1542/1640,

Page 41

Ciprico RF5500, Rancho Technology RT10-AT/RT1000, and Western

Digital WD7000.

NetWare Essentials Kit

NetWare Essentials is a toolbox of useful utilities for systems

running Novell NetWare 2.x. This collection contains the software

we feel is so important for installation, maintenance, and

protection of a NetWare 2.x hard disk.

The NetWare Essentials Kit consists of several Ontrack products:

NetOptimizer 2, NetUtils, Cognito VADDs, Disk Manager -N, and

Dr. Solomon's Anti-Virus Toolkit.

Products For DOS

Disk Manager

Disk Manager hard disk installation software is a complete package

that will initialize, partition, and prepare virtually any hard

disk.

Disk Manager has automatic or manual installation procedures, and

lets you get around the normal DOS 1024 cylinder limit.

Disk Manager will recognize virtually any hard disk including IDE,

SCSI, ESDI, MFM and RLL.

DOSUtils

DOSUtils is a collection of expert utilities designed to provide

diagnostics, maintenance, and recovery utilities for problems in

the DOS environment.

DOSUtils consists of five programs: Disk Look, File Look, Scan,

File Recovery, and DNA.

Disk Look is a sector editor that lets you examine the entire

hard disk.

File Look lets you access and modify data within any file in any

DOS partition.

Page 42

Scan will examine and repair corrupt DOS structures, and test used

and unused areas of the hard disk media for defects.

File Recovery is an easy-to-use utility designed to locate and

undelete any files unintentionally erased.

DNA is a diagnostic program that will help you determine if

software or hardware problems are causing errors.  Additionally,

DNA will do a non-destructive low-level format of ST-506 (MFM or

RLL) and ESDI hard disks.

Dr. Solomon's Anti-Virus Toolkit

Dr. Solomon's Anti-Virus Toolkit provides the most comprehensive

treatment and preventative maintenance available to combat virus

infections in your computer. It readily detects boot, partition,

stealth, and polymorphic viruses, and is updated on a regular

basis to protect you against new viruses.

Superproms

Superproms are an add-in set of ROM (Read Only Memory) chips

that add up to another 255 table entries for hard disks.

Products For Macintosh

Disk Manager Mac

Disk Manager Mac installs and completely prepares virtually any

SCSI hard disk drive or magneto optical for use in a Macintosh. It

comes with utilities featuring advanced defect management,

initialization, hard partitioning, password protection, multiple

start-up partitions, and more.  System 7 compatible.

Products For Unix

RecoverEase

The RecoverEase Data Recovery and Protection Utilities for Unix

Systems software is a toolbox of useful utilities for systems

Page 43

running SCO Unix 3.2 or later, SCO XENIX 2.3 or later, SCO ODT 1.0

or later, and Interactive System V/386 Release 3.x. RecoverEase

consists of four programs: Disk Look, File Look, Scan, and UXDNA.

Disk Look is a powerful sector editor that lets you examine and

modify data anywhere on a hard disk. Disk Look will access the

entire hard disk, or it will access data only within a specific

Unix/Xenix filesystem (including the superblock and inode list).

File Look is a file editor that lets you access and save data from

a Unix/Xenix filesystem, change any file attributes, or print the

contents of any Unix/Xenix file.

Scan will examine and repair corrupt Unix/Xenix structures in a

filesystem, and test used and unused areas of the hard disk media

for defects.

UXDNA is a diagnostic program that can help you determine if

software or hardware problems are causing errors.  UXDNA will

additionally nondestructively low-level format ST-506 (MFM or RLL)

and ESDI hard disks.

Technical Support Infobase

Technical Support Infobase Information

Ontrack's Technical Support Tools Ontrack's technical support

infobases contain accurate and hard-to-find information about hard

disk drives, networks, systems, and the complicated interactions

between hardware, software, and operating systems.  Designed for use

by support technicians, Ontrack's infobases are published using the

FolioVIEWS engine, and provide access to information as fast as you

can type.

TechSource

TechSource: TechSource provides information about the inner

workings of DOS, Novell, Macintosh, and Unix environments, based on

years of technical support documentation, with detailed information

about Ontrack's product and services.  Special attention has been paid

to incorporating product and system field reports, Ontrack Data

Recovery engineering expertise, and solutions to real-life technical

problems - the type the manufacturers don't give you in the manuals.

Page 44

DiskSource

DiskSource: DiskSource will save you loads of time if you install,

or maintain hard disk drives, by putting the exact hard drive

information you need at your fingertips.  DiskSource contains

technical specifications on more than 3,000 hard disks and 200

controller cards, plus more than 60 BIOS charts.  This comprehensive

hard disk and controller reference source allows you to search by

drive capacity, to choose replacement drives by searching for the BIOS

make, version, and drive type.  Also included are descriptions of

drive interfaces, and much more.

SystemSource

SystemSource:  SystemSource is a comprehensive collection of

technical information on more than 500 system boards from more

than 35 manufacturers. It gives you a quick and easy reference tool

in a user friendly format.  Search by manufacturer, bus type, and

processor speed, and you'll find clear, concise diagrams detailing

full configuration of jumper and dip switch settings.  This infobase

also provides full error code descriptions to assist you with

troubleshooting.

NetSource

Available Fall of 1993

NetSource: NetSource is the most comprehensive collection

of technical information ever assembled for networks.

Specifications are included for more than 500 networking cards, and

more than 90 manufacturers in all networking environments.  You can

also tap into worldwide listings of the Novell Authorized Resellers,

and Novell Authorized Education Centers, plus a listing of BBSs around

the world that you can contact with questions regarding your networks.

Page 45

Drive Rocket License Agreement

ONTRACK COMPUTER SYSTEMS, INC.

License Agreement

This is a legal agreement between you, the end user, and Ontrack

Computer Systems, Inc. ("Ontrack"). BY USING THIS SOFTWARE, YOU

ARE AGREEING TO BE BOUND BY THE TERMS OF THIS AGREEMENT, PROMPTLY

REMOVE THE SOFTWARE FROM YOUR MACHINE AND RETURN ANY DISKETTES AND

THE ACCOMPANYING ITEMS (INCLUDING WRITTEN MATERIALS AND BINDERS

OR OTHER CONTAINERS) TO THE PLACE YOU OBTAINED THEM.

1.  GRANT OF LICENSE. Ontrack grants to you the right to use

one copy of this Ontrack software program (the "SOFTWARE") on

a single computer (i.e. with a single CPU at a single

location).  If you wish to put the SOFTWARE on a network

server, or if you wish to contact Ontrack for additional

requirements prior to such intended use of the SOFTWARE.

2.  COPYRIGHT. The SOFTWARE is owned by Ontrack or its suppliers

and is protected by United States copyright laws and

international treaty provisions. All rights reserved

worldwide.  Therefore, you must treat the SOFTWARE like any

other copyrighted material (e.g.  a book or a musical

recording) except that you may (a) archival purposes, and (b)

the transfer of the SOFTWARE to hard disk drives provided that

only one copy of the SOFTWARE is used at any time. You may not

copy any written materials accompanying the SOFTWARE without

written permission from Ontrack.

3.  OTHER RESTRICTIONS. You may not: (a) rent or lease or

sublicense the SOFTWARE, (b) decompile, reverse assemble or

reverse engineer the SOFTWARE, (c) make any telecommunications

transmissions of the SOFTWARE, or (d) permit use of the

SOFTWARE in a computer service business, network, timesharing,

multiple CPU or multiple user arrangement to users that are

not individually licensed by Ontrack. You may transfer the

SOFTWARE and any accompanying written materials on a permanent

basis provided you retain no copies and the recipient agrees

to the terms of this Agreement.

4.  TERMINATION. This license is effective until terminated. This

license will terminate immediately without notice from Ontrack

if you fail to comply with any provision of this license. Upon

termination you must destroy any SOFTWARE and any related

documentation, including all copies. Such termination shall be

in addition to and not in lieu of any criminal, civil, or

other remedies available to Ontrack.

5.  DISCLAIMER OF WARRANTY AND LIMITATION OF WARRANTIES. THE

SOFTWARE AND WRITTEN MATERIALS ARE PROVIDED AND LICENSED "AS

IS" WITHOUT WARRANTY OF ANY KIND. FURTHER, NEITHER ONTRACK

NOR ITS SUPPLIERS WARRANT, GUARANTEE, OR MAKE ANY

REPRESENTATIONS REGARDING THE USE, OR THE RESULTS OF THE USE,

OF THE SOFTWARE OR DOCUMENTATION IN TERMS OF CORRECTNESS,

Page 46

ACCURACY, RELIABILITY, CURRENTNESS, OR OTHERWISE. THE ENTIRE

RISK AS TO THE RESULT OR PERFORMANCE OF THE SOFTWARE OR

DOCUMENTATION IS ASSUMED BY YOU.  THERE ARE NO OTHER

WARRANTIES, EXPRESS OF IMPLIED, INCLUDING ANY IMPLIED

WARRANTIES OF MERCHANTABILITY AND/OR FITNESS FOR A PARTICULAR

PURPOSE, MADE BY ONTRACK OR ITS SUPPLIERS ON THE SOFTWARE OR

DOCUMENTATION.

IN NO EVENT SHALL EITHER ONTRACK OR ITS SUPPLIERS BE LIABLE

FOR ANY DIRECT, INDIRECT OR CONSEQUENTIAL DAMAGES (INCLUDING

BUT NOT LIMITED TO SPECIAL, INCIDENTAL OR OTHER DAMAGES FOR

LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF

BUSINESS INFORMATION, AND THE LIKE) ARISING OUT OF THE USE OR

INABILITY TO USE THE SOFTWARE OR DOCUMENTATION EVEN IF

ONTRACK HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

BECAUSE SOME STATES DO NOT ALLOW THE EXCLUSION OR LIMITATION

FOR CONSEQUENTIAL OR INCIDENTAL DAMAGES, THE ABOVE LIMITATION

MAY NOT APPLY TO YOU.

6.  GENERAL.  Any questions concerning the Agreement should be

referred in writing to Ontrack Computer Systems, Inc.  at 6321

Bury Drive, Eden Prairie, Mn. 55346.

All Ontrack brand and product names are trademarks or

registered trademarks of Ontrack Computer Systems, Inc.  All

other brand and product names are trademarks or registered

trademarks of their respective holders.

This Agreement is governed by the laws of the State of

Minnesota.

The SOFTWARE and related documentation are Proprietary to

Ontrack Computer Systems, Inc. and its suppliers. Use,

Duplication, or Disclosure by the Government is Subject to

Restrictions as Set Forth in Paragraph (c) (1) (ii) of the

Rights in Technical Data and Computer Software Clause at

DFARS 252.227-7013. Ontrack Computer Systems, Inc.

Page 47