Direct DOS 5.0 support for disks with more than 1024 cylinders

--------------------------------------------------------------

This document describes my experience with adding support to DOS 5.0

for drives with more than 1024 cylinders. I used NO third-party

partitioning software to accomplish this. Instead, I patched

DOS 5.0 to directly support large drives. An actual step-by-step

procedure and the DEBUG patches used are contained in the

GT1024.ZIP file.

A WARNING first: Unless you are experienced with patching and testing

software, you should not try the process I will describe! Even

experienced programmers may shy away from the modifications I have

made due to their sensitive nature.

In any case, you should read and understand this process.  The drive

limitations imposed on MS-DOS users are vestiges of the original IBM

AT BIOS. Microsoft should have completely eliminated them when support

for drives with more then 64K sectors was added to DOS.

- - - - - - - - - - - - -

I have recently purchased an additional hard drive which has 1700+

cylinders.  In the past, I have experienced compatability problems

with third-party partitioning schemes. Also, I have two large (>300M)

hard drives on my system and don't like to define multiple partitions.

In fact, I voluntarily gave up using the last 200 cylinders of my

first hard drive to stay within the DOS limit of 1024 cylinders.

Rather than give up additional disk space or add unusual disk access

software to my system, I chose the examine the possibility of having

DOS directly access the entire hard disk. After all, DOS no longer has

a 64K limit on the number of disk sectors. Most disk controllers

support up to 4096 cylinders, not the 1024 cylinder maximum which was

coded into the original (and poorly written) IBM AT BIOS.

Futhermore, many BIOS manufacturers have added support for INT 13

access to drives with more than 1024 cylinders. The original INT 13

specification uses a 10-bit cylinder number for a maximum value of

1023 (hex 3FF).  An additional 2 bits of the cylinder number may be

placed in the DH register on an INT 13 call, for example, resulting in

a maximum value of 4095 (hex FFF).  This is the case on my primary

system, which uses a Hauppauge Computer Works 4860 motherboard with

Award BIOS.

(My second system is an original IBM AT; On this system, I found it

fairly easy to modify the BIOS to support a 12-bit cylinder number for

INT 13 calls.)

Since DOS is so very close to supporting really large disks anyway,

I found it possible to patch the DOS disk device driver and eliminate

the 1024-cylinder maximum. Until an INT 13 is performed, DOS identifies

a sector on the disk by its 32-bit sector number. Prior to issuing

the INT 13 request, the sector number is converted to a physical

sector address on the disk. By slightly modifying this calculation,

up to 4096 cylinders can be accessed.

During DOS startup, the disk driver reads the disk partition table

to identify the DOS partition(s) it should access. If there is a

single DOS partition which starts at a cylinder number less than

1024, the 32-bit sector count in the partition table is the only value

used to determine the end of the partition. The ending cylinder and

head values in the parition table are ignored. Therefore, setting

a large sector count is all that is needed for DOS to access more

then 1024 cylinders.

The FDISK.EXE program builds the partition table using the values

returned from INT 13 function 08 ("get drive parameters"). Unfortunately,

for compatibility reasons, this function returns a maximum cylinder

value of 1022 (cylinder 1023 is a "diagnostic" cylinder). To modify

FDISK, I chose to examine the drive parameter tables pointed to by

INT 41 and INT 46 directly. These contain the actual number of usable

cylinders on the drive.

After modifying the disk driver in IO.SYS and the FDISK.EXE program,

I was able to successfully FORMAT and use both of my entire drives

as single large partitions.

Next, I began to test this setup for compatibility with various

applications. Since well-behaved applications access the hard disk

via DOS calls, I expected few problems. However, I was unsure of

how MS-Windows would work; accessing its 386-mode swap file in

particular.

I have been running for almost two months now, and have encountered

only a few difficulties:

1. Disk caching programs do not usually recognize drives with more

than 1024 cylinders and unusual values in DH on an INT 13 call

can make them fail.

Initially, I tried SMARTDRV, which ignores INT 13 calls for which

the high bits of DH are nonzero. Everything operates properly,

but the area of the drive beyond cylinder 1023 is not cached.

This is not too serious, but I was uncomfortable with the

thought of slow access to that part of my drives.

I had previously purchased a commercial cache program (I hesitate

to mention names here), but gave it up for SMARTDRV after experiencing

problems under DOS 5.0 and Windows. I realized that SMARTDRV is a very

poor performer, but I was willing to trade performance for a working

system.

Several shareware cache programs are available on COMPUSERV. I tried

some of these, even one which claimed to support more than 1024

cylinders directly. I gave that one up after getting "General Failure"

errors on my C: drive.

Finally, unwilling to compromise, I wrote my own disk caching program

which supports most of the advanced features available in commercial

program. I would be willing to make this available as "freeware" or

"shareware" to interested parties.

2. I use SPINRITE to regularly low-level format my hard drive. I love

this program, but can no longer use it. For now, SPINRITE only

formats cylinders 0-1022 and uses cylinder 1023 as a diagnostic

cylinder, overwritting any data contained there.

Actually, SPINRITE is very close to having the support which I need.

Perhaps a future version will work.

As a general precaution, I have written a utility called MARKCYL to

mark cylinder 1023 as "bad" in the DOS FAT. This prevents it from

being used by DOS programs and so safe for use as a diagnostic

cylinder. This utility is included in GT1024.ZIP

3. Most PARK utilities use the cylinder value returned from INT 13

function 08. I have written a PARK.COM program which uses the

cylinder counts from INT 41 and INT 46 instead. This utility is

included in GT1024.ZIP.

4. I use a temporary swap file for Windows. When a permanent swap file

is allocated, Windows uses INT 13 for read and write it. A temporary

swap file is accessed through DOS.

In the future, I may modify Windows' PAGESWAP and VHD drivers to

allow direct support for cylinders > 1024. Windows 3.1 is reported

to directly support drives with more than 1024 cylinders, so this

may not be needed.

- - - - - - - - - - - - -

Uploaded 10/13/91 by:

Jerry Metcoff

ID 70540,566