------------------------------------------------------------

IDEINFO.EXE             Version 1.0                5/26/91

------------------------------------------------------------

(C) Copyright 1991 Thomas J. Newman.  All Rights Reserved

Purpose:

IDEINFO displays information about the IDE (Integrated Drive

Electronic) drives in an AT computer.  It issues a standard

IDE drive command that requests information about the drive.

This information is then displayed on the screen.

The information returned by the drive includes the physical

number of cylinders/heads/sectors; buffer size; number of

ECC bytes transferred; and the drive's model number, serial

number, and firmware revision level.

This program would be useful to someone who wants to know

more information about the IDE drives installed in his/her

system.  It can also be used as a troubleshooting aid since

it by-passes the system BIOS and accesses the hardware

directly.

Format:  IDEINFO    (no parameters required)

Remarks:

The program first displays the numbers of hard disk drives

reported by the AT System BIOS (this value is stored in

memory at location 0040:0075).  This value will be 0 if no

drives found; 1 for one drive; and 2 for two drives.

Next a check is made to determine if any IDE drives exist.

The main hard disk controller address is check (1F0 to 1F7)

to determine the number of hard drives (0, 1, or 2).  The

number of drives found is then displayed.

If any IDE drives are found, a special command (Identify

Drive) is sent to each drive and the information returned by

the drive is displayed.

Note:  An error message may be displayed if drives are found

but are connected to an older AT disk controller that

doesn't support the Identify Drive command.  This is not a

fatal error, it just means that the drive information can't

be displayed.

*** Identify command was rejected by drive x

The Identify Drive command is not used by the BIOS or the

System for normal operations.

Notes:

This program is only useful when used on AT computers that

use a standard AT drive interface.  If it's used with

non-standard disk controllers/drives, it will report the

number of hard drives the BIOS found and report zero IDE/AT

drives found.  The program could be run in these systems to

confirm that there is no IDE/AT drives or in systems that

may have an IDE drive plus another type (e.g., SCSI drives).

This program accesses the disk drives by directly reading

and writing hardware registers.  This program does not write

to the disk drive nor does it alter any of the drive's data

or parameters.

The IDE Drive interface only supports two drives:  Drive 0

and Drive 1.  The program does a special check to see if

Drive 1 is present without Drive 0.  This means that the

drive(s) are jumpered incorrectly and a warning message is

displayed:

*** Warning:  Only one drive found and Its addressed

as Drive 1.

The number of cylinders/heads/sectors displayed is generally

the drive's "native" values.  These are the physical values

used internally by the drive.  The interface accepts logical

parameters from the System (as defined in the CMOS drive

type and Drive Parameter tables) and translates them into

the drive's physical parameters.

Some drives (Seagate ST-1144 for example) will return the

same parameters as the System's.  This occurs because the

drive varies the number of sectors per track based on it

physical location on the disk (called "zones").

-----------------------------------------------------------

Copyright Notice

-----------------------------------------------------------

This program and documentation is copyrighted by Thomas J.

Newman.  The program and documentation in its complete and

unmodified form may be used freely and uploaded to other

BBSs, but may not be resold without written permission from

the author.

If you have any comments, suggestions, or question about

this program, please send an e-mail message to the author

through Compuserve: 71246,1573