OS2IDE2X Driver Documentation

----------------------------------

Enclosed is the OS2IDE2X.ADD. It contains a number

of changes from the previous driver. This note is

intended to help explain those changes.

Section 1:

Summary of Changes

------------------

Multiple Controller support

- 2 independent controllers can be supported per instance

of this driver.

- Multiple instances of the driver may be loaded

Improved Geometry Support and  >1024 Cyl support

- The driver will attempt to determine drive geometry

by the following methods:

-- Via command line switches

-- Via an IDENTIFY command issued to the drive

-- Via the Fixed Drive Tables

This driver will support disks >1024 cylinders

if indicated by one of the above geometry sources.

- The driver can display the BIOS (Int 13), IDENTIFY,

Command Line geometries and the geometry selected

by the driver.

Error Handling/Recovery

- The driver no longer issues "Power Management"

commands.

Regardless of what CAM states there appears to

be no industry wide consensus on what these commands

should be. In addition sufficient attention has

not been paid to compatibility issues with older

MFM/RLL and IDE controllers.

- Error recovery is driven by elapsed time

Error recovery in this driver has been restructured.

When an error occurs, retries will be done periodically

over a set time interval. This should allow drives which

have "spun-down" to recover.

- Soft-Reset used to help clear error conditions

The driver will issue a soft reset via 3F6 to recover

from timeouts and from persistent error conditions.

Command Line Switches

---------------------

Sample Command Lines:

BASEDEV=OS2IDE2X.ADD /V /A:0 /U:0 /T:10 /GEO:(1314,7,26,-1) /A:1 /I

BASEDEV=OS2IDE2X.ADD /V /A:1 /P:120 /IRQ:11 /U:0 /T:10

BASEDEV=OS2IDE2X.ADD /V /A:1 /U:0 /GEO:20

BASEDEV=OS2IDE2X.ADD /V /A:0 /U:0 /SMS

BASEDEV=OS2IDE2X.ADD /V /A:0 /!R

Switch definitions:

/V = Verbose Mode

Displays controller status and drive geometry information

during OS/2 initialization.

/A:0 = Adapter number

Adapter 0 has default parameters for a primary adapter.

Adapter 1 has default parameters for a secondary adapter.

/U:0 = Unit Id

Unit 0 indicates parameters apply to the master unit.

Unit 1 indicates parameters apply to the slave unit.

/T:10 = Error retry time in seconds. (dec)

Indicates the total allowable error recover time for

a request. Error recovery times < 5 seconds will be

ignored. This parameter defaults to 30 seconds. A shorter

interval may be desirable for fault tolerant applications.

/GEO:(1314,7,26,-1) or /GEO:20 = Drive geometry override (dec)

Indicates the Cylinder/Head/Sector geometry for the drive.

The fourth parameter is the Write Precompensation Cylinder

which may be omitted for drives which do not require

precompensation.

As an alternate format standard BIOS drive types may

be used. Types (0-47) are supported. User defined types

48, 49 should be entered directly by in the previous

format.

When specified, this parameter overrides other geometry sources.

If this parameter is absent, other geometry sources discussed

above will be used.

/I = Ignore adapter

The driver automatically attempts to locate and initialize

both the primary and secondary adapters. In some cases other

DASD controllers may appear between the primary and secondary

IDE controllers. In these cases the system should be configured

as follows:

BASEDEV=OS2IDE2X.ADD /V /A:1 /I

BASEDEV=MOREDASD.ADD

BASEDEV=OS2IDE2X.ADD /V /A:0 /I

/!R = Disable adapter resets

Disables adapter resets. In most cases resets are beneficial

to assist in recovering from transient hardware problems

such as lost interrupts, timeouts, or commands a particular

adapter may not support.

However, for some ESDI adapters, options set by vendor

unique commands such as 'Sector Sparing' may be lost after

after a reset. Setting this switch is recommended for ESDI

adapters with disks formatted using 'Sector Sparing'.

/P:120 or /PORT:120 = Base Port address (hex)

Overrides the default Base Port address for this adapter.

Ports are still assigned in this case would be (120-127) and

(326). The default port addresses for Adapter 0 is (1F0) and

for Adapter 1 is (170).

/IRQ:11 = IRQ Number (dec)

Overrides the default IRQ Number for this adapter. The default

IRQ address for Adapter 0 is (14) and for Adapter 1 is (15).

/SMS = Enable Set Multiple Mode support

This switch enables Set Multiple Support which the improves

performance of some IDE drives. If the drive does not have

this feature, this switch will be ignored. The /V - (Verbose)

option will indicate whether this feature has been enabled

on a particular drive.

Section 2:

Install Method :

There are two methods to install this driver to OS/2

Method 1. Put this driver disk in Drive A:

In OS2 Command Prompt type

C:>ddinstall

and press ENTER key

OS2 will automatic install this driver to system !

Method 2. Copy OS2IDE2X.ADD from driver diskette to \OS2

Edit \CONFIG.SYS

Add a REM to the line contain BASEDEV=IBM1S506.ADD

BASEDEV=IBM1S506.ADD ---> REM BASEDEV=IBM1S506.ADD

ADD another line to CONFIG.SYS

BASEDEV=OS2IDE2X.ADD

You can add options as described in Section 1 Command

Line Switch !