(1) SISIDE.DSK  Support netware 3.1x and 4.x Hard disk and ASPI interface

(2) SISIDED.DSK support netware 3.11. (Without CD-ROM).

(3) By editing c:\server\startup.ncf file, you can force the default DMA

master mode into PIO mode by using /d0, and /d1 command. For example

load siside.dsk /d0 /d1 int=E port=1f0  ==> Force Channel 0 into PIO mode.

load siside.dsk /d0 /d1 int=F port=170  ==> Force channel 1 into PIO mode.

*/D0 : force HDD (master) to PIO mode

*/D1 : force HDD (slave) to PIO mode

Command Usage as follows :

:load siside port=1f0 irq=e  /D0 /D1

*I/O port :

port=1f0 => I/O port 1f0-1f7

port=170 => I/O port 170-177

*IRQ :

irq=e =>  interrupt request 14

irq=f =>  interrupt request 15

******************** installation Note *********************************

(1) If you install two channel netware driver, please install as follows:

LOAD SISIDE.DSK INT=E PORT=IF0

LOAD SISIDE.DSK /d0 /d1 INT=F PORT=170

(2) If you want to mount the CD-ROM as one of your volumes, Novell lab.

recommends user to use the HD & CD-ROM in different Channel.

for example on Netware 4.1:

LOAD SISIDE.DSK INT=E PORT=1F0  (Using channel 0 for HDs)

LOAD SISIDE.DSK INT=F PORT=170  (using channel 1 for CD-ROM)

LOAD ASPICD

LOAD CD-ROM

CD DEVICE LIST   ; (There will show a CD-ROM device number on the screen)

CD MOUNT #       ; (# is the device number of the CD-ROM)

(3) SiS Netware driver can support the CD-ROM as follows:

OTI             Dolphin 8001

SONY            CDU55E

SONY            CDU76E

TOSHIBA         XM-5302B

NEC             CDR-273

CREATIVE        CD420E

CREATIVE        CD620E

MITSUMI         FX400

WEARNESS        CDD220

************************************************************************