Table of Contents:

Section 1.  Introduction

1.1 UltraStor UltraTAPE Software Package

1.2 License

1.3 Limited Warranty

1.4 Copyright

Section 2.  Preparations and Precautions

2.1 Protecting Your Data

2.2 Ensuring the Proper Attachment of Peripherals

2.3 Power Requirements

2.4 DOS Operating Environment

Section 3.  Technical Assistance

3.1 Resellers

3.2 Compuserve

3.3 UltraStor Technical Support

Section 4.  Description of files for UltraTape Software Package

Section 5.  Installation

5.1 Warning

5.2 Installing the UltraTape Program

Section 6. Tested SCSI Peripherals

PLEASE READ THIS MANUAL BEFORE PROCEEDING TO INSTALL. THERE ARE

MANY PRECAUTIONS TO CONSIDER AND YOUR TIME WILL BE WELL SERVED

BY STUDYING THIS README FILE BEFORE ATTEMPTING TO INSTALL.

Section 1.  Introduction

1.1 UltraStor UltraTAPE Software Package

UltraStor UltraTAPE Software Package contains :

* ASPI Managers for UltraStor ULTRA 14N, ULTRA 14F and ULTRA 24F

Host Adapters.

* UltraTape Utility to backup and restore disk data to tape

drives.

1.2 License

You may use the programs on a single machine which contains an

UltraStor Host Adapter.  You may make backup copies for the same

machine.  This license is only transferable to another party if

the other party agrees to accept all of the conditions of this

license.  If you transfer the programs, all programs, disks,

materials and copies of the programs must be transferred at the

same time.  If you transfer or yield possession of the programs

or any portion thereof, your license is automatically

terminated.  This license is effective until terminated.  You may

terminate the license at any time by sending all programs,

disks, materials, and copies to UltraStor.  This license will

also terminate if you fail to comply with the terms or

conditions of this license or if you infringe upon UltraStor's

copyrights.  You agree upon termination to destroy the programs,

disks, materials, and copies in any form.

1.3 Limited Warranty

The programs are provided "as is" without any warranty of any

kind, either expressed or implied.  UltraStor assumes no risk for

the use of the programs.  If your use of the programs, disks,

materials and copies proves defective, you assume the total cost

for repair or correction.  UltraStor does not warrant that the

programs, disks, materials, and copies will meet your

requirements or that the operations of said programs, disks,

material, and copies will be error-free.  UltraStor warrants that

the diskette(s) containing the programs to be free from defects

in material and workmanship under normal and intended use for a

period of 90 days from the date of delivery to you.

1.4 Copyright

The programs, disks, materials, and copies are proprietary

products of UltraStor Corporation and are protected by the

Copyright Laws of the United States.  Except as allowed in the

license, you may not copy any portion of the programs, disks,

and materials.

Section 2.  Preparations and Precautions

2.1 Protecting Your Data

In order to protect your data, we strongly recommend that you

backup all data on your hard drive(s).  Loss of data is never

expected and the best protection is a good set of backups.

2.2 Ensuring the Proper Attachment of Peripherals

During the installation of various SCSI peripherals, it

will be necessary for you to locate and determine whether the

peripheral has terminating resistors installed, which SCSI ID

has been set on the SCSI peripheral(s), and where "Pin 1" exists

on the peripheral's SCSI interface connector.

A proper placement of terminating resistors will ensure

reliable and unimpeded data flow on the SCSI bus.  Only the two

peripherals, including the UltraStor SCSI Host Adapter,

at each end of the SCSI bus will have terminating

resistors installed.  All other SCSI peripherals will have their

terminating resistors removed or disabled.

The SCSI bus provides for eight (8) physical unit IDs, starting

with zero (0) and running upto seven (7).  The last ID (7) is

usually reserved for the SCSI Host Adapter.  The first two IDs,

zero (0) and one (1) are usually reserved for the booting hard

drive and second physical hard drive; more than two (2) hard

drives can be supported with the UltraStor SCSI Host Adapters with

either a special jumper setting or an EISA configuration board

option selection.  Keep in mind that you have only seven (7) IDs

to choose since the UltraStor SCSI Host Adapter requires one.  It

is usually a good idea to write out the intended assignments for

IDs before installation.

The SCSI cable can only be properly attached to peripherals in

one way; attaching the cable incorrectly will cause problems

with the installation.  Use the peripheral's installation guide

or manual to locate "Pin 1" for its SCSI connector.  This is

important as some damage may occur to the peripheral if the SCSI

cable is reversed and power is turned onto the system.  Many SCSI

peripherals set "Pin 1" on its SCSI connector to be closest to

the power supply; in all cases, verify its location.  The cable

provided by UltraStor has one side of the cable marked by

writing on the entire length.  This marking indicates where "Pin

1" is located; line up "Pin 1" on all peripherals and your SCSI

bus will operate properly.

2.3 Power Requirements

Ensuring proper power supply to the entire system, including the

SCSI bus peripherals is extremely important.  Make sure your

system's power supply is adequate; should your power supply not

be able to supply sufficient power to all peripherals, the

system will not operate properly and may cause damage to some

peripherals and perhaps to the system.  Some SCSI peripherals

require a peak power supply at power on time; this is a key

point for many older SCSI drives and tape drives as the peak

power necessary to start spinning a 15 platter drive or start

spinning a tape cartridge is rather high.  If the system

successfully recognizes all peripherals after the power up

sequence is completed, then your system will most likely contain

sufficient power to continue operations.

2.4 DOS Operating Environment

The UltraStor UltraTAPE Software has been tested with MS DOS 3.3 and

above.  Certain versions of MS DOS operating system utilities may

operate differently; please refer to the actual documentation.

Use of memory managers and caching software may alter the

behavior of some UltraStor utilities; every effort has been made

to ensure compatibility with standard operating environments.

However, it is possible to configure memory managers and caching

software in a way that impedes operation or slows performance of

the SCSI peripherals.  Should you experience problems, we

strongly recommend that you limit or omit your usage of these

types of software until such time you have successfully

installed the SCSI subsystem.

Section 3.  Technical Assistance

3.1 Resellers

If you purchased this UltraStor product from a reseller, please

contact that reseller for further assistance.  Since the reseller

may have provided additional equipment and software, it is

always best to contact the reseller first.

3.2 Compuserve

Additionally, technical assistance may be obtained by accessing

the Compuserve Information Network.  Once online, issue the "GO

IBMHW" command to enter the IBM Compatible Hardware forum.

Select the section on Host Adapters and drives (as of this

writing, that section is "4") and leave a message to all or

UltraStor.  An UltraStor technical expert will generally read

those messages and offer assistance.

3.3 UltraStor Technical Support

UltraStor maintains a technical support hotline for our

resellers and for those customers who are unable to obtain

support from the other methods listed above.  That telephone

number is 1-510-623-8955.  As the staff of this organization is

continuously called upon for help, please be patient when

requesting assistance.  It is not uncommon for your name and

telephone number to be requested so that a call back can be

scheduled.  Since call backs are scheduled on a first come first

served, please expect delays of one or more days.  Again, we

strongly recommend that you contact the reseller or log on to

Compuserve for assistance.

In addition to the voice hotline, UltraStor maintains a Driver

Download Facility.  Licensed UltraStor users may download the

latest operating system level drivers from this system.  Note:

UltraCD, UltraREM, UltraTape, and MSCDEX are licensed products

which may not be downloaded from this facility; the latest

versions of the USPI driver and the third floppy driver may be

downloaded.  Access the UltraStor Driver Facility by having your

modem set for 8bits, No parity, 1 start/stop bit, 2400 to

14400bps speed, and then dialing 1-510-623-9091.  Please do not

leave requests for technical support on this system as they

cannot be answered by the technical support organization.

Section 4.  Description of files for UltraStor UltraTAPE Software Package

README  .DOC - This readme file.

USPI14N .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 14N AT SCSI

Host Adapter.

USPI14  .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 14F AT Bus Master

SCSI Host Adapter.

USPI24  .SYS - DOS device driver for supporting ASPI compatible device

module interface used with ULTRA 24F EISA Bus Master

SCSI Host Adapter.

UTAPE   .EXE - Provides full functions for tape backup and restore

utility.

UTAPE   .HLP - The help file for UTAPE.EXE.

Section 5.  Installation

5.1 Warning

Warning: this installation procedure will modify the AUTOEXEC.BAT

and CONFIG.SYS files on your booting hard drive.  Please make copies

so that you can recover should problems occur.  Please make

backups of all data and programs on your hard drive.

This procedure is not a replacement for the installation

instructions contained in the manual but only a brief summary of

the necessary steps.  Please read the installation instructions

detailed in Chapter 2 to insure a proper and successfull

installation.

Install the UltraStor SCSI Host Adapter per the installation guide

supplied.

5.2 Installing the UltraTape Program

Insert the UltraTape floppy into drive A: (or B:) and copy the

appropriate USPIxx.SYS file (where xx = 14N, 14 or 24) to the

root directory of your hard disk.

Add the following line to your config.sys:

Device=USPIxx.SYS  where xx = 14N, 14 or 24

Remove the UltraTape floppy and reboot your computer to allow the

USPIxx.SYS to take control of the UltraStor Host Adapter.

After rebooting, re-insert the UltraTape floppy and key in the

following statement at the DOS prompt:

A:>Utape

UltraTape will now present a screen.  Press the letter "C" to

pull down the "CONFIG" menu option at the top of the screen.  Your

first line option will be to select the SCSI ID of the tape drive

which you will be using.  Key in the appropriate ID.  You may leave

all other options at their default.  Press the F10 key (function key)

to continue to the next step.

UltraTape will now prompt you to decide if these changes are

permanent.  Answer "Y" if this is the first installation.

Next, the install program will request the target drive and

directory where all of the UltraTape utility will reside.  A

default name is displayed but you may change it to any valid

drive and directory name.  The directory name does not have to

exist as the install program will create it, if necessary.

If you choose to continue, the install program will modify the

CONFIG.SYS and AUTOEXEC.BAT files.

After the modules have been installed and after the CONFIG.SYS

and AUTOEXEC.BAT files have been modified, the install program

will finish with a prompt asking if you would like to reboot the

system so that the new modules and the modified files will be

loaded.

After the system reboots, you should now have the selected

supported as requested during the install procedure.

Enjoy!

Section 6. Tested SCSI Peripherals

The following hard drive manufacturers have supplied a  representative

sampling of all their SCSI drive families for compatibility testing

with UltraStor SCSI Host Adapters:

Connor

Fujitsu

Hitachi

HP

Magtron

Maxtor

Micropolis

Microscience

NEC

Quantum

Seagate

Toshiba

TAPE Drives

Archive 2060S

Archive 2150S

Archive 2525S

Caliper 150/525

HP 35470

HP 35480

Wangtek 5525ES

Wangtek 5150ES

DAT/4mm Tape Drives

Exabyte 8200

Exabyte 8200SX

WangDAT 1300

Sony SDT-1000

Sony SDT-1020

Archive Python 4520NT