# MicrIO Device Driver for the

# WD7000FASST SCSI Controller

Copyright MicrIO 1994

email address; micrio@mv.com

# Introduction

This manual describes the installation procedure for the MicrIO device driver for the Western Digital WD7000FASST SCSI controller board.   The driver will support up to three boards in one system.   The driver is a full multitasking device driver that supports scatter/gather on those boards that have the scatter/gather feature.

## Disribution

The MicrIO shareware device driver for the WD7000FASST SCSI controller board and associated program files must distributed together along with this manual.

This manual may be distributed seperately from the MicrIO device driver but the manual may not be changed and must be distributed in it's entirety.

## The shareware driver

The driver is available in two forms; the commercial product and the shareware product.   The shareware driver may be freely distributed, posted to bulletin boards and included in shareware CDROMs so long as all files are included and no file is altered.   The shareware driver has the following limitations;

*	Only one Winchester disk is supported (must be at SCSI id 0)

*	Only one CD-ROM device is supported (may be at any SCSI id)

*	No other devices are supported, i.e.; tape drives, scanners, etc.

*	Performance is lower than that of the commercial driver.

*	Scatter/gather is not supported.

*	Must be registered after a one month evaluation period.

## Registration

To register the shareware version of the driver send $19.00 to the address below.

MicrIO

P.O. Box 1202

Londonderry, N.H. 03053-1202

The full featured commercial driver may be purchased for $39.00 plus $5.00 shipping and handling.   Registered users of the shareware driver may upgrade to the commercial version by sending the difference, ie; $20.00 plus $5.00 shipping and handling.

## Initial installation of the WD7000FASST

### Hardware installation steps

*	Set the I/O address to 350.

*	Set the interrupt level to 15.

*	Set the DMA port to 6.

*	Set the ROM BIOS address to CE00 or any acceptable address and insure that the BIOS ROM is installed.

*	Remove the terminators if both internal and external devices are to be used

*	Install the terminator power jumper W5.

*	Do not install the floppy disable jumper W6 if you are going to use the on board floppy controller

*	Install the dual speed floppy jumper W7

*	Install the floppy write precomp jumper W8

*	Install the floppy primary address jumper W9.

*	Install the board in the system.

### Driver installation steps

*	Boot system off the Windows NT 3.1 boot disk

*	Select "C" for custom configure when prompted

*	Select "S" to configure additional SCSI adapters

*	Select "Other" SCSI adapter.

*	Insert the MicrIO driver disk

*	Choose the WD7000FASST driver (only choice)

*	Continue with installation normally.

## Installing a second card

### Hardware installation steps

*	Set the I/O address to 358.

*	Set the interrupt level to 11.

*	Set the DMA port to 5.

*	Set the ROM BIOS address to D000 or acceptable address and remove the BIOS ROM.

*	Install the terminator power jumper W5.

*	Install the floppy disable jumper W6.

### Driver installation steps

*	Select Windows NT Setup from the Main window

*	Select Options; Add/Remove SCSI Adapter

*	Select Add

*	Answer yes to the Warning message

*	Select Other from the list of SCSI controllers

*	Insert the MicrIO I/O WD7000FASST driver floppy

*	Answer Yes after the floppy is inserted

*	After the driver is listed in the selection window, answer Yes

*	Then select Install

*	The system will detect that the driver is already on the system and ask if you want to replace it with a new one.   If there is no difference then answer Current.

*	The driver will be loaded then select close

*	Close the setup window

*	You will be informed that the change will not take effect until the computer is restarted, answer OK

## Installing a third card

### Hardware installation steps

*	Set the I/O address to 360.

*	Set the interrupt level to 10.

*	Set the DMA port to 7.

*	Set the ROM BIOS address to D200 or acceptable address and remove the BIOS ROM.

*	Install the terminator power jumper W5.

*	Install the floppy disable jumper W6.

### Driver installation steps

There is nothing special to be done in NT when installing a third card.   The driver will detect the presence of the third card and automatically support it.

## Debugging

If, you are adding a second board and you can run Windows NT, the WinMsd program can give some information as to your setup.

Displaying "Devices" will show you if all SCSI drivers are loaded.   They are called \Device\ScsiPort0 for the first and \Device\ScsiPort1 for the second and so on.   If you double click on the driver line, more information will be displayed.   The DMA channel, the interrupt level, the I/O port, and the memory address of the BIOS are all displayed.   The second and third WD7000FASST cards do not claim any BIOS memory address so this area should be empty.   The second and third WD7000FASST cards do use the memory area whose address is set by the ROM BIOS address jumpers.   The addresses are used even though the ROM BIOS EPROM is not installed..

## Termination

The SCSI bus requires that there be a termination at each end of the bus.   The termination is a resistor package that can take a few forms.   It can be 2 or 3 resistor single inline package (SIP)  modules that are generally plugged into the circuit board of the SCSI devices themselves.   The terminator can be a module that plugs into the SCSI connector that exits the case of the SCSI device.   Or it can be 50 pin DIP module that connects inline with the SCSI 50 pin ribbon cables.   All of these are essentially equivalent.   The important thing being that there be a terminator at each physical end of  the SCSI bus.

If you only have internal devices then one termination should be at the WD7000FASST controller board.   This termination can be either the two SIP resistor packages that plug into the board or a terminator that plugs into the "Centronics" connector on the board rail.   The SIP resistors must be plugged into the controller in the correct direction.   The small dot near the pin on one end of the device indicates pin one of the SIP.   This must be plugged in such that it aligns with pin one on the controller.   Pin one of the controller is toward the top of the board, that is, the edge away from the PC bus connector.

## WD7000FASST Jumper Settings

### I/O Address

This table describes the jumpers that set the I/O address.   These pins are in jumper block W3.   The default setting is I/O address 350.   This setting is required for system booting using the on board BIOS.

Address	Pins:	Pins:	Pins:	Pins:	Pins:

in hex	1-2	3-4	5-6	7-8	9-10

300	short	short	short	short	short

308	open	short	short	short	short

310	short	open	short	short	short

318	open	open	short	short	short

320	short	short	open	short	short

328	open	short	open	short	short

330	short	open	open	short	short

338	open	open	open	short	short

340	short	short	short	open	short

348	open	short	short	open	short

350 	short	open	short	open	short

358	open	open	short	open	short

360	short	short	open	open	short

368	open	short	open	open	short

370	short	open	open	open	short

378	open	open	open	open	short

380	short	short	short	short	open

388	open	short	short	short	open

390	short	open	short	short	open

398	open	open	short	short	open

3A0	short	short	open	short	open

3A8	open	short	open	short	open

3B0	short	open	open	short	open

3B8	open	open	open	short	open

3C0	short	short	short	open	open

3C8	open	short	short	open	open

3D0	short	open	short	open	open

3D8	open	open	short	open	open

3E0	short	short	open	open	open

3E8	open	short	open	open	open

3F0	short	open	open	open	open

3F8	open	open	open	open	open

### ROM BIOS Address

This table describes the jumpers that set the BIOS ROM memory address.   These pins are in jumper block W4.   The default setting is address CE00.   This setting can be changed so that it does not conflict with any other devices, most commonly; video boards.   The BIOS occupies 8192 bytes of address space.   For any particular address setting, the WD7000FASST occupies the address up to the next higher address in the following table.   For example; if the address was set to CE00 the board would occupy the address up to CFFF.

A second or third board should be addressed so as to not conflict with other devices in the system.   The second and third boards must have their BIOS ROM removed.   The system will detect two or more BIOS ROMs in the system and may not boot properly.   Even though the BIOS ROM is physically removed the address selected for it is still occupied.   This is because the BIOS address decoding circuitry cannot be disabled, it can only have it's address changed.

In extreme cases the BIOS address of a second board can sometimes be set to the same address of the first board.   This results in address contention on the bus but often the system will work anyway.   This is only an issue during booting, during normal running the BIOS ROM is not used.   This generally cannot be done with a third board because the contention becomes greater and the system will not boot.

Address	Pins:	Pins:	Pins:	Pins:

in hex	1-2	3-4	5-6	7-8

C000	short	short	short	short

C200	open	short	short	short

C400	short	open	short	short

C600	open	open	short	short

C800	short	short	open	short

CA00	open	short	open	short

CC00	short	open	open	short

CE00	open	open	open	short

D000	short	short	short	open

D200	open	short	short	open

D400	short	open	short	open

D600	open	open	short	open

D800	short	short	open	open

DA00	open	short	open	open

DC00	short	open	open	open

DE00	open	open	open	open

### DMA Channel

This table describes the jumpers that set the DMA channel.   These pins are in jumper block W2.   The default setting is DMA channel 6.   This setting is required for booting using the on board BIOS.

DMA 	Pins:	Pins:	Pins:	Pins:	Pins:	Pins:

Channel	1-2	3-4	5-6	7-8	9-10	11-12

5	open	open	short	open	open	short

6	open	short	open	open	short	open

7	short	open	open	short	open	open

### Interrupt Channel

This table describes the jumpers that set the Interrupt channel.   These pins are in jumper blocks W1 and W2.   The default setting is Interrupt channel 15.

Interrupt	Short	Jumper

Channel	Pins	Block

3	1-2	W1

4	3-4	W1

5	5-6	W1

7	7-8	W1

9	9-10	W1

10	21-22	W2

11	19-20	W2

12	17-18	W2

14	15-16	W2

15	13-14	W2

### Terminator Power

The terminator power jumper is W5.   It should be inserted if you want the WD7000FASST board to supply the terminators with power.   The default is for the jumper to be in.

### Floppy Enable

Jumper W6 controls floppy enable.   Installed disables the floppy controller.   The default is not installed.   Second and third controllers installed in one system should disable their floppy controllers by installing a jumper.

### Dual Speed Floppy

Jumper W7 controls the dual speed floppy.   With the jumper installed both 360 and 300 RPM speeds are enabled.   Not installed enables only 300 RPM.   The default is installed.

### Floppy Write Pre compensation Select

Jumper W8 controls floppy write pre compensation.   Installed selects 187 ns and not installed selects 125 ns.   The default is installed.

### Floppy Controller Address

Jumper W9 controls the I/O address of the floppy controller.   Installed selects the primary address.   Not installed selects the secondary address.   The default is installed.