Cyrix 486SLC/486DLC

Cache Controller

The SETCACHE.EXE program initializes the Cyrix 486 chips internal Cache

to the desired mode. When the Cyrix chips are reset, cache is disabled.

This program allows the user to setup cache for maximum operation speed.

The SETCACHE program may be executed in the AUTOEXEC.BAT file to enable

cache operation at startup, or may be executed at any time from the

command line. The default values have proven to be the fastest operation,

however each user can establish their own setup.

Command Line options are as follows:(Options Help Screen)

Cache Control program for Cyrix 486SLC/486DLC Microprocessors. Ver. 1.2

COPYRIGHT Dec. 14, 1993 BY JOHN HICKEY of J. JENKS ENTERPRISES.

}SETCACHE [options]   (Options as follows:)

o - display options help screen, overrides v t l m h r options.[New v1.2]

v - display current settings, overrides t l m h r options.[New v1.2]

t - DOS memory area A0000-FFFFF is not cached.[Changed v1.2]

l - Cache inhibit 64K at 1 meg boundaries.

m - Cache mode is direct-mapped.

h - Flush Cache on CPU Hold.

r#aaaaas - Inhibit cache at region #, starting

at address aaaaa X 1000h, for s size, where:

#     - 1 to 4

aaaaa - 00000-FFFFF hex = address bits 31-12 of inhibited memory.

s     - 0-F size code as follows:

size-code     Range  size-code     Range  size-code     Range

0 - none       N/A   1 - 4KBytes   aaaaa  2 - 8KBytes   aaaaj

3 - 16KBytes  aaaak  4 - 32KBytes  aaaal  5 - 64KBytes  aaaa0

6 - 128KBytes aaaj0  7 - 256KBytes aaak0  8 - 512KBytes aaal0

9 - 1MBytes   aaa00  A - 2MBytes   aaj00  B - 4MBytes   aak00

C - 8MBytes   aal00  D - 16MBytes  aa000  E - 32MBytes  aj000

F - 4GBytes   00000(Cache is totally inhibited)

a = 0-F Hex  j = 0, 2, 4, 6, 8, A, C or E Hex  k = 0, 4, 8 or C Hex

l = 0 or 8 Hex     Spaces unnecessary between options.

Defaults to 2-way assoc., all mem. cached except Video RAM A0000-BFFFF.

SETCACHE with no options will initialize the Cache to the default values

of 2-way associative cache and all memory cached except Video RAM at

A0000 through BFFFF.

Option

o      - Displays the Options Help screen. Overrides all other options.

v      - Read Cache settings and display them. Overrides all options except

option 'o'.

t      - Inhibit Cache of DOS memory area A0000-FFFFF.

l      - Inhibit Caching of 64k bytes at each 1meg boundary.

m      - Cache Mode is direct mapped.

h      - Flush cache on CPU HOLD.

r#aaaaas - Inhibit Cache in specific memory regions.

#     - Non-Cacheable Region number 1 to 4.

aaaaa - Region beginning boundary address in 4k segments (1000 hex).

The boundary address must start on specific power of 2

address, depending upon the size code. Valid addresses

will be rounded down to the nearest appropriate value.

s     - Size code 0-F hex specifying region granularity.

Size           Boundary   Boundary

code           addr range ending digit(s)

0 - not        N/A

1 - 4KBytes   aaaaa(000) 0-F

2 - 8KBytes   aaaaj  "   0,2,4,6,8,A,C,E

3 - 16KBytes  aaaak  "   0,4,8,C

4 - 32KBytes  aaaal  "   0,8

5 - 64KBytes  aaaa0  "   00-F0

6 - 128KBytes aaaj0  "   00,20,40,60,80,A0,C0,E0

7 - 256KBytes aaak0  "   00,40,80,C0

8 - 512KBytes aaal0  "   00,80

9 - 1MBytes   aaa00  "   000-F00

A - 2MBytes   aaj00  "   000,200,400,600,800,A00,C00,E00

B - 4MBytes   aak00  "   000,400,800,C00

C - 8MBytes   aal00  "   000,800

D - 16MBytes  aa000  "   0000-F000

E - 32MBytes  aj000  "   0000,2000,4000,6000,8000,A000,C000,E000

*F - 4GBytes   00000  "   Cache is totally inhibited.

After the SETCACHE program executes, the values currently in the

Cache Controller registers are read back and are displayed as follows:

RX RY area-1 area-2 area-3 area-4

where

RX     = Cache Control Register CCR0

RY     = Cache Control Register CCR1

area-n = aaaaas for Non-Cacheable Region n.

SETCACHE Register display: (Default settings)

Cache Control program for Cyrix 486SLC/486DLC Microprocessors. Ver. 1.1

COPYRIGHT Dec. 8, 1993 BY JOHN HICKEY of J. JENKS ENTERPRISES.

00 00 000A06 000000 000000 000000

The Cache Controller settings will remain unchanged until a System Reset

is done or SETCACHE is rerun with new parameters.

!!CAUTION!!!

Do not run the program on any 80486 chip except the Cyrix or Texas

Instrument 486SLC or 486DLC chips. The Cache Control registers are

not compatible with INTEL 80486 chips.

Do not attempt to run this program on any CPU other than a 80486.

It uses 486 unique instructions and will cause illegal instruction

faults on lesser CPU's!!

NOTE:

It may be that some motherboards or BIOS's are incompatible with

enabling the 486 internal cache. One user reported that enabling the

cache did not increase the operation speed of his TI DLC chip (AMI BIOS,

UMC chip set), however moving the chip into a different motherboard

worked properly. (V1.2 - Found that the BIOS was disabling Cache by setting

bit 30 in CPU Control Register 0(CR0). Added code to enable cache in CPU

register CR0 if disabled.)

Another user (AMI BIOS, FOREX chip set) reported that

the Cache speeded up CPU operation, but prevented proper operation of the

Ctrl-Alt-Del reboot function. Further investigation is being made to

determine whether it is a chip set or BIOS problem. One possible answer is a

conflict between the BIOS and Cache. (V1.2 - Add the field 'R2000F05' to the

SETCACHE command to inhibit cacheing the BIOS ROM area, F0000 through FFFFF,

as one possible solution.)

The program has worked well with PHOENIX BIOS, AMI BIOS, VLSI chip set,

Chips and Technology chip set, and OPTI chip set.

WHATS NEW!

___________________________________________________________________________

Changes in Version 1.2.

It was found that some BIOS's set the Cache Disable bit 30 in the CPU

Control Register 0(CR0), preventing the internal 486DLC cache from working.

Code was added to Version 1.2 to test for Cache Disable in CR0 and clear

the Disable bit if necessary.

NOTE: This program can only be run in DOS Real mode and not in protected

mode such as a DOS window under WINDOWS.

___________________________________________________________________________

Changes in Version 1.1.

Added 'o' option to display help screen. Cache Registers are unchanged and

are not displayed with the help screen.

Added 'v' option to read current status of cache registers. Cache Registers

are unchanged.

Changed default settings to all memory cached except video RAM at A0000-BFFFF.

____________________________________________________________________________

SETCACHE.EXE is the Copyrighted work of John Hickey and J. JENKS Enterprises.

Copyright June, 1993

Copyright December, 1993

All Rights Reserved

Author         : John C. Hickey

Date of release: 06/04/93

Version 1.1    : 12/08/93

Version 1.2    : 12/14/93

Please let me know if you have any problems or find the program to be

useful. Registered users may call if they have any problems, and will

be notified of any upgrades.

John Hickey

J. Jenks Enterprises

2155 Philippe Pkwy

Safety Harbor, Fl, 34695

(813) 726-0648