MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4131G  GREEN  VL-BUS  MAINBOARD  TEST  REPORT

January  31,  1994

MAINBOARD	MS-4131G

CHIPSET	ALI M1429G

VERSION	Ver.  3

BIOS		Phoenix  PD51

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel P24S (80486DX2-66 S-series) CPU

·	16MB Memory which consists of:

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-20)

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel P24S (80486DX2-66) CPU

·	28MB Memory which consists of:

(Micron MT4C4001JDJ-7)  	8MB x 1	72-pin double-sided

(Micron MT4C16257DJ-7)  	4MB x 1	72-pin single-sided

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Qtronix SDX-85 Tape Drive

·	Quantum 240AT (240MB) IDE Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| DOS MODE | OK |  |
| CP FILE | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel P24S (80486DX2-66) CPU

·	16MB Memory which consists of:

(Micron MT4C4001JDJ-7)  	8MB x 1	72-pin double-sided

·	256KB Cache (Winbond W24257AK-20)

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Conner CP3104 (104MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel 80486DX2-66 CPU

·	28MB Memory which consists of:

(Micron MT4C4001JDJ-7)  	8MB x 1	72-pin double-sided

(Micron MT4C16257DJ-7)  	4MB x 1	72-pin single-sided

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-20)

·	Ultrastor U14F ISA SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Quantum 240S (240MB) SCSI Hard Disk

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| FILE COMPARE | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

* Using Intel P24S (80486DX2-66) CPU will show the CPU type as Pentium CPU.

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| ACUMOS | AVGA2 | ISA |
| IIT | AGX-014 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| S3 | S3801 | ISA* |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA8900CL | ISA |
| TRIDENT | TVGA8900D | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA* |
| WD | PARADISE | ISA |
| WD | WD90C30 | ISA |
| ATI | MACH 32 | VESA |
| DIAMOND STEALTH PRO REV. 3.1 | S3928 | VESA |
| DIAMOND VIPER REV. B1 | WEITEK P9000 | VESA |
| MICRO-STAR MS-3405 | CIRRUS LOGIC 5426 | VESA |
| MICRO-STAR MS-3406 | IIT AGX-015 | VESA |
| MICRO-STAR MS-3409 | CIRRUS LOGIC 5428 | VESA |
| S3 | S3805 | VESA |
| TSENG LABS | ET4000/W32 | VESA |

*Note:		When  using  S3801  and  ET400/W32  ISA  VGA  cards,  set  the  option  "Hidden  Refresh"  				in  the  BIOS  Chipset  Setup  to  "Disabled."

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NUMBER | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| SMC | FDC37C65BLJP | ISA |
| UMC | UM82C863F | ISA |
| WINBOND | W83757AF | ISA |
| MS-3802 | PROMISE PDC20230B | VESA |
| PROMISE DC4030VL-2 | PROMISE PDC20130A | VESA W/ CACHE |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ULTRASTOR | U14F | ISA |
| ULTRASTOR | U34F | VESA |

D.	NETWORK  ADAPTERS

| TYPE | CHIPSET | REMARKS |
| --- | --- | --- |
| ARCNET | SMC COM90C65LJ | ISA |
| 3COM | 3C503 | ISA |
| 3COM | 3C509 | ISA |
| NE-2000 |  | ISA |
| NE-2100 |  | ISA |

E.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| CONNER | CP3100 | 100 MB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |

F.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| PANASONIC | MN41C1000SJ-08 | 30-pin | 1 MB |
| TOSHIBA | TC511000AJ-80 | 30-pin | 1 MB |
| TOSHIBA | TC5141000ASJL-70 | 30-pin | 4 MB |
| PANASONIC | MN41C4000SJ-08 | 30-pin | 4 MB |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| CYPRESS | CY7C199-20 | 32K * 8 |
| MICRON TECH | MT5C2568-20 | 32K * 8 |
| TOSHIBA | TC55328P-20 | 32K * 8 |
| WINBOND | W24257AK-20 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |

III.	PERFORMANCE  TEST

Configuration:

·	16MB Memory  (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-20)

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| TEST PROGRAMS | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 12.65 | 8.65 |  |
| FPU Index | 62.16 | 30.50 |  |
| Disk Index | 12.34 | 11.49 |  |
| Video Index | 13.68 | 9.24 |  |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.72 | 1.28 |  |
| FPU Index | 4.61 | 2.31 |  |
| Disk Index | 1.04 | 1.04 |  |
| Video Index | 3.74 | 2.18 |  |

| TEST PROGRAMS | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| PC LABS BENCHMARK 7.0 |  |  |  |  |
| DOSMark | 12.64 | 11.93 |  |
| Processor Harmonic | 12647.85 | 8867.25 |  |
| Video Harmonic | 8192.19 | 8168.97 |  |
| Memory Harmonic | 6746.06 | 5266.53 |  |
| Disk Harmonic | 8.89 | 8.84 |  |
| NORTON SYSTEM INFO 4.5 |  |  |  |  |
| Computing Index | 110.2 | 51.5 |  |
| Disk Index | 3.9 | 3.4 |  |
| Performance Index | 74.7 | 35.4 |  |
| NORTON SYSTEM INFO 7.0 |  |  |  |  |
| Computing Index | 132.0 | 72.0 |  |
| Disk Index | 7.6 | 7.5 |  |
| Performance Index | 90.4 | 50.5 |  |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 151.9 | MHz |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 223.0 | 111.5 | MHz |
| FPU Speed | 545.6 | 272.6 | MHz |
| Video Speed | 945.2 | 945.2 | CHR/MS |

| QA PLUS 4.61 | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| SYSTEM PERFORMANCE |  |  |  |  |
| CPU Speed | 45517 | 22758 | Dhrystones |
| Direct Video Speed | 757390 | 284021 | CPS |
| Math Speed | 10525.5 | 5683.5 | K-Whetstones |
| DISK PERFORMANCE |  |  |  |
| Trk-Trk Seek | 5.46 | 5.23 | ms |
| Average Seek | 12.5 | 13.0 | ms |
| Transfer Rate | 39.0 | 38.9 | K/sec |

| POWER METER 1.7 | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| SYSTEM TEST |  |  |  |
| Aggregate Hardware Test | 1879.7 | 1069.6 | PMUs |
| Spreadsheet Simulation | 3782.1 | 2808.2 | PMUs |
| Word Processing Simulation | 3819.6 | 4402.0 | PMUs |
| Database App'n. Simulation | 2538.9 | 2936.3 | PMUs |
| CAE/CAD Simulation | 44619.2 | 24246.8 | PMUs |
| Throughput Test | 13590.6 | 7159.4 | PMUs |
| CPU MEMORY TEST |  |  |  |
| Agg. CPU/Memory Test | 26712.0 | 13254.3 | PMUs |
| Clock Speed Test | 65.5 | 33.0 | MHertz |
| MIPS Test | 26.9 | 14.8 | MIPS |
| Dhrystone Test | 34.5 | 19.0 | K-Dhrystones/S |
| Whetstone Test | 6531.5 | 3276.4 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | Seconds |
| DISK DRIVE TEST |  |  |  |
| Aggregate Disk Test | 196.9 | 170.7 | PMUs |
| Track-to-Track Seek Test | 2.4 | 2.4 | MS/Seek |
| Mean Seek Test | 5.8 | 5.9 | MS/Seek |
| Data Transfer Rate Test | 900.5 | 748.8 | Kb/Sec. |
| File Open/Close Test | 3.8 | 3.7 | MS/Op. |
| Sequential File Rd/Wrt Test | 0.2 | 0.2 | MS/Op. |
| Random File Rd/Wrt Test | 12.6 | 12.7 | MS/Op. |
| VIDEO DISPLAY TEST |  |  |  |
| Aggregate Video Test | 7494.8 | 6686.9 | PMUs Mode 3 |
| Character Write Test | 0.5 | 0.6 | Sec. Mode 3 |
| Line Drawing Test | 2.3 | 2.9 | Sec. Mode 18 |
| Fill Test | 11.3 | 13.1 | Sec. Mode 18 |

| QA PLUS 4.70 | DX2-66 | DX-33 | UNITS |  |
| --- | --- | --- | --- | --- |
| SYSTEM PERFORMANCE |  |  |  |  |  |
| CPU Speed | 45.5 | 22.8 | K-Dhrystones |  |
| Direct Video Speed | 757.4 | 473.4 | K-CPS |  |
| Math Speed | 11.1 | 5.7 | M-Whetstones |  |
| DISK PERFORMANCE |  |  |  |  |
| Trk-Trk Seek | 5.42 | 5.15 | ms |  |
| Average Seek | 12.2 | 12.9 | ms |  |
| Transfer Rate | 37.8 | 37.7 | K/s |  |
| POWER METER 1.81 | DX2-66 | DX-33 | UNITS |  |
| SYSTEM TEST |  |  |  |  |
| Aggregate Hardware Test | 1851.3 | 1047.8 | PMUs |  |
| Spreadsheet Simulation | 3884.6 | 3254.1 | PMUs |  |
| Word Processing Simulation | 939.9 | 2654.3 | PMUs |  |
| Database App'n. Simulation | 2546.2 | 1709.4 | PMUs |  |
| CAE/CAD Simulation | 44575.8 | 23959.0 | PMUs |  |
| Throughput Test | 13712.6 | 8448.0 | PMUs |  |
| CPU MEMORY TEST |  |  |  |  |
| Agg. CPU/Memory Test | 25724.7 | 12889.7 | PMUs |  |
| Clock Speed Test | 66.0 | 33.0 | MHertz |  |
| MIPS Test | 19.0 | 10.9 | MIPS |  |
| Dhrystone Test | 34.9 | 19.1 | K-Dhrystones/S |  |
| Whetstone Test | 6531.5 | 3265.7 | K-Whetstones/S |  |
| Sieve Test | 0.1 | 0.1 | Seconds |  |
| DISK DRIVE TEST |  |  |  |  |
| Aggregate Disk Test | 198.6 | 173.1 | PMUs |  |
| Track-to-Track Seek Test | 2.4 | 2.3 | MS/Seek |  |
| Mean Seek Test | 5.8 | 5.9 | MS/Seek |  |
| Data Transfer Rate Test | 910.2 | 752.6 | Kb/Sec. |  |
| File Open/Close Test | 3.6 | 3.8 | MS/Op. |  |
| Sequential File Rd/Wrt Test | 1.2 | 0.2 | MS/Op. |  |
| Random File Rd/Wrt Test | 20.4 | 13.3 | MS/Op. |  |
| VIDEO DISPLAY TEST |  |  |  |  |
| Aggregate Video Test | 7494.8 | 6686.9 | PMUs Mode 3 |  |
| Character Write Test | 0.5 | 0.6 | Sec. Mode 3 |  |
| Line Drawing Test | 2.3 | 2.9 | Sec. Mode 18 |  |
| Fill Test | 11.3 | 13.1 | Sec. Mode 18 |  |

| CHECKIT 3.0 | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| MAIN SYSTEM |  |  |  |
| CPU Speed | 33609 | 16170 | Dhrystones |
| Math Speed | 11419.9 | 5741.3 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |
| BIOS Video Speed (VGA) | 24332 | 19315 | Char/S |
| Direct Video Speed (VGA) | 319793 | 211486 | Char/S |
| HARD DISK |  |  |  |
| Transfer Speed | 732.4 | 732.4 | K/Second |
| Average / Track Seek Time | 14.2 / 6.0 | 14.5 / 5.6 | ms |

The following programs below were tested under Windows 3.1, using Ver. 1.32B driver, 1024 x 768 x 256C resolution.

| WINDOWS TEST | DX2-66 | DX-33 | UNITS |
| --- | --- | --- | --- |
| SPEEDY | 27.14 | 18.23 | SPEEDMARK |
| WINTACH 1.2 | 33.77 | 23.82 | RPM |
| WINBENCH 3.11 | 21.95 | 16.13 | WINMARK |
| WINBENCH 4.0 | 7.58 | 5.58 | WINMARK |

| WINDOWS TEST | DX2-66 | DX-33 |
| --- | --- | --- |
| WINDSOCK 3.1 |  |  |
| CPU | 500 | 240 |
| Video | SU - 465 | GR - 282 |
| Disk | GR - 248 | GO - 196 |
| Memory | GO - 177 | GO - 173 |
| Windows (Overall) | GR - 385 | GR - 237 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good

IV.	POWER  SAVING  FUNCTION  TEST

Common  Configurations:

System  Memory	12MB Memory which consists of:

(NEC 424260-80)  	4MB x 2	72-pin single-sided

(TI TMS4C1024DJ-80) 	1MB x 4	30-pin

Cache  Memory	128KB Cache (Winbond W24257AK-20)

Hard  Disk	Conner CP30254 (254MB) Hard Disk

Floppy  Drives	One 1.44MB FDD  (Mitsubishi  MF335C-255MG)

One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

Serial  Mouse	Genius Mouse

Power  Supply	Seasonic SSA-200G

Testing  Voltage	104  volts

Power  Meter	Topward  Digital  Watt-ammeter  1301

Configuration  1: 	Ordinary CPU + Ordinary Monitor + Ordinary Power Supply 					+ All-in-one Cirrus Logic 5428 and UMC I/O Controller

CPU			Intel  80486DX2-66

IDE  &  VGA	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 89.4 | 89.4 | 82.9 | 82.9 |
| WINDOWS | 100.7 | 100.1 | 83.2 | 83.2 |

µ §

Configuration  2: 	CPU with SMM + Ordinary Monitor + Ordinary Power 							Supply + All-in-one Cirrus Logic 5428 and UMC I/O Card

CPU			Intel  P24S  (80486DX2-66 with SMM)

IDE  &  VGA	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 88.8 | 88.6 | 82.2 | 74.1 |
| WINDOWS | 99.9 | 98.8 | 82.2 | 74.5 |

µ §

Configuration  3: 	Ordinary CPU + Ordinary Monitor + Ordinary Power Supply 					+ Diamond Viper + Promise IDE

CPU			Intel  80486DX2-66

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	Diamond Viper VGA Card

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 94.8 | 94.8 | 86.7 | 86.7 |
| WINDOWS | 102.2 | 101.2 | 86.9 | 86.9 |

µ §

Configuration  4: 	CPU with SMM + Ordinary Monitor + Ordinary Power 							Supply 	+ Diamond Viper + Promise IDE

CPU			Intel  P24S  (80486DX2-66  with  SMM)

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	Diamond Viper VGA Card

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 93.4 | 92.6 | 87.1 | 80.2 |
| WINDOWS | 102.2 | 100.9 | 86.8 | 79.8 |

µ §

Configuration  5: 	Ordinary CPU + Ordinary Monitor + Ordinary Power Supply 					+ Cirrus Logic 5428 + Promise IDE

CPU			Intel  80486DX2-66

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	MS-3409 Cirrus Logic 5428 VGA Card

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 89.4 | 89.4 | 82.8 | 82.8 |
| WINDOWS | 101.1 | 100.1 | 83.1 | 83.1 |

µ §

Configuration  6: 	CPU with SMM + Ordinary Monitor + Ordinary Power 							Supply + Cirrus Logic 5428 + Promise IDE

CPU			Intel  P24S  (80486DX2-66  with  SMM)

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	MS-3409 Cirrus Logic 5428 VGA Card

Monitor		CTX  CMS-1461  (consumes  56  watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 91.8 | 91.8 | 82.4 | 75.1 |
| WINDOWS | 101.1 | 99.7 | 82.6 | 74.8 |

µ §

Configuration  7: 	Ordinary CPU + Green Monitor + Ordinary Power Supply + 					All-in-one Cirrus Logic 5428 and UMC IDE Controller

CPU			Intel  80486DX2-66

IDE  &  VGA	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 80.5 | 80.5 | 36.5 | 36.5 |
| WINDOWS | 89.4 | 88.4 | 37.2 | 37.2 |

µ §

Configuration  8:	CPU with SMM + Green Monitor + Ordinary Power Supply + 					All-in-one Cirrus Logic 5428 and UMC IDE Controller

CPU			Intel  P24S  (80486DX2-66 with SMM)

IDE  &  VGA	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 80.7 | 80.3 | 36.4 | 29.1 |
| WINDOWS | 89.3 | 88.1 | 36.9 | 29.1 |

µ §

Configuration  9: 	Ordinary CPU + Green Monitor + Ordinary Power Supply + 					Diamond Viper + Promise IDE

CPU			Intel  80486DX2-66

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	Diamond Viper VGA Card

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 84.3 | 84.1 | 40.8 | 40.8 |
| WINDOWS | 91.1 | 90.2 | 41.2 | 41.2 |

µ §

Configuration  10: 	CPU with SMM + Green Monitor + Ordinary Power Supply + 					Diamond Viper + Promise IDE

CPU			Intel  P24S  (80486DX2-66  with  SMM)

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	Diamond Viper VGA Card

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 84.4 | 83.8 | 40.7 | 33.8 |
| WINDOWS | 91.1 | 89.9 | 40.9 | 34.5 |

µ §

Configuration  11: 	Ordinary CPU + Green Monitor + Ordinary Power Supply + 					Cirrus Logic 5428 + Promise IDE

CPU			Intel  80486DX2-66

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	MS-3409 Cirrus Logic 5428 VGA Card

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 81.3 | 81.1 | 37.1 | 37.1 |
| WINDOWS | 89.9 | 89.1 | 37.3 | 37.3 |

µ §

Configuration  12: 	CPU with SMM+ Green Monitor + Ordinary Power Supply + 					Cirrus Logic 5428 + Promise IDE

CPU			Intel  P24S  (80486DX2-66  with  SMM)

I/O  Controller	MS-3802 Promise PDC20230B Super I/O Controller

VGA  Card	MS-3409 Cirrus Logic 5428 VGA Card

Monitor		HOB D-1560 Intelligent monitor (consumes 46 watts)

| MODES | ON | DOZE | STANDBY | SUSPEND |
| --- | --- | --- | --- | --- |
| DOS | 81.7 | 81.5 | 37.1 | 24.6 |
| WINDOWS | 90.1 | 88.8 | 37.3 | 29.5 |

µ §

V.		IMPORTANT  REMINDERS  FOR  THIS  VERSION

1.		In  this  version,  only  ordinary  CPU  (CPU  without  SMM  function)  and  Intel  			CPU  with  SMM  function  are  supported.  Cyrix  M6,  M7  and  AMD  DXL  are  		not  yet  supported.  	(This  is  already  corrected  in  the  next  version.)

2.		When  using  Intel  CPU  with  SMM  function,  use  only  the  one  with  the  mark  		"5V1X".

3.		When  running  multi-tasking  operating  systems  like  UNIX,  OS/2  and  Windows  		NT,  be  sure  to  use  CPU  with  SMM  function.

4.		When  using  CPU  with  SMM  function  and  Suspend  Mode  is  enabled,  be  sure  		to  install  APM  drivers  or  softwares.  	If  APM  softwares  or  drivers  is  not  				installed, when  the  system  wakes  up  after  	entering  suspend  mode,  the  DOS  			clock  will  not  be  updated.

Solution:	A.		When  the  system  wakes  up  after  entering  Suspend  Mode,  						turn  off  the  system  and  reboot.  The  DOS  clock  will  then  						get  its  data  anew  from  the  Real  Time  Clock.              OR

B.		Do  not  enter  Suspend  Mode.  You  can  disable  this  option  						in  the  BIOS  Setup.  This  way,  the  CPU  will  not  be  								stopped  and  thus,  the  DOS  clock  will  not  stop  									functioning.

5.		When  connecting  to  a  network  (as  a  server  or  as  a  workstation),  do  not  				enable  Suspend  Mode.

6.		If  possible,  do  not  use  the  Power  Saving  Switch  to  immediately  enter  					Suspend  Mode  when  the  system  is  running  important  tasks  like  diskcopy  or  			large  operating  systems  or  programs.

Note:		These  conditions  are  seen  at  the  time  of  testing.  It  may  not  be  true     			for  	all  versions.  Please  check  with  your  dealers  regularly  for  updates.