MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4132G  GREEN  VL-BUS  MAINBOARD  TEST  REPORT

Date:  May  9,  1994

MAINBOARD	MS-4132G

CHIPSET	SiS  471

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Testing  Configuration:

Processor		Intel P24S (80486DX2-66 S-series) CPU

System  Memory	16MB Memory (Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

Cache  Memory	256KB Cache (Winbond W24257AK-20)

VGA & I/O Controller	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Hard  Disk	Conner CP30254 (254MB) Hard Disk

Floppy  Drives	One 1.44MB FDD  (Mitsubishi  MF335C-255MG)

One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

Serial  Mouse	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Testing  Configuration:

Processor		Intel P24S (80486DX2-66 S-series) CPU

System  Memory	20MB Memory

(Micron MT4C16257DJ-7)  	4MB x 1	72-pin single-sided

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

Cache  Memory	256KB Cache (Winbond W24257AK-20)

VGA & I/O Controller	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

SCSI Controller	Adaptec AHA-1542C SCSI Controller

Tape Drive	Qtronix SDX-85 Tape Drive

Hard  Disk	Quantum ProDrive LPS 240AT (240MB) Hard Disk

Floppy  Drives	One 1.44MB FDD  (Mitsubishi  MF335C-255MG)

One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

Serial  Mouse	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| DOS MODE | OK |  |
| CP FILE | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Testing  Configuration:

Processor		Intel P24S (80486DX2-66 S-series) CPU

System  Memory	32MB Memory (NEC D424100GS-70L-9JD)  	32MB x 1	72-pin double-sided

Cache  Memory	256KB Cache (Winbond W24257AK-20)

VGA & I/O Controller	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

SCSI Controller	Adaptec AHA-1542C SCSI Controller

CD-ROM		NEC Multi-Spin 3XE

Hard  Disk	Conner CP3104 (104MB) Hard Disk

Floppy  Drives	One 1.44MB FDD  (Mitsubishi  MF335C-255MG)

One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

Serial  Mouse	Genius Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |

E.  NOVELL  NETWARE

Testing  Configuration:

Processor		Intel 80486DX2-66 CPU

System  Memory	32MB Memory (NEC D424100GS-70L-9JD)  	32MB x 1	72-pin double-sided

Cache  Memory	256KB Cache (Winbond W24257AK-20)

VGA & I/O Controller	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Hard  Disk	Conner CP3044 (40MB) Hard Disk

Floppy  Drives	One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| TOKEN RING | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| FILE COMPARE | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.		VIDEO  CARDS

| MANUFACTURER & MODEL NUMBER | CHIPSET | REMARKS |
| --- | --- | --- |
| ACUMOS | AVGA2 | ISA |
| DFI | CIRRUS LOGIC 5422 | ISA |
| IIT | AGX-014 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| OAK | OTI-087 | ISA |
| OAK | OTI-087X | ISA |
| S3 | S3801 | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA8900CL | ISA |
| TRIDENT | TVGA8900D | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | PARADISE | ISA |
| WD | WD90C30 | ISA |
| ATI | MACH 32 | VESA |
| DIAMOND SPEEDSTAR PRO | CIRRUS LOGIC 5426 | VESA |
| DIAMOND STEALTH PRO REV. 3.1 | S3928 | VESA |
| DIAMOND VIPER REV. B1 | WEITEK P9000 | VESA |
| LEADTEK | S3805 | VESA |
| MICRO-STAR MS-3405 | CIRRUS LOGIC 5426 | VESA |
| MICRO-STAR MS-3409 | CIRRUS LOGIC 5428 | VESA |
| PROLINK | S3805 | VESA |
| TSENG LABS | ET4000/W32 | VESA |

B.		I/O  CONTROLLERS

| MANUFACTURER & MODEL NUMBER | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| SMC | FDC37C65BLJP | ISA |
| UMC | UM82C863F | ISA |
| WINBOND | W83757AF | ISA |
| MICRO-STAR MS-3802 | PROMISE PDC20230B | VESA |
| SiS | SiS 83C611 | VESA |
| HORNET TECH. VL-200 | ADAPTEC AIC-25VL | VESA W/ CACHE |
| PROMISE DC4030VL-2 | PROMISE PDC20130A | VESA W/ CACHE |

C.		ALL-IN-ONE  (VGA  AND  I/O)  CONTROLLERS

| MANUFACTURER & MODEL NUMBER | CHIPSET | REMARKS |
| --- | --- | --- |
| GENOA M5 VELOCE | APPIAN ADI2 CIRRUS LOGIC 5426 | VESA |
| MICROSTAR MS-3408 | UMC UM82C863F/65F CIRRUS LOGIC 5422 | VESA |

D.		SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| BUSTEK | BT-542B | ISA |
| ULTRASTOR | U14F | ISA |
| ULTRASTOR | U34F | VESA |

E.		NETWORK  ADAPTERS

| TYPE | CHIPSET | REMARKS |
| --- | --- | --- |
| ARCNET | SMC COM90C65LJ | ISA |
| 3COM | 3C503 | ISA |
| 3COM | 3C509 | ISA |
| IBM | TOKEN RING | ISA |
| NE-2000 |  | ISA |
| NE-2100 |  | ISA |

F.		HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| QUANTUM | ProDrive LPS 240AT | 240 MB | IDE |
| QUANTUM | ProDrive LPS 540AT | 540 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| CONNER | CP3100 | 100 MB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |

G.		FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

H.		MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| PANASONIC | MN41C1000SJ-08 | 30-pin | 1 MB |
| TOSHIBA | TC511000AJ-80 | 30-pin | 1 MB |
| TOSHIBA | TC5141000ASJL-70 | 30-pin | 4 MB |
| PANASONIC | MN41C4000SJ-08 | 30-pin | 4 MB |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

I.		STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| MICRON TECH | MT5C2568-20 | 32K * 8 |
| NKK | N341256P-20 | 32K * 8 |
| PARADIGM | PDM41256SA20P | 32K * 8 |
| WINBOND | W24257AK-20 | 32K * 8 |

J.		MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE | SCSI |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |

III.	PERFORMANCE  TEST

Testing  Configuration:

System  Memory	8MB Memory (MS-4909 Samsung KM416C1200J-7)

Cache  Memory	256KB Cache (NKK N341256P-20)

VGA & I/O Controller	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

Hard  Disk	Conner CP30254 (240MB) Hard Disk

Floppy  Drives	One 1.44MB FDD  (Mitsubishi  MF335C-255MG)

One 1.2MB FDD  (Mitsubishi  MF504C-315MG)

Serial  Mouse	Genius Mouse

|  | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| BYTE 1.3 |  |  |  |  |  |
| CPU Index | 11.30 | 10.88 | 9.95 | 7.91 |  |
| FPU Index | 59.33 | 44.34 | 33.92 | 29.39 |  |
| Disk Index | 25.44 | 21.41 | 24.64 | 19.37 |  |
| Video Index | 12.99 | 12.20 | 9.84 | 8.73 |  |
| BYTE 2.2 |  |  |  |  |  |
| CPU Index | 1.51 | 1.57 | 1.43 | 1.16 |  |
| FPU Index | 4.61 | 3.46 | 2.96 | 2.31 |  |
| Disk Index | 1.16 | 1.15 | 1.14 | 1.15 |  |
| Video Index | 3.84 | 3.17 | 2.35 | 2.21 |  |
| PC LABS 7.0 |  |  |  |  |  |
| DOSMark | 24.04 | 23.77 | 22.31 | 21.66 |  |
| Processor Harmonic | 14965.64 | 13144.28 | 9740.87 | 8943.22 |  |
| Video Harmonic | 8782.844 | 9649.99 | 9414.69 | 8757.73 |  |
| Memory Harmonic | 9803.44 | 10650.76 | 9866.63 | 8150.30 |  |
| Disk Harmonic | 18.34 | 18.30 | 17.95 | 17.90 |  |
| NORTON SI 4.5 |  |  |  |  |  |
| Computing Index | 110.3 | 78.6 | 88.1 | 51.5 |  |
| Disk Index | 4.3 | 4.3 | 4.3 | 4.3 |  |
| Performance Index | 74.9 | 53.8 | 60.1 | 35.7 |  |
| NORTON SI 7.0 |  |  |  |  |  |
| Computing Index | 132.0 | 108.0 | 67.9 | 72.0 |  |
| Disk Index | 7.1 | 7.1 | 7.1 | 7.1 |  |
| Performance Index | 90.3 | 74.1 | 47.4 | 50.3 |  |
| LANDMARK 1.14 |  |  |  |  |  |
| CPU Speed | 200+ | 200+ | 121.4 | 151.3 | MHz |
| LANDMARK 2.0 |  |  |  |  |  |
| CPU Speed | 223.01 | 167.26 | 131.94 | 111.50 | MHz |
| FPU Speed | 545.51 | 408.82 | 355.44 | 272.81 | MHz |
| Video Speed | 10802 | 12766 | 12136 | 10802 | CHR/MS |

| POWER METER VER 1.7 | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| SYSTEM TEST |  |  |  |  |  |
| Agg. Hardware Test | 1810.3 | 1485.5 | 1311.9 | 1022.7 | PMUs |
| Spreadsheet Simulation | 4399.0 | 4125.4 | 3036.0 | 3823.0 | PMUs |
| Word Processing Sim. | 4769.8 | 6015.8 | 3180.4 | 3751.5 | PMUs |
| Database App'n. Sim. | 2885.7 | 3068.9 | 2147.1 | 2165.9 | PMUs |
| CAE/CAD Simulation | 45583.2 | 35290.6 | 30844.4 | 23685.5 | PMUs |
| Throughput Test | 14278.7 | 11094.1 | 10110.4 | 8320.6 | PMUs |
| CPU / MEMORY TEST |  |  |  |  |  |
| Agg. CPU/Memory | 26712.0 | 19989.0 | 17554.0 | 13256.0 | PMUs |
| Clock Speed Test | 65.0 | 49.0 | 39.0 | 33.0 | MHertz |
| MIPS Test | 25.8 | 21.8 | 15.8 | 14.8 | MIPS |
| Dhrystone Test | 33.1 | 27.9 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 6531.5 | 4898.6 | 4481.2 | 3265.7 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE TEST |  |  |  |  |  |
| Agg. Disk Test | 185.2 | 189.3 | 188.8 | 184.1 | PMUs |
| Trk-to-Trk Seek Test | 3.9 | 3.9 | 3.9 | 3.9 | MS/Seek |
| Mean Seek Test | 10.0 | 10.1 | 10.1 | 10.0 | MS/Seek |
| Data Transfer Rate | 986.4 | 1000.5 | 1006.0 | 974.1 | Kb/Sec. |
| File Open/Close Test | 6.3 | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq. File Rd/Wrt Test | 0.2 | 0.1 | 0.2 | 0.2 | MS/Op. |
| Ran. File Rd/Wrt Test | 15.6 | 13.2 | 13.3 | 13.5 | MS/Op. |
| VIDEO DISPLAY TEST |  |  |  |  |  |
| Agg. Video Test | 5936.5 | 6945.0 | 6613.1 | 5500.8 | PMUs Mode 3 |
| Character Write Test | 0.6 | 0.5 | 0.5 | 0.7 | Sec. Mode 3 |
| Line Drawing Test | 3.1 | 2.7 | 3.4 | 3.6 | Sec. Mode 18 |
| Fill Test | 13.9 | 11.8 | 14.8 | 15.5 | Sec. Mode 18 |

| QA PLUS 4.61 | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| SYSTEM PERFORMANCE |  |  |  |  |  |
| CPU Speed | 45517 | 45517 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 757390 | 473369 | 473369 | 284021 | CPS |
| Math Speed | 10525.5 | 8525.2 | 7372.6 | 5683.5 | K-Whetstones |
| DISK PERFORMANCE |  |  |  |  |  |
| Trk-to-Trk Seek | 4.56 | 4.57 | 4.5 | 4.65 | ms |
| Average Seek | 11.1 | 11.1 | 11.1 | 11.3 | ms |
| Transfer Rate | 356.2 | 390.1 | 309.1 | 240.9 | K/sec |
| POWER METER VER 1.81 | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| SYSTEM TEST |  |  |  |  |  |
| Agg. Hardware Test | 1782.1 | 1452.3 | 1271.2 | 1000.9 | PMUs |
| Spreadsheet Simulation | 4815.3 | 4456.1 | 3060.1 | 3376.5 | PMUs |
| Word Processing Sim. | 4191.6 | 4688.6 | 3282.5 | 3818.8 | PMUs |
| Database App'n. Sim. | 2865.9 | 3024.9 | 2355.9 | 2307.1 | PMUs |
| CAE/CAD Simulation | 45130.3 | 35051.6 | 31395.3 | 23841.5 | PMUs |
| Throughput Test | 14060.8 | 11539.1 | 9051.5 | 8211.6 | PMUs |
| CPU / MEMORY TEST |  |  |  |  |  |
| Agg. CPU/Memory | 25698.0 | 19433.7 | 16854.7 | 12889.7 | PMUs |
| Clock Speed Test | 66.0 | 50.0 | 39.1 | 33.0 | MHertz |
| MIPS Test | 19.3 | 16.0 | 11.6 | 10.8 | MIPS |
| Dhrystone Test | 34.0 | 28.1 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 6531.5 | 4898.6 | 4461.2 | 3265.7 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE TEST |  |  |  |  |  |
| Agg. Disk Test | 184.1 | 188.8 | 188.8 | 185.8 | PMUs |
| Trk-to-Trk Seek Test | 3.9 | 3.9 | 3.9 | 3.7 | MS/Seek |
| Mean Seek Test | 10.0 | 10.1 | 10.1 | 10.0 | MS/Seek |
| Data Transfer Rate | 990.3 | 1012.1 | 1012.1 | 979.8 | Kb/Sec. |
| File Open/Close Test | 6.2 | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq. File Rd/Wrt Test | 0.4 | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran. File Rd/Wrt Test | 15.6 | 13.4 | 12.8 | 13.4 | MS/Op. |
| VIDEO DISPLAY TEST |  |  |  |  |  |
| Agg. Video Test | 5936.5 | 6945.0 | 6613.1 | 5500.8 | PMUs Mode 3 |
| Character Write Test | 0.6 | 0.5 | 0.5 | 0.7 | Sec. Mode 3 |
| Line Drawing Test | 3.2 | 2.8 | 3.5 | 3.7 | Sec. Mode 18 |
| Fill Test | 14.0 | 12.0 | 14.9 | 15.6 | Sec. Mode 18 |

| QA PLUS 4.70 | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| SYSTEM PERFORMANCE |  |  |  |  |  |
| CPU Speed | 45.5 | 45.5 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 757.4 | 473.4 | 473.4 | 473.4 | K-CPS |
| Math Speed | 11.1 | 8.6 | 7.1 | 5.7 | M-Whetstones |
| DISK PERFORMANCE |  |  |  |  |  |
| Trk-to-Trk Seek | 4.56 | 4.55 | 4.5 | 4.65 | ms |
| Average Seek | 11.2 | 11.1 | 11.1 | 11.2 | ms |
| Transfer Rate | 321.3 | 606.8 | 292.6 | 264.3 | K/sec |

| CHECK IT 3.0 | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| MAIN SYSTEM |  |  |  |  |  |
| CPU Speed | 33609 | 23807 | 18415 | 16170 | Dhrystones |
| Math Speed | 11419.9 | 8378.2 | 7379.2 | 5741.3 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |  |
| BIOS Video Speed | 18793 | 20624 | 18061 | 15858 | Char/S |
| Direct Video Speed | 280059 | 264038 | 243530 | 210846 | Char/S |
| HARD DISK |  |  |  |  |  |
| Transfer Speed | 554.3 | 554.3 | 554.3 | 554.3 | K/Second |
| Average Seek Time | 13.3 | 13.3 | 13.3 | 13.3 | ms |
| Track Seek Time | 4.7 | 4.7 | 4.5 | 5.1 | ms |

The following programs were tested under Windows 3.1, using Cirrus Logic Ver 1.32B Windows drivers, 1024 x 768 x 256C resolution.

|  | Intel DX2-66 | Intel DX-50 | Cyrix M7 DX-40 | Intel DX-33 | Units |
| --- | --- | --- | --- | --- | --- |
| SPEEDY | 27.21 | 25.92 | 20.67 | 19.34 | SPEEDMARK |
| WINTACH 1.2 | 34.38 | 33.19 | 26.00 | 24.60 | RPM |
| WINBENCH 3.11 | 22.18 | 21.29 | 17.62 | 16.56 | WINMARK |
| WINBENCH 4.0 | 7.83 | 7.73 | 6.23 | 5.63 | WINMARK |
| WINDSOCK 3.1 |  |  |  |  |  |
| CPU | 492 | 374 | 361 | 240 |  |
| Video | SU ® 487 | GR ® 409 | GR ® 299 | GR ® 288 |  |
| Disk | GR ® 262 | GR ® 260 | GR ® 212 | GO ® 194 |  |
| Memory | GO ® 119 | GO ® 142 | GO ® 140 | GO ® 112 |  |
| Windows (Overall) | GR ® 377 | GR ® 313 | GR ® 276 | GR ® 222 |  |

NOTE:

SU = Superb

GR = Great

GO = Good

IV.		IMPORTANT  REMINDERS  FOR  THIS  VERSION

1.		All  CPUs  (ordinary  CPU  and  CPU  with  SMM  function)  are  supported.  				However,  when  using  Intel  non-SMM,  Cyrix  M7  and  AMD  DXL,  only  the  			STANDBY  mode  is  supported.

2.		When  using  Intel  CPU  with  SMM  function,  use  only  the  one  with  the  mark  		"5V1X".

3.		When  using  CPU  with  SMM  function  and  Suspend  Mode  is  enabled,  be  sure  		to  install  APM  drivers  or  softwares.  	If  APM  softwares  or  drivers  is  not  				installed, when  the  system  wakes  up  after  	entering  suspend  mode,  the  DOS  			clock  will  not  be  updated.

Solution:	A.		When  the  system  wakes  up  after  entering  Suspend  Mode,  						turn  off  the  system  and  reboot.  The  DOS  clock  will  then  						get  its  data  anew  from  the  Real  Time  Clock.              OR

B.		Do  not  enter  Suspend  Mode.  You  can  disable  this  option  						in  the  BIOS  Setup.  This  way,  the  CPU  will  not  be  								stopped  and  thus,  the  DOS  clock  will  not  stop  									functioning.

4.		When  connecting  to  a  network  (as  a  server  or  as  a  workstation),  do  not  				enable  Suspend  Mode.

5.		If  possible,  do  not  use  the  Power  Saving  Switch  to  immediately  enter  					Suspend  Mode  when  the  system  is  running  important  tasks  like  diskcopy  or  			large  operating  systems  or  programs.

Note:		These  conditions  are  seen  at  the  time  of  testing.  It  may  not  be  true     			for  	all  versions.  Please  check  with  your  dealers  regularly  for  updates.