MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4134  PCI / VL / ISA  GREEN  MAINBOARD

TEST  REPORT

Date:  June  27,  1994

MAINBOARD	MS-4134

CHIPSET	ALI M1429G + M1435

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel 486DX2-66 CPU

·	40MB Memory which consists of:

(Panasonic MN41C4000SJ-08) 	32MB x 1	72-pin

(Panasonic MN41C4000SJ-08) 	4MB x 4	72-pin

(Panasonic MN41C4000SJ-08)	1MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-15)

·	MS-4803 ALI PCI IDE Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	Winbond Multi-I/O Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel P24S (80486DX2-66) CPU

·	28MB Memory which consists of:

(Micron MT4C4001JDJ-7)  	8MB x 1	72-pin double-sided

(Micron MT4C16257DJ-7)  	4MB x 1	72-pin single-sided

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Qtronix SDX-85 Tape Drive

·	Quantum 240AT (240MB) IDE Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| DOS MODE | OK |  |
| CP FILE | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel P24S (80486DX2-66) CPU

·	32MB Memory which consists of:

(Panasonic MN41C4000SJ-08) 	32MB x 1	72-pin

·	256KB Cache (NKK N341256D-20)

·	MS-3805 NCR PCI SCSI Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	UMC Multi-I/O Controller

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel 80486DX2-66 CPU

·	28MB Memory which consists of:

(Micron MT4C4001JDJ-7)  	8MB x 1	72-pin double-sided

(Micron MT4C16257DJ-7)  	4MB x 1	72-pin single-sided

(Panasonic MN41C4000SJ-08) 	4MB x 4	30-pin

·	256KB Cache (Winbond W24257AK-20)

·	Ultrastor U14F ISA SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Quantum 240S (240MB) SCSI Hard Disk

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

* Using Intel P24S (80486DX2-66) CPU will show the CPU type as Pentium CPU.

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| ACUMOS | AVGA2 | ISA |
| IIT | AGX-014 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| S3 | S3801 | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA8900CL | ISA |
| TRIDENT | TVGA8900D | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | PARADISE | ISA |
| WD | WD90C30 | ISA |
| ATI | MACH 32 | VESA |
| CHAINTECH | ALI ALG2228 | VESA |
| DIAMOND STEALTH PRO REV. 3.1 | S3928 | VESA |
| DIAMOND VIPER REV. B1 | WEITEK P9000 | VESA |
| MICRO-STAR MS-3405 | CIRRUS LOGIC 5426 | VESA |
| MICRO-STAR MS-3409 | CIRRUS LOGIC 5428 | VESA |
| S3 | S3805 | VESA |
| TSENG LABS | ET4000/W32 | VESA |
| 2 THE MAX | ET4000/W32P | PCI |
| ATI MACH 32 | MACH 32 | PCI |
| ATI MACH 64 | MACH 64 | PCI |
| CARDEX | ET4000/W32P | PCI |
| DIAMOND SPEEDSTAR 64 | CIRRUS LOGIC 5434 | PCI |
| DIAMOND STEALTH 32 | ET4000/W32P | PCI |
| DIAMOND VIPER PCI | WEITEK P9000/P9001 | PCI |
| ELSA WINNER 1000PRO | S3 864 | PCI |
| GENOA PHANTOM 32I | ET4000/W32P | PCI |
| INTEL SPEA V7 MERCURY LITE | S3 928 | PCI |
| LEADTEK | S3 805 | PCI |
| MATROX MGA | MATROX MGA | PCI |
| MICRO-STAR MS-4401 | ALI ALG2201 | PCI |
| MICRO-STAR MS-4402 | ALI ALG2301 | PCI |
| MICRO-STAR MS-4403 | IIT AGX-016 | PCI |
| ORCHID KELVIN 64 | CIRRUS LOGIC 5434 | PCI |
| PROLINK | S3 928 | PCI |

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| MICRO-STAR MS-3806 | SMC FDC37C665 (666) | ISA |
| MICRO-STAR MS-3807 | WINBOND W83757 / 58 | ISA |
| UMC | UM82C863F | ISA |
| MICRO-STAR MS-3802 | PROMISE PDC20230B | VESA |
| PROMISE DC4030VL-2 | PROMISE PDC20130A | VESA W/ CACHE |
| CMD |  | PCI |
| MICRO-STAR MS-4803 | ALI M5215 | PCI |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ULTRASTOR | U14F | ISA |
| ULTRASTOR | U34F | VESA |
| ADAPTEC | AHA-2940 | PCI |
| MICRO-STAR MS-3805 | NCR | PCI |

D.	NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C503 | ISA |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |
| DEC |  | 21040 | PCI |

E.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| CONNER | CP3100 | 100 MB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| PANASONIC | MN41C1000SJ-08 | 30-pin | 1 MB |
| TOSHIBA | TC511000AJ-80 | 30-pin | 1 MB |
| TOSHIBA | TC5141000ASJL-70 | 30-pin | 4 MB |
| PANASONIC | MN41C4000SJ-08 | 30-pin | 4 MB |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| CYPRESS | CY7C199-20 | 32K * 8 |
| MICRON TECH | MT5C2568-20 | 32K * 8 |
| TOSHIBA | TC55328P-20 | 32K * 8 |
| WINBOND | W24257AK-20 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| AUDIO CARD | OAK | MOZART |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-4134 Ver 1.1

System BIOS	:	AMI AL53

System Chipset	:	ALI M1429G + M1435

VGA Card		:	MS-4402 Avance Logic ALG2301A PCI VGA Card

IDE Controller		:	MS-4803 ALI PCI IDE Controller

I/O Controller		:	MS-3806 SMC Super I/O Controller

System Memory		:	8 MB (MS-4904 Micron Tech MT4C4001JDJ-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive		:	Conner CP30254 HDD (240 MB)

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |
| --- | --- | --- | --- | --- |
|  | DX2-66 | DX2-50 | DX-40 | DX-33 |
| BYTE BENCHMARK 1.3 |  |  |  |  |
| CPU Index | 11.99 | 9.63 | 8.27 | 8.42 |
| FPU Index | 62.36 | 42.62 | 33.83 | 29.43 |
| Disk Index | 28.82 | 27.66 | 19.55 | 20.07 |
| Video Index | 12.47 | 8.96 | 7.48 | 8.79 |
| BYTE BENCHMARK 2.2 |  |  |  |  |
| CPU Index | 1.69 | 1.31 | 1.18 | 1.24 |
| FPU Index | 4.59 | 3.68 | 2.95 | 2.30 |
| Disk Index | 1.17 | 1.27 | 1.27 | 1.34 |
| Video Index | 3.25 | 2.31 | 1.93 | 1.99 |
| PC BENCHMARK 7.0 |  |  |  |  |
| DOSMark | 27.70 | 32.40 | 25.00 | 24.71 |
| Processor Harmonic | 14934.01 | 10006.03 | 8800.72 | 8803.92 |
| Video Harmonic | 5904.10 | 4606.31 | 3484.62 | 5883.66 |
| Memory Harmonic | 13327.79 | 10909.68 | 9313.20 | 10182.34 |
| Disk Harmonic | 21.34 | 29.49 | 21.63 | 21.02 |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |
| --- | --- | --- | --- | --- |
|  | DX2-66 | DX2-50 | DX-40 | DX-33 |
| NORTON SI 4.5 |  |  |  |  |
| Computing Index | 104.2 | 115.7 | 92.6 | 51.5 |
| Disk Index | 7.3 | 6.9 | 6.5 | 7.2 |
| Performance Index | 71.9 | 79.4 | 63.9 | 36.7 |
| NORTON SI 7.0 |  |  |  |  |
| Computing Index | 136.9 | 84.7 | 67.9 | 70.3 |
| Disk Index | 12.6 | 9.7 | 8.8 | 12.4 |
| Performance Index | 95.2 | 59.5 | 48.1 | 50.9 |
| LANDMARK SPEED 1.14 |  |  |  |  |
| CPU Speed | 200+ | 151.9 | 121.4 | 147.1 |
| LANDMARK SPEED 2.0 |  |  |  |  |
| CPU Speed | 218.98 | 164.91 | 131.92 | 110.99 |
| FPU Speed | 536.96 | 443.89 | 355.25 | 271.00 |
| Video Speed | 7801 | 5886 | 4165 | 7801 |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| MAIN SYSTEM |  |  |  |  |  |
| CPU Speed | 33609 | 22554 | 17918 | 15785 | Dhrystones |
| Math Speed | 11281.0 | 8039.9 | 6759.0 | 5690.1 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |  |
| BIOS Video Speed | 34636 | 25566 | 22088 | 26276 | Char/S |
| Direct Video Speed | 330688 | 277496 | 227508 | 208923 | Char/S |
| HARD DISK |  |  |  |  |  |
| Transfer Speed | 1139.3 | 1139.3 | 831.4 | 1139.3 | K/Second |
| Average Seek Time | 13.3 | 13.6 | 13.3 | 13.6 | ms |
| Track Seek Time | 4.7 | 5.3 | 4.8 | 5.3 | ms |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |  |
| Agg. Hardware | 1852.4 | 1519.6 | 1229.0 | 1049.9 | PMUs |
| Spreadsheet | 4219.6 | 4254.9 | 3488.8 | 1792.5 | PMUs |
| Word Processing | 4810.2 | 3801.3 | 3802.2 | 4327.8 | PMUs |
| Database App'n. | 3324.0 | 2518.9 | 2244.2 | 2848.4 | PMUs |
| CAE / CAD | 45106.5 | 38105.3 | 31064.3 | 24168.2 | PMUs |
| Throughput Test | 14334.1 | 11955.1 | 10232.2 | 7857.4 | PMUs |
| CPU MEMORY |  |  |  |  |  |
| Agg. CPU/Memory | 26139.7 | 21642.3 | 17310.3 | 13226.7 | PMUs |
| Clock Speed Test | 65.5 | 48.0 | 39.0 | 32.5 | MHertz |
| MIPS Test | 28.2 | 19.5 | 15.7 | 14.6 | MIPS |
| Dhrystone Test | 36.2 | 24.9 | 20.1 | 18.7 | K-Dhrystones/S |
| Whetstone Test | 6489.0 | 5521.1 | 4441.4 | 3255.1 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.2 | 0.2 | Seconds |
| DISK DRIVE |  |  |  |  |  |
| Agg. Disk Test | 401.4 | 368.5 | 315.8 | 403.3 | PMUs |
| Trk-to-Trk Seek | 3.8 | 3.9 | 3.8 | 3.7 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | 10.1 | MS/Seek |
| Data Transfer Rate | 2464.9 | 2192.8 | 1858.8 | 2372.5 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 12.3 | 12.7 | 13.3 | 13.1 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |  |
| Agg Video Test | 6686.9 | 5154.1 | 4441.4 | 6181.3 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.8 | 0.9 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 1.8 | 2.5 | 3.0 | 2.1 | sec. Md 18 |
| Fill Test | 7.5 | 11.1 | 12.7 | 9.0 | Sec. Md 18 |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |  |
| Agg. Hardware | 1822.6 | 1456.7 | 1188.5 | 1027.6 | PMUs |
| Spreadsheet | 4192.1 | 3247.0 | 3284.0 | 1747.7 | PMUs |
| Word Processing | 6657.8 | 2837.7 | 4208.2 | 4416.8 | PMUs |
| Database App'n. | 4079.1 | 2027.2 | 2118.2 | 2773.4 | PMUs |
| CAE / CAD | 45029.8 | 37627.3 | 31556.8 | 24269.5 | PMUs |
| Throughput Test | 14749.2 | 11707.3 | 10626.0 | 8462.8 | PMUs |
| CPU MEMORY |  |  |  |  |  |
| Agg. CPU/Memory | 25640.3 | 20930.3 | 16632.0 | 12860.3 | PMUs |
| Clock Speed Test | 66.0 | 47.9 | 39.2 | 33.0 | MHertz |
| MIPS Test | 20.0 | 14.1 | 11.4 | 10.7 | MIPS |
| Dhrystone Test | 35.2 | 24.9 | 20.1 | 18.7 | K-Dhrystones/S |
| Whetstone Test | 6447.2 | 5490.7 | 4402.3 | 3244.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.2 | 0.2 | Seconds |
| DISK DRIVE |  |  |  |  |  |
| Agg. Disk Test | 415.6 | 366.1 | 318.5 | 397.3 | PMUs |
| Trk-to-Trk Seek | 3.8 | 3.9 | 3.8 | 3.8 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | 10.2 | MS/Seek |
| Data Transfer Rate | 2507.6 | 2211.9 | 1883.0 | 2400.6 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 13.3 | 13.3 | 16.0 | 13.0 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |  |
| Agg Video Test | 6686.9 | 5154.1 | 4441.4 | 6181.3 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.8 | 0.9 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 1.8 | 2.5 | 3.0 | 2.1 | sec. Md 18 |
| Fill Test | 7.6 | 11.1 | 12.9 | 9.0 | Sec. Md 18 |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |  |
| CPU Speed | 45517 | 22758 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 757390 | 473369 | 473369 | 284021 | CPS |
| Math Speed | 11211.1 | 9133.9 | 7340.0 | 5683.5 | K-Whetstones |
| DISK PERF. |  |  |  |  |  |
| Trk-Trk Seek | 4.45 | 4.58 | 4.56 | 4.72 | ms |
| Average Seek | 11.1 | 11.1 | 11.1 | 11.4 | ms |
| Transfer Rate | 862.3 | 1489.5 | 1365.3 | 963.8 | K/sec |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |  |
| CPU Speed | 45.5 | 22.8 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 757.4 | 473.4 | 473.4 | 284.0 | K-CPS |
| Math Speed | 10.6 | 8.9 | 7.1 | 5.4 | M-Whetstones |
| DISK PERF. |  |  |  |  |  |
| Trk-Trk Seek | 4.47 | 4.58 | 45.7 | 4.57 | ms |
| Average Seek | 11.1 | 11.1 | 11.1 | 11.2 | ms |
| Transfer Rate | 862.3 | 682.7 | 1365.3 | 1365.3 | K/sec |

The following programs below were tested under Windows 3.1, using Avance Logic ALG2301A Ver. 1.2 Windows driver, 1024 x 768 x 256C resolution.

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |  |
| --- | --- | --- | --- | --- | --- |
| WINDOWS TEST | DX2-66 | DX2-50 | DX-40 | DX-33 | UNITS |
| SPEEDY | 61.72 | 46.21 | 36.68 | 44.22 | SPEEDMARK |
| WINTACH 1.2 | 43.20 | 31.02 | 27.76 | 30.15 | RPM |
| WINBENCH 3.11 | 20.55 | 16.76 | 13.98 | 15.39 | WINMARK |
| WINBENCH 4.0 | 9.98 | 7.43 | 6.56 | 7.09 | WINMARK |

|  | INTEL/AMD | CYRIX M7 | CYRIX M7 | INTEL |
| --- | --- | --- | --- | --- |
| WINDSOCK | DX2-66 | DX2-50 | DX-40 | DX-33 |
| CPU | 471 | 445 | 310 | 244 |
| VIDEO | SU ¾ 837 | SU ¾ 619 | SU ¾ 507 | SU ¾ 606 |
| DISK | GR ¾ 304 | GR ¾ 221 | GR ¾ 208 | GR ¾ 226 |
| MEMORY | GO ¾ 177 | GO ¾ 130 | GO ¾ 112 | GO ¾ 176 |
| WINDOWS (OVERALL) | GR ¾ 449 | GR ¾ 377 | GR ¾ 292 | GR ¾ 296 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good