MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  May 5, 1994

MS-4134  PCI / VL / ISA  MAINBOARD

SUPPLEMENTS  (Rel. 1)

MAINBOARD	MS-4134

CHIPSET        	ALI  M1429G + M1435

Please find below the proposed jumper and BIOS settings for different configurations.  Use default values and settings for the other jumpers and BIOS not specified.

MS-3805  is  the  NCR  PCI  SCSI  Controller.

MS-4803  is  the  ALI  PCI  IDE  Controller.

MS-4402  is  the  Avance  Logic  PCI  VGA  Card.

CASE I:		MS-4134 + MS-3805

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  2-3.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  Phoenix  BIOS:

NCR  SCSI  PCI  Device:

Enable  Device								:	Enabled

Enable  Master								:	Enabled

Use  Default  Latency  Timer  Value	:	Yes

Latency  Timer  Value  					:	0040

IRQ  Line										:	IRQ9

CASE II:		MS-4134 + MS-4803

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP4  to  ON.

Set  JP12  to  2-3.

2.		For  MS-4803:

Set  JPIDE1,  short  INTA  2-3.

Install  MS-4803  in  PCI  Slot  2.

3.		For  Phoenix  BIOS:

PCI  Device, Slot # 2:

Enable  Device								:	Enabled

Enable  Master								:	Disabled

Use  Default  Latency  Timer  Value	:	Yes

Latency  Timer  Value  					:	0040

IRQ  Line										:	None

CASE III:		MS-4134 + MS-3805 + MS-4803

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  2-3.

Set  JP4  to  ON.

Set  JP12  to  2-3.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  MS-4803:

Set  JPIDE1,  short  INTA  2-3.

Install  MS-4803  in  PCI  Slot  2.

4.		For  Phoenix  BIOS:

NCR  SCSI  PCI  Device:

Enable  Device								:	Enabled

Enable  Master								:	Enabled

Use  Default  Latency  Timer  Value	:	Yes

Latency  Timer  Value  					:	0040

IRQ  Line										:	IRQ9

PCI  Device, Slot # 2:

Enable  Device								:	Enabled

Enable  Master								:	Disabled

Use  Default  Latency  Timer  Value	:	Yes

Latency  Timer  Value  					:	0040

IRQ  Line										:	None

F	Note:	 	When  using  MS-4402  in  MS-4134  and  you  want  to  use  the  power  						saving  mode,  set  the  "VGA  with  Power  Down  Feature"  to  								"Standard"  or  "Cirrus  5426".  Do  not  use  "VESA  DPMS".