MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  August 15, 1994

MS-4134  PCI / VL / ISA  MAINBOARD

SUPPLEMENTS  (Rel. 2)

MAINBOARD	MS-4134

CHIPSET        	ALI  M1429G + M1435

Please find below the proposed jumper and BIOS settings for different configurations.  Use default values and settings for the other jumpers and BIOS not specified.

MS-3805  is  the  NCR  PCI  SCSI  Controller.

MS-4803  is  the  ALI  PCI  IDE  Controller.

CASE I:		MS-4134 + MS-3805

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  2-3.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

PCI  Slot  1  INTA#'s  IRQ  Sel				:	IRQ11

On  Board  SCSI  IRQ  Select					:	IRQ11

4.		For  AMI  BIOS (Ver. AL54):

On  Board  SCSI  IRQ  Select					:	IRQ11

CASE II:		MS-4134 + MS-4803

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP4  to  ON.

Set  JP12  to  2-3.

2.		For  MS-4803:

Set  JPIDE1,  short  INTA  2-3.

Install  MS-4803  in  PCI  Slot  2.

3.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

Match  PCI  Slot  2's  IRQ  Select				:	None

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

4.		For  AMI  BIOS (Ver. AL54):

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

CASE III:		MS-4134 + MS-3805 + MS-4803

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  2-3.

Set  JP4  to  ON.

Set  JP12  to  2-3.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  1.

3.		For  MS-4803:

Set  JPIDE1,  short  INTA  2-3.

Install  MS-4803  in  PCI  Slot  2.

4.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

PCI  Slot  1  INTA#'s  IRQ  Sel				:	IRQ11

On  Board  SCSI  IRQ  Select					:	IRQ11

Match  PCI  Slot  2's  IRQ  Select				:	None

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

5.		For  AMI  BIOS (Ver. AL54):

On  Board  SCSI  IRQ  Select					:	IRQ11

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

CASE IV:		MS-4134 + Other  PCI  SCSI  Controller

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  1-2.

2.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

Match  PCI  Slot  1's  IRQ  Select				:	IRQ11

PCI  Slot  1  INTA#'s  IRQ  Sel				:	IRQ11

3.		For  AMI  BIOS (Ver. AL54):

PCI  Slot  1  INTA#'s  IRQ  Sel				:	IRQ11

CASE V:		MS-4134 + Other  PCI  IDE  Controller

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP4  to  ON.

Set  JP12  to  2-3.

2.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

Match  PCI  Slot  2's  IRQ  Select				:	None

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

3.		For  AMI  BIOS (Ver. AL54):

PCI  Slot  2  INTA#'s  IRQ  Sel				:	None

CASE VI:		MS-4134 + AMD PCI Network Card

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

Match  PCI  Slot  3's  IRQ  Select				:	IRQ9

PCI  Slot  3  INTA#'s  IRQ  Sel				:	IRQ9

2.		For  AMI  BIOS (Ver. AL54):

PCI  Slot  3  INTA#'s  IRQ  Sel				:	IRQ9

3.		For  AMD  Network  Card:

Install  in  PCI  Slot  3.

For  the  AMD  Network  Driver  to  install  correctly:

A.		Create a NET.CFG file containing the following:

link driver pcntnw

frame ethernet_802.3

bustype pci1

B.		Put the NET.CFG file in the directory which contains the files:  lsl.com, 							pcntnw.com, ipxodi.com and netx.com.

CASE VII:	MS-4134 + 2  MS-3805

For MS-4134 Version 0B, 1.0 and 1.1 Mainboards:

1.		For  MS-4134:

Set  JP6  to  2-3.

Set  JP12  to  open.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  1st  MS-3805  in  PCI  Slot  1.

Install  2nd  MS-3805  in  PCI  Slot  2.

3.		For  AMI  BIOS (Ver. AL51, AL52, AL53):

PCI  Slot  1  INTA#'s  IRQ  Sel				:	IRQ11

On  Board  SCSI  IRQ  Select					:	IRQ11

Match  PCI  Slot  2's  IRQ  Select				:	IRQ9

PCI  Slot  2  INTA#'s  IRQ  Sel				:	IRQ9

4.		For  AMI  BIOS (Ver. AL54):

PCI  Slot  2  INTA#'s  IRQ  Sel				:	IRQ9

On  Board  SCSI  IRQ  Select					:	IRQ11

Install the DOSCAM and SCSIDISK in CONFIG.SYS.  Take note, however, that upon boot-up, only one hard disk is shown in the BIOS.  The second hard disk can only be shown upon the installation of drivers.