MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

June  27,  1994

MS-4134  PCI/VL/ISA

µ §

MAINBOARD

NEWS  UPDATES (Rel. 1)

MAINBOARD	MS-4134

CHIPSET        	ALI M1429G + M1435

IMPORTANT  REMINDERS:

For Mainboard Version 0B, 1 and 1.1:

1.		The mainboard has been reworked to provide greater stability and to support most 			of 	the ET4000/W32P PCI VGA cards.  The modifications are as follows:

Change R85 from 22 Ohms to 100 Ohms.

Change U34 from FCT244 to Signetics F244.

If you are experiencing problems with your mainboard, please follow the above 				modifications.

2.		When installing network card, do not use the port address 300H.  This is used by the 		PCI Slot 1.

For Phoenix BIOS PL51 and PL52:

1.		These two versions of BIOS might cause CMOS Checksum error in some 						configurations.

2.		These two versions of BIOS might cause memory count error or extended memory 			error in some configurations when running DX2-50.

3.		When using Cyrix M7 CPU, software reset will fail.

4.		The system cannot boot up when using Diamond Viper + PCI Bus Master (like

MS-3805 NCR SCSI and Adaptec AHA-2940).

5.		The system cannot boot up when using Intel Spea V7 Mercury Lite + PCI Bus 				Master (like MS-3805 NCR SCSI and Adaptec AHA-2940).

6.		When using Matrox MGA and ATI Mach 64 VGA cards, the screen will be garbled 			after booting up in DOS prompt.

For Phoenix BIOS PL53:

1.		This BIOS solves the CMOS Checksum error in PL51 and PL52.

2.		This BIOS might cause memory count error or extended memory 	error in some 				configurations when running DX2-50.

3.		When using Cyrix M7 CPU, software reset will fail.

4.		The following options have been added in the BIOS "Advanced Chipset Setup" for 			greater flexibility:

Dynamic Fast Cycle

I/O Recovery Time

5.		The system cannot boot up when using Diamond Viper + PCI Bus Master (like

MS-3805 NCR SCSI and Adaptec AHA-2940).

6.		The system cannot boot up when using Intel Spea V7 Mercury Lite + PCI Bus 				Master (like MS-3805 NCR SCSI and Adaptec AHA-2940).

7.		When using Matrox MGA and ATI Mach 64 VGA cards, the screen will be garbled 			after booting up in DOS prompt.

For AMI BIOS AL51:

1.		This BIOS solves the memory count error or extended memory error in some 					configurations when running DX2-50.

2.		The system cannot boot up when using Diamond Viper + PCI Bus Master (like

MS-3805 NCR SCSI and Adaptec AHA-2940).

3.		The system cannot boot up when using Intel Spea V7 Mercury Lite + PCI Bus 				Master (like MS-3805 NCR SCSI and Adaptec AHA-2940).

4.		When using Matrox MGA and ATI Mach 64 VGA cards, the screen will be garbled 			after booting up in DOS prompt.

For AMI BIOS AL52:  (Not formally released)

1.		This BIOS provides greater compatibility with PCI Bus Master.

2.		This BIOS solves the boot-up problem when using Diamond Viper + PCI Bus 				Master (like MS-3805 NCR SCSI and Adaptec AHA-2940).

3.		This BIOS solves the boot-up problem when using Intel Spea V7 Mercury Lite + 			PCI Bus Master (like MS-3805 NCR SCSI and Adaptec AHA-2940).

4.		This BIOS solves the "garbled screen" problem when using Matrox MGA and ATI 			Mach 64 VGA cards.

For AMI BIOS AL53:

1.		This BIOS provides greater compatibility with PCI VGA cards.

F Note:

1.		The Buslogic BT-946C PCI SCSI Controller is not yet supported in the above 				mentioned version of mainboard and BIOS.

2.		The AMD PCI Network Adapter is not yet supported in the above mentioned 					version of  mainboard and BIOS.