MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4135  486CT1  PCI / ISA  MAINBOARD

TEST  REPORT

Date:  July  29,  1994

MAINBOARD	MS-4135

CHIPSET	CONTAQ 82C596 + 82C599

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel 80486DX4-100 CPU

·	8MB Memory (Micron Tech MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	MS-4803 ALI PCI IDE Controller

·	MS-4402 Avance Logic ALG2301A PCI VGA Card

·	Winbond Multi-I/O Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN OS/2 | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel 80486DX4-100 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	Adaptec AHA-1542CF SCSI Controller

·	MS-4402 Avance Logic ALG2301A PCI VGA Card

·	Wangtek 5525ES Tape Drive

·	MAxtor LXT-340S (340MB) SCSI Hard Disk

·	One 1.2MB FDD

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| DOS MODE | OK |  |
| CP FILE | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel 80486DX4-100 CPU

·	32MB Memory (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-15)

·	MS-3805 NCR PCI SCSI Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	UMC Multi-I/O Controller

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Logitech Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel 80486DX4-100 CPU

·	12MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-15)

·	MS-4803 ALI PCI IDE Controller

·	MS-4402 Avance Logic ALG2301 PCI VGA Card

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| CIRRUS LOGIC | GD-5422 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| OAK | OTI-087 | ISA |
| OAK | OTI-087X | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | WD90C30 | ISA |
| ATI MACH 32 | MACH 32 | PCI |
| ATI MACH 64 | MACH 64 | PCI |
| CARDEX | ET4000/W32P | PCI |
| DIAMOND SPEEDSTAR 64 | CIRRUS LOGIC 5434 | PCI |
| DIAMOND STEALTH 32 | ET4000/W32P | PCI |
| DIAMOND VIPER PCI | WEITEK P9000/P9001 | PCI |
| ELSA WINNER 1000PRO | S3 864 | PCI |
| GENOA PHANTOM 32I | ET4000/W32P | PCI |
| INTEL SPEA V7 MERCURY LITE | S3 928 | PCI |
| MATROX MGA | MATROX MGA | PCI |
| MICRO-STAR MS-4401 | ALI ALG2201 | PCI |
| MICRO-STAR MS-4402 | ALI ALG2301 | PCI |
| MICRO-STAR MS-4403 | IIT AGX-016 | PCI |
| MICRO-STAR MS-4405 | ET4000/W32P | PCI |
| ORCHID KELVIN 64 | CIRRUS LOGIC 5434 | PCI |
| PROLINK | CIRRUS LOGIC 5434 | PCI |
| PROLINK | S3 928 | PCI |
| TNC 2 THE MAX | ET4000/W32P | PCI |

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| MICRO-STAR MS-3806 | SMC FDC37C665 (666) | ISA |
| MICRO-STAR MS-3807 | WINBOND W83757 / 58 | ISA |
| UMC | UM82C863F | ISA |
| ARTOP | AECLAB ATP8400 | PCI |
| CMD TECHNOLOGY |  | PCI |
| MICRO-STAR MS-4803 | ALI M5215 | PCI |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ADAPTEC | AHA-2940 | PCI |
| MICRO-STAR MS-3805 | NCR | PCI |

D.	NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |
| AMD |  | PCNET | PCI |
| DEC |  | 21040 | PCI |

E.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| CONNER | CFS540A | 540 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| QUANTUM | 540AT | 540 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| WD | CAVIAR 2540 | 540 MB | IDE |
| FUJITSU | M2694ESA | 1 GB | SCSI |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |
| SEAGATE | ST31200N | 1 GB | SCSI |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| WINBOND | W24257AK-15 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE |  |
| SCANNER | LOGITECH | SCANMAN 256 |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-4135 Ver 1

System BIOS	:	AWARD AW2

System Chipset:	CONTAQ 82C596 + 82C599

VGA Card		:	MS-4402 Avance Logic ALG2301A PCI VGA Card

IDE Controller		:	MS-4803 ALI PCI IDE Controller

I/O Controller		:	MS-3806 SMC Super I/O Controller

System Memory	:	8 MB (MS-4904 Micron Tech MT4C4001JDJ-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CP30254 HDD (240 MB)

|  | INTEL | INTEL | INTEL/AMD |
| --- | --- | --- | --- |
|  | DX4-100 | DX4-75 | DX2-66 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 16.93 | 14.88 | 12.69 |
| FPU Index | 88.29 | 69.17 | 58.07 |
| Disk Index | 23.03 | 20.23 | 22.01 |
| Video Index | 12.05 | 9.45 | 10.45 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 2.10 | 1.77 | 1.78 |
| FPU Index | 6.89 | 5.19 | 4.59 |
| Disk Index | 1.15 | 1.14 | 1.19 |
| Video Index | 4.19 | 3.21 | 3.30 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 35.53 | 32.21 | 32.69 |
| Processor Harmonic | 20856.09 | 15852.46 | 16036.44 |
| Video Harmonic | 6618.18 | 5155.83 | 6614.51 |
| Memory Harmonic | 5010.78 | 4411.50 | 4713.71 |
| Disk Harmonic | 32.84 | 31.15 | 31.07 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 11.19 | 10.81 | 8.77 |
| FPU Index | 42.41 | 33.94 | 30.84 |
| Disk Index | 22.44 | 18.77 | 17.61 |
| Video Index | 8.00 | 7.88 | 7.69 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.52 | 1.58 | 1.32 |
| FPU Index | 3.70 | 2.96 | 2.30 |
| Disk Index | 1.15 | 1.14 | 1.18 |
| Video Index | 2.31 | 2.02 | 2.01 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 29.45 | 24.84 | 27.29 |
| Processor Harmonic | 10960.28 | 9669.86 | 9203.48 |
| Video Harmonic | 5154.86 | 6477.06 | 6599.67 |
| Memory Harmonic | 4025.08 | 4282.70 | 3952.33 |
| Disk Harmonic | 30.66 | 23.74 | 28.72 |
|  | INTEL | INTEL | INTEL/AMD |
|  | DX4-100 | DX4-75 | DX2-66 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 206.1 | 143.0 | 109.3 |
| Disk Index | 4.3 | 3.7 | 4.3 |
| Performance Index | 138.8 | 96.5 | 74.3 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 131.4 | 99.0 | 131.4 |
| Disk Index | 7.1 | 6.9 | 7.1 |
| Performance Index | 89.7 | 68.1 | 89.7 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 177.3 | 200+ |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 358.02 | 269.73 | 221.97 |
| FPU Speed | 806.46 | 607.55 | 542.03 |
| Video Speed | 9544.00 | 7175.00 | 9544.00 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 109.1 | 87.7 | 51.5 |
| Disk Index | 3.8 | 4.2 | 4.2 |
| Performance Index | 74.0 | 59.8 | 35.7 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 84.9 | 67.9 | 71.7 |
| Disk Index | 7.0 | 7.0 | 7.0 |
| Performance Index | 58.7 | 47.5 | 49.9 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 151.3 | 121.4 | 151.3 |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 164.90 | 131.92 | 110.98 |
| FPU Speed | 444.23 | 355.41 | 271.25 |
| Video Speed | 7175.00 | 9544.0 | 9544.00 |

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX4-100 | DX4-75 | DX2-66 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 41282 | 33609 | 33609 | Dhrystones |
| Math Speed | 14728.8 | 10791.3 | 11417.5 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 22088 | 10315 | 22088 | Char/S |
| Direct Video Speed | 563964 | 425536 | 419769 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 554.3 | 554.3 | 554.3 | K/Second |
| Average Seek Time | 13.6 | 13.6 | 13.6 | ms |
| Track Seek Time | 4.7 | 4.8 | 4.8 | ms |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX2-50 | DX-40 | DX-33 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 22554 | 18415 | 16170 | Dhrystones |
| Math Speed | 8629.6 | 7379.2 | 5703.7 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 18793 | 20569 | 18813 | Char/S |
| Direct Video Speed | 304412 | 244171 | 210205 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 554.3 | 554.3 | 554.3 | K/Second |
| Average Seek Time | 13.6 | 13.3 | 13.6 | ms |
| Track Seek Time | 4.8 | 5.0 | 4.7 | ms |

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX4-100 | DX4-75 | DX2-66 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2476.2 | 1875.8 | 1750.5 | PMUs |
| Spreadsheet | 3177.5 | 4072.8 | 4970.1 | PMUs |
| Word Processing | 2460.9 | 5106.0 | 5361.6 | PMUs |
| Database App'n. | 2491.3 | 3477.5 | 4324.0 | PMUs |
| CAE / CAD | 63729.5 | 50261.6 | 47016.0 | PMUs |
| Throughput Test | 19462.5 | 15554.4 | 15070.6 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 38251.0 | 28775.0 | 26708.7 | PMUs |
| Clock Speed Test | 97.0 | 73.0 | 65.0 | MHertz |
| MIPS Test | 33.0 | 25.5 | 27.1 | MIPS |
| Dhrystone Test | 43.4 | 32.7 | 34.7 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 7214.4 | 6531.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 182.6 | 184.0 | 183.0 | PMUs |
| Trk-to-Trk Seek | 4.0 | 3.9 | 4.0 | MS/Seek |
| Mean Seek Test | 10.1 | 10.1 | 10.1 | MS/Seek |
| Data Transfer Rate | 974.1 | 977.9 | 973.2 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.1 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 12.7 | 15.5 | 14.0 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 4334.4 | 4272.6 | 4272.6 | PMUs Md 3 |
| Char Write Test | 0.9 | 0.8 | 0.8 | Sec. Md 3 |
| Line Drawing Test | 2.6 | 2.7 | 2.7 | sec. Md 18 |
| Fill Test | 11.0 | 11.3 | 11.3 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1457.2 | 1219.7 | 959.0 | PMUs |
| Spreadsheet | 3148.3 | 4354.9 | 3162.3 | PMUs |
| Word Processing | 4389.8 | 3190.3 | 3898.2 | PMUs |
| Database App'n. | 3032.0 | 1707.0 | 1344.4 | PMUs |
| CAE / CAD | 39695.2 | 31643.0 | 21848.8 | PMUs |
| Throughput Test | 12463.9 | 10690.9 | 8832.8 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 22020.7 | 17555.7 | 13252.7 | PMUs |
| Clock Speed Test | 49.0 | 39.5 | 33.0 | MHertz |
| MIPS Test | 19.5 | 15.9 | 14.8 | MIPS |
| Dhrystone Test | 25.0 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 5582.8 | 4481.2 | 3255.1 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 182.7 | 183.2 | 184.3 | PMUs |
| Trk-to-Trk Seek | 4.0 | 4.0 | 3.9 | MS/Seek |
| Mean Seek Test | 10.0 | 10.1 | 10.0 | MS/Seek |
| Data Transfer Rate | 976.9 | 977.9 | 974.1 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 16.1 | 13.5 | 13.7 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 3778.9 | 4334.4 | 4125.6 | PMUs Md 3 |
| Char Write Test | 0.9 | 0.8 | 0.9 | Sec. Md 3 |
| Line Drawing Test | 3.4 | 3.1 | 3.0 | sec. Md 18 |
| Fill Test | 15.0 | 13.3 | 12.6 | Sec. Md 18 |

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX4-100 | DX4-75 | DX2-66 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2376.8 | 1796.7 | 1721.3 | PMUs |
| Spreadsheet | 5013.0 | 4223.3 | 5810.4 | PMUs |
| Word Processing | 3729.3 | 4141.3 | 4539.7 | PMUs |
| Database App'n. | 4265.1 | 2952.5 | 3509.0 | PMUs |
| CAE / CAD | 65170.7 | 49153.0 | 46619.3 | PMUs |
| Throughput Test | 19714.8 | 15902.5 | 17754.0 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 37183.3 | 28169.0 | 25718.0 | PMUs |
| Clock Speed Test | 97.2 | 73.2 | 66.0 | MHertz |
| MIPS Test | 24.3 | 18.6 | 19.5 | MIPS |
| Dhrystone Test | 43.1 | 32.7 | 34.2 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 7241.4 | 6489.0 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 183.7 | 183.4 | 183.4 | PMUs |
| Trk-to-Trk Seek | 4.0 | 4.0 | 4.0 | MS/Seek |
| Mean Seek Test | 10.1 | 10.1 | 10.1 | MS/Seek |
| Data Transfer Rate | 980.7 | 984.5 | 980.7 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.1 | MS/Op. |
| Ran File Rd/Wrt | 13.1 | 18.1 | 12.4 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 4344.8 | 3670.9 | 4272.6 | PMUs Md 3 |
| Char Write Test | 0.8 | 1.0 | 0.8 | Sec. Md 3 |
| Line Drawing Test | 2.6 | 3.2 | 2.7 | sec. Md 18 |
| Fill Test | 11.0 | 13.2 | 11.3 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1414.4 | 1179.0 | 939.0 | PMUs |
| Spreadsheet | 3199.7 | 3868.6 | 3933.5 | PMUs |
| Word Processing | 3435.3 | 4358.5 | 3959.9 | PMUs |
| Database App'n. | 2374.0 | 2523.2 | 2263.0 | PMUs |
| CAE / CAD | 38679.7 | 30806.0 | 23526.2 | PMUs |
| Throughput Test | 12523.3 | 10356.5 | 8072.3 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 21278.0 | 16856.3 | 12886.3 | PMUs |
| Clock Speed Test | 48.9 | 40.0 | 33.0 | MHertz |
| MIPS Test | 14.1 | 11.6 | 10.8 | MIPS |
| Dhrystone Test | 24.9 | 20.3 | 18.9 | K-Dhrystones/S |
| Whetstone Test | 5582.8 | 4461.2 | 3244.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 183.3 | 184.0 | 184.1 | PMUs |
| Trk-to-Trk Seek | 3.9 | 3.9 | 3.8 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | MS/Seek |
| Data Transfer Rate | 983.6 | 983.6 | 979.8 | Kb/Sec. |
| File Open/Close | 6.2 | 6.2 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 18.1 | 13.2 | 13.7 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 3778.9 | 4334.4 | 4116.2 | PMUs Md 3 |
| Char Write Test | 0.9 | 0.8 | 0.9 | Sec. Md 3 |
| Line Drawing Test | 3.4 | 3.1 | 3.0 | sec. Md 18 |
| Fill Test | 15.0 | 13.3 | 12.6 | Sec. Md 18 |

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX4-100 | DX4-75 | DX2-66 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45517 | 45517 | 45517 | Dhrystones |
| Direct Video Speed | 1136085 | 757390 | 757390 | CPS |
| Math Speed | 14876.9 | 10871.1 | 10524.0 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.59 | 4.69 | 4.64 | ms |
| Average Seek | 11.2 | 11.7 | 11.2 | ms |
| Transfer Rate | 420.1 | 327.7 | 315.1 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22758 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 473369 | 473369 | 284021 | CPS |
| Math Speed | 9167.5 | 7372.6 | 5683.5 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.56 | 4.65 | 4.64 | ms |
| Average Seek | 11.4 | 11.4 | 11.3 | ms |
| Transfer Rate | 309.1 | 297.9 | 321.3 | K/sec |

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX4-100 | DX4-75 | DX2-66 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45.5 | 45.5 | 45.5 | K-Dhrystones |
| Direct Video Speed | 1000.0 | 757.4 | 757.4 | K-CPS |
| Math Speed | 14.4 | 11.3 | 10.6 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.60 | 4.71 | 4.64 | ms |
| Average Seek | 11.5 | 11.8 | 11.3 | ms |
| Transfer Rate | 420.1 | 334.4 | 282.5 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22.8 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 473.4 | 473.4 | 284.0 | K-CPS |
| Math Speed | 8.7 | 7.1 | 5.7 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.55 | 4.66 | 4.63 | ms |
| Average Seek | 11.5 | 11.3 | 11.4 | ms |
| Transfer Rate | 341.3 | 297.9 | 334.4 | K/sec |

The following programs below were tested under Windows 3.1, using Avance Logic ALG2301A Ver. 1.2 Windows driver, 1024 x 768 x 256C resolution.

|  | INTEL | INTEL | INTEL/AMD |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX4-100 | DX4-75 | DX2-66 | UNITS |
| SPEEDY | 64.82 | 59.78 | 62.29 | SPEEDMARK |
| WINTACH 1.2 | 57.76 | 46.59 | 44.03 | RPM |
| WINBENCH 3.11 | 22.23 | 19.81 | 19.90 | WINMARK |
| WINBENCH 4.0 | 12.9 | 11.4 | 9.76 | WINMARK |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX2-50 | DX-40 | DX-33 | UNITS |
| SPEEDY | 47.93 | 42.74 | 43.50 | SPEEDMARK |
| WINTACH 1.2 | 32.48 | 29.88 | 29.65 | RPM |
| WINBENCH 3.11 | 16.37 | 15.78 | 15.22 | WINMARK |
| WINBENCH 4.0 | 7.69 | 7.38 | 7.41 | WINMARK |

|  | INTEL | INTEL | INTEL/AMD |
| --- | --- | --- | --- |
| WINDSOCK | DX4-100 | DX4-75 | DX2-66 |
| CPU | 857 | 638 | 590 |
| VIDEO | SU ¾ 993 | SU ¾ 838 | SU ¾ 834 |
| DISK | GR ¾ 395 | GR ¾ 303 | GR ¾ 261 |
| MEMORY | GO ¾ 108 | GO ¾ 102 | GO ¾ 108 |
| WINDOWS (OVERALL) | SU ¾ 646 | SU ¾ 509 | GR ¾ 445 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
| WINDSOCK | DX2-50 | DX-40 | DX-33 |
| CPU | 452 | 361 | 246 |
| VIDEO | SU ¾ 653 | SU ¾ 591 | SU ¾ 607 |
| DISK | GO ¾ 207 | GO ¾ 199 | GO ¾ 208 |
| MEMORY | GO ¾ 99 | GO ¾ 116 | GO ¾ 104 |
| WINDOWS (OVERALL) | GR ¾ 377 | GR ¾ 329 | GR ¾ 285 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good