MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

Date:  July 29, 1994

MS-4135  PCI / ISA  MAINBOARD

SUPPLEMENTS  (Rel. 1)

MAINBOARD	MS-4135

CHIPSET        	CONTAQ  82C596 + 82C599

Please find below the proposed jumper and BIOS settings for different configurations.  Use default values and settings for the other jumpers and BIOS not specified.

MS-3805  is  the  NCR  PCI  SCSI  Controller.

MS-4803  is  the  ALI  PCI  IDE  Controller.

CASE I:		MS-4135 + MS-3805

For MS-4135 Version 1 Mainboard:

1.		For  MS-4135:

Set  JINTA  to  1-2.

Set  JIRQ11  to  1-2.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  2.

3.		For  Award  BIOS:

In  the PCI  Configuration  Setup:

Slot  2  IRQ  Line								:	11

On  Board  PCI/SCSI  BIOS				:	Enabled

4.		For  AMI  BIOS:

In  the Chipset  Setup:

PCI  Slot # 2  IRQ  Line						:	IRQ 11

On  Board  SCSI  BIOS  for  Offboard	:	Enabled

CASE II:		MS-4135 + MS-4803

For MS-4135 Version 1 Mainboard:

1.		For  MS-4135:

Set  JINTD  to  2-3.

Set  JIQ14#  to  1-2.

2.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  PCI  Slot  1.

3.		For  Award  BIOS:

In  the PCI  Configuration  Setup:

Slot  1  IRQ  Line				:	14

4.		For  AMI  BIOS:

In  the Chipset  Setup:

PCI  Slot # 1  IRQ  Line		:	IRQ 14

CASE III:		MS-4135 + MS-3805 + MS-4803

For MS-4135 Version 1 Mainboard:

1.		For  MS-4135:

Set  JINTA  to  1-2.

Set  JINTD  to  2-3.

Set  JIRQ11  to  1-2.

Set  JIQ14#  to  1-2.

2.		For  MS-3805:

Set  JP2  to  1-2.

Install  MS-3805  in  PCI  Slot  2.

3.		For  MS-4803:

Set  JPIDE1,  short  INTA  1-2.

Install  MS-4803  in  PCI  Slot  1.

4.		For  Award  BIOS:

In  the PCI  Configuration  Setup:

Slot  1  IRQ  Line								:	14

Slot  2  IRQ  Line								:	11

On  Board  PCI/SCSI  BIOS				:	Enabled

5.		For  AMI  BIOS:

In  the Chipset  Setup:

PCI  Slot # 1  IRQ  Line						:	IRQ 14

PCI  Slot # 2  IRQ  Line						:	IRQ 11

On  Board  SCSI  BIOS  for  Offboard	:	Enabled

CASE IV:		MS-4135 + AMD PCI Network Card

For MS-4135 Version 1 Mainboard:

1.		For  MS-4135:

Set  JINTB  to  2-3.

Set  JIRQ9  to  2-3.

2.		For  AMD  Network  Card:

Install  in  PCI  Slot  3.

For  the  AMD  Network  Driver  to  install  correctly:

A.		Create a NET.CFG file containing the following:

link driver pcntnw

frame ethernet_802.3

bustype pci1

B.		Put the NET.CFG file in the directory which contains the files:  lsl.com, 							pcntnw.com, ipxodi.com and netx.com.

3.		For  Award  BIOS:

In  the PCI  Configuration  Setup:

Slot  3  IRQ  Line				:	9

4.		For  AMI  BIOS:

In  the Chipset  Setup:

PCI  Slot # 3  IRQ  Line		:	IRQ 9