MICRO-STAR INTERNATIONAL CO., LTD.

7F, NO. 4, LANE 497, CHUNG CHENG RD.HSIN TIEN CITY, TAIPEI HSIEN, TAIWAN, R.O.C.TEL:  886-2-218-2292 (REP)FAX:  886-2-218-5552, 218-3361, 218-6049µ §

MS-4137  VL-BUS  GREEN  MAINBOARD

TEST  REPORT

Date:  July  26,  1994

MAINBOARD	MS-4137

CHIPSET	ALI M1429G

I.		SOFTWARE COMPATIBILITY TEST

A.  DOS

| MS-DOS | VER 5.0 | VER 6.0 | VER 6.2 | REMARKS |
| --- | --- | --- | --- | --- |
| INSTALL | OK | OK | OK |  |
| BOOT | OK | OK | OK |  |
| FORMAT A: | OK | OK | OK |  |
| BOOT FROM A | OK | OK | OK |  |
| COPY | OK | OK | OK |  |
| COMP | OK | OK | OK |  |
| XCOPY | OK | OK | OK |  |
| DISKCOPY | OK | OK | OK |  |
| DISKCOMP | OK | OK | OK |  |
| M.C.R. DIR | OK | OK | OK |  |
| EMM386 | OK | OK | OK |  |

B.  IBM  OS/2

Configuration:

·	Intel 80486DX4-100 CPU

·	32MB Memory (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-15)

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Conner CP30254 (254MB) Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| OS/2 | VER 2.0 | VER 2.1 | REMARKS |
| --- | --- | --- | --- |
| INSTALL | OK | OK |  |
| BOOT | OK | OK |  |
| FORMAT A: | OK | OK |  |
| COPY | OK | OK |  |
| COMP | OK | OK |  |
| XCOPY | OK | OK |  |
| DISKCOPY | OK | OK |  |
| DISKCOMP | OK | OK |  |
| M.C.R. DIR | OK | OK |  |
| TASK TEST | OK | OK |  |
| WIN-OS/2 | OK | OK |  |
| WIN PROGRAMS | OK | OK |  |

C.  SCO  UNIX

Configuration:

·	Intel 80486DX4-100 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Wangtek 5525ES Tape Drive

·	Maxtor LXT-340S (340MB) SCSI Hard Disk

·	One 1.44MB and one 1.2MB FDDs

·	Genius Mouse

| SCO UNIX | REL 3.2.4 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| O.D.T. | OK |  |
| TASK TEST | OK |  |

D.  WINDOWS  N.T.

Configuration:

·	Intel 80486DX4-100 CPU

·	32MB Memory (Panasonic MN41C4000SJ-08)

·	256KB Cache (Winbond W24257AK-20)

·	Adaptec AHA-1542C SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Maxtor LXT340SY (340MB) SCSI Hard Disk

·	NEC Multi-Spin 3XE CD-ROM

·	One 1.44MB and one 1.2MB FDDs

·	Logitech Mouse

| WINDOWS N.T. | VER 3.1 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| FORMAT A: | OK |  |
| COMP | OK |  |
| XCOPY | OK |  |
| DISKCOPY | OK |  |
| DISKCOMP | OK |  |
| TASK TEST | OK |  |
| HCT | OK |  |

E.  NOVELL  NETWARE

Configuration:

·	Intel 80486DX2-66 CPU

·	16MB Memory (Micron MT4C4001JDJ-7)

·	256KB Cache (Winbond W24257AK-20)

·	Ultrastor U14F ISA SCSI Controller

·	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

·	Quantum 240S (240MB) SCSI Hard Disk

·	Conner CP3044 (40MB) Hard Disk

| NETWARE | 386 V3.11 | REMARKS |
| --- | --- | --- |
| INSTALL | OK |  |
| BOOT | OK |  |
| 3COM | OK | File Server and Workstation |
| ARCNET | OK | File Server and Workstation |
| NE2000 | OK | File Server and Workstation |
| NE2100 | OK | File Server and Workstation |
| BOOT ROM | OK |  |
| FILE TRANSFER | OK |  |
| NWTEST | OK |  |

F.  PACKETS  AND  UTILITIES

| S O F T W A R E | REMARKS |
| --- | --- |
| AutoCAD R10 c10 | OK |
| AutoCAD R11 | OK |
| AutoCAD R12 | OK |
| Central Point Anti-Virus 1.4 | OK |
| Clipper 5.01 | OK |
| CorelDraw V3.0 | OK |
| DBase III Plus | OK |
| DBase 4 | OK |
| GeoWorks Ensemble V1.2 | OK |
| Harvard Graphics V2.12 | OK |
| Lotus 1-2-3 R2.4 | OK |
| Lotus 1-2-3 R3.1+ | OK |
| Microsoft Entertainment Pack Vol 1 | OK |
| Microsoft Entertainment Pack Vol 2 | OK |
| Microsoft Entertainment Pack Vol 3 | OK |
| Microsoft Excel V4.0 | OK |
| Microsoft PowerPoint for Windows V3.0 | OK |
| Microsoft Video for Windows | OK |
| Microsoft Visual Basic V2.0 | OK |
| Microsoft Windows 3.1 | OK |
| Microsoft Word for Windows V2.0 | OK |
| Microsoft Works for Windows V2.0 | OK |
| Norton Desktop for Windows V1.2 | OK |
| Norton Utilities V6.0 / V7.0 | OK |
| PC Tools V8.0 | OK |
| PE 2 | OK |
| QEMM 386 V5.11 | OK |
| QEMM 386 V6.0 | OK |
| QEMM 386 V7.01 | OK |
| QEMM 386 V7.02 | OK |
| Symantec Entertainment Pack for Windows | OK |
| TopDOS 3.00 | OK |
| Ventura Publisher V2.0 | OK |
| WordPerfect 5.1 | OK |
| XTree Gold V2.51 | OK |

G.  DIAGNOSTIC  PROGRAMS

| S O F T W A R E | REMARKS |
| --- | --- |
| '92 System Mark | OK |
| '93 System Mark | OK |
| Byte Benchmark V1.3 | OK |
| Byte Benchmark V2.2 | OK |
| CheckIt V3.0 | OK |
| CheckIt Pro | OK |
| Core Test V2.92 | OK |
| IBM Diagnostics V2.03 | OK |
| Landmark Speed Test V1.14 | OK |
| Landmark Speed Test V2.0 | OK |
| Microsoft Software Diagnostics (MSD) | OK |
| Norton Diagnostics V7.0 | OK |
| Norton System Information V 4.5 | OK |
| Norton System Information V7.0 | OK |
| PC Magazine Labs Benchmark 6.0 | OK |
| PC Magazine Labs Benchmark 7.0 | OK |
| PC Week Labs System Benchmarks V1.10 | OK |
| Power Meter V1.7 | OK |
| Power Meter V1.81 | OK |
| QA Plus V4.61 | OK |
| QA Plus V4.70 | OK |
| QA Plus/FE V5.03 | OK |
| QA Plus/FE V5.04 | OK |
| QA Plus/FE V5.12 | OK |
| Speedy | OK |
| Superscale 3D Benchmark | OK |
| WinBench V2.5 | OK |
| WinBench V3.11 | OK |
| WinBench V4.0 | OK |
| WindSock 3.1 | OK |
| WinStone 94 | OK |
| WinTach V1.0 | OK |
| WinTach V1.2 | OK |

II.		HARDWARE COMPATIBILITY TEST

A.	VIDEO  CARDS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| ACUMOS | AVGA2 | ISA |
| IIT | AGX-014 | ISA |
| OAK | OTI-067 | ISA |
| OAK | OTI-077 | ISA |
| S3 | S3801 | ISA |
| TRIDENT | TVGA8900C | ISA |
| TRIDENT | TVGA8900CL | ISA |
| TRIDENT | TVGA8900D | ISA |
| TRIDENT | TVGA9000B | ISA |
| TSENG LABS | ET4000/W32 | ISA |
| WD | PARADISE | ISA |
| WD | WD90C30 | ISA |
| ATI | MACH 32 | VESA |
| CHAINTECH | ALI ALG2228 | VESA |
| DIAMOND SPEEDSTAR REV A2 | CIRRUS LOGIC 5426 | VESA |
| DIAMOND STEALTH PRO REV. 3.1 | S3928 | VESA |
| DIAMOND VIPER REV. B1 | WEITEK P9000 | VESA |
| MICRO-STAR MS-3405 | CIRRUS LOGIC 5426 | VESA |
| MICRO-STAR MS-3409 | CIRRUS LOGIC 5428 | VESA |
| ORCHID FAHRENHEIT VLB | S3805 | VESA |
| S3 | S3805 | VESA |
| TSENG LABS | ET4000/W32 | VESA |

B.	I/O  CONTROLLERS

| MANUFACTURER & MODEL NO. | CHIPSET | REMARKS |
| --- | --- | --- |
| GOLDSTAR | PRIME 2 | ISA |
| MICRO-STAR MS-3806 | SMC FDC37C665 (666) | ISA |
| MICRO-STAR MS-3807 | WINBOND W83757 / 58 | ISA |
| UMC | UM82C863F | ISA |
| DTC DTC2278VL |  | VESA |
| MICRO-STAR MS-3802 | PROMISE PDC20230B | VESA |
| HORNET VL-200 | ADAPTEC AIC-5VL01 | VESA W/ CACHE |
| PROMISE DC4030VL-2 | PROMISE PDC20130A | VESA W/ CACHE |
| TEKRAM DC-680C |  | VESA W/ CACHE |

C.	SCSI CONTROLLERS

| MANUFACTURER | MODEL NO. | REMARKS |
| --- | --- | --- |
| ADAPTEC | AHA-1542C | ISA |
| ADAPTEC | AHA-1542CF | ISA |
| ULTRASTOR | U14F | ISA |
| ULTRASTOR | U34F | VESA |

D.	NETWORK  ADAPTERS

| MANUFACTURER | TYPE | CHIPSET | REMARKS |
| --- | --- | --- | --- |
| 3COM | 3COM | 3C503 | ISA |
| 3COM | 3COM | 3C509 | ISA |
| SMC | ARCNET | COM90C65LJ | ISA |
|  | NE-2000 |  | ISA |
|  | NE-2100 |  | ISA |

E.	FLOPPY  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY |
| --- | --- | --- |
| MITSUBISHI | MF504C-315MG | 1.2 MB |
| MITSUBISHI | MF355C-255MG | 1.44 MB |
| TEAC | FD235HF | 1.44 MB |

F.	HARD  DISK  DRIVES

| MANUFACTURER | MODEL NUMBER | CAPACITY | REMARKS |
| --- | --- | --- | --- |
| CONNER | CP3104 | 104 MB | IDE |
| CONNER | CP3204F | 204 MB | IDE |
| CONNER | CP30254 | 254 MB | IDE |
| CONNER | CFS420A | 420 MB | IDE |
| CONNER | CFA540A | 540 MB | IDE |
| QUANTUM | 240AT | 240 MB | IDE |
| SEAGATE | ST3243A | 214 MB | IDE |
| WD | CAVIAR 2250 | 250 MB | IDE |
| WD | CAVIAR 2420 | 420 MB | IDE |
| WD | CAVIAR 2540 | 540 MB | IDE |
| MAXTOR | 7120S | 120 MB | SCSI |
| MAXTOR | 4170S | 170 MB | SCSI |
| MAXTOR | LXT340SY | 340 MB | SCSI |
| QUANTUM | ELS 170S | 170 MB | SCSI |
| QUANTUM | ELS 240S | 240 MB | SCSI |

G.	MEMORY  MODULES

| MANUFACTURER | MODEL NUMBER | TYPE | SIZE |
| --- | --- | --- | --- |
| MITSUBISHI | M5M44260AJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN41C41000SJ-8 | 72-pin single-sided | 4 MB |
| PANASONIC | MN414400ASJ-7 | 72-pin single-sided | 4 MB |
| SAMSUNG | KM416C256AJ-7 | 72-pin single-sided | 4 MB |
| MICRON TECH | MT4C4001JDJ-7 | 72-pin double-sided | 8 MB |
| NEC | D424100GS-70L-9JD | 72-pin double-sided | 32 MB |

H.	STATIC  RAMs

| MANUFACTURER | MODEL NUMBER | SIZE |
| --- | --- | --- |
| WINBOND | W24257AK-20 | 32K * 8 |

I.	MISCELLANEOUS  PERIPHERAL  DEVICES

| TYPE | MANUFACTURER | MODEL NUMBER | REMARKS |
| --- | --- | --- | --- |
| AUDIO CARD | CREATIVE LABS | SOUND BLASTER PRO |  |
| AUDIO CARD | OAK | MOZART |  |
| CD-ROM DRIVE | CREATIVE TECH. | CR-521-C |  |
| CD-ROM DRIVE | NEC | MULTI-SPIN 3XE |  |
| TAPE DRIVE | IRWIN BACKUP | ACCUTRAK PLUS + | 250 MB |
| TAPE DRIVE | QTRONIX | SDX-85 | 150 MB |
| TAPE DRIVE | WANGTEK | 5525ES | 525 MB |
| VIDEO CARD | CREATIVE LABS | VIDEO BLASTER |  |

III.	PERFORMANCE  TEST

Testing  Configuration:

Mainboard		:	MS-4137 Ver 1.2

System BIOS	:	AMI AD5H

System Chipset:	ALI M1429G

VGA & I/O Card	:	MS-3408 All-in-one Cirrus Logic 5428 VGA and UMC IDE Controller

System Memory	:	8 MB (MS-4904 Micron Tech MT4C4001JDJ-7)

Cache Memory		:	256 KB (Winbond W24257AK-15)

Hard Disk drive	:	Conner CP30254 HDD (240 MB)

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
|  | DX4-100 | DX2-66 | DX-50 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 17.31 | 12.46 | 9.29 |
| FPU Index | 90.28 | 58.50 | 45.11 |
| Disk Index | 23.03 | 21.64 | 19.73 |
| Video Index | 16.32 | 13.69 | 11.09 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.94 | 1.67 | 1.28 |
| FPU Index | 6.89 | 4.59 | 3.46 |
| Disk Index | 0.99 | 1.30 | 1.00 |
| Video Index | 4.87 | 3.72 | 3.11 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 38.28 | 39.79 | 34.25 |
| Processor Harmonic | 17713.15 | 14557.75 | 11906.47 |
| Video Harmonic | 8203.40 | 8200.14 | 9186.34 |
| Memory Harmonic | 14771.57 | 13540.67 | 9313.26 |
| Disk Harmonic | 30.88 | 34.30 | 30.85 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| BYTE BENCHMARK 1.3 |  |  |  |
| CPU Index | 11.19 | 8.68 | 8.65 |
| FPU Index | 41.00 | 32.41 | 28.43 |
| Disk Index | 23.07 | 19.00 | 23.25 |
| Video Index | 10.30 | 9.02 | 9.14 |
| BYTE BENCHMARK 2.2 |  |  |  |
| CPU Index | 1.51 | 1.22 | 1.26 |
| FPU Index | 3.68 | 2.95 | 2.30 |
| Disk Index | 1.30 | 1.20 | 1.29 |
| Video Index | 2.61 | 2.32 | 2.17 |
| PC BENCHMARK 7.0 |  |  |  |
| DOSMark | 34.67 | 30.76 | 33.15 |
| Processor Harmonic | 9672.69 | 8469.25 | 8832.73 |
| Video Harmonic | 6424.91 | 8831.14 | 8173.10 |
| Memory Harmonic | 13032.45 | 9630.43 | 10447.17 |
| Disk Harmonic | 32.23 | 29.36 | 32.05 |
|  | INTEL | INTEL/AMD | INTEL |
|  | DX4-100 | DX2-66 | DX-50 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 208.7 | 104.3 | 76.8 |
| Disk Index | 5.7 | 5.6 | 5.1 |
| Performance Index | 141.0 | 71.4 | 52.9 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 131.4 | 131.4 | 99.0 |
| Disk Index | 7.7 | 7.6 | 7.6 |
| Performance Index | 90.0 | 90.0 | 68.5 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 200+ | 200+ | 177.3 |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 358.05 | 221.99 | 167.23 |
| FPU Speed | 806.52 | 542.55 | 407.96 |
| Video Speed | 9452.00 | 9452.00 | 11702.00 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
|  | DX2-50 | DX-40 | DX-33 |
| NORTON SI 4.5 |  |  |  |
| Computing Index | 110.0 | 87.8 | 52.3 |
| Disk Index | 5.5 | 5.1 | 5.6 |
| Performance Index | 75.1 | 60.2 | 36.7 |
| NORTON SI 7.0 |  |  |  |
| Computing Index | 84.9 | 67.9 | 71.7 |
| Disk Index | 7.7 | 7.7 | 7.7 |
| Performance Index | 59.0 | 47.7 | 50.3 |
| LANDMARK SPEED 1.14 |  |  |  |
| CPU Speed | 151.9 | 121.4 | 151.3 |
| LANDMARK SPEED 2.0 |  |  |  |
| CPU Speed | 164.91 | 131.92 | 110.99 |
| FPU Speed | 444.26 | 355.38 | 271.56 |
| Video Speed | 7135.00 | 11045.00 | 9452.00 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX4-100 | DX2-66 | DX-50 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 41282 | 33609 | 23164 | Dhrystones |
| Math Speed | 14892.0 | 11417.5 | 8363.3 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 26276 | 24590 | 18061 | Char/S |
| Direct Video Speed | 335815 | 334533 | 281341 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 750.3 | 754.0 | 754.0 | K/Second |
| Average Seek Time | 13.3 | 13.0 | 13.0 | ms |
| Track Seek Time | 5.4 | 4.8 | 4.7 | ms |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| CHECKIT 3.0 | DX2-50 | DX-40 | DX-33 | UNITS |
| MAIN SYSTEM |  |  |  |  |
| CPU Speed | 22554 | 18415 | 16170 | Dhrystones |
| Math Speed | 86710.3 | 6759.0 | 5703.7 | M-Whetstones |
| VIDEO SYSTEM |  |  |  |  |
| BIOS Video Speed | 20013 | 16255 | 19315 | Char/S |
| Direct Video Speed | 292877 | 235199 | 210205 | Char/S |
| HARD DISK |  |  |  |  |
| Transfer Speed | 754.0 | 574.0 | 754.0 | K/Second |
| Average Seek Time | 13.0 | 13.3 | 13.6 | ms |
| Track Seek Time | 5.3 | 5.0 | 5.8 | ms |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2526.7 | 1873.0 | 1406.6 | PMUs |
| Spreadsheet | 2770.4 | 2744.6 | 2228.3 | PMUs |
| Word Processing | 4286.0 | 2328.2 | 4356.8 | PMUs |
| Database App'n. | 2533.5 | 2955.6 | 2327.5 | PMUs |
| CAE / CAD | 65022.1 | 45559.1 | 35123.8 | PMUs |
| Throughput Test | 20247.4 | 14834.6 | 11267.1 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 38251.0 | 26712.0 | 19957.3 | PMUs |
| Clock Speed Test | 98.0 | 65.5 | 49.0 | MHertz |
| MIPS Test | 33.6 | 27.3 | 20.5 | MIPS |
| Dhrystone Test | 43.1 | 34.9 | 26.3 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 6531.5 | 4898.6 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 261.5 | 263.2 | 260.7 | PMUs |
| Trk-to-Trk Seek | 3.9 | 3.6 | 4.0 | MS/Seek |
| Mean Seek Test | 10.1 | 10.0 | 10.1 | MS/Seek |
| Data Transfer Rate | 1483.3 | 1483.3 | 1492.0 | Kb/Sec. |
| File Open/Close | 6.5 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.1 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 13.2 | 12.8 | 13.5 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 7402.3 | 7253.1 | 5467.4 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 2.3 | 2.4 | 3.2 | sec. Md 18 |
| Fill Test | 11.6 | 11.9 | 16.2 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.7 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1577.5 | 1276.1 | 1057.0 | PMUs |
| Spreadsheet | 4039.5 | 3007.6 | 2866.4 | PMUs |
| Word Processing | 4248.6 | 4504.2 | 2930.8 | PMUs |
| Database App'n. | 1538.2 | 2530.7 | 2829.8 | PMUs |
| CAE / CAD | 38300.4 | 31107.1 | 24342.3 | PMUs |
| Throughput Test | 12865.4 | 9955.5 | 8455.4 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 21645.7 | 17312.0 | 13254.3 | PMUs |
| Clock Speed Test | 49.0 | 39.0 | 33.0 | MHertz |
| MIPS Test | 19.7 | 15.8 | 14.8 | MIPS |
| Dhrystone Test | 25.2 | 20.2 | 19.0 | K-Dhrystones/S |
| Whetstone Test | 5551.7 | 4441.4 | 3255.1 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 260.3 | 265.9 | 261.2 | PMUs |
| Trk-to-Trk Seek | 4.0 | 3.5 | 3.8 | MS/Seek |
| Mean Seek Test | 10.1 | 10.0 | 10.0 | MS/Seek |
| Data Transfer Rate | 1483.3 | 1489.8 | 1481.1 | Kb/Sec. |
| File Open/Close | 6.3 | 6.3 | 6.3 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 15.1 | 13.4 | 13.0 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 6711.8 | 5674.3 | 6470.4 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.6 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 3.1 | 3.7 | 2.9 | sec. Md 18 |
| Fill Test | 13.6 | 17.0 | 13.7 | Sec. Md 18 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 2528.9 | 1845.0 | 1374.5 | PMUs |
| Spreadsheet | 6147.3 | 4660.0 | 2787.0 | PMUs |
| Word Processing | 5496.8 | 3578.3 | 3308.3 | PMUs |
| Database App'n. | 2027.7 | 4627.2 | 2594.2 | PMUs |
| CAE / CAD | 63156.5 | 45866.9 | 34060.2 | PMUs |
| Throughput Test | 19001.2 | 15197.2 | 11223.6 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 37176.7 | 25721.3 | 19402.0 | PMUs |
| Clock Speed Test | 98.1 | 66.0 | 50.0 | MHertz |
| MIPS Test | 24.7 | 19.7 | 15.0 | MIPS |
| Dhrystone Test | 43.4 | 34.7 | 26.3 | K-Dhrystones/S |
| Whetstone Test | 9608.8 | 6489.0 | 4898.6 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.1 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 258.8 | 262.8 | 264.0 | PMUs |
| Trk-to-Trk Seek | 4.0 | 3.9 | 3.9 | MS/Seek |
| Mean Seek Test | 10.2 | 10.1 | 10.1 | MS/Seek |
| Data Transfer Rate | 1492.0 | 1498.6 | 1500.8 | Kb/Sec. |
| File Open/Close | 6.5 | 6.3 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.1 | 0.1 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 12.4 | 13.5 | 12.2 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 7402.3 | 7253.1 | 5407.4 | PMUs Md 3 |
| Char Write Test | 0.5 | 0.5 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 2.3 | 2.4 | 3.2 | sec. Md 18 |
| Fill Test | 11.6 | 11.9 | 16.2 | Sec. Md 18 |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| POWER METER 1.81 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM TEST |  |  |  |  |
| Agg. Hardware | 1513.7 | 1276.1 | 1035.7 | PMUs |
| Spreadsheet | 5034.8 | 3007.6 | 3037.9 | PMUs |
| Word Processing | 4609.8 | 4504.2 | 3455.9 | PMUs |
| Database App'n. | 2584.0 | 2530.7 | 2073.5 | PMUs |
| CAE / CAD | 38438.0 | 31107.1 | 23521.8 | PMUs |
| Throughput Test | 11982.7 | 9955.5 | 8436.5 | PMUs |
| CPU MEMORY |  |  |  |  |
| Agg. CPU/Memory | 20933.7 | 17312.0 | 12888.0 | PMUs |
| Clock Speed Test | 50.0 | 39.0 | 33.0 | MHertz |
| MIPS Test | 14.4 | 15.8 | 10.8 | MIPS |
| Dhrystone Test | 25.2 | 20.2 | 19.0 | K-Dhrystones/S |
| Whetstone Test | 5521.1 | 4441.4 | 3244.5 | K-Whetstones/S |
| Sieve Test | 0.1 | 0.2 | 0.1 | Seconds |
| DISK DRIVE |  |  |  |  |
| Agg. Disk Test | 261.3 | 265.9 | 264.4 | PMUs |
| Trk-to-Trk Seek | 3.9 | 3.5 | 3.7 | MS/Seek |
| Mean Seek Test | 10.0 | 10.0 | 10.0 | MS/Seek |
| Data Transfer Rate | 1500.8 | 1489.8 | 1498.6 | Kb/Sec. |
| File Open/Close | 6.2 | 6.3 | 6.2 | MS/Op. |
| Seq File Rd/Wrt | 0.2 | 0.2 | 0.2 | MS/Op. |
| Ran File Rd/Wrt | 21.1 | 13.4 | 13.3 | MS/Op. |
| VIDEO DISPLAY |  |  |  |  |
| Agg Video Test | 6711.8 | 5674.3 | 6470.4 | PMUs Md 3 |
| Char Write Test | 0.6 | 0.6 | 0.6 | Sec. Md 3 |
| Line Drawing Test | 3.1 | 3.7 | 2.9 | sec. Md 18 |
| Fill Test | 13.6 | 17.0 | 13.7 | Sec. Md 18 |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45517 | 45517 | 22758 | Dhrystones |
| Direct Video Speed | 757390 | 757390 | 473369 | CPS |
| Math Speed | 14876.9 | 10524.0 | 8525.2 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.62 | 4.53 | 4.51 | ms |
| Average Seek | 11.1 | 11.1 | 11.1 | ms |
| Transfer Rate | 910.2 | 963.8 | 512.0 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.61 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22758 | 22758 | 22758 | Dhrystones |
| Direct Video Speed | 473369 | 473369 | 284021 | CPS |
| Math Speed | 9167.5 | 7340.0 | 5683.5 | K-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.68 | 4.57 | 4.85 | ms |
| Average Seek | 11.1 | 11.1 | 11.5 | ms |
| Transfer Rate | 1365.3 | 252.1 | 1092.3 | K/sec |

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX4-100 | DX2-66 | DX-50 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 45.5 | 45.5 | 45.5 | K-Dhrystones |
| Direct Video Speed | 757.4 | 757.4 | 473.4 | K-CPS |
| Math Speed | 14.4 | 10.6 | 8.1 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.60 | 4.55 | 4.47 | ms |
| Average Seek | 11.3 | 11.1 | 11.2 | ms |
| Transfer Rate | 364.1 | 910.2 | 512.0 | K/sec |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| QA PLUS 4.70 | DX2-50 | DX-40 | DX-33 | UNITS |
| SYSTEM PERF. |  |  |  |  |
| CPU Speed | 22.8 | 22.8 | 22.8 | K-Dhrystones |
| Direct Video Speed | 473.4 | 473.4 | 284.0 | K-CPS |
| Math Speed | 8.7 | 7.1 | 5.7 | M-Whetstones |
| DISK PERF. |  |  |  |  |
| Trk-Trk Seek | 4.68 | 4.56 | 4.83 | ms |
| Average Seek | 11.1 | 11.1 | 11.3 | ms |
| Transfer Rate | 682.7 | 348.6 | 655.4 | K/sec |

The following programs below were tested under Windows 3.1, using Cirrus Logic

GD-5428 Ver. 1.32B Windows driver, 1024 x 768 x 256C resolution.

|  | INTEL | INTEL/AMD | INTEL |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX4-100 | DX2-66 | DX-50 | UNITS |
| SPEEDY | 33.98 | 27.19 | 25.31 | SPEEDMARK |
| WINTACH 1.2 | 45.75 | 34.12 | 30.83 | RPM |
| WINBENCH 3.11 | 25.69 | 22.37 | 20.77 | WINMARK |
| WINBENCH 4.0 | 11.0 | 7.91 | 7.28 | WINMARK |

|  | CYRIX | CYRIX | INTEL |  |
| --- | --- | --- | --- | --- |
| WINDOWS TEST | DX2-50 | DX-40 | DX-33 | UNITS |
| SPEEDY | 19.54 | 18.58 | 19.54 | SPEEDMARK |
| WINTACH 1.2 | 24.59 | 22.78 | 24.59 | RPM |
| WINBENCH 3.11 | 17.25 | 15.99 | 16.56 | WINMARK |
| WINBENCH 4.0 | 5.81 | 5.51 | 5.69 | WINMARK |

|  | INTEL | INTEL/AMD | INTEL |
| --- | --- | --- | --- |
| WINDSOCK | DX4-100 | DX2-66 | DX-50 |
| CPU | 857 | 492 | 374 |
| VIDEO | SU ¾ 650 | SU ¾ 478 | GR ¾ 397 |
| DISK | GR ¾ 397 | GR ¾ 281 | GR ¾ 234 |
| MEMORY | GO ¾ 181 | GO ¾ 177 | GO ¾ 119 |
| WINDOWS (OVERALL) | SU ¾ 586 | GR ¾ 385 | GR ¾ 302 |

|  | CYRIX | CYRIX | INTEL |
| --- | --- | --- | --- |
| WINDSOCK | DX2-50 | DX-40 | DX-33 |
| CPU | 374 | 277 | 374 |
| VIDEO | GR ¾ 280 | GR ¾ 259 | GR ¾ 397 |
| DISK | GO ¾ 205 | GO ¾ 183 | GR ¾ 234 |
| MEMORY | GO ¾ 167 | GO ¾ 117 | GO ¾ 119 |
| WINDOWS (OVERALL) | GR ¾ 288 | GR ¾ 257 | GR ¾ 302 |

Note:

SU	=	Superb

GR	=	Great

GO 	=	Good